| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0088 M30822 |
| --- | --- |

| Title: | AhG5: Memory Bandwidth Reduction for HEVC RExt |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Vadim Seregin, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, USA 92121 | Tel: Email: | 1-858-845-8893 wooshikk@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

HEVC Range Extensions 4:4:4 potential applications include consumer applications, so the complexity increase due to the 4:4:4 processing has to be taken into account. The scope of this contribution is the memory bandwidth increase for 4:4:4 motion estimation and compensation, which is around 37% more than the 4:2:0 case for 4×2 memory blocks. Specifically, in this contribution, restriction of bi-directional prediction for 8×8 prediction unit is studied. This modification increases 4:4:4 bandwidth by 8%, as opposed to the current 37% with respect to 4:2:0. It is reported that the impact on coding performance is 0.8% and 0.6% luma BD-rate loss for RA and LB, respectively, in YUV 4:4:4.

# Introduction

High Efficiency Video Coding (HEVC) version 1 has been developed mainly targeting video in the 4:2:0 chroma format, while the HEVC Range Extensions (RExt) considers video in 4:2:2 and 4:4:4 chroma formats as well. As the block size of chroma components has been increased, the memory bandwidth to access reference pixels for inter-prediction is increased as well. Accordingly, the implementation complexity of HEVC RExt is increased, which can be a problem, especially for consumer applications.

This problem has been analysed in detail in the previous contributions, JCTVC-M0298 [1] and JCTVC-N0261 [2]. In 14th JCT-VC meeting, it was suggested to consider restricting bi-prediction for 8×8 prediction unit (PU) when finalizing RExt. This method is implemented and analysed on top of HM-12.0_RExt4.1 and the results are reported in this contribution.

Table 1 compares memory bandwidth in 4:2:0 and 4:4:4 formats in the current HM, where the analysis is performed based on the method in [3]. JCTVC-M0298 includes further details.

Table 1. Comparison of memory bandwidth in 4:2:0 and 4:4:4 formats.

| Memory configuration | 4:2:0 | 4:4:4 | Ratio to 4:2:0 |
| --- | --- | --- | --- |
| 4×2 | 15.0 | 20.5 | 137% |
| 8×2 | 18.0 | 24.0 | 133% |

# Proposed Method

As analyzed in JCTVC-M0298, the bandwidth worst case is 8×8 PU. By disallowing bi-directional prediction for 8×8 PUs, the bandwidth is reduced by 21% in case of 4×2 memory configuration with respect to the current HM. When restriction of bi-directional prediction is applied, the motion vector associated with the reference picture list RefPicList0 is always selected. This method is applied to all color components. Note that this is the same method applied to 4×8/8×4 PUs in HEVC version 1. Table 2 compares memory bandwidth in the current HM and the proposed method.

Table 2. Memory bandwidth of the proposed method compared to HM.

| Memory configuration | HM | Proposed | Ratio to HM | Ratio to 4:2:0 |
| --- | --- | --- | --- | --- |
| 4×2 | 20.5 | 16.3 | 79% | 108% |
| 8×2 | 24.0 | 21.0 | 88% | 117% |

# Experimental Results

To evaluate the coding efficiency of the proposed methods, they are implemented on top of HM-12.0_RExt4.1, and experiments are performed using the common test conditions [4].

Experiments are performed except for all intra coding case, where no change is proposed. Table 3 shows the summary results of YUV 4:4:4 and 4:2:2 cases. Full results can be found in the accompanying spread sheets.

Table 3. Summary results of the proposed method.

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.5% | 0.9% | 1.4% | 1.6% | 0.8% | 1.4% |
| YCbCr 4:4:4 | 0.8% | 0.8% | 0.8% | 1.0% | 0.9% | 1.2% |
| YCbCr 4:2:2 | 0.6% | 0.6% | 0.6% | 0.9% | 0.7% | 1.0% |
| Overall | 1.0% | 0.8% | 0.9% | 1.2% | 0.8% | 1.2% |
|  | 1.0% | 0.6% | 1.0% | 1.2% | 0.6% | 1.2% |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 1.3% | 0.9% | 1.2% | 1.4% | 0.8% | 1.3% |
| YCbCr 4:4:4 | 0.6% | 0.6% | 0.6% | 0.9% | 0.7% | 0.9% |
| YCbCr 4:2:2 | 0.7% | 0.4% | 0.4% | 0.9% | 0.5% | 0.8% |
| Overall | 0.9% | 0.6% | 0.8% | 1.1% | 0.7% | 1.0% |
|  | 0.9% | 0.8% | 0.8% | 1.0% | 0.6% | 1.0% |
| Enc Time[%] | 96% |  |  | 96% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

# Conclusion

In this contribution, the method to reduce memory bandwidth for inter-prediction is proposed, where the bi-prediction is restricted in 8×8 PU. Experimental results show that the reduced bandwidth is 79% of that of HM at the cost of a coding efficiency loss of about 0.7% for Luma in YUV 4:4:4 case.

# References

- V. Seregin, X. Wang, J. Sole, and M. Karczewicz, “Bandwidth reduction for range extension,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-M0298, Apr. 2013.

- W.-S. Kim, V. Seregin, J. Sole, and M. Karczewicz, “AhG5: Memory Bandwidth Reduction for HEVC RExt,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N0261, Jul. 2013.

- E. François, A. Tabatabai, and E. Alshina, “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L0440, Jan. 2013.

- D. Flynn, K. Sharman, and C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1006, Jul. 2013.

# Proposed Text Changes

8.5.3.2.1	Derivation process for luma motion vectors for merge mode

…

10. When predFlagL0 is equal to 1 and predFlagL1 is equal to 1, and ( nOrigPbW + nOrigPbH ) is equal to 12 or equal to 16 and ChromaArrayType is 3, the following applies:

refIdxL1 = −1		(8-1)

predFlagL1 = 0		(8-2)

##### Table 7-1 – Name association to inter prediction mode

| inter_pred_idc | Name of inter_pred_idc |  |
| --- | --- | --- |
|  | ( nPbW + nPbH ) != 12 && ( ( nPbW + nPbH ) != 16 ¦¦ ChromaArrayType != 3 ) | ( nPbW + nPbH ) == 12 ¦¦ (( nPbW + nPbH ) == 16 && ChromaArrayType == 3 ) |
| 0 | PRED_L0 | PRED_L0 |
| 1 | PRED_L1 | PRED_L1 |
| 2 | PRED_BI | na |

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).