| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0089 M30823 |
| --- | --- |

| Title: | AhG5: Deblocking Filter in 4:4:4 Chroma Format |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Wei Pu, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, USA 92121 | Tel: Email: | 1-858-845-8893 wooshikk@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, methods are proposed to improve the performance of deblocking filter of chroma components in 4:4:4 format. In the first method, the luma deblocking filter is applied to the chroma components. In the second method, the luma deblocking filter is applied to the chroma components without the strong filter. In the third method, the chroma filter is used when the boundary strength is larger than 0. It is also proposed to signal deblocking control parameters separately for chroma components. Experimental results report visual quality improvements as well as coding efficiency improvements of 0.2/2.2/2.2 and 0.4/2.2/1.8 (Y/U/V BD-rate (%)) for RA and LB Main-tier in Method 1, 0.2/1.9/1.9 and 0.4/1.9/1.7 in Method 2, and 0.1/0.5/0.5 and 0.0/0.3/0.4 in Method 3, respectively.

# Introduction

High Efficiency Video Coding (HEVC) version 1 [1] has been developed mainly targeting video in 4:2:0 chroma format, while its Range Extension (RExt) work considers video in 4:2:2 and 4:4:4 formats as well [2]. In case of 4:4:4, test sequences in both YCbCr and RGB color spaces are included in the common test conditions [3].

In HEVC version 1, different deblocking schemes are applied for luma and chroma components. For luma component, there are the strong filter and two types of weak filters. For chroma, there is only one weak filter which modifies one pixel at each side around the boundary when one of the blocks forming the boundary is coded with intra prediction. Therefore, the extent of deblocking filter in chroma components is very weak compared to that of luma.

In HEVC RExt, chroma components in 4:4:4 format can contain much more information compared to those in 4:2:0 format. This implies that the blocking artifacts in chroma components can degrade the overall visual quality. In addition it can cause coding efficiency degradation as the deblocking filter is applied as the in-loop filter, which improves the coding efficiency. In this case, it is desirable to apply stronger filtering to the chroma components.

In this contribution, the visual quality of 4:4:4 content is examined, which reveals that the blocking artifact in chroma components are severe and causes degradation of overall visual quality, especially for RGB sequences. Three methods are proposed to solve the problem. In the first method, the luma deblocking filter method is applied to chroma components. In the second method, the luma deblocking filter is applied without the strong filtering. In the third method, the chroma filter is used when the boundary strength is larger than 0, so that deblocking can be applied to both intra and inter coded block boundaries. Note that these were proposed in our previous contribution JCTVC-N0263 [4].

In addition it is also proposed to signal deblocking filter control parameters for chroma components separately from those of luma component.

# Proposed Methods

In case of 4:4:4 chroma format, a flag deblock_chroma_as_luma is signaled in the SPS. If this flag is 1, one of the methods described below is applied for chroma deblocking. If this flag is 0, the chroma deblocking filter in the current HM is applied.

## Method 1: Chroma deblocking as Luma

When deblock_chroma_as_luma is 1, the chroma components are deblocked in the same way the luma component is deblocked. This means that the strong filter and two types of the weak filter are available for chroma blocks and they are applied according to the conditions defined for luma component (but checking them using the chroma samples). The deblocking filter is applied to both intra- and inter-coded blocks.

## Method 2: Chroma deblocking as Luma without the strong filter

When deblock_chroma_as_luma is 1, the same filtering scheme is applied to chroma components as in the first method above, except that the strong filtering is not used. When the conditions to apply the strong filter are satisfied for chroma components, the next strongest filter is applied, which modifies two samples in each block along the boundary. As the strong filter is not used, the size of line buffer is reduced, which may facilitate implementations.

## Method 3: Chroma deblocking when Bs > 0

In the current HM, the chroma deblocking filter is applied only when the boundary strength (Bs) is larger than 1. This means that the filtering is applied only when one of the blocks forming the boundary is coded with intra prediction. In the third method, when deblock_chroma_as_luma is 1, the chroma deblock filtering is applied when Bs is larger than 0. Note that there is no change in the boundary strength calculation and the chroma filter. Therefore, this method requires minimal changes to the current HM.

## Signaling of chroma deblock control parameters

In the current HM, one set of deblocking filter control parameters can be signaled in picture parameter set (PPS) and shared between luma and chroma. Also another set of parameters can be signaled in slice header and shared between luma and chroma. To provide flexibility for chroma deblocking, it is proposed to signal beta offset and tc offset for Cb and Cr component separately in both PPS and slice header.

# Experimental Results

To evaluate the coding efficiency of the proposed methods, they are implemented on top of HM-12.0_RExt4.1 and experiments are performed using the common test conditions [3].

Experiments are performed except for the 4:2:2 case, where no change is proposed. Below tables are the summary results of RGB and YUV 4:4:4 cases. Full results can be found in the accompanying spread-sheets. The flag deblock_chroma_as_luma is always set to 1.

## Method 1: Chroma deblocking as Luma

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | -0.3% | 0.2% | 0.0% | -0.9% | -0.3% | 0.0% | -1.8% | -1.2% |
| YCbCr 4:4:4 | 0.0% | -1.1% | -1.2% | 0.0% | -0.6% | -0.5% | 0.0% | -0.7% | -0.3% |
| Overall 4:4:4 | 0.0% | -0.7% | -0.5% | 0.0% | -0.8% | -0.4% | 0.0% | -1.3% | -0.8% |
|  | 0.0% | -0.7% | -0.5% | 0.0% | -1.1% | -0.3% | 0.0% | -1.4% | -0.9% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.2% | -1.6% | -2.2% | -0.2% | -1.3% | -1.8% |  |  |  |
| YCbCr 4:4:4 | -0.2% | -2.9% | -2.3% | -0.1% | -1.8% | -1.3% |  |  |  |
| Overall 4:4:4 | -0.2% | -2.2% | -2.2% | -0.2% | -1.5% | -1.6% |  |  |  |
|  | -0.2% | -1.9% | -2.1% | -0.2% | -2.5% | -1.8% |  |  |  |
| Enc Time[%] | 100% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | 106% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.7% | -1.8% | -2.6% | -0.4% | -1.5% | -1.9% |  |  |  |
| YCbCr 4:4:4 | -0.2% | -2.6% | -0.8% | -0.2% | -1.3% | 0.2% |  |  |  |
| Overall 4:4:4 | -0.4% | -2.2% | -1.8% | -0.3% | -1.4% | -0.9% |  |  |  |
|  | -0.4% | -1.5% | -1.5% | -0.4% | -1.5% | -1.0% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 106% |  |  | 104% |  |  |  |  |  |

## Method 2: Chroma deblocking as Luma without the strong filter

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.0% | -0.3% | 0.3% | 0.0% | -0.9% | -0.3% | 0.0% | -1.8% | -1.2% |
| YCbCr 4:4:4 | 0.0% | -0.7% | -0.7% | 0.0% | -0.5% | -0.3% | 0.0% | -0.7% | -0.3% |
| Overall 4:4:4 | 0.0% | -0.5% | -0.2% | 0.0% | -0.7% | -0.3% | 0.0% | -1.3% | -0.8% |
|  | 0.0% | -0.5% | -0.2% | 0.0% | -1.0% | -0.2% | 0.0% | -1.4% | -0.9% |
| Enc Time[%] | 103% |  |  | 103% |  |  | 103% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  | 101% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.2% | -1.5% | -2.1% | -0.2% | -1.2% | -1.7% |  |  |  |
| YCbCr 4:4:4 | -0.1% | -2.3% | -1.7% | -0.1% | -1.5% | -1.0% |  |  |  |
| Overall 4:4:4 | -0.2% | -1.9% | -1.9% | -0.1% | -1.4% | -1.4% |  |  |  |
|  | -0.2% | -1.8% | -1.8% | -0.2% | -2.0% | -1.6% |  |  |  |
| Enc Time[%] | 101% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -0.6% | -1.6% | -2.5% | -0.4% | -1.4% | -1.9% |  |  |  |
| YCbCr 4:4:4 | -0.1% | -2.2% | -0.9% | -0.2% | -1.3% | -0.1% |  |  |  |
| Overall 4:4:4 | -0.4% | -1.9% | -1.7% | -0.3% | -1.4% | -1.0% |  |  |  |
|  | -0.4% | -1.2% | -1.5% | -0.3% | -1.5% | -1.1% |  |  |  |
| Enc Time[%] | 96% |  |  | 96% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |  |  |  |

## Method 3: Chroma deblocking when Bs > 0

|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | -0.1% | -0.1% | -0.4% | 0.0% | 0.3% | 0.5% |
| YCbCr 4:4:4 | -0.1% | -0.9% | -0.7% | -0.1% | -0.3% | 0.1% |
| Overall 4:4:4 | -0.1% | -0.5% | -0.5% | -0.1% | 0.0% | 0.3% |
|  | -0.1% | -0.3% | -0.5% | 0.0% | 0.8% | 0.6% |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 105% |  |  | 114% |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |
|  | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.1% | 0.3% | 0.0% | 0.2% | 0.7% | 1.0% |
| YCbCr 4:4:4 | -0.1% | -1.1% | -0.8% | -0.1% | -0.1% | 0.1% |
| Overall 4:4:4 | 0.0% | -0.3% | -0.4% | 0.1% | 0.3% | 0.6% |
|  | 0.0% | 0.2% | -0.4% | 0.0% | 0.8% | 0.7% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 116% |  |  | 116% |  |  |

# Conclusions

In this contribution, the visual quality of 4:4:4 content has been investigated and the problem with the deblocking filter for chroma components is reported. Three methods are proposed to improve the performance of deblocking of chroma components in 4:4:4 format. In addition it is proposed to signal deblock control parameters for chroma. The experimental results show visual quality improvement as well as coding efficiency improvements.

# References

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, Y.-K. Wang, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 10,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-L1003, Jan. 2013.

- D. Flynn, J. Sole, and T. Suzuki, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 4,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1005, Jul. 2013.

- D. Flynn, K. Sharman, and C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N1006, Jul. 2013.

- W.-S. Kim, W. Pu, J. Sole, and M. Karczewicz, “AhG5: Deblocking Filter in 4:4:4 Chroma Format,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N0263, Jul. 2013.

# Proposed Text Changes

## Method 1: Chroma deblocking as Luma

#### 7.3.2.2 Sequence parameter set RBSP syntax

| if( chroma_format_idc = = 3 ) |  |
| --- | --- |
| deblock_chroma_as_luma_flag | u(1) |

#### 7.4.3.2 Sequence parameter set RBSP semantics

deblock_chroma_as_luma_flag equal to 1 specifies that the chroma components are deblocked as the luma component. When separate_colour_plane_flag is not present, it is inferred to be equal to 0.

##### Vertical edge filtering process

…

When bS[ xDk * SubWidthC ][ yDm * SubHeightC ] equal to 2 larger than 0 and ( ( ( xCb / SubWidthC + xDk )  >>  3 )  <<  3 ) is equal to xCb / SubWidthC + xDk, the following ordered steps apply:

- The filtering process for chroma block edges as specified in subclause Error: Reference source not found is invoked if deblock_chroma_as_luma_flag is 0 and bS[ xDk * SubWidthC ][ yDm * SubHeightC ] is equal to 2, or as specified in subclause 8.7.2.5.4 with the decision made for the luma block if deblock_chroma_as_luma_flag is 1 with the chroma picture sample array recPictureCb, …

- The filtering process for chroma block edges as specified in subclause Error: Reference source not found is invoked if deblock_chroma_as_luma_flag is 0 or as specified in subclause 8.7.2.5.4 with the decision made for the luma block if deblock_chroma_as_luma_flag is 1 with the chroma picture sample array recPictureCr, …

##### 8.7.2.5.2 Horizontal edge filtering process

…

When bS[ xDk * SubWidthC ][ yDm * SubHeightC ] is equal to 2 larger than 0 and ( ( ( yCb / SubHeightC + yDm )  >>  3 )  <<  3 ) is equal to yCb / SubHeightC + yDm, the following ordered steps apply:

- The filtering process for chroma block edges as specified in subclause Error: Reference source not found is invoked if deblock_chroma_as_luma_flag is 0 and bS[ xDk * SubWidthC ][ yDm * SubHeightC ] is equal to 2, or as specified in subclause 8.7.2.5.4 with the decision made for the luma block if deblock_chroma_as_luma_flag is 1 with the chroma picture sample array recPictureCb, …

- The filtering process for chroma block edges as specified in subclause Error: Reference source not found is invoked if deblock_chroma_as_luma_flag is 0 or as specified in subclause 8.7.2.5.4 with the decision made for the luma block if deblock_chroma_as_luma_flag is 1 with the chroma picture sample array recPictureCr, …

## Method 2: Chroma deblocking as Luma without the strong filter

In addition to the text changes in section 6.1, additional change is as below.

or as specified in subclause 8.7.2.5.4 after the decision process as specified in Error: Reference source not found with bS set to Min( bs, 1 ) if deblock_chroma_as_luma_flag is 1

## Method 3: Chroma deblocking when Bs > 0

##### 8.7.2.5.1 Vertical edge filtering process

…

When bS[ xDk * SubWidthC ][ yDm * SubHeightC ] is equal to 2 or it is larger than 0 when chroma_format_idc is 3 and…

##### 8.7.2.5.2 Horizontal edge filtering process

…

When bS[ xDk * SubWidthC ][ yDm * SubHeightC ] is equal to 2 or it is larger than 0 when chroma_format_idc is 3 and…

## Signaling of chroma deblock control parameters

#### 7.3.2.3 Picture parameter set RBSP syntax

| if( pps_extension1_flag ) { |  |
| --- | --- |
| if(deblocking_filter_control_present_flag && !pps_deblocking_filter_disabled_flag ) |  |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |

#### 7.3.6.1 Picture parameter set RBSP syntax

| if( slice_segment_header_extension_present_flag ) { |  |
| --- | --- |
| if( deblocking_filter_override_flag && !slice_deblocking_filter_disabled_flag ) |  |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |

* Note that signalling of beta offset is not necessary for Method 3.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).