| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0091 M30825 |
| --- | --- |

| Title: | AhG 5 and 18: Sample Adaptive Offset on High Bit-depth |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Woo-Shik Kim, Joel Sole, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, USA 92121 | Tel: Email: | 1-858-845-8893 wooshikk@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this document it is reported that the sample adaptive offset (SAO) does not work properly when bit-depth is high. It is reported that in case of 16 bit video coding SAO is used in 0% of LCUs in a picture, thus providing 0% coding gain. As HEVC Range Extension covers high bit-depth video coding, it is encouraged to perform further study to resolve the issue.

# Introduction

The sample adaptive offset (SAO) is a new coding tool introduced in High Efficiency Video Coding (HEVC) version 1. The motivation of SAO is to include a deringing filter into the coding loop like in-loop deblock filter. To improve the performance a set of SAO parameters is signaled for each LCU.

The main benefit of SAO is known as subjective quality improvement rather than objective quality improvement. As SAO corrects pixel value at the cost of bitrate to code SAO parameters, it can be inferred that if SAO is used, the picture quality can be improved even though there is no bitrate savings. In other words, it is better to use SAO to improve the subjective quality, if the bitrate overhead is not too much.

In the last meeting, there were two proposals that propose to improve SAO performance for high bit-depth video coding. In this document the SAO coding performance on high bit-depth video is evaluated, and it is studied whether the previous proposals can improve the performance.

# SAO performance evaluation

First, it is tested how much coding gain can be achieved by SAO for high bit-depth video coding. HM12.1_RExt4.1 is used with 16 bit test sequences, which were used to study the internal precision for high bit-depth coding [1]. 50 frames are coded with the low-delay B configuration. For 10 bit case, QP values of 22, 27, 32, and 37 are used, and for higher bit-depth cases, QP is decreased by 6 for 1 bit-depth. For example, for 16 bit coding QP values of -14, -9, -4, and 1 are used. This implies that the same quantization step size is used regardless to the bit-depth. The HM anchor is compared to the case when SAO is off. Table 1 shows the summary results.

Table 1. Performance of SAO off compared to SAO on in HM.

|  | LB HBD |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 16-bits | 0.0% | 0.0% | 0.0% |
| 14-bits | 0.0% | 0.0% | 0.0% |
| 12-bits | 0.4% | 0.3% | 0.3% |
| 10-bits | 1.7% | 1.8% | 1.4% |

This reveals that SAO does not provide the expected coding gain when it is used for high bit-depth coding.

To further verify its performance, the SAO Lagrange multiplier is set to one, so that the selection of SAO usage can be increased. Table 2 shows the summary results.

Table 2. Summary results of SAO performance when SAO Lagrange multiplier is set to one.

|  | LB HBD |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 16-bits | 0.0% | 0.0% | 0.0% |
| 14-bits | 0.1% | 0.1% | 0.1% |
| 12-bits | 0.2% | 0.1% | 0.1% |
| 10-bits | 4.6% | 2.8% | 3.4% |

By doing this it is expected that PSNR would increase at the cost of increment of bitrate, and as bitrate increment would be large, there would be loss in terms of BD-bitrate. This is what happened in case of 10 bit video coding. However, in case of 16 bit video coding this does not happen, which tells that the current SAO design has a problem to be used for high bit-depth video coding.

# Evaluation of previous proposals

In the last meeting there were two proposals [2, 3] to deal with SAO for high bit-depth video coding. The methods in those proposals can be categorized into three. First, change of edge classification scheme for edge offset; second, offset scaling part; and third, offset coding part.

## Edge classification

In the current HM, the edge shape is classified into five categories, and the offset is applied to four of them. Fig. 1 shows the edge shapes of the four edge categories.

Fig. 1. Edge classification in SAO.

To determine the edge category, the difference between the current pixel value and the neighboring pixel value is taken, and its sign value is checked as shown below.

edgeIdx = 2 + Sign( recPicture[ xSi ][ ySj ] − recPicture[ xSi + hPos[ 0 ] ][ ySj + vPos[ 0 ] ] ) +
		Sign( recPicture[ xSi ][ ySj ] − recPicture[ xSi + hPos[ 1 ] ][ ySj + vPos[ 1 ] ] ).

This means the effective interval is pixel value 1 regardless to input bit-depth. This would make the edge classification process too much sensitive to the noise in case of high bit-depth coding.

Currently offset is calculated as:

o = (S/N) >> Min(0, BitDepth – 10),

where S is the sum of difference between the original pixel and the reconstructed pixel in the same category, and N is the number of pixels in the same category. Experimental examination on high bit-depth video coding revealed that category 1 and 4 are selected too many times with small value of S, which always makes the offset value as 0 by dividing small S by large N. Similarly no meaningful offset was selected for the other categories.

To fix this problem it was proposed in [2, 3] to increase the interval according to bit-depth. There were three different approaches, and we verified that they all behave in the similar way. Considering its complexity it would be good to scale down each pixel before subtraction. This does not require any additional condition check, while the others do due to sign check.

It is also proposed to provide flexibility by signaling the interval. This will provide flexibility, but it may be enough to apply fixed number of right shift according to bit-depth, e.g., Max(0, BitDepth – 10).

## Offset scaling

Even though the edge classification problem is fixed, it is noticed that often the offset becomes zero. This is because the offset is scaled down according to bit-depth, if bit-depth is larger than 10. As scaling of offset can reduce bitrate to code offset, this can be related to rate-distortion performance.

In our experiments, we reduced QP value by 6 when bit-depth is increased by 1 with the base QP values of 22, 27, 32, and 37 for 10 bit case. This implies that similar mean square error would be resulted with the same quantization step size regardless to bit-depth. In this case, it is expected the SAO offset magnitude would be similar, too. With this assumption, the current SAO design will make the offset too small for high bit-depth by scaling down according to bit-depth. Therefore, it may be reasonable not to apply the scaling operation for the offset according to bit-depth.

In [3], it is proposed to signal the number of bits for right shift operation to be applied to offset, which will provide flexibility.

## Offset coding

In [3], it is also proposed to signal maximum value of offset, which is necessary for TU binarization. Within the limited experiments we performed, we observed no benefit from increased maximum value. On the other hand, it would increase the maximum number of bins for offset coding.

However, it may be possible to achieve better performance by improving the entropy coding of SAO parameters. Therefore, further investigation is encouraged.

## SAO Usage

In the table below, the usage of SAO is compared between the current HM and the case where fixed scaling is used for edge classification and no scaling is applied for offset.

Table 3. SAO usage of HM and the case where fixed scaling is applied for edge classification and no scaling is applied for offset.

(%)

| bit-depth | QP | HM |  |  |  |  |  | edge scaling/offset no scaling |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | luma |  |  | chroma |  |  | luma |  |  | chroma |  |  |
|  |  | Off | EO | BO | Off | EO | BO | Off | EO | BO | Off | EO | BO |
| 16-bits | -14 | 100 | 0 | 0 | 100 | 0 | 0 | 86 | 13 | 1 | 90 | 0 | 10 |
|  | -9 | 100 | 0 | 0 | 100 | 0 | 0 | 97 | 3 | 0 | 97 | 0 | 3 |
|  | -4 | 100 | 0 | 0 | 100 | 0 | 0 | 99 | 1 | 0 | 99 | 0 | 1 |
|  | 1 | 100 | 0 | 0 | 100 | 0 | 0 | 100 | 0 | 0 | 100 | 0 | 0 |
| 14bits | -2 | 100 | 0 | 0 | 100 | 0 | 0 | 99 | 1 | 0 | 100 | 0 | 0 |
|  | 3 | 100 | 0 | 0 | 100 | 0 | 0 | 99 | 1 | 0 | 100 | 0 | 0 |
|  | 8 | 100 | 0 | 0 | 100 | 0 | 0 | 96 | 4 | 0 | 97 | 3 | 0 |
|  | 13 | 96 | 4 | 0 | 96 | 4 | 0 | 83 | 17 | 0 | 87 | 13 | 0 |
| 12bits | 10 | 100 | 0 | 0 | 100 | 0 | 0 | 88 | 12 | 0 | 91 | 9 | 0 |
|  | 15 | 72 | 27 | 1 | 91 | 8 | 2 | 58 | 42 | 0 | 83 | 15 | 2 |
|  | 20 | 20 | 79 | 1 | 45 | 52 | 3 | 25 | 75 | 0 | 56 | 44 | 0 |
|  | 25 | 34 | 64 | 2 | 22 | 66 | 12 | 45 | 55 | 0 | 54 | 44 | 2 |
| 10bits | 22 | 22 | 76 | 1 | 26 | 66 | 8 | 22 | 76 | 1 | 26 | 66 | 8 |
|  | 27 | 45 | 53 | 2 | 36 | 57 | 8 | 45 | 53 | 2 | 36 | 57 | 8 |
|  | 32 | 76 | 23 | 1 | 56 | 43 | 1 | 76 | 23 | 1 | 56 | 43 | 1 |
|  | 37 | 92 | 8 | 0 | 89 | 11 | 0 | 92 | 8 | 0 | 89 | 11 | 0 |

This statistics show that the usage of SAO is a little bit increased by applying scaling for edge classification and not applying offset scaling. However, its increment is very limited, thus it provides no improvement in terms of BD-bitrate.

# Conclusion

In this document the problem of SAO for high bit-depth coding is reported. The previous proposals to resolve this issue are studied. Some of them look reasonable for SAO to work for high bit-depth video coding. It is encouraged to further investigate the performance of SAO to make it work for high bit-depth video coding.

# References

- K. Sharman, N. Saunders, and J. Gamei, “AHG 5 and 18: Internal precision for high bit depths,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N0188, Jul. 2013.

- A. Tourapis, “SAO extension for higher bit-depth coding,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N0201, Jul. 2013.

- S.-T. Hsiang, C.-M. Fu, Y.-W. Huang, and S. Lei, “AHG5: Modified SAO for range extensions,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-N0246, Jul. 2013.