| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0093 |
| --- | --- |

| Title: | AHG9: On inter-layer dependency |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yan Ye, Xiaoyu Xiu, Yuwen He 9710 Scranton Road, #250 San Diego, CA 92121 USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc |  |  |

_____________________________

# Abstract

This contribution proposes constraint and syntax modifications to simplify the high level signaling of inter-layer dependency type. The constraint is to mandate all direct dependent layers are used for inter-layer sample prediction. Based on this constraint, two options are proposed to simplify the current layer dependency signaling: (1) removing syntax element indicating dependency type length and merging the dependency flag syntax and the dependency type syntax together; (2) removing dependency type syntax and signaling inter-layer motion prediction using a new flag.

# Constraint on inter-layer dependency type

SHVC working draft v3 specifies 3 inter-layer dependency types with the syntax element direct_dependency_type:

- sample prediction only, when direct_dependency_type is equal to 0

- motion prediction only, when direct_dependency_type is equal to 1

- both sample and motion prediction, when direct_dependency_type is equal to 2

In addition, SHVC main profile constrains that the inter-layer sample and motion prediction shall be from the same reference layer picture when both sample prediction and motion prediction are enabled.

However, the current SHVC design allows the case where a direct dependent layer is used to provide only inter-layer motion prediction, but not inter-layer sample prediction. We consider the case of decoding a dependent layer and using it only in motion prediction but not in sample prediction to have very limited practical meaning. Instead, it is more logical to use inter-layer sample prediction as a base form of inter-layer prediction, and to allow inter-layer motion prediction from a reference layer only if inter layer sample prediction is allowed from the same layer.

# Proposed simplification of layer dependency signaling

## Merge the signaling of direct_dependency_flag and direct_dependency_type

Currently the syntax elements direct_dependency_flag and the syntax element direct_dependency_type are both used to indicate the dependency between two layers, which results in somewhat redundant signaling. We propose to remove direct_dep_type_len_minus2 and merge direct_dependency_flag and direct_dependency_type into one syntax element, direct_dependency_type, as shown in Table 1.

- Dependency signaling consolidation in VPS extension

| vps_extension( ) { | Descriptor |
| --- | --- |
| … | … |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| … | … |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v)ue(v) |
| … | … |
| } |  |

The semantics of direct_dependency_type is modified as follows:

direct_dependency_type[ i ][ j ] indicates the type of dependency between the layer with nuh_layer_id equal layer_id_in_nuh[ i ] and the layer with nuh_layer_id equal to layer_id_in_nuh[ j ].

direct_dependency_type[ i ][ j ] equal to 0 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[j] is not a direct reference layer for the layer with nuh_layer_id equal to layer_id_in_nuh[i];

direct_dependency_type[ i ][ j ] equal to 1 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[j] is a direct reference layer for the layer with nuh_layer_id equal to layer_id_in_nuh[i], and layer with nuh_layer_id equal to layer_id_in_nuh[j] is used for inter-layer sample prediction;

direct_dependency_type[ i ][ j ] equal to 2 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[j] is a direct reference layer for the layer with nuh_layer_id equal to layer_id_in_nuh[i], and layer with nuh_layer_id equal to layer_id_in_nuh[j] is used for both inter-layer sample prediction  and inter-layer motion prediction;

The variables NumDirectRefLayers[ i ], RefLayerId[ i ][ j ] SamplePredEnabledFlag[ i ][ j ], MotionPredEnabledFlag[ i ][ j ] and DirectRefLayerIdx[ i ][ j ] derivation specified in Error: Reference source not found can be modified as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	iNuhLId = layer_id_in_nuh[ i ]
	NumDirectRefLayers[ iNuhLId ] = 0
	for( j = 0; j < i; j++ )
		if( direct_dependency_type[ i ][ j ] ) {
			RefLayerId[ iNuhLId ][ NumDirectRefLayers[ iNuhLId ]++ ] = layer_id_in_nuh[ j ]

SamplePredEnabledFlag[ iNuhLId ][ j ] = ( ( direct_dependency_type[ i ][ j ] + 1 ) & 1 )
			MotionPredEnabledFlag[ iNuhLId ][ j ] = ( ( ( direct_dependency_type[ i ][ j ] + 1 ) & 2 ) >> 1 )
			SamplePredEnabledFlag[ iNuhLId ][ j ] = 1
			MotionPredEnabledFlag[ iNuhLId ][ j ] = (( direct_dependency_type[ i ][ j ] & 2)>>1)
			DirectRefLayerIdx[ iNuhLid ][ layer_id_in_nuh[ j ] ] = NumDirectRefLayers[ iNuhLId ] – 1

}
}

## Signaling on inter layer motion prediction

In the second option, the syntax element direct_dependency_flag is used to indicate inter layer sample prediction, and the syntax element direct_dependency_type is removed from the VPS extension. To indicate inter layer motion prediction on top of inter layer sample prediction, a flag is added conditionally if direct_dependency_flag is set to 1.

Table 2 shows the proposed syntax structure for the second option.

- Motion prediction flag proposed in VPS extension

| vps_extension( ) { | Descriptor |
| --- | --- |
| … | … |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| if(direct_dependency_flag[i][j]) |  |
| il_motion_prediction_flag[i][j] | u(1) |
| … | … |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| … | … |
| } |  |

direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. direct_dependency_flag[ i ][ j ] equal to 1 specifies that the layer with index j may be a direct reference layer for the layer with index i, and the layer with its nuh_layer_id equal to layer_id_in_nuh[j] is used for inter-layer sample prediction of the layer with nuh_layer_id equal to layer_id_in_nuh[i]. When direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

il_motion_prediction_flag[ i ][ j ] equal to 0 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is not used for inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. il_motion_prediction_flag[ i ][ j ] equal to 1 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. When il_motion_prediction_flag[i][j] is not present for i and j in the range of 0 to vps_max_layers_minus1, it is inferred to be equal to 0.

The variables NumDirectRefLayers[ i ], RefLayerId[ i ][ j ] SamplePredEnabledFlag[ i ][ j ], MotionPredEnabledFlag[ i ][ j ] and DirectRefLayerIdx[ i ][ j ] are derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	iNuhLId = layer_id_in_nuh[ i ]
	NumDirectRefLayers[ iNuhLId ] = 0
	for( j = 0; j < i; j++ )
		if( direct_dependency_flag[ i ][ j ] ) {
			RefLayerId[ iNuhLId ][ NumDirectRefLayers[ iNuhLId ]++ ] = layer_id_in_nuh[ j ]

SamplePredEnabledFlag[ iNuhLId ][ j ] = ( ( direct_dependency_type[ i ][ j ] + 1 ) & 1 )
			MotionPredEnabledFlag[ iNuhLId ][ j ] = ( ( ( direct_dependency_type[ i ][ j ] + 1 ) & 2 ) >> 1 )
			SamplePredEnabledFlag[ iNuhLId ][ j ] = 1
			MotionPredEnabledFlag[ iNuhLId ][ j ] = il_motion_prediction_flag[ i ][ j ]
			DirectRefLayerIdx[ iNuhLid ][ layer_id_in_nuh[ j ] ] = NumDirectRefLayers[ iNuhLId ] – 1

}
}

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).