| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0094 |
| --- | --- |

| Title: | AHG9: On SHVC Main Profile Constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yan Ye, Xiaoyu Xiu, Yuwen He 9710 Scranton Road, #250 San Diego, CA 92121 USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc |  |  |

_____________________________

# Abstract

This contribution proposes two constraints for the SHVC main profile: (1) to restrict the total number of dependent layers, including direct dependent and indirect dependent layers, that can be used for inter-layer prediction for SHVC so as to limit the decoder complexity; (2) to require the base layer with TemporalID equal to 0 to be the dependent layer, direct or indirect, for all the enhancement layers so as to fulfill the conformance requirement of sub-bitstream extraction process.

# Constraint on the number of dependent layers

SHVC working draft v3 specifies 3 bitstream conformance constraints on SHVC main profile in clause H.11.1.2 as copied below:

–	The picture resampling process of picture sample values as specified in subclause Error: Reference source not found shall not be invoked more than once for decoding of each particular picture and the resampling process of picture motion field as specified in subclause Error: Reference source not found shall not be invoked more than once for decoding of each particular picture. When both picture sample values and picture motion field resampling processes are invoked for decoding of a particular picture, they shall be applied to the same reference layer picture.

–	When DependencyId[ i ] is equal to 1, ScalabilityId[ LayerIdxInVps [ i ]][ smIdx ] shall be equal to 0 for any smIdx value from 0 to 15, inclusive, that is not equal to 2, for any coded picture with nuh_layer_id equal to i.

–	When avc_base_layer_flag equal to 1, it is a requirement of bitstream conformance that MotionPredEnabledFlag[ iNuhLId ][ 0 ] shall be equal to 0 for iNuhLId equal to any value of nuh_layer_id present in the bitstream.

For spatial scalability, the main profile specifies that, when decoding a particular picture, at most one reference picture from one of the direct dependent layers can be resampled for inter-layer prediction. However, this main profile constraint does not apply to SNR scalability scenario. For SNR scalability, when decoding a particular picture, up to 63 direct dependent layers can be used for inter-layer prediction, since neither sample resampling nor motion resampling is needed.

There are a few reasons why the current constraint cannot be used to limit the decoder complexity. First of all, the above constraint is specified at the picture level; this means different pictures can still have different inter layer reference pictures from different direct dependent layers within a CVS, as shown in Figure 1. At the sequence level, due to temporal prediction within each layer, the decoder may still need to fully decode up to 63 active reference layers.

- An example of inter-layer prediction structure for SHVC main profile

Additionally, the impact of indirect dependent layers on decoder complexity is not addressed with the current constraint. Figure 2 shows such an example: Layer-63 has only one direct dependent layer, layer-62, layer-62 has only one direct dependent layer, layer-61 layer-61 has only one direct dependent layer, layer-60, …, so on and so forth. In order to correctly decode any picture in layer-63 (e.g. the picture highlighted in orange), not only must the collocated picture of its direct dependent layer, layer-62, be decoded, all the pictures (shadowed by green in Figure 2) from all indirect dependent layers must also be decoded. In the worst case, the number of indirect dependent layers could be up to 62 layers. Given SHVC adopts the multi-loop design, without proper constraint, the decoder complexity will be too high.

- An example of direct/indirect dependency structure for SHVC main profile

In order to properly constrain the complexity of an  SHVC decoder conforming to the SHVC main profile, it is desirable to specify the total number of dependent layers (direct and indirect) allowed.

The following pseudo-code is proposed to derive the total number of direct and indirect dependent layers:

The variables NumDirectRefLayers[ i ], NumIndirectRefLayers[i], RefLayerId[ i ][ j ] SamplePredEnabledFlag[ i ][ j ], MotionPredEnabledFlag[ i ][ j ] and DirectRefLayerIdx[ i ][ j ] are derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	iNuhLId = layer_id_in_nuh[ i ]
	NumDirectRefLayers[ iNuhLId ] = 0
	NumIndirectRefLayers[ iNuhLId ] = 0
	iRefLayerMarkedFlag[ iNuhLId ] = 0
	for( j = 0; j < i; j++ )
		if( direct_dependency_flag[ i ][ j ] ) {
			RefLayerId[ iNuhLId ][ NumDirectRefLayers[ iNuhLId ]++ ] = layer_id_in_nuh[ j ]
			SamplePredEnabledFlag[ iNuhLId ][ j ] = ( ( direct_dependency_type[ i ][ j ] + 1 ) & 1 )
			MotionPredEnabledFlag[ iNuhLId ][ j ] = ( ( ( direct_dependency_type[ i ][ j ] + 1 ) & 2 ) >> 1 )
			DirectRefLayerIdx[ iNuhLid ][ layer_id_in_nuh[ j ] ] = NumDirectRefLayers[ iNuhLId ] – 1

iRefLayerMarkedFlag[iNuhLId] = iRefLayerMarkedFlag[iNuhLId] | ( 1 << j)

getIndirectDependentLayers(j, iRefLayerMarkedFlag[iNuhLId], NumIndirectRefLayers[iNuhLId])

}
}

Where getIndirectDependentLayers() is defined below:

getIndirectDependentLayers(layer_index, iRefLayerMarkedFlag, NumIndirectRefLayers) {

for( i = 0; i < layer_index; i++) {

if(direct_dependency_flag[layer_index][i] && !(iRefLayerMarkedFlag & (1 << i))) {

iRefLayerMarkedFlag = iRefLayerMarkedFlag | (1 << i)

NumIndirectRefLayers ++

getIndirectDependentLayers(i, iRefLayerMarkedFlag, NumIndirectRefLayers)

}

}

}

The total number of direct and indirect dependent layers of the layer with nuh_layer_id equal to iNuhLId, NumRefLayers[iNuhLId], is the summation of NumDirectRefLayers[iNuhLId] and NumIndirectRefLayers[iNuhLId].

NumRefLayers[iNuhLId] = NumDirectRefLayers[iNuhLId] + NumIndirectRefLayers[iNuhLId]

The constraint to SHVC main profile is proposed as follows:

- For a particular layer with nuh_layer_id equal to iNuhLId, the total number of the direct and indirect dependent layers, NumRefLayers[iNuhLId], shall be equal to 1.

# Constraint on the base layer

The sub-bitstream extraction process specified in HEVC standard clause 10 requires that a conforming bitstream contains one or more coded slice segment NAL units with nuh_layer_id equal to 0 and TemporalId equal to 0.

SHVC WD 3 applies the same sub-bitstream extraction process to an SHVC bitstream with the bitstream, the highest temporal layer Id (HighestTid) and the layer identification list TargetDecLayerIdList as inputs.

Figure 3 shows a layer dependency example with two simulcast layers, layer-0 and layer-1. Base layer, layer-0, is the dependent layer of layer-2 and layer-4; while layer-1 is the dependent layer of layer-3. When layer-2 or layer-4 is the targeting decoding layer, the base layer would be included in the output bitstream after sub-bitstream extraction. However, the base layer would not be included in the output bitstream if the targeting decoding layer is layer-3.

- Layer dependency example with 2 simulcast layers

In order to guarantee the input and output bitstreams of sub-bitstream extraction process are conforming bitstreams as specified in sub-bitstream extraction, for all the layers in a conforming bitstream, the base layer with nuh_layer_id equal to 0 shall always be a dependent layer.

An additional constraint is proposed to be added to SHVC main profile as follows:

- A layer comprising a picture with nuh_layer_id equal to 0 and TemporalId equal to 0 shall be a direct or indirect dependent layer of the layer for each nuh_layer_id value in the TargetDecLayerIdList.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).