| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0095 |
| --- | --- |

| Title: | AHG9: Skipped Slice Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yan Ye, Xiaoyu Xiu, Yuwen He 9710 Scranton Road, #250 San Diego, CA 92121 USA | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc |  |  |

_____________________________

# Abstract

This contribution proposes syntax elements and decoding process to define enhancement layer skipped picture or skipped slice. This contribution is a follow-up of JCTVC-N0209.

# Introduction

In SVC, the syntax element, slice_skip_flag, is signaled in the slice header in slice layer extension to specify if a particular slice is skipped or not. Such kind of slice skip feature in SVC has been found useful in SVC implementations.  It is useful to an encoder that is CPU starved or bitrate starved, as a low CPU/low bitrate method to encode an enhancement layer slice.  It is also useful to a decoder or middle box for error resiliency, by feeding to the decoder a NAL unit containing the syntax for a skipped slice in place of a missing coded slice, as an error concealment strategy.

In this contribution, we propose the syntax signaling to indicate either all enhancement layer slices in a picture is skipped or a particular slice is skipped. The contribution is an update of the previous contribution JCTVC-N0209 and the indicators are only present in enhancement layer (nuh_layer_id > 0) and when at least one direct reference layer is available (NumDirectReflayers[nuh_layer_id] > 0).

# Proposed syntax

## Proposed slice segment header syntax modification

Three syntax elements, skip_picture_flag, skip_slice_flag and num_ctbs_in_slice_minus1are proposed to be added to the slice segment header as below:

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| if( nal_unit_type >= BLA_W_LP && nal_unit_type <= RSV_IRAP_VCL23 ) |  |
| no_output_of_prior_pics_flag | u(1) |
| slice_pic_parameter_set_id | ue(v) |
| if( !first_slice_segment_in_pic_flag ) { |  |
| if( dependent_slice_segments_enabled_flag ) |  |
| dependent_slice_segment_flag | u(1) |
| slice_segment_address | u(v) |
| } |  |
| if( !dependent_slice_segment_flag ) { |  |
| i = 0 |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| poc_reset_flag | u(1) |
| } |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| discardable_flag | u(1) |
| } |  |
| for( i = 1; i < num_extra_slice_header_bits; i++ ) |  |
| slice_reserved_flag[ i ] | u(1) |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( nuh_layer_id > 0 ¦ ¦ ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| if( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) { |  |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| short_term_ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( long_term_ref_pics_present_flag ) { |  |
| if( num_long_term_ref_pics_sps > 0 ) |  |
| num_long_term_sps | ue(v) |
| num_long_term_pics | ue(v) |
| for( i = 0; i < num_long_term_sps + num_long_term_pics; i++ ) { |  |
| if( i < num_long_term_sps ) { |  |
| if( num_long_term_ref_pics_sps > 1 ) |  |
| lt_idx_sps[ i ] | u(v) |
| } else { |  |
| poc_lsb_lt[ i ] | u(v) |
| used_by_curr_pic_lt_flag[ i ] | u(1) |
| } |  |
| delta_poc_msb_present_flag[ i ] | u(1) |
| if( delta_poc_msb_present_flag[ i ] ) |  |
| delta_poc_msb_cycle_lt[ i ] | ue(v) |
| } |  |
| } |  |
| if( nuh_layer_id > 0 && NumDirectRefLayers[ nuh_layer_id ] > 0 && first_slice_segment_in_pic_flag) |  |
| skip_picture_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag && !skip_picture_flag) |  |
| slice_temporal_mvp_enabled_flag | u(1) |
| } |  |
| if (!skip_picture_flag) { |  |
| skip_slice_flag | u(1) |
| if (skip_slice_flag) |  |
| num_ctbs_in_slice_minus1 | ue(v) |
| } |  |
| if (!skip_picture_flag && !skip_slice_flag) { |  |
| if( nuh_layer_id > 0 && ! all_ref_layers_active_flag && NumDirectRefLayers[ nuh_layer_id ] > 0 ) { |  |
| inter_layer_pred_enabled_flag | u(1) |
| if( inter_layer_pred_enabled_flag && NumDirectRefLayers[ nuh_layer_id ] > 1) { |  |
| if( !max_one_active_ref_layer_flag ) |  |
| num_inter_layer_ref_pics_minus1 | u(v) |
| if( NumActiveRefLayerPics != NumDirectRefLayers[ nuh_layer_id ] ) |  |
| for( i = 0; i < NumActiveRefLayerPics; i++ ) |  |
| inter_layer_pred_layer_idc[ i ] | u(v) |
| } |  |
| } |  |
| if( sample_adaptive_offset_enabled_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| slice_sao_chroma_flag | u(1) |
| } |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPicTotalCurr > 1 ) |  |
| ref_pic_lists_modification( ) |  |
| if( slice_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( slice_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_idx_l0_active_minus1 > 0 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_idx_l1_active_minus1 > 0 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( weighted_pred_flag && slice_type = = P ) ¦ ¦ ( weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| five_minus_max_num_merge_cand | ue(v) |
| } |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( pps_loop_filter_across_slices_enabled_flag && ( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ¦ ¦ !slice_deblocking_filter_disabled_flag ) ) |  |
| slice_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| if( tiles_enabled_flag ¦ ¦ entropy_coding_sync_enabled_flag ) { |  |
| num_entry_point_offsets | ue(v) |
| if( num_entry_point_offsets > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_entry_point_offsets; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| if( slice_segment_header_extension_present_flag ) { |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

skip_picture_flag equal to 1 specifies that slice_segment_data( ) is not present for the current slice.  skip_picture_flag equal to 0 specifies that slice_segment_data( ) is present for the current slice.  When skip_picture_flag is not present, it is inferred to be equal to 0. When skip_picture_flag is equal to 1, it is a requirement of bitstream conformance that NumSamplePredRefLayers[ nuh_layer_id ] shall be greater than 0 and the number of slices of the skipped picture is 1.

skip_slice_flag equal to 1 specifies that slice_segment_data( ) is not present for the current slice.  skip_slice_flag equal to 0 specifies that slice_segment_data( ) is present for the current slice.  When skip_slice_flag not present, it is inferred to be equal to 0. When skip_slice_flag is equal to 1, it is a requirement of bitstream conformance that NumSamplePredRefLayers[ nuh_layer_id ] shall be greater than 0.

num_ctbs_in_slice_minus1 plus 1 specifies the number of coding tree blocks for a slice with skip_slice_flag equal to 1. The value of num_ctus_in_slice_minus1 shall be in the range of 0 to PicSizeInCtbsY-1.

## Proposed slice segment layer RBSP modification

| slice_segment_layer_rbsp( ) { | Descriptor |
| --- | --- |
| slice_segment_header( ) |  |
| if (!skip_picture_flag && !skip_slice_flag) |  |
| slice_segment_data( ) |  |
| rbsp_slice_segment_trailing_bits( ) |  |
| } |  |

## Proposed decoding process

The following text specifies the decoding process for skipped picture, skipped slice with and without tile partition as shown in Figure 1.

- H.8.1.1 Decoding process for a coded picture with nuh_layer_id greater than 0

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in subclause Error: Reference source not found.

- The processes in subclause Error: Reference source not found and Error: Reference source not found specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Prior to decoding the first slice of the current picture, subclause Error: Reference source not found is invoked.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause Error: Reference source not found is invoked for derivation of reference picture list 0 (RefPicList0), and when decoding a B slice, reference picture list 1 (RefPicList1).

- If skip_picture_flag is equal to 1, the skipped picture skipPic is derived by invoking the process in subclause H.8.1.4 with the decoded reference layer picture of the reference layer with layer id equal to  RefLayerId[nuh_layer_id][ NumDirectRefLayers[nuh_layer_id] -1 ] and the layer id of RefLayerId[nuh_layer_id][ NumDirectRefLayers[nuh_layer_id] – 1 ] as inputs.

- Otherwise, if skip_picture_flag is equal to 0, the processes in subclauses H.8.3.5, Error: Reference source not found, Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each form a partitioning of the picture.

- After all slices of the current picture have been decoded, the marking process for ending the decoding of a coded picture with nuh_layer_id greater than 0 specified in subclause Error: Reference source not found is invoked.

H.8.3.5 Decoding process for a skipped slice with nuh_layer_id greater than 0

The decoding process operates as follows for the current skipped slice with skip_slice_flag is set equal to 1.

Input to this process is:

- The resampled reference layer picture, rsPic, of the layer with layer id equal to RefLayerId[nuh_layer_id][ NumDirectRefLayers[nuh_layer_id] – 1 ].

Output of this process is the decoded samples of the skipped slice, skipSliceSample.

The following steps are applied to derive the reconstructed samples of the skipped slice when skip_slice_flag is equal to 1.

- When tiles_enabled_flag is equal to 0, the number of CTBs in the slice specified by num_ctbs_in_slcie_minus1 is in CTB raster scan of the picture.

- The variable, skipCtbAddrInRs, specifying a coding tree block address in coding tree block raster scan of the picture, is in the range of slice_segment_address to (slice_segment_address + num_ctbs_in_slice_minus1).

- The location ( xCtb, yCtb ) which specifies the top-left sample of the current coding tree block is derived as follows:

xCtb = ( skipCtbAddrInRs % PicWidthInCtbsY )  <<  CtbLog2SizeY

yCtb = ( skipCtbAddrInRs / PicWidthInCtbsY )  <<  CtbLog2SizeY

- Otherwise, the number of CTBs in the slice specified by num_ctbs_in_slice_minus1 is in tile scan.

- The variable skipCtbAddrInTs, specifying a coding tree block address in tile scan, is in the range of CtbAddrRsToTs[slice_segment_address] to (CtbAddrRsToTs[slice_segment_address] + num_ctbs_in_slice_minus1)

- The location (xCtb, yCtb), specifying the top-left sample of the current coding tree block which can be derived as follows:

xCtb = ( CtbAddrTsToRs(skipCtbAddrInTs) % PicWidthInCtbsY )  <<  CtbLog2SizeY

yCtb = ( CtbAddrTsToRs(skipCtbAddrInTs) / PicWidthInCtbsY )  <<  CtbLog2SizeY

- The sample value of skipSliceSampleL[x][y], with x = xCtb … (xCtb+CtbSizeY-1), and y = xCtb … (xCtb + CtbSizeY-1), inclusive, is derived by copying the corresponding sample value rsPicSampleL[x][y] of resampled reference layer picture, rsPic.

- The sample value of skipSliceSampleCb[x][y], with x = xCtb … (xCtb + (CtbSizeY/SubWidthC)-1), and y = xCtb … (xCtb + (CtbSizeY/SubHeightC)-1), is derived by copying the corresponding sample value rsPicSampleCb[x][y] of resampled reference layer picture, rsPic.

- The sample value of skipSliceSampleCr[x][y], with x = xCtb … (xCtb + (CtbSizeY/ SubWidthC)-1), and y = xCtb … (xCtb + (CtbSizeY/SubHeightC)-1), is derived by copying the corresponding sample value rsPicSampleCr[x][y] of resampled reference layer picture, rsPic.

- Skipped picture (left) skipped slice w/o tile partition (middle) and skipped slice w/ tile partition.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).