| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0099r1 |
| --- | --- |

| Title: | Time code from AVC pic_timing() SEI for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg Harmonic Inc. Brian Lewis, Jeff Lassahn, Tim Prins Elemental Technologies 225 SW Broadway, Suite 600 Portland, OR 97205 Matteo Naccari Marta Mrak BBC – R&D Centre House, Block D 56 Wood Lane, W12 7SB London | Email: Tel: Email Tel: Email: | chadfogg@gmail.com +1 503 222 3212 brian@elementaltechnologies.com jeffl@elementaltechnologies.com timp@elementaltechnologies.com +44 (0)303 0409640 matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | Harmonic Inc., Elemental Technologies Inc., BBC – R&D |  |  |

_____________________________

# Abstract

This proposal requests that all time code elements from AVC pic_struct() SEI be copied into a new HEVC SEI message, time_code(), which can accompany each coded picture in the elementary HEVC Main, Main10 and future versions of HEVC video bitstreams. Time code is informative, and has no impact on the decoding, HRD, or any output processes covered within the scope of the HEVC specification. Although some application-specific specifications have defined their own time code syntax, or relied on user_data() convetions, the MPEG-2 GOP header and AVC pic_timing() SEI message have served as universal embedded syntax that are relied upon in practice by several encoders across applications to synthesize systems timing information, in particular PTS and timelines. Time codes are also passed through the elementary stream for various uses downstream. Although the authors of this proposal suggested earlier that a simplified SEI time code be re-introduced into the next HEVC draft, it has since been learned that DVB participants wish to retain the full AVC time code flexibility, including time code for any repeat pictures (indicated by pic_struct and DeltaToDivisor in HEVC spec. Table E-6) as per the AVC clock_timestamp[i] array. While not all AVC time code elements and semantics may make sense in HEVC, the co-authors of this proposal suggest at this time to adopt all elements in order to accommodate users who do not track JCT development, and allow translation or pass-through of time code from AVC to HEVC. This proposal suggests necessary but minor adaptations of the AVC time code syntax for HEVC, including: (1) the iterations of the clock_timestamp_flag loop should be determined by DeltaToDivisor. (2) increase the drop count n_frame to 9 bits to accommodate 300 fps in HEVC Table A.4 (3)  time_offset_length is inserted in the proposed time_code() SEI to signal the bit length of time_offset. JCT editors may wish to consider whether  (1) time_offset_length should instead be derived from vui_time_scale; (2) n_frames be greater than 9-bits (3) adapt the semantics of time code, particularly drop frame, to more closely reflect the SAFF paradigm of HEVC.

# Introduction

After a clean slate from the BoG recommendation [1] ,the pic_timing() SEI message was first filled into draft 7 (JCTVC-I1003_d4.doc) of the HEVC version 1 text specification [2] with the sub-picture delays adopted at the San Jose meeting and CPB and DPB delay from AVC. Later, pic_struct was added [3] to the HEVC draft, but no time code elements were carried over from AVC due to lack of interest for them in several subsequent meetings. Since then, feedback from vendors and DVB participants revealed that encoders generate A/V timing info such as PTS from the embedded elementary stream time codes for a number of commercial stream application formats, including: Microsoft Smooth Streaming T0 tracks, Quicktime timecode tracks, RTMP onFI/onSMPTE, HLS PROGRAM-DATE-TIME, TS segment markers (customer-specific), and HDS Manifests.

Several applications also rely on AVC SEI elementary stream embedded time codes, including: cut lists (sections of input files to be concatenated), time code burn in, input clipping & seeking, output locking (synchronization between multiple encoders) and SCC and SRT caption inputs.

# Proposal

The proposed HEVC SEI copies all relevant time code elements from AVC pic_timing() SEI, including “Full time code” syntax, as recommended by DVB in section 5.5.4.1 [4].  Unlike the earlier proposal in [6], no simplified time code is suggested here.

A verbatim copy of AVC time code syntax and semantics is not sufficient by itself for inclusion in HEVC. Some adaptation is needed to accommodate small differences between the relevant HEVC and AVC aspects that time code is dependent upon.  These include:

- HEVC DeltaToDivisor replaces NumClockTS from HEVC.

- The bit length of the element n_frames is suggested to be increased to 9-bits to accommodate higher frames rates of HEVC compared to AVC, in particular: 300 Hz (Table A-4 of the HEVC version 1 specification [5]).  Since “picture rate” is a derived value in Annex A of HEVC, it may be desirable to increase the length to greater than 9 bits.

- Currently, no such time range can be expressed anywhere in HEVC high level syntax, so the time_offset_length is copied from AVC hrd_parameters and pasted in the proposed HEVC time_code() SEI message to signal the length of the time_offset element .Since HEVC has no variable length integers (while AVC does), the type is proposed to be u(v).  Could the length of time_offset is derived from vui_time_scale instead?

## Syntax of proposed time_code() SEI message

The syntax presented below copies over the various drop frame conventions from AVC.

What is not copied from AVC:

- CPB, DPB delays and pic_struct since they are already in pic_timing() SEI message.

- ct_type is essentially source_scan_type in pic_timing()

Elements that may not make sense in HEVC [for future review]:

- The drop frame element nuit_field_based_flag may be unnecessary and confusing in the HEVC sequence adaptive field frame (SAFF) paradigm.

- It may be better to call n_frames:  n_pictures, and reword dependent semantics accordingly

| time_code ( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| for( i = 0; i < DeltaToDivisor ; i++ ) { |  |  |
| clock_timestamp_flag[ i ] | 5 | u(1) |
| if( clock_timestamp_flag[ i ] ) { |  |  |
| nuit_field_based_flag | 5 | u(1) |
| counting_type | 5 | u(5) |
| full_timestamp_flag | 5 | u(1) |
| discontinuity_flag | 5 | u(1) |
| cnt_dropped_flag | 5 | u(1) |
| n_frames | 5 | u(9) |
| if( full_timestamp_flag ) { |  |  |
| seconds_value /* 0..59 */ | 5 | u(6) |
| minutes_value /* 0..59 */ | 5 | u(6) |
| hours_value /* 0..23 */ | 5 | u(5) |
| } else { |  |  |
| seconds_flag | 5 | u(1) |
| if( seconds_flag ) { |  |  |
| seconds_value /* range 0..59 */ | 5 | u(6) |
| minutes_flag | 5 | u(1) |
| if( minutes_flag ) { |  |  |
| minutes_value /* 0..59 */ | 5 | u(6) |
| hours_flag | 5 | u(1) |
| if( hours_flag ) |  |  |
| hours_value /* 0..23 */ | 5 | u(5) |
| } |  |  |
| } |  |  |
| } |  |  |
| time_offset_length | 5 | u(5) |
| if(time_offset_length>0) |  |  |
| time_offset | 5 | u(v) |
| } |  |  |
| } |  |  |
| } |  |  |

The semantics section below is a modification of section D.2.2 from the 2013 publication of AVC.

## Semantics of time_code() SEI message

DeltaToDivisor is determined by pic_struct as specified in Table E-6 (Divisor for computation of DbpOutputElementalInterval[n]). There are up to DeltaToDivisor sets of clock timestamp information for a picture, as specified by clock_timestamp_flag[ i ] for each set. The sets of clock timestamp information apply to the field(s) or the frame(s) associated with the picture by pic_struct.

[ for the derived value DeltaToDivisor, note text bug report where DeltaToDivisor should be 1 when pic_struct=0:  https://hevc.hhi.fraunhofer.de/trac/hevc/ticket/1083.]

The contents of the clock timestamp syntax elements indicate a time of origin, capture, or alternative ideal display. This indicated time is computed as

clockTimestamp = ( ( hH * 60 + mM ) * 60 + sS ) * vui_time_scale + 
			                nFrames * ( vui_num_units_in_tick * ( 1 + nuit_field_based_flag ) ) + tOffset,	(D-[X])

in units of clock ticks of a clock with clock frequency equal to vui_time_scale Hz, relative to some unspecified point in time for which clockTimestamp is equal to 0. Output order and DPB output timing are not affected by the value of clockTimestamp. When two or more frames with pic_struct equal to 0 are consecutive in output order and have equal values of clockTimestamp, the indication is that the frames represent the same content and that the last such frame in output order is the preferred representation.

NOTE [X] – clockTimestamp time indications may aid display on devices with refresh rates other than those well-matched to DPB output times.

clock_timestamp_flag[ i ] equal to 1 indicates that a number of clock timestamp syntax elements are present and follow immediately. clock_timestamp_flag[ i ] equal to 0 indicates that the associated clock timestamp syntax elements are not present. When NumClockTS is greater than 1 and clock_timestamp_flag[ i ] is equal to 1 for more than one value of i, the value of clockTimestamp shall be non-decreasing with increasing value of i.

nuit_field_based_flag is used in calculating clockTimestamp, as specified in Equation D-[X]

Equation D-.

counting_type specifies the method of dropping values of the n_frames as specified in Table D-.]

- Table D-[X] – Definition of counting_type values

| Value | Interpretation |
| --- | --- |
| 0 | no dropping of n_frames count values and no use of time_offset |
| 1 | no dropping of n_frames count values |
| 2 | dropping of individual zero values of n_frames count |
| 3 | dropping of individual MaxFPS − 1 values of n_frames count |
| 4 | dropping of the two lowest (value 0 and 1) n_frames counts when seconds_value is equal to 0 and minutes_value is not an integer multiple of 10 |
| 5 | dropping of unspecified individual n_frames count values |
| 6 | dropping of unspecified numbers of unspecified n_frames count values |
| 7..31 | reserved |

full_timestamp_flag equal to 1 specifies that the n_frames syntax element is followed by seconds_value, minutes_value, and hours_value. full_timestamp_flag equal to 0 specifies that the n_frames syntax element is followed by seconds_flag.

discontinuity_flag equal to 0 indicates that the difference between the current value of clockTimestamp and the value of clockTimestamp computed from the previous clock timestamp in output order can be interpreted as the time difference between the times of origin or capture of the associated frames or fields. discontinuity_flag equal to 1 indicates that the difference between the current value of clockTimestamp and the value of clockTimestamp computed from the previous clock timestamp in output order should not be interpreted as the time difference between the times of origin or capture of the associated frames or fields. When discontinuity_flag is equal to 0, the value of clockTimestamp shall be greater than or equal to all values of clockTimestamp present for the preceding picture in DPB output order.

cnt_dropped_flag specifies the skipping of one or more values of n_frames using the counting method specified by counting_type.

n_frames specifies the value of nFrames used to compute clockTimestamp. n_frames shall be less than

MaxFPS = Ceil( vui_time_scale  ( 2 * vui_num_units_in_tick ) )	(D-[X])

NOTE [X] – n_frames is a frame-based counter. For field-specific timing indications, time_offset should be used to indicate a distinct clockTimestamp for each field.

When counting_type is equal to 2 and cnt_dropped_flag is equal to 1, n_frames shall be equal to 1 and the value of n_frames for the previous picture in output order shall not be equal to 0 unless discontinuity_flag is equal to 1.

NOTE [X] – When counting_type is equal to 2, the need for increasingly large magnitudes of tOffset in Equation D- when using fixed non-integer frame rates (e.g., 12.5 frames per second with vui_time_scale equal to 50 and vui_num_units_in_tick equal to 2 and nuit_field_based_flag equal to 0) can be avoided by occasionally skipping over the value n_frames equal to 0 when counting (e.g., counting n_frames from 0 to 12, then incrementing seconds_value and counting n_frames from 1 to 12, then incrementing seconds_value and counting n_frames from 0 to 12, etc.).

When counting_type is equal to 3 and cnt_dropped_flag is equal to 1, n_frames shall be equal to 0 and the value of n_frames for the previous picture in output order shall not be equal to MaxFPS − 1 unless discontinuity_flag is equal to 1.

NOTE [X] – When counting_type is equal to 3, the need for increasingly large magnitudes of tOffset in Equation D- when using fixed non-integer frame rates (e.g., 12.5 frames per second with vui_time_scale equal to 50 and vui_num_units_in_tick equal to 2 and nuit_field_based_flag equal to 0) can be avoided by occasionally skipping over the value n_frames equal to MaxFPS − 1 when counting (e.g., counting n_frames from 0 to 12, then incrementing seconds_value and counting n_frames from 0 to 11, then incrementing seconds_value and counting n_frames from 0 to 12, etc.).

When counting_type is equal to 4 and cnt_dropped_flag is equal to 1, n_frames shall be equal to 2 and the specified value of sS shall be zero and the specified value of mM shall not be an integer multiple of ten and n_frames for the previous picture in output order shall not be equal to 0 or 1 unless discontinuity_flag is equal to 1.

NOTE [X] – When counting_type is equal to 4, the need for increasingly large magnitudes of tOffset in Equation D- when using fixed non-integer frame rates (e.g., 300001001 frames per second with vui_time_scale equal to 60000 and vui_num_units_in_tick equal to 1 001 and nuit_field_based_flag equal to 1) can be reduced by occasionally skipping over the values of n_frames equal to 0 and 1 when counting (e.g., counting n_frames from 0 to 29, then incrementing seconds_value and counting n_frames from 0 to 29, etc., until the seconds_value is zero and minutes_value is not an integer multiple of ten, then counting n_frames from 2 to 29, then incrementing seconds_value and counting n_frames from 0 to 29, etc.). This counting method is well known in industry and is often referred to as "NTSC drop-frame" counting.

When counting_type is equal to 5 or 6 and cnt_dropped_flag is equal to 1, n_frames shall not be equal to 1 plus the value of n_frames for the previous picture in output order modulo MaxFPS unless discontinuity_flag is equal to 1.

NOTE [X] – When counting_type is equal to 5 or 6, the need for increasingly large magnitudes of tOffset in Equation D- when using fixed non-integer frame rates can be avoided by occasionally skipping over some values of n_frames when counting. The specific values of n_frames that are skipped are not specified when counting_type is equal to 5 or 6.

seconds_flag equal to 1 specifies that seconds_value and minutes_flag are present when full_timestamp_flag is equal to 0. seconds_flag equal to 0 specifies that seconds_value and minutes_flag are not present.

seconds_value specifies the value of sS used to compute clockTimestamp. The value of seconds_value shall be in the range of 0 to 59, inclusive. When seconds_value is not present, the previous seconds_value in decoding order shall be used as sS to compute clockTimestamp.

minutes_flag equal to 1 specifies that minutes_value and hours_flag are present when full_timestamp_flag is equal to 0 and seconds_flag is equal to 1. minutes_flag equal to 0 specifies that minutes_value and hours_flag are not present.

minutes_value specifies the value of mM used to compute clockTimestamp. The value of minutes_value shall be in the range of 0 to 59, inclusive. When minutes_value is not present, the previous minutes_value in decoding order shall be used as mM to compute clockTimestamp.

hours_flag equal to 1 specifies that hours_value is present when full_timestamp_flag is equal to 0 and seconds_flag is equal to 1 and minutes_flag is equal to 1.

hours_value specifies the value of hH used to compute clockTimestamp. The value of hours_value shall be in the range of 0 to 23, inclusive. When hours_value is not present, the previous hours_value in decoding order shall be used as hH to compute clockTimestamp.

time_offset_length greater than 0 specifies the length in bits of the time_offset syntax element. time_offset_length equal to 0 specifies that the time_offset syntax element is not present. When the time_offset_length syntax element is present in more than one hrd_parameters( ) syntax structure within the VUI parameters syntax structure, the value of the time_offset_length parameters shall be equal in both hrd_parameters( ) syntax structures. When the time_offset_length syntax element is not present, it shall be inferred to be equal to 24.  [should this be derived instead from vui_time_scale?]

time_offset specifies the value of tOffset used to compute clockTimestamp. The number of bits used to represent time_offset shall be equal to time_offset_length. When time_offset is not present, the value 0 shall be used as tOffset to compute clockTimestamp.

# References

[1] R. Sjöberg, “Notes from break-out session on TMuC and AVC high-level syntax,” JCTVC-B121, 2nd Meeting: Geneva, CH, 21–28 July 2010.

[2] B. Bross, W-J. Han, J-R Ohm, G. Sullvian, T. Wiegand, “HEVC text specification draft 7,” JCTVC-I1003, 9th Meeting: Geneva, CH, 27 April – 7 May 2012.

[3] O. Bar-Nir, C. Fogg, “Proposal to add pic_struct information to HEVC, with simulation results,” JCTVC-K0165, 11th Meeting: Shanghai, 10-19 Oct. 2012

[4] ETSI TS 101 154, “Digital Video Broadcasting (DVB):  Specification for the use of Video and Audio Coding in Broadcasting Applications based on the MPEG-2 Transport Stream.” September 2009.

[5] B. Bross, W-J. Han, J-R Ohm, G. Sullvian, Y-K. Wang, T. Wiegand,”High Efficiency Video Coding (HEVC) text specification drat 10 (for FDIS and Consent)”,  JCTVC-L1003, March 2013.

[6] C. Fogg, B. Lewis, J. Lassahn, T. Prins, “Time Code SEI”, JCTVC-N0286, 14th meeting: Vienna, July 2013.

# Patent rights declaration(s)

Harmonic Inc.,Elemental Technologies, and The British Broadcasting Corporation (BBC)  do not have any current or pending patent rights relating to the technology described in this contribution.