| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0101r3 |
| --- | --- |

| Title: | High Dynamic Range video coding results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bill Mandel 100 Universal City Plaza Los Angeles, CA 91608 USA Chad Fogg 4300 N. 1st Street San Jose, CA 95134 USA Jim Helman 530 Lytton Ave, Suite 300 Palo Alto, CA 94301 USA | Email: | bill.mandel@nbcuni.com chadfogg@gmail.com jhelman@movielabs.com |
| Source: | Universal Pictures, Harmonic Inc., Movielabs Inc. |  |  |

_____________________________

# Abstract

Results from preliminary tests exploring combinations of bitdepths, profiles, color space, and Electro-Optic Transfer Function (EOTF) indicate unmodified HEVC version 1 Main12 is well suited for coding High Dynamic Range (HDR), large color volume (wide gamut) digital 1080p 24 frames/sec cinematography sequences. Movie clips “Tears of Steel” (from Roosendaal & Hubert (CC) Blender Foundation | mango.blender.org) and “Oblivion” (from Universal Pictures) derived from 16-bit master source video originally captured from the Sony F65 camera are used in this study. In the “Oblivion” clip, Main12 YDzDx-color PQ-EOTF coded video shows very similar R/D curves to that of Main10 YCC Rec. 2020-color-transformed video mapped to the same PQ-EOTF. In tests with RExt12 (which adds the screen content coding tools adopted so far by JCT), plus several extra precision modes (extended_precision_processing_flag=1 and encoder-only tools: #define HIGH_BIT_DEPTH_SUPPORT and HIGH_PRECISION_FORWARD_TRANSFORM) measured a BD-Rate reduction of -2.66% in the Qp 7,12,17,22 range of Oblivion, while BD-Rate increased by +0.11% in the “mid-Qp” range of 22,27,32,37 compared to HEVC version 1 Main12. In a cursory test, additional AHG 18 precision tools (extended_precision_processing_flag=1) being studied in AHG18 showed only small benefits in the tested sequence range of this study (Qp 7 to 37), though are expected to yield greater gains below Qp=7. Future sequences will be tested across a wide range of Qp and a set of practical tool/profile combinations.

# Introduction

The primary test of this study converts 16-bit precision master source to 12-bit 4:2:0 Y’DzDx perceptual quantization (PQ) mapped encoder input [1] designed to be both display-independent and more closely approximate constant perceived brightness than traditional gamma transfer YCbCr. The Y’DzDx frames are then concatenated into a single input file and passed through HEVC encoding as per HEVC common test conditions (CC) [2].  The basic workflow is similar to Blu-Ray and Digital Cinema authoring.

The sequences were coded according to Random Access (High Efficiency) Main10 (encoder_randomaccess_main10.cfg), but with InputBitDepth=12 and InternalBitDepth=12 (“Main12”).  Additional Qp points (Qp=7,12,17) representative of Blu-Ray and cinema distribution quality were added to the standard four CC set (Qp=22,27,32,37). A “cross check” of these results are documented in JCTVC-O0366 (Peng Yin, Taoran Lu).

# Results

The tables from which the following charts are derived are included in the accompanying JCTVC-O0101_r2.xls file.

| Title | Profile | Encoder & settings | Color matrix | EOTF |
| --- | --- | --- | --- | --- |
| Tears of Steel 1080p 24 fps (downscaled from 2160p) | Main12 | HM 12 encoder_randomaccess_main10.cfg InputBitDepth=12, InternalBitDepth=12 | Y’DzDx | PQ |
| Tears of Steel 2160p 24 fps | Main12 | HM 12 encoder_randomaccess_main10.cfg InputBitDepth=12, InternalBitDepth=12 | Y’DzDx | PQ |
| Oblivion 1080p 24fps | Main12 | HM 12 encoder_randomaccess_main10.cfg InputBitDepth=12, InternalBitDepth=12 | Y’DzDx | PQ |
| Oblivion 1080p 24fps | Main10 | HM 12 encoder_randomaccess_main10.cfg InputBitDepth=10, InternalBitDepth=10 | YCC Rec.2020 | PQ |
| Oblivion 1080p 24fps | Main10 | HM 12 encoder_randomaccess_main10.cfg InputBitDepth=10, InternalBitDepth=10 | YCbCr Rec.709 | Rec. 709 |
| Oblivion 1080p 24fps | RExt12 | HM-12.0+RExt-4.1 encoder_randomaccess_main_rext.cfg InputBitDepth=12, InternalBitDepth=12 | Y’DzDx | PQ |

Table 1 Summary of conducted experiments

## “Tears of Steel”

Both full 2160p and 1080p downscaled-from-2160p (exported by the rescaler within Sony’s F65 tools) were tested on this 24 frame/sec public domain clip. Main12 encodings of the selected sequence (as shown in Fig. 2 photo below) demonstrates desirable monotonic R-D behaviour.

Figure 1a --  “Tears of Steel”, Main12 RA (CC), QP=37,32,27,22,17,12 and 7

On the other hand, the 2160p full-resolution version RD curve (Fig. 1b) appears to show the effects of blurrier areas and more noise than the 1080p derivation.

Figure 1b

Figure 2 -- First frame from selected “Tears of Steel” clip

## “Oblivion”

All scenes from Oblivion are 1920x1080 (1080p) 24 frames/sec. A total of 192 frames were encoded in the test sequence with the same “Main12” variant of the CC settings applied in “Tears of Steel”

### 12-bit Oblivion test

.
Figure 3 – “Oblivion” (192 frames) for Qp 37,32,27,22,17,12,7

### Main12 & RExt 12 comparison

This test applies HM RExt 4.1 with 12-bit settings representing the RExt12 analogy to Main12, as documented in RExt CC [3] (encoder_randomaccess_main_rext.cfg).  A BD-Rate reduction of -2.66% was measured in the QP 7-22 range (Fig.4), while a small +0.11% increase was measured for QP 22-37 (not shown here).

Figure 4

A cursory test with extended_processing_precision=1, and other relevant AHG18  modes [4] set in RExt 4.1 TypeDef.h was run to check whether these tools had any impact beyond RExt12, but no Y-BD-Rate benefits were measured greater than +/- 0.2%.

### EOTF behaviour comparison

Though “apples to oranges”, the following graph shows behavior of PQ and Rec.709 that translates through encoding.

Figure 6

Figure 6a Oblivion 12-bit YDzDx PQ-EOTF histogram

Figure 6b Oblivion 10-bit YCbCr 709-EOTF histogram

# References

[1] S. Miller, M. Nezamabadi, S. Daly, “Perceptual Signal Coding for More Efficient Usage of Bit Codes”,  SMPTE Journal, Vol. 122, No. 4, May-June 2013.

[2]  F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-L1100, Geneva, January 2013.

[3] D. Flynn, K. Sharman, C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-N1006, July 2013.

[4] K. Sharman, N. Saunders, “AHG5 and AHG18: Transform Matrix Precision for High Bit Depths”, JCTVC-O0068, Oct. 2013

# Patent rights declaration(s)

Universal, Harmonic, and MovieLabs do not have any current or pending patent rights relating to the technology described in this contribution.