| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0103r1 |
| --- | --- |

| Title: | Intra-only SHVC rescaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg 4300 N. 1st Street San Jose, CA 95134 USA | Email: | chadfogg@gmail.com |
| Source: | Harmonic Inc. |  |  |

_____________________________

# Abstract

It is proposed to add a mandate to and adhoc to investigate a combination of Intra-base-only, high-level-syntax enhancement-layer only SHVC combination that would lead to a HEVC version 2 profile to signal the logical constraints of such a combination. Use cases and operating examples were most recently described by Nokia in Incheon document JCTVC-M0040. This combination would allow smoother, more coding efficient trick mode operation in VoD, and offer an alternative to field structured sequences to reduce buffer delay and intra coded picture bit size. Such a combination offers a less drastic rate control buffer overflow prevention “panic” mode for multi-program multiplexing than a more visible ramping of Qp.

# Proposal

In a Intra-base-layer-only, high-level-syntax enhancement-layer (IBLOHLSEL) scheme, there is only one coded frame for each temporal frame period (either base or enhancement, but not both), rather than the traditional SHVC scenario where there is potentially one corresponding temporally co-incident frame in each of the base layer (BL) and enhancement layer (EL). In other SHVC studies, the BL is AVC or HEVC Main, and the EL makes use of full SHVC syntax, i.e. non high level only syntax SHVC.

## Common Conditions

Towards the above adhoc mandate, the following parameters are suggested in the fashion of the SHVC common conditions document [1], for the proposed IBLOHLSEL combination:

| Optional configurations for the Common Test conditions |  |
| --- | --- |
| HEVC base layer (BL) | HEVC enhancement layer (EL) |
| - Intra only (IRAP such as CRA, IDR, etc.) | - I, P or B, spatial 1.5x - I, P or B, spatial 2x - “High level syntax only” SHVC |

## Profile

The level of the BL bitstream shall be the same level of the EL.  The BL VCL shall consist only of IRAP NALs.

The coded picture size of the EL shall be 1x, 1.5x, or 2x of the BL in each of the horizontal and vertical dimensions.

Avc_ base_layer_flag shall be equal to 0.

# References

[1] X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, JCTVC-N1009, Vienna, Austria, July 2013.

[2] K. Ugur, “AHG-9: Using SHVC for adaptive resolution change and efficient trick mode,” JCTVC-M0040, Incheon, Korea, 18-26 April 2013.

# Patent rights declaration(s)

Harmonic does not have any current or pending patent rights relating to the technology described in this contribution.