| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0109 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0042 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: VPS extension clean-up |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjin Cho Byeongdoo Choi Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Tel: Email: | {yongjin9.cho, b.d.choi, m.w.park, jinyoung79, hc.wey, dionism}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes improvements and modifications of vps extension syntax elements. Some of syntax elements in the current vps extension require improvements or modifications in syntax and semantics. This contribution addresses such issues.

# Proposal

| vps_extension( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( NumViews > 1 ) { |  |
| view_id_len_minus1 | u(4) |
| for( i = 01; i < NumViews; i++ ) |  |
| view_id_val[i ] | u(v) |
| } |  |
| …… |  |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] profile_ref [i] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| …… |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flagdefault_output_layer_idx | u(1)u(2) |
| …… |  |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| vps_num_rep_formats_minus1 | u(4) |
| for( i = 0; i <= vps_num_rep_formats_minus1; i++ ) |  |
| rep_format( ) |  |
| if( rep_format_idx_present_flag ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| if( vps_num_rep_formats_minus1 > 0) |  |
| vps_rep_format_idx[ i ] | u(4) |
| rep_format ( ) |  |
| single_rep_format_flag | u(1) |
| if ( !single_ref_format_flag ) { |  |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| vps_num_rep_formats_minus12 | u(4) |
| for( i = 0; i <= vps_num_rep_formats_minus12; i++ ) |  |
| rep_format( ) |  |
| if( rep_format_idx_present_flag ) |  |
| for( i = 1; i <= vps_max_layers_minus12; i++ ) |  |
| if( vps_num_rep_formats_minus1 > 0) |  |
| vps_rep_format_idx[ i ] | u(4) |
| } |  |
| …… |  |
| cross_layer_irap_aligned_flag | u(1) |
| single_layer_for_non_irap_flag | u(1) |
| …… |  |
| } |  |

- When NumViews is equal to 1, the length of view_id_val syntax element is not known. Moreover, the syntax element is not necessary when NumViews is equal to 1.

- profile_ref_minus1[i] is broken for i equal to 1. That is, when i is equal to 1 and profile for the 1st layer set is not present, profile_ref_minus1[1] should have a value corresponding to 0. However, even the minimum value of the syntax element will refer to the profile index value of 1. Also, it conflicts with the current semantics that profile_ref_minus1[i]+1 shall be less than i, because profile_ref_minus1[1]+1 cannot be less than i even though the value of profile_ref_minus1[1] is 0.

  - Semantics change

profile_ref_minus1[ i ] specifies that the profile and tier information for the i-th profile_tier_level( ) syntax structure is inferred to be equal to the profile and tier information for the (profile_ref_minus1[ i ] + 1)-th layer set. The value of profile_ref_minus1[ i ] + 1 shall be less than i.

  - Option 2, not signaling when i is equal to 1.

| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| --- | --- |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] && i > 1) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |

profile_ref_minus1[ i ] specifies that the profile and tier information for the i-th profile_tier_level( ) syntax structure is inferred to be equal to the profile and tier information for the (profile_ref_minus1[ i ] + 1)-th layer set. The value of profile_ref_minus1[ i ] + 1 shall be less than i. When i is equal to 1 and vps_profile_present_flag[1] is equal to 0, profile reference of the first layer is inferred to be 0.

- default_one_target_output_layer_flag

  - Removal of the flag

    - The syntax element can be derived/inferred from the type of scalability and the number of scalability (i.e., by scalability_mask_flag[i]). No reason to signal default output layer.

      - When NumScalabilityTypes is equal to 1 and scalability_mask_flag[1] is equal to 1 (Multiview scalability only), then the output of default layer sets can be inferred to be all layers.

      - When NumScalabilityTypese is equal to 1 and scalability_mask_flag[2] is equal to 1 (spatial/SNR scalability only), then output of default layer sets can be inferred to be the highest layer.

      - For other scalability types, the default behavior might be either all or one, and thus can be inferred as well.

      - When a layer set contains more than 1 scalability types, the default behavior can be pre-defined. For example, when multiview and shvc scalability are present in a layer set, then the default output can be defined as all layers with highest DependencyId.

    - A variable DefaultOneTargetOutputLayerFlag can be introduced to store the inferred value.

  - Substitution of the flag with default_output_layer_idx

    - If more default output behaviors other than one or all are expected, an index is introduced to specify the default target output layer(s).

    - default_output_layer_idx semantics

default_output_layer_idx equal to 0 specifies that all layers in each of the default output layer sets are target output layers. default_output_layer_idx equal to 1 specifies that the highest layer in each of the default output layers is a target output layer. The values of default_output_layer_idx greater than 1 are reserved for future use.

- single_rep_format_flag

  - equal to 1 specifies that all layers share a common representation format. single_ref_format_flag equal to 0 specifies that the number of representation formats is greater than one.

  - Semantics

single_rep_format_flag equal to 1 specifies that only one representation format is present in the vps extension and that all layers referring to the corresponding vps extension share the same representation format. single_rep_format_flag equal to 0 specifies that the number of representation formats may or may not be greater than 0.

- Removal of cross_layer_irap_aligned_flag

  - If the value of cross_layer_irap_aligned_flag is dependent on the type of the scalability, it can be inferred from scalability_mask_flag syntax element. That is, the cross-layer IRAP alignment is required for multi-view extension, and is not essential for SHVC extension. Hence, the value of cross_layer_irap_aligned_flag can be inferred to be 1 for MV-HEVC (scalability_mask_flag[1] is equal to 1 and NumScalabilityTyps is equal to 1), and 0 for SHVC.

  - A variable CrossLayerIrapAlignedFlag can be introduced to store the inferred value.

- Removal of single_layer_for_non_irap_flag

  - Change of the highest nuh_layer_id value across AUs [1].

  - Where can a new CVS start ?

    - According to the current definition of CVS, the beginning of a new CVS should be an IRAP picture at the base layer (nuh_layer_id is equal to 0). Hence, when step-wise layer up-switching happens, a new CVS will always have to get back to the first layer (base layer) or the second layer from the bottom, as demonstrated in the figure.

(a) A new CVS begins from the base layer

(b) A new CVS begins from the second layer

Fig. 1 Beginning of a new CVS when single_layer_for_non_irap_flag is equal to 1

  - Any specific usage of the flag is also questioned. That is, it is not used for the sample reconstruction (by the decoder).

- Relocation of vps_vui_present_flag such that the vps_vui_offset is conditional on the presence flag.

  - No reason to waste 16 bits for the offset when vps_vui ( ) syntax structure is not present.

| vps_extension( ) { | Descriptor |
| --- | --- |
| …… |  |
| vps_vui_present_flag | u(1) |
| if ( vps_vui_present_flag ) |  |
| vps_vui_offset | u(16) |
| …... |  |
| } |  |

# Reference

- Y.-K. Wang et al., “MV-HEVC/SHVC HLS: On changing of the highest layer ID across Aus and multi-mode bitstream extraction,” the 14th JCTVC meeting, Vienna, AT, Jul. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).