| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0111 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0041 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: On VPS extension syntax element arrangement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjin Cho Byeongdoo Choi Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Tel: Email: | {yongjin9.cho, b.d.choi, m.w.park, jinyoung79, hc.wey, dionism}@samsung.com |
| Source: | Samsung Electronics, Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes to group the syntax elements of the vps extension into those necessary for decoding (sample reconstruction), and those for session negotiation. Also, it is proposed to have a presence flag and an offset to a group of side information, such that information unnecessary for sample reconstruction can be optional.

# Introduction

In the current vps extension, syntax elements not necessary on the decoder side are present to signal the characteristics of the corresponding bitstream. They are asserted to be used for session negotiation purpose etc. However, not all applications require session negotiation. For example, in a broadcasting application, no session negotiation process occurs between transmitters and receivers. In such applications, vps extension syntax elements for session negotiation information would result in bandwidth waste. Moreover, because there is no such process as session negotiation, it is important for video bitstreams for broadcasting applications to have parameter sets transmitted regularly. In the ATSC standard, it is described that AVC bit stream shall contain one SPS at each Access Point, which is defined as the point at which a decoder can begin decoding process successfully [1]. That is, it would be required to have parameter sets at every IRAP period in the HEVC context, and thus, transmitting unused data should be avoided. Hence, this contribution proposes to have such information optional in the vps extension.

# Proposal

## Allowing optional use of syntax elements for session negotiation

| vps_extension( ) { | Descriptor |
| --- | --- |
| vps_sni_present_flag | u(1) |
| if (vps_sni_present_flag) |  |
| vps_sni_offset | u(16) |
| syntax elements necessary for decoding (sample reconstruction) |  |
| … |  |
| if ( vps_sni_present_flag ) { |  |
| while (!byte_aligned()) |  |
| vps_sni_alignment_bit_equal_to_one | u(1) |
| vps_sni() |  |
| } |  |
| } |  |

vps_sni_present_flag equal to 1 specifies that the vps_sni ( ) syntax structure is present in the VPS. vps_sni_present_flag equal to 0 specifies that the vps_sni ( ) syntax structure is not present in the VPS.

vps_sni_offset specifies the byte offset, starting from the beginning of the VPS NAL unit, of the set of fixed-length coded information starting from avc_base_layer_flag, when present, in the VPS NAL unit. When present, emulation prevention bytes that appear in the VPS NAL unit are counted for purposes of byte offset identification.

vps_sni_alignment_bit_equal_to_one shall be equal to 1.

vps_extension_offset specifies the byte offset, starting from the beginning of the VPS NAL unit, of the next set of fixed-length coded information starting from avc_base_layer_flag vps_sni_present_flag, when present, in the VPS NAL unit. When present, emulation prevention bytes that appear in the VPS NAL unit are counted for purposes of byte offset identification.

| vps_sni ( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| // more side information syntax elements |  |
| } |  |

## Grouping of VPS extension syntax elements

Group 1: Syntax elements for sample reconstruction decoding

splitting_flag, scalability_mask_flag, dimension_id_len_minus1, vps_nuh_layer_id_present_flag, layer_id_in_nuh, dimension_id, view_id_len_minus1, view_id_val, direct_dependency_flag, max_tid_ref_present_flag, max_tid_il_ref_pics_plus1, all_ref_layers_active_flag, vps_num_layer_sets_minus1, more_output_layer_sets_than_default_flag, num_add_output_layer_sets_minus1, default_one_target_output_layer_flag, output_layer_set_idx_minus1, output_layer_flag, ref_format_idx_present_flag, vps_num_rep_formats_minus1, rep_format(), vps_rep_format_idx, max_one_active_ref_layer_flag

Group 2: Syntax elements for session negotiation etc.

avc_base_layer_flag, vps_vui_offset, vps_num_profile_tier_level_minus1, vps_profile_present_flag, profile_ref_minus1, profile_tier_level ( ), cross_layer_irap_aligned_flag, single_layer_for_non_irap_flag, vps_vui_present_flag, vps_vui_alignment_bit_equal_to_one, vps_vui ( )

# Reference

- Advanced Television Systems Committee, Inc., “A/72: Video System Characteristics of AVC in the ATSC Digital Television System,” Jul 2008

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).