| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0112 |
| --- | --- |

| Title: | AHG5: Extension of intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunil Lee Elena Alshina Chanyul Kim | Email: | sunil.lee@samsung.com elena_a.alshina@samsung.com dionism@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, a method to extend intra block copy is proposed. When the current CU to be predicted overlaps the reference CU, the prediction samples in the overlapped region are generated by copying the available samples from either vertical or horizontal direction. This reference sample generation method extends the range of valid motion vectors in intra block copy, and provides the coding gain of -0.8% in AI-MT, -0.6% in RA-MT, and -1.1% in LDB-MT for SC YUV 444, respectively. In the proposed method, intra block copy is further extended by applying the motion vector prediction from left CU coded in intra block copy mode. When this motion vector prediction is combined with the reference sample generation, the coding gain is -2.5% in AI-MT, -1.8% in RA-MT, and -2.0% in LDB-MT for SC YUV 444, respectively.

# Introduction

At the last JCT-VC meeting in Vienna, intra block copy was adopted [1]. As shown in Fig. 1, the prediction samples for the current CU are copied from the reference CU searched within the current and left CTU. One flag to indicate whether the current CU is coded in intra block copy mode and motion vector (MV) with integer-pel accuracy are signaled for intra block copy.

Figure 1. Intra block copy

Since motion estimation and compensation (ME/MC) should be performed in a same slice in intra block copy, only already reconstructed samples in the same slice can be used for the prediction, and this leads to the restriction of MV range. Fig. 2 shows an example of intra block copy in which the current CU to be predicted overlaps the reference CU. In this example, the reference samples in yellow region are not available yet and cannot be used for the prediction. In the current design of intra block copy, the MV referring to the reference CU in Fig. 2 is considered as an invalid MV.

Figure 2. Intra block copy - Invalid MV

In this contribution, a method to extend intra block copy is proposed. When the reference CU overlaps the current CU to be predicted, the samples in the overlapped region are generated by copying the available samples from either horizontal or vertical direction. This reference sample generation extends the range of valid MVs which can be used for intra block copy. In the proposed method, intra block copy is further extended by applying MV prediction from left CU coded in intra block copy mode.

Figure 3. Reference samples of the overlapped region

# Algorithm description

Let MVx, MVy, W, and H be horizontal and vertical components of the current MV, width and height of the current CU, respectively. The proposed reference sample generation method is invoked only when the following condition is satisfied,

( –W < MVx ≤ 0 ) AND ( –H < MVy ≤ 0 ).

When the reference sample generation process is invoked, the reference samples are generated by copying the available samples from either vertical or horizontal direction. Fig. 3 shows the reference samples of the MxN overlapped region in which M = H – | MVy |, N = W – | MVx |, and ri,j is the reference sample at the ith row and jth column. When | MVy | > | MVx |, the reference samples are generated by copying the reference samples of the bottommost row of the above CU,

ri,j=r−1,j   ,0≤i≤(M−1),0≤j≤(N−1).

Otherwise, when | MVy | ≤ | MVx |, the reference samples are generated by copying the reference samples of the rightmost column of the left CU,

ri,j=ri,−1   ,0≤i≤(M−1),0≤j≤(N−1).

The reference sample generation process is more intuitively described in Fig. 4.

Figure 4. Reference samples copied from either (a) left or (b) above CU

Figure 5. The range of the valid MVs: (a) in the current design of intra block copy, 
(b) after the proposed reference sample generation is applied

Fig. 5 shows how the range of the valid MVs in intra block copy can be extended. By applying the proposed reference sample generation method, the MVs whose components are within the range of ( –W < MVx ≤ 0 ) AND ( –H < MVy ≤ 0 ) can be now used without being declared as invalid MVs.

In this contribution, intra block copy is further extended by applying the MV prediction from the left CU coded in intra block copy mode. When left CU is available and coded in intra block copy mode, its MV is used as a predictor for the MV of the current CU, and MV difference is signaled. When left CU is not available or not coded in intra block copy mode, MV predictor is set to a zero vector. By combining the reference sample generation method and the MV prediction, the coding gain of intra block copy could be improved.

# Test results

The coding gain of the proposed method is evaluated using the common test condition of RCE2 and RCE3 [2]. Table 1 and 2 show the coding gain of the reference sample generation in lossy and lossless coding configurations, respectively. As shown in Table 1, the coding gain of the reference sample generation is -0.8% in AI-MT, -0.6% in RA-MT, and -1.1% in LB-MT for SC YUV 444, respectively. Table 2 shows that its coding gain in lossless coding is 0.5% in AI-Main, 0.4% in RA-Main, and 0.3% in LB-Main for SC YUV 444, respectively.

Table 1. Coding gain of the reference sample generation

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | -0.4% | -0.4% | -0.3% | -0.3% | -0.3% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.9% | -0.9% | -0.9% | -0.6% | -0.8% | -0.7% | -0.6% |
| Animation RGB | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.8% | -0.9% | -0.9% | -0.6% | -0.6% | -1.1% | -0.9% |
| Animation YUV | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR | -3.0% | -3.2% | -3.1% | -2.1% | -2.3% | 1.0% | 0.2% |
| SC(444) YUV | -1.7% | -1.7% | -1.7% | -2.0% | -1.9% | -2.5% | -1.9% |
| Enc Time[%] | 102% | 102% | 101% | 101% | 101% | 101% | 101% |
| Dec Time[%] | 96% | 96% | 96% | 102% | 102% | 105% | 105% |

Table 2. Coding gain of the reference sample generation in lossless coding

| Bit-rate saving (Average) | AI-Main | RA-Main | LB-Main |
| --- | --- | --- | --- |
| Class F | 0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.6% | 0.3% | 0.2% |
| Animation RGB | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.5% | 0.4% | 0.3% |
| Animation YUV | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC(444) GBR | 1.1% | 0.8% | 0.3% |
| SC(444) YUV | 1.3% | 1.4% | -0.1% |
| Enc Time[%] | 101% | 100% | 100% |
| Dec Time[%] | 98% | 99% | 100% |

Table 3 and 4 show the coding gain of the combination of the reference sample generation and MV prediction in lossy and lossless coding configurations, respectively. As shown in Table 3, the proposed method provides the coding gain of -2.5% in AI-MT, -1.8% in RA-MT, and -2.0% in LB-MT for SC YUV 444, respectively. Table 4 also shows that the combined coding gain in lossless coding is 0.8% in AI-Main, 0.7% in RA-Main, and 0.6% in LB-Main for SC YUV 444, respectively.

Table 3. Combined coding gain of the reference sample generation and MV prediction

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class F | -1.1% | -0.8% | -0.6% | -0.8% | -0.6% | -0.7% | -0.5% |
| Class B | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.9% | -1.7% | -1.5% | -1.4% | -1.3% | -1.3% | -1.2% |
| Animation RGB | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| SC YUV 444 | -2.5% | -2.1% | -1.8% | -1.8% | -1.5% | -2.0% | -2.0% |
| Animation YUV | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR | -6.3% | -6.3% | -5.9% | -5.1% | -5.0% | -2.4% | -3.4% |
| SC(444) YUV | -4.9% | -4.3% | -3.8% | -5.0% | -4.4% | -4.9% | -3.9% |
| Enc Time[%] | 103% | 102% | 102% | 101% | 101% | 101% | 101% |
| Dec Time[%] | 97% | 96% | 96% | 102% | 101% | 102% | 103% |

Table 4. Combined coding gain of the reference sample generation and MV prediction in lossless coding

| Bit-rate saving (Average) | AI-Main | RA-Main | LB-Main |
| --- | --- | --- | --- |
| Class F | 0.4% | 0.3% | 0.3% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.9% | 0.7% | 0.4% |
| Animation RGB | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.8% | 0.7% | 0.6% |
| Animation YUV | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC(444) GBR | 1.8% | 1.5% | 0.7% |
| SC(444) YUV | 2.5% | 2.5% | 0.8% |
| Enc Time[%] | 101% | 100% | 101% |
| Dec Time[%] | 99% | 100% | 101% |

# Conclusion

In this contribution, the extension of intra block copy is proposed. The proposed extension includes the reference sample generation and MV prediction from left CU. The combined coding gain for lossy coding is -2.5% in AI-MT, -1.8% in RA-MT, and -2.0% in LDB-MT for SC YUV 444, respectively. It is suggested that the proposed method is adopted into the next version of the HEVC Range Extensions text and reference software.

# Proposed text changes

7.4.9.9	Motion vector difference semantics

…

The variable BvdIntra[ x0 ][ y0 ][ compIdx ] specifies the difference between a vector component to be used for the intra block copying prediction mode and its prediction. The value of BvdIntra[ x0 ][ y0 ] shall be in the range of −215 to 215 − 1, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 1), MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 2), BvdIntra[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

8.4.1	General decoding process for coding units coded in intra prediction mode

…

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a variable log2CbSize specifying the size of the current luma coding block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The derivation process for quantization parameters as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

A variable nCbS is set equal to 1  <<  log2CbSize.

The derivation process for motion vector components of intra block copoy specified in subcluase 8.4.5 is invoked with the luma coding block location ( xCb, yCb ) as an input, and the motion vector BvIntra for intra block copy as an output.

Depending on the values of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for luma samples is specified as follows:

…

8.4.4.2.7	Specification of intra block copying prediction mode

Inputs to this process are:

–	a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	a variable nTbS specifying the transform block size,

–	a variable bvIntra specifying the block copying vector,

–	a variable cIdx specifying the colour component of the current block.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The variable bv representing the block vector for prediction in full-sample units is derived as follows:

bv[ 0 ] = bvIntra[ 0 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubWidthC ) − 1 )	(8-1)

bv[ 1 ] = bvIntra[ 1 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubHeightC ) − 1 )	(8-2)

The (nTbS)x(nTbS) array of predicted samples samples, with x, y = 0..nTbS − 1, are derived as follows:

–	The temporary reference sample location ( xRefCmpTmp, yRefCmpTmp ) is specified by:

( xRefCmpTmp, yRefCmpTmp ) = ( xTbCmp + x + bv[ 0 ], yTbCmp + y + bv[ 1 ] )	(8-3)

–	The variable intraBcExtFlag is derived as follows

–  If all of the following conditions are true, intraBcExtFlag is set equal to 1:

–  xRefCmpTmp is greater than or equal to xTbCmp

–  xRefCmpTmp is less than xTbCmp + nTbS

–  yRefCmpTmp is greater than or equal to yTbCmp

–  yRefCmpTmp is less than yTbCmp + nTbS

–  Otherwise, intraBcExtFlag is set equal to 0.

–	If intraBcExtFlag is equal to 1, the reference sample location ( xRefCmp, yRefCmp ) is specified by:

( xRefCmp, yRefCmp ) = ( Abs( bv[ 1 ] ) > Abs ( bv[ 0 ] ) ) ? ( xTbRefCmpTmp, yTbCmp - 1 ) : ( xTbCmp -1, yTbRefCmpTmp ) 		(8-66)

–	Otherwise (intraBcExtFlag is equal to 0), the reference sample location ( xRefCmp, yRefCmp ) is specified by:

( xRefCmp, yRefCmp ) = ( xTbRefCmpTmp, yTbRefCmpTmp )	(8-67)

- Each sample at the location ( xRefCmp, yRefCmp ) is assigned to predSamples[ x ][ y ].

8.4.5	Derivation process for motion vector components of intra block copy

- Input to this process is a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Output of this process is a motion vector BvIntra for intra block copy.

The location ( xNbL, yNbL ) is set equal to ( xCb − 1, yCb ) and the variable availableL, specifying the availability of the block located directly to the left of the current block, is derived by invoking the availability derivation process for a block in z-scan order as specified in subclause Error: Reference source not found with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbL, yNbL ) as inputs, and the output is assigned to availableL.

For the derivation of the variable BvIntram the following applies:

- If all of the following conditions are true, BvIntra [ xCb ][ yCb ] = BvIntra [ xNbL ][ yNbL ] + BvdIntra[ xCb ][ yCb ]

- availableL is equal to TRUE,

- intra_bc_flag[ xNbL ][ yNbL ] is equal to 1.

- Otherwise, BvIntra [ xCb ][ yCb ] = BvdIntra[ xCb ][ yCb ].

# References

- C. Pang, J. Sole, L. Guo, M. Karczewicz, R. Joshi, “Non-RCE3: Intra motion compensation with 2-D MVs,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0256, Jul. 2013.

- A. Saxena, D. Kwon, M. Naccari, C. Pang, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques,” Document of Joint Collaborative Team on Video Coding, JCTVC-N1123, Jul. 2013.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).