| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0113 |
| --- | --- |

| Title: | AHG5: On context modelling for intra block copy mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

At the 14th JCT-VC meeting, the adoption of JCTVC-N0256 introduced the ‘intra block copy’ mode into HEVC Range extensions.  This feature is signaled at the coding unit level using intra_bc_flag.  This flag uses three contexts, selected using above and left neighbouring flags. This proposal removes two contexts and the dependency on the neighbouring flags. The results are as follows: class F main tier was 0.1% luma loss in MT under AI and 0.0% for HT and SHT of AI and 0.1% for MT and HT of RA and LB configs. Class B was 0.0% loss in luma under all configs and tiers. The worst case loss observed as 0.6% in luma for MT under the LB config with SC RGB444, due to one sequence ‘sc_video_conferencing_doc_sharing_1280x720_30_8bit_300_rgb.rgb’.

# Introduction

At the 14th JCT-VC meeting, the adoption of JCTVC-N0256 introduced the ‘intra block copy’ mode into HEVC Range extensions.  This feature is signaled at the coding unit level using intra_bc_flag.  This flag uses three contexts, selected using above and left neighbouring flags. This proposal removes two contexts and the dependency on the neighbouring flags. The results are as follows.

# Description of the proposed change

In HM-12.0-RExt4.0, intra_bc_flag uses three contexts. These are selected according to the following formula (see table 9-40 of [2]):

( condL  &&  availableL ) + ( condA  &&  availableA )

The ‘condA’ dependency introduces a requirement for a line buffer of intra_bc_flag values for coding units from the above row of coding tree blocks, e.g. approximately 500 flags for a 4K2K frame size.

In this contribution, dependencies on the left and above coding units for selecting the context for intra_bc_flag are removed.  Consequently, only one context is used by this flag.

# Experiment Results

Testing conditions are defined in [3] and [4].

Table 1 and Table 2 show the results for using one context for intra_bc_flag under RCE2/3 lossy and lossless conditions respectively.

Table 1. Context reduction results under RCE2/3 lossy conditions with HM-12.0-RExt4.1 as anchor.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% |
| SC(444) YUV Optional | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 97% |  |  | 96% |  |  | 96% |  |  |
| Dec Time[%] | 91% |  |  | 91% |  |  | 90% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | 0.1% | 0.0% | 0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | 0.1% | 0.0% | -0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC(444) YUV Optional | -0.2% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.1% | -0.6% | -0.7% | 0.1% | 0.0% | -0.4% |  |  |  |
| Class B | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | 0.6% | 0.6% | 0.6% | 0.4% | 0.5% | 0.3% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.2% | -0.2% | -0.4% | -0.2% | -0.2% | -0.3% |  |  |  |
| Animation YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | -0.6% | -0.5% | -0.6% | 0.2% | 0.3% | 0.3% |  |  |  |
| SC(444) YUV Optional | -0.6% | -0.4% | -0.5% | 0.0% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 103% |  |  | 104% |  |  |  |  |  |

Table 2. Context reduction results under RCE2/3 lossless conditions with HM-12.0-RExt4.1 as anchor.

|  | AI Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 4.57 | 4.57 | 5.57 | 5.57 | 2.27 | 2.27 | 11.14 | 11.14 | 0.0% | 0.0% | -0.1% | 0.0% |
| Class B | 2.24 | 2.24 | 2.26 | 2.26 | 2.08 | 2.08 | 2.44 | 2.44 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 7.88 | 7.88 | 9.5 | 9.51 | 5.23 | 5.22 | 14.73 | 14.73 | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation RGB 444 | 2.48 | 2.48 | 2.5 | 2.52 | 2.15 | 2.15 | 3.05 | 3.05 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 11.09 | 11.09 | 13.2 | 13.22 | 7.87 | 7.86 | 19.32 | 19.32 | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation YUV 444 | 3.00 | 3.00 | 3.2 | 3.16 | 2.57 | 2.57 | 3.93 | 3.93 | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 1.92 | 1.92 | 2.38 | 2.38 | 1.46 | 1.46 | 4.37 | 4.37 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 20.30 | 20.29 | 24.82 | 24.81 | 9.69 | 9.69 | 36.10 | 36.07 | 0.0% | 0.0% | -0.1% | 0.0% |
| SC YUV 444 Optional | 31.85 | 31.83 | 34.39 | 34.37 | 22.30 | 22.30 | 51.97 | 51.93 | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] |  |  | 100% |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  | 100% |  |  |  |  |  |  |  |  |  |

|  | RA Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.66 | 8.66 | 36.22 | 36.22 | 3.01 | 3.01 | 90.48 | 90.43 | 0.0% | 0.0% | -0.1% | 0.1% |
| Class B | 2.60 | 2.60 | 2.60 | 2.60 | 2.57 | 2.57 | 2.64 | 2.64 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 53.71 | 53.69 | ##### | 116.92 | 19.48 | 19.48 | 225.68 | 225.52 | 0.0% | -0.1% | -0.3% | 0.0% |
| Animation RGB 444 | 3.58 | 3.58 | 3.60 | 3.60 | 3.43 | 3.43 | 3.70 | 3.70 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 73.79 | 73.78 | ##### | 157.04 | 24.42 | 24.42 | 287.13 | 287.03 | 0.0% | 0.0% | -0.1% | 0.1% |
| Animation YUV 444 | 3.8 | 3.84 | 4.48 | 4.48 | 2.75 | 2.75 | 5.70 | 5.70 | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 2.1 | 2.1 | 2.5 | 2.5 | 1.5 | 1.53 | 4.41 | 4.41 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 42.0 | 42.0 | 254.1 | 254.0 | 11.5 | 11.45 | 395.52 | 395.35 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 Optional | 101.1 | 101.0 | 314.9 | 314.7 | 30.5 | 30.45 | 531.53 | 531.17 | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |

|  | LB Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.9 | 8.9 | 57.8 | 57.7 | 3.0 | 3.0 | 169.5 | 169.2 | 0.0% | 0.0% | -0.1% | 0.0% |
| Class B | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 62.2 | 62.2 | 355.6 | 355.4 | 20.5 | 20.5 | 1618.0 | 1617.4 | 0.0% | 0.0% | -0.2% | 0.0% |
| Animation RGB 444 | 3.6 | 3.6 | 3.6 | 3.6 | 3.4 | 3.4 | 3.7 | 3.7 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 85.1 | 85.1 | 469.0 | 468.9 | 25.7 | 25.6 | 2105.0 | 2104.5 | 0.0% | 0.0% | -0.1% | 0.0% |
| Animation YUV 444 | 4 | 3.9 | 4.5 | 4.5 | 2.8 | 2.8 | 5.8 | 5.8 | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 2.1 | 2.1 | 2.5 | 2.5 | 1.5 | 1.5 | 4.4 | 4.4 | 0.0% | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 45 | 45.0 | ##### | 1007.7 | 11.6 | 11.6 | 2253.7 | 2247.7 | 0.0% | -0.2% | -0.4% | 0.0% |
| SC YUV 444 Optional | 117.1 | 117.0 | ##### | 1138.9 | 31.0 | 31.0 | 2208.6 | 2208.8 | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 99% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  |  |  |  |  |  |  |  |

# Working Draft changes

Working draft text changes are on top of [2].

9.3.2.2	Initialization process for context variables

Table 9-3 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| coding_unit( ) | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | intra_bc_flag[ ][ ] | Table 9-4 | 0..2 | 3..51 | 6..82 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 | 1..4 | 5..8 |
|  | prev_intra_luma_pred_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | rqt_root_cbf | Error: Reference source not found |  | 0 | 1 |

Table 9-4 – Values of initValue for ctxIdx of intra_bc_flag

| Initialization variable | ctxIdx of intra_bc_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 185 | 185197 | 201197 | 197 | 197 | 185 | 197 | 197 | 185 |

##### 9.3.4.2.2	Derivation process of ctxInc using left and above syntax elements

Table 9-5 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxInc |
| --- | --- | --- | --- |
| split_cu_flag[ x0 ][ y0 ] | CtDepth[ xNbL ][ yNbL ] > cqtDepth | CtDepth[ xNbA ][ yNbA ] > cqtDepth | ( condL && availableL ) + ( condA && availableA ) |
| cu_skip_flag[ x0 ][ y0 ] | cu_skip_flag[ xNbL ][ yNbL ] | cu_skip_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| intra_bc_flag[ x0 ][ y0 ] | intra_bc_flag[ xNbL ][ yNbL ] | intra_bc_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |

# Conclusion

This contribution proposes a simplified binarisation for intra_bc_flag. This simplified binarisation results in the removal of two contexts and a removal of dependencies on neighbouring flag values.

# References

- C. Pang, J. Sole, L. Guo, M. Karczewicz, R. Joshi “Non-RCE3: Intra Motion Compensation with 2-D MVs”, Joint Collaborative Team on Video Coding, JCTVC-N0256, Vienna, AT, July 2013.

- D. Flynn, J. Sole, T. Suzuki “Range Extensions Draft 4”, Joint Collaborative Team on Video Coding, JCTVC-N1005, Vienna, AT, July 2013.

- C. Rosewarne, J. Sole, K. Sharman, S. Kim, “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialisation and update methods,” JCTVC-N1122, 14th JCT-VC Meeting, Vienna, AT, July 2013.

- A. Saxena, D. Kwon, M. Naccari, C. Pang, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques,” JCTVC-N1123, 14th JCT-VC Meeting, Vienna, AT, July 2013.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).