| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0114 |
| --- | --- |

| Title: | RCE2: Test B2. Golomb-rice parameter initialization for transform-skip and transquant-bypass modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | V. Kolesnikov, C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 vladimir.kolesnikov@cisra.canon.com.au chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

RCE2 of the 14th JCT-VC meeting included a number of sub-tests relating to entropy coding of the residual for transform skip and transquant bypass blocks. In particular, the update formula for the golomb-rice parameter is modified, which is applied once per instance of coef_abs_level_remaining (up to 16 instances per sub-block).  This contribution proposes to modify the initialisation of the golomb-rice parameter, which is applied once per sub-block.

# Introduction

During the Incheon meeting proposals were submitted to address the issue of efficient entropy modelling of mixed content. In proposals [1] and [2] the Golomb-Rice parameter (cRiceParam) update mechanism was modified for the transform-skip and transquant-bypass modes.

This contribution proposes a different approach whereby the initial value is modified for this blocks, while the existing update mechanism is retained.

# Description of the proposed change

This contribution proposes to use the following formula to initialize cRiceParam prior to coding each sub-block:

cRiceParam = 2, if any of transform-skip or transquant-bypass modes is active and the number of greater-than-one coefficients in the current sub-set is equal to or greater than 8;

cRiceParam = 0, otherwise.

The condition on the number of greater-than-one coefficients is already present in the design of residual coefficient coding (for the ‘high throughput binarisation’ of coeff_abs_level_greater1_flag) and therefore the existing condition check may be used for this new purpose.

# Experiment Results

Table 1 and Table 2 provide results for RCE2 Test 21 under AHG8/RCE2/3 lossy and lossless conditions respectively (as defined in [2] and [3]). Table 3 and Table 4 provide results for RCE2 Tes B21 under AHG18 conditions, with the HM forward transform and the high precision forward transform respectively.

Table 1. RCE2 Test B2 results under RCE2/3 lossy conditions.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% | -0.4% | -0.4% | -0.5% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | -0.2% | -0.1% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -0.1% | -0.1% | -0.1% | -0.3% | -0.2% | -0.3% | -0.5% | -0.5% | -0.6% |
| SC(444) YUV Optional | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.3% | -0.4% | -0.4% |
| Enc Time[%] | 102% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.04% | 0.00% | -0.09% | -0.08% | -0.03% | -0.05% |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 0.01% | 0.00% | -0.01% |  |  |  |
| SC RGB 444 | 0.10% | 0.07% | 0.10% | -0.15% | -0.21% | -0.19% |  |  |  |
| Animation RGB 444 | 0.03% | 0.03% | 0.02% | 0.02% | 0.03% | 0.02% |  |  |  |
| SC YUV 444 | 0.09% | 0.07% | 0.08% | 0.11% | 0.06% | 0.02% |  |  |  |
| Animation YUV 444 | 0.00% | 0.00% | -0.01% | 0.00% | -0.01% | -0.02% |  |  |  |
| RangeExt | 0.01% | 0.05% | -0.03% | 0.00% | 0.04% | -0.05% |  |  |  |
| SC(444) GBR Optional | 0.00% | -0.01% | 0.01% | -0.11% | -0.10% | -0.08% |  |  |  |
| SC(444) YUV Optional | -0.07% | -0.06% | -0.12% | -0.10% | -0.11% | -0.07% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.6% | -0.4% | 0.0% | -0.4% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.1% | 0.0% | -0.2% | -0.1% | 0.0% | -0.2% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | 1.5% | 1.3% | 1.2% | 0.9% | 0.8% | 0.6% |  |  |  |
| SC(444) YUV Optional | -0.4% | -0.3% | -0.5% | 0.0% | 0.2% | -0.2% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 95% |  |  |  |  |  |

Table 2. RCE2 Test B2 results under RCE2/3 lossless conditions.

|  | AI Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 4.57 | 4.59 | 5.57 | 5.60 | 2.27 | 2.27 | 11.14 | 11.19 | 0.4% | 0.4% | 0.2% | 0.6% |
| Class B | 2.24 | 2.25 | 2.26 | 2.26 | 2.08 | 2.08 | 2.44 | 2.44 | 0.0% | 0.0% | -0.1% | 0.1% |
| SC RGB 444 | 7.88 | 7.99 | 9.5 | 9.64 | 5.23 | 5.32 | 14.73 | 14.94 | 1.4% | 1.3% | 0.6% | 1.8% |
| Animation RGB 444 | 2.48 | 2.48 | 2.5 | 2.52 | 2.15 | 2.15 | 3.05 | 3.05 | 0.1% | 0.0% | -0.1% | 0.2% |
| SC YUV 444 | 11.09 | 11.18 | 13.2 | 13.34 | 7.87 | 7.93 | 19.32 | 19.58 | 0.8% | 0.9% | 0.3% | 1.4% |
| Animation YUV 444 | 3.00 | 3.00 | 3.2 | 3.16 | 2.57 | 2.56 | 3.93 | 3.93 | 0.0% | 0.0% | -0.1% | 0.0% |
| RangeExt | 1.92 | 1.96 | 2.38 | 2.43 | 1.46 | 1.51 | 4.37 | 4.44 | 2.3% | 2.1% | 0.6% | 3.3% |
| SC GBR 444 Optional | 20.30 | 20.44 | 24.82 | 25.05 | 9.69 | 9.73 | 36.10 | 36.32 | 0.7% | 0.8% | 0.4% | 1.4% |
| SC YUV 444 Optional | 31.85 | 32.19 | 34.39 | 34.71 | 22.30 | 22.46 | 51.97 | 52.28 | 1.1% | 1.0% | 0.6% | 1.6% |
| Enc Time[%] |  |  | 103% |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  | 101% |  |  |  |  |  |  |  |  |  |

|  | RA Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.66 | 8.66 | 36.22 | 36.33 | 3.01 | 3.01 | 90.48 | 90.82 | 0.0% | 0.2% | 0.0% | 0.4% |
| Class B | 2.60 | 2.60 | 2.60 | 2.60 | 2.57 | 2.57 | 2.64 | 2.64 | -0.1% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | 53.71 | 54.15 | 117.01 | 117.89 | 19.48 | 19.66 | 225.68 | 225.84 | 0.8% | 0.8% | 0.1% | 1.4% |
| Animation RGB 444 | 3.58 | 3.58 | 3.60 | 3.59 | 3.43 | 3.43 | 3.70 | 3.70 | 0.0% | 0.0% | -0.1% | 0.0% |
| SC YUV 444 | 73.79 | 74.20 | 157.08 | 158.67 | 24.42 | 24.53 | 287.13 | 290.97 | 0.6% | 0.8% | 0.1% | 1.7% |
| Animation YUV 444 | 3.8 | 3.83 | 4.48 | 4.48 | 2.75 | 2.75 | 5.70 | 5.70 | -0.1% | -0.1% | -0.2% | 0.0% |
| RangeExt | 2.1 | 2.1 | 2.5 | 2.6 | 1.5 | 1.57 | 4.41 | 4.48 | 1.6% | 1.6% | -0.1% | 2.8% |
| SC GBR 444 Optional | 42.0 | 42.1 | 254.1 | 256.4 | 11.5 | 11.48 | 395.52 | 401.03 | 0.3% | 0.7% | 0.3% | 1.4% |
| SC YUV 444 Optional | 101.1 | 101.4 | 314.9 | 317.5 | 30.5 | 30.53 | 531.53 | 533.84 | 0.4% | 0.7% | 0.2% | 1.4% |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |

|  | LB Main |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | compression ratio (Total) |  | compression ratio (Average) |  | compression ratio (min) |  | compression ratio (max) |  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) |
|  | Ref. | Tested | Ref. | Tested | Ref. | Tested | Ref. | Tested |  |  |  |  |
| Class F | 8.9 | 8.9 | 57.8 | 57.9 | 3.0 | 3.0 | 169.5 | 169.8 | 0.0% | 0.1% | 0.0% | 0.2% |
| Class B | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | 2.6 | -0.1% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | 62.2 | 62.7 | 355.6 | 359.5 | 20.5 | 20.7 | 1618.0 | 1637.2 | 0.8% | 0.8% | 0.2% | 1.2% |
| Animation RGB 444 | 3.6 | 3.6 | 3.6 | 3.6 | 3.4 | 3.4 | 3.7 | 3.7 | 0.0% | 0.0% | -0.1% | 0.0% |
| SC YUV 444 | 85.1 | 85.5 | 469.0 | 473.9 | 25.7 | 25.8 | 2105.0 | 2132.2 | 0.4% | 0.6% | 0.1% | 1.3% |
| Animation YUV 444 | 4 | 3.9 | 4.5 | 4.5 | 2.8 | 2.7 | 5.8 | 5.8 | -0.2% | -0.1% | -0.3% | 0.0% |
| RangeExt | 2.1 | 2.1 | 2.5 | 2.6 | 1.5 | 1.6 | 4.4 | 4.5 | 1.6% | 1.6% | -0.1% | 2.8% |
| SC GBR 444 Optional | 45 | 45.2 | 1010.8 | 1040.0 | 11.6 | 11.6 | 2253.7 | 2346.9 | 0.3% | 1.2% | -0.8% | 4.0% |
| SC YUV 444 Optional | 117.1 | 117.2 | 1139.4 | 1116.9 | 31.0 | 31.1 | 2208.6 | 2138.2 | 0.1% | -0.9% | -3.3% | 0.2% |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |  |

Table 3. RCE2 Test B2 simulation results under AHG18 conditions with the HM forward transform.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -2.0% | -1.7% | -1.7% |
| 14-bit | -3.3% | -3.3% | -3.1% |
| 16-bit | -2.9% | -3.0% | -3.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

Table 4. RCE2 Test B2 simulation results under AHG18 conditions with the high precision forward transform.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| 12-bit | -1.3% | -1.2% | -1.2% |
| 14-bit | -1.6% | -1.6% | -1.6% |
| 16-bit | -1.1% | -1.1% | -1.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

# Conclusion

This contribution presents an approach to the Rice parameter adaptation whereby the initialisation is modified for transform skip and transquant bypass blocks, while leaving the update mechanism unchanged.

# References

- V. Kolesnikov, C. Rosewarne, M. Maeda, “Non-RCE2: Golomb-rice parameter initialization for transform-skip and transquant-bypass modes,”, Joint Collaborative Team on Video Coding, JCTVC-N0137, Vienna, AT, July 2013.

- Chris Rosewarne, Joel Sole, Karl Sharman, S.-H. Kim “HEVC Range Extensions Core Experiment 2 (RCE2): Rice parameter initialisation and update methods”, Joint Collaborative Team on Video Coding, JCTVC-N1122, Vienna, AT, 25 July – 2 Aug. 2013.

- A. Saxena, D. Kwon, M. Naccari, C. Pang, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques,” JCTVC-N1123, 14th JCT-VC Meeting, Vienna, AT, July 2013.

# Patent rights declaration(s)

Canon Information Systems Research Australia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).