| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0121 |
| --- | --- |

| Title: | SHVC: On Inter Layer Reference frame and motion derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Gisquet, G. Laroche, P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

The ILR is a frame inserted into a reference list and that can be used for both texture and motion prediction. In the case of (at least) 3 cascaded layers, the motion of the second layer may contain motion pointing to said ILR frames. As a result, the third layer may use for motion prediction that kind of motion, that is asserted to be inefficient for motion prediction.

# Introduction

In SHVC, interlayer prediction is performed through the use of an Inter-Layer Reference frame obtained through upsampling of a collocated frame in a reference layer.

In JCTVC-N0252 and JCTVC-M0256, it was proposed to use another reference frame from the current layer, and in particular, to replace motion referring the Reference Layer sample data (i.e. the ILR frame) by Reference Layer motion. Indeed, this motion is usually null so as to select the collocated position in the Reference Layer, and the SHM encoder software never tests any other motion. This kind of motion is particularly unsuited for motion prediction and may affect negatively the performance of the Merge mode.

This issue of using motion referring ILR frames for motion prediction is however not limited to that scenario. Cases can be made for both SHVC and MV-HEVC.

# Motion syntax prediction in case of inserted reference frames

## SHVC case

The coding efficiency loss for SHVC is difficult to estimate because of the following:

- It is not clear if the SHM software correctly supports more than 2 layers, as the maximum number of layers (MAX_LAYERS) is set to 2;

- There is no CTC and no sequences for tests using more than 2 layers;

- It is unclear if the upsampling filters for sample data are adapted to this situation.

### Situation

The issue arises as soon as 3 layers are cascaded, the second one using (at least) sample prediction (i.e. SamplePredEnabledFlag[1][0] is equal to 1) and the third using (at least) motion prediction from the second layer (i.e. MotionPredEnabledFlag[2][1] is equal to 1). The processes of resampling sample values and picture motion field is described in section 8.1.4 of the specification.

Figure 1 – Using SHVC to code 3 cascaded layers

We start with the L0 layer. Once an image at time t2 has been encoded, the encoding of the corresponding frame in the L1 layer (at time t2) requires the upsampling of the decoded information of said frame in the L0 layer. This information is put into the ILR frame of the L1 layer, and may consist of samples values and motion information. The L1 image may then access the ILR frame through a reference list, containing other frames (here, frames already coded at times t0 and t1, as represented by the dashed lines).

When encoding e.g. the blue block of the L1 layer, the ILR frame may be used to predict it. Therefore, the null MV and the ref_idx of the ILR frame will be used to signal this.

Again, when encoding the frame from layer L2 at time t2, an ILR frame may be obtained from upsampling (where our invention can be located) of the collocated frame in layer L1, and placed in the reference list for said frame in layer L2. However, when encoding the gray block in that frame, motion prediction may use the collocated position from the ILR frame of current frame.

Therefore, the gray block may use motion in the orange block, which is upsampled from blue block of layer L1. As a consequence, it contains the null MV, and this renders the derived candidate useless as it does not constitute an actual motion.

### Potential solution

One possibility is to mark the motion pointing to an ILR frame as unavailable, i.e. for a given list LX and according to section H.8.1.4.2, predFlagLX is forced to be 0 and refIdxLX is forced to be -1. To detect whether a frame is an ILR frame, one can have additional input to the process in the shape of:

- Either POC values (PicOrderCntVal) of current frame and those from the RL:

  - For a given value of refIdxLXRL, if the corresponding value of PicOrderCntVal for the corresponding RL frame is equal to the current frame, then the frame is an ILR

- Or the value of num_ref_idx_lx_minus1 for the RL: if refIdxLXRL > num_ref_idx_lx_minus1, then the frame is an ILR

The later was chosen to illustrate a potential modification to the specification:

H.8.1.3.2.1	Derivation process for inter layer motion parameters

Inputs to this process are

–	the number of reference frames in each reference list of the RL frame, num_ref_idx_l0_minus1RL and num_ref_idx_l1_minus1RL

[…]

- The variables mvL0[ xP ][ yP ], mvL1[ xP ][ yP ], refIdxL0[ xP ][ yP ], refIdxL1[ xP ][ yP ], predFlagL0[ xP ][ yP ] and predFlagL1[ xP ][ yP ] are derived as follows:

  - If predMode[ xP ][ yP ] is equal to MODE_INTER, the following applies

- The variable colPb specifies the luma prediction block covering the location given by ( xRL, yRL ) inside the reference layer picture specified by rlPic. The 16 x 16 coding block with top-left luma location ( xP, yP ) relative to the top-left luma sample of the resampled picture is set associated with the slice containing the prediction block colPb inside the reference layer picture specified by rlPic.

- For each X = 0, 1, the following applies:

        - The variables refIdxLX[ xP ][ yP ] and predFlagLX[ xP ][ yP ] are derived as follows:
if refIdxLXRL[ xRL ][ yRL ] > num_ref_idx_lx_minus1RL then
    refIdxLX[ xP ][ yP ] = -1
    predFlagLX[ xP ][ yP ] = 0
else
    refIdxLX[ xP ][ yP ] = refIdxLXRL[ xRL ][ yRL ]
    predFlagLX[ xP ][ yP ] = predFlagLXRL[ xRL ][ yRL ]

## MV-HEVC case

The case has been studied to a lesser extend so no solution is proposed, only an illustration of the issue is given:

Figure 2 – Using MV-HEVC to code 3 cascaded views

For a given image of View 1, the collocated image from View 0 is inserted in the reference lists for said image of View 1. Therefore, in View 1, the block B1 may use the content of block B0 in View 0 for sample prediction. The relative positions of B1 and B0 are related to the disparity between the views, but do not constitute an actual motion. Therefore, the B1 block motion consists in a particular MV and the ref_idx of the inserted image from View 0.

Now for a given image of View 2, the collocated image from View 1 may be inserted in the reference lists for said image of View 2. Therefore, to encode the motion of a block B2 of View 2, motion from View 1 may be used. But in that case, block B1 again contains unnatural motion (related to e.g. the disparity between Views 0 and 1) and is likely to be unsuited for accurate prediction of motion.

# Conclusion

An issue with the reuse of motion pointing to interlayer/interview pictures has been raised when several layers are used. We recommend the adoption of the very simple solution consisting in not using motion information pointing on an ILR frame for further motion prediction in upper layer.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).