| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0123r2 |
| --- | --- |

| Title: | AhG5: Vector transformation for Intra Block Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Gisquet, G. Laroche, P. Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

During the previous JCTVC meeting in Geneva, several proposals around the Intra Block copy method were made, using various methods to encode the displacement vector. It was however preferred to reuse the MVD entropy coding. In this proposal is presented a simple transformation of the vector components to better suit the properties of the entropy coder. It is reported that the technique yields bitrate savings for lossless coding of SCC classes from 0.1% to 0.6% on AI Main, 0.1% to 0.5% on RA Main, and 0.0% to 0.5% on LDB Main. Gains of up to respectively 2.4%, 2.0% and 2.0% for lossy coding of respectively AI, RA and LDB are also reported.

# Introduction

Intra Block Copy uses a causal area as source for the prediction block, which is simply a copy of pixels from said area. An encoder limitation restricts the search window to the current CTB and its 2 neighbors on the left. From this situation we can already define simple restrictions on the position:

Figure 1 – position restriction

Indeed, the red block overlaps the block to code, and therefore can’t be used for predicting the current CU. This happens whenever, for a block of size NxN, MVx>-N and MVy>-N. This was acknowledged by providing a fast encoder search, which also takes into account the Z-scan coding order of the CUs as follows:

Figure 2 – encoder search window

Given the CU 6 to encode, all greyed CUs are not yet available and cannot be used for prediction. As a consequence, the red line representing the bottom-right part of the fast search window has been defined.

# Proposal

## Description

Using the knowledge of this “impossible” area when coding the vector can improve coding efficiency. However, there are 2 important things that we focused on:

- The entropy coding shall not be modified;

- The change should not adversely affect the decoding, and should be very simple.

Our proposal is therefore to use the knowledge of the most obvious and simplest area below:

Figure 3 – allowed area taken into account

In this case, for a block of size NxN, the rule is simply that any vector MV whose components lead to MVx+N > 0 and MVy+N > 0 is unlikely. Therefore, if MVy+N > 0, then MVx < -N. We can therefore simply derive this transformation: if MVy+N > 0, then MVx += P, P being some prediction value.

The most obvious value of P is N. However, because HEVC MVD coding is used, -MVx consumes as many bits as MVx: it is interesting to define P so as to recenter the occurrences of MVx+P around 0, or rather minimizing the overall magnitude of MVx+P.

Figure 4 – example of conditional occurrences

Taking into account, we have found P=2N to be the best solution. Indeed, it may happen that if MVx<-2N, the CU of lower depth (and thus size 2Nx2N) might be selected instead, therefore placing a higher bound on P.

Applying to the vertical component instead of the horizontal one yielded worse results. Although this may be due to the search window, it can be expected that most encoders will restrict the vertical search size, still justifying the choice of predicting the horizontal component.

Further investigations on how to apply better the knowledge of the forbidden area did not yield much improvement, and did not justify the increased complexity.

## Proposed specification modification

The variable BvIntra[ x0 ][ y0 ][ compIdx ] specifies the vector used for the intra block copying prediction mode.  The value of BvIntra[ x0 ][ y0 ] shall be in the range of 0 to 128, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 1), MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 2):

  - If lMvd[ 1 ] + nS > 0, then BvIntra[ x0 ][ y0 ][ 0 ] is set to lMvd[ 0 ] - 2*nS and BvIntra[ x0 ][ y0 ][ 1 ] is set to lMvd[ 1 ].

  - Otherwise BvIntra[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

# Results

## Lossless CTC

Positive values are gains.

## Lossy CTC

Negative values are gains.

# Conclusion

By modifying the vector components to code for intra block copy, bitrate savings of SCC classes are reported to be up  to 0.5% on lossless coding and up to 2.4% on lossy coding. In addition, the required modification is asserted to be both a small change specification-wise and implementation-wise, so it is recommended to adopt this change into the test model and the specifications.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).