| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0124 |
| --- | --- |

| Title: | SCE1: Cross-check of JCTVC-O0076 on |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | P. Onno Canon Research France | Tel: Email: | patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This document provides the cross-check results of the proposal JCTVC-O0076 related to an alternative variant of 6/16-pel Luma up-sampling filter for arbitrary spatial scalability support. An independent implementation of the proposal has been made by the cross-checker. All the BD-rate results as well the complexity assessment numbers obtained from this independent implementation are reported to match exactly those provided by the proponent. Almost no difference compared to the anchor results (generated from SHM3.0.1 with macro ARBITRARY_SPATIAL_RATIO set on) is observed. Complexity assessment numbers also show no difference in complexity between the reference and the proposal.

# Introduction

JCTVC-O0076 relates to upsampling filters for spatial scalability with any ratio between 1x and 2x.

Reference filters recommended in JCTVC-N0375 are integrated in the SHM3.0.1 (enabled by macro ARBITRARY_SPATIAL_RATIO).

Table 1 and Table 2 provide the 16-phases upsampling filters for Luma and Chroma for the reference (used as anchor, cf JCTVC-N0375), and for the proposal. Filters for phase p > 8 are not depicted since they are the symmetrical version of filters of phase (8-p).

Filters from the proposal differing from the reference filters are highlighted in yellow. The proposal changes filters coefficients for only phases 6 and 10.

Table 1: luma upsampling filters

| phase | Reference (N0375) | Proposal |
| --- | --- | --- |
| 0/16 | 0,0, 0,64, 0, 0,0, 0 | 0,0, 0,64, 0, 0,0, 0 |
| 1/16 | 0,1, -3,63, 4, -2,1, 0 | 0,1, -3,63, 4, -2,1, 0 |
| 2/16 | -1,2, -5,62, 8, -3,1, 0 | -1,2, -5,62, 8, -3,1, 0 |
| 3/16 | -1,3, -8,60,13, -4,1, 0 | -1,3, -8,60,13, -4,1, 0 |
| 4/16 | -1,4,-10,58,17, -5,1, 0 | -1,4,-10,58,17, -5,1, 0 |
| 5/16 | -1,4,-11,52,26, -8,3,-1 | -1,4,-11,52,26, -8,3,-1 |
| 6/16 | -1,3, -9,47,31,-10,4,-1 | -1,4,-11,45,34,-10,4,-1 |
| 7/16 | -1,4,-11,45,34,-10,4,-1 | -1,4,-11,45,34,-10,4,-1 |
| 8/16 | -1,4,-11,40,40,-11,4,-1 | -1,4,-11,40,40,-11,4,-1 |

Table 2: chroma upsampling filters

| phase | Reference (N0375) | Proposal |
| --- | --- | --- |
| 0/16 | 0, 64, 0, 0 | 0, 64, 0, 0 |
| 1/16 | -2, 62, 4, 0 | -2, 62, 4, 0 |
| 2/16 | -2, 58, 10, -2 | -2, 58, 10, -2 |
| 3/16 | -4, 56, 14, -2 | -4, 56, 14, -2 |
| 4/16 | -4, 54, 16, -2 | -4, 54, 16, -2 |
| 5/16 | -6, 52, 20, -2 | -6, 52, 20, -2 |
| 6/16 | -6, 46, 28, -4 | -6, 46, 28, -4 |
| 7/16 | -4, 42, 30, -4 | -4, 42, 30, -4 |
| 8/16 | -4, 36, 36, -4 | -4, 36, 36, -4 |

# Cross-checking description

## Implementation verification

An independent implementation of the filters changes have been made by the cross-checker.

The complexity assessment patch and the BDR and complexity numbers have been sent by the proponent on Sept 9, 2013.

The added complexity assessment software modules have been verified.

SCE1 basically aims at evaluating different spatial scalability ratios. 3 ratios are actually considered: 1.5x, 2x, and around ~1.75x. This later ratio is adjusted in order to have picture dimensions multiple of 8.

A new dowsampler was required for this new ratio. Therefore it has been decided to test two downsamplers:

- the JSVM downsampler with a phase of 0,

- the SHVC downsampler (already used for ratios 1.5x and 2x), completed by filters for missing phases to address the 1.75x case (16 phases are considered).

4 tests are performed in SCE1, using different versions of base layer sequences, generated by the two different downsamplers:

- Performance using non-modified base layer with ratios 2x and 1.5x 
This corresponds to the Common test conditions, without any change to the base layer sequences.

- Performance using base layer with ratios 2x and 1.5x generated by JSVM downsampler 
This corresponds to following modified JSVM settings:

- #define ZERO_PHASE        1

- #define SHVC_DOWNSAMPLER  0

- Performance using base layer with ratio ~1.75x generated by SHVC downsampler
This corresponds to following modified JSVM settings:

- #define ZERO_PHASE        1

- #define SHVC_DOWNSAMPLER  1

- Performance using base layer with ratio ~1.75x generated by JSVM downsampler
This corresponds to following modified JSVM settings:

- #define ZERO_PHASE        1

- #define SHVC_DOWNSAMPLER  0

## Simulations

Tests were done using the SCE1 test conditions. The source code was compiled successfully using windows32, VC2008 and using Linux 2.6.18-194.el5, gcc version 4.1.2 20080704 (Red Hat 4.1.2-48) 64 bits.

The full cross-check results are in the attached xls files. Results provided in JCTVC-O0076 are confirmed.

## Complexity assessment

The complexity modules, integrated into the software, have been checked. To the best knowledge of the cross-checker, no major issue has been detected in this implementation. Complexity results generated when decoding the streams with these complexity modules are confirmed by the cross-checker.

# Conclusions

The implementation, BDR results and complexity evaluation of JCTVC-O0076 presented by Samsung are confirmed by the cross-checker.

No difference, both in terms of coding efficiency and in terms of complexity numbers, is observed between the reference version (JCTVC-N0375) and the proposal. The cross-checker is therefore not able to make any recommendation on which version should be finally adopted in the SHVC design.