| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0125_r2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0046_r2 |
| --- | --- |

| Title: | SHVC/MV-HEVC HLS: On use of splitting_flag with combinations of scalability dimensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-3V and JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Andrey Norkin Thomas Rusert Färögatan 6 16480 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In HEVC extensions, use of splitting_flag (signalling scalability identifiers in the layer_id_in_nuh) is constrained by the order of scalability dimensions in the scalability_mask.  This restriction does not allow using the splitting_flag with different combinations of scalability extensions which do not follow the order of scalability dimensions in the scalability_mask. The document proposes two solutions that enable different coding orders of scalability layers and therefore provide compatibility of the splitting_flag syntax and semantics with the future scalability extensions. Solution 1 is capable of supporting any coding order of scalability layers with the splitting_flag while at the same time simplifying the syntax and semantics of the working draft (see Solution 1, Syntax variant 2). Solution 2 enables more efficient use of nuh_layer_id bits in some cases and can be combined with Solution 1.

The revision of the document provides the Solution 1 without changes of the syntax. The specification text changes for MV-HEVC and SHVC are attached as separate files.

# Introduction

The syntax element splitting_flag in the VPS extensions [2], [3], [4] is used to indicate that the ScalabilityId values of a layer are signaled in the NAL unit header instead of signaling them with the dimension_id syntax element. This gives network elements a possibility to learn about the scalability identifiers of a layer from the NAL unit header. In order to achieve this, the nuh_layer_id in NAL unit header (NUH) is split into several "fields", which are used to derive the values of ScalabilityId from the specified bits of nuh_layer_id. It is a requirement for layer_id_in_nuh that for every i greater than 0, layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i – 1 ].

The present syntax for ScalabilityId makes it impossible to use splitting_flag with coding order different from that specified in scalability mask (e.g. texture and depth alternate at the lowest level, then the view_id_changes, then the scalaiblity (spatial or SNR) etc).

At the Vienna meeting, the order of ScalabilityId mapping for multiview and depth components has been changed in order to allow usage of splitting_flag with 3D-HEVC common test conditions [1]. However, this sonly solves the problem with this particular case while still constraining the future use of splitting_flag to a particular order of scalability dimensions order in the scalability mask table.

A drawback the current syntax is that it restricts the coding order for other types of scalability which potentially can make use of splitting flag incompatible with other adopted types of scalability as well as and with the current scalability types. For example, if the desired coding order is T0 D0 D1 T1, it is still not possible with such a solution. Moreover, when other types of scalability are added in the future, the use of splitting_flag will be incompatible with certain coding orders of scalability dimensions.

Two solutions are proposed in this document, which allows using the splitting_flag with any order of scalability types.

# Proposed solution 1

A flexible coding order when using the splitting flag can be achieved by interpreting the most significant bits in the layer_id_in_nuh as "reordering bits". These “reordering bits” are the most significant bits in the nuh_layer_id or layer_id_in_nuh and are not used for signaling scalability dimensions information but are used instead to get a layer_id_in_nuh[ i ] value that is greater than the layer_id_in_nuh[ i – 1] value in case when this otherwise would not hold. Reordering bits can be freely selected by the encoders and since these bits utilize the most significant bits in the layer_id, they can be set in a way that the condition “the layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i – 1 ]” is fulfilled.

Example: One tries to encode the layers in the order T0, D0, D1, T1. The least significant bit is used to signal the depth dimension and the next two least significant bits are used to signal the ViewId dimension. When the three most significant bits are set equal to 0, the following layer_id_in_nuh values would be used according to the current working draft.

Table 1 Using splitting flag causes decreasing values of nuh_layer_id

| Coding order | Component | nuh_layer_id (binary), 6 bits bit 0 (rightmost group): 1 bit depth signaling bit 1-2 (central group): 2 bits viewId signaling bit 3-5 (leftmost group): 3 MSB equal to 0 | nuh_layer_id (decimal) |
| --- | --- | --- | --- |
| 0 | T0 | 000 00 0 | 0 |
| 1 | D0 | 000 00 1 | 1 |
| 2 | D1 | 000 01 1 | 3 |
| 3 | T1 | 000 01 0 | 2 (Smaller than the previous layer_id value) |

One can see that the layer_id_in_nuh does not follow the constraint to have increasing values in the coding order of layers. Therefore, splitting_flag cannot be used in this case.

Now, suppose that it is allowed to use the three most significant bits for resolving this coding order and nuh_layer_id dependency. This allows the encoder to force the increasing values of nuh_layer_id when the splitting_flag is used (see Table 2). The example in Table 2 uses depth and viewId dimensions, while this approach can also be used to also resolve the dependencies between other scalability dimensions.

Table 2 Three upper bits in nuh_layer_id are used to force increasing values of nuh_layer_id

| Coding order | Component | nuh_layer_id (binary), 6 bits bit 0 (rightmost group): 1 bit – depth signaling bit 1-2 (central group): 2 bits multiview signaling bit 3-5 (leftmost group): 3 MSB used to resolve coding order dependecies | nuh_layer_id (decimal) |
| --- | --- | --- | --- |
| 0 | T0 | 000 00 0 | 0 |
| 1 | D0 | 000 00 1 | 1 |
| 2 | D1 | 000 01 1 | 3 |
| 3 | T1 | 001 01 0 | 10 (Coding order resolved) |

The proposed solution can be realized by one of the two variants of the syntax. Both variants provide exactly the same capabilities. The only difference between the two variants is that the first variant uses the explicit reorder bits while the second variant does not force assigning nuh_layer_id most significant bits to any scalability dimension, which gives the encoder a possibility to use these bits to ensure the increasing values of nuh_layer_id.

## Syntax variant 1 (explicit reordering bits)

These reordering bits can be defined as one of the scalability dimensions, and assigned to the scalability_mask_index equal to 15 to the “reordering” dimension. Encoders can choose to utilize that scalability type by indicating its use in the scalability_mask in the VPS. Since scalability_mask index equal to 15 is used, when splitting_flag is equal to 1 and the reordering information is signaled, the reordering information will always occupy the most significant bits in nuh_layer_id.

Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| vps_vui_offset | u(16) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16 15 + splitting_flag; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |

- Video parameter set extension semantics

scalability_mask_flag[ i ] equal to 1 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension in Table F-1 are present. scalability_mask_flag[ i ] equal to 0 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension are not present. When splitting_flag is equal to 0, the value of scalability_mask_flag [ 15 ] is inferred to be equal to 0.

Table F–1 – Mapping of ScalabiltyId to scalability dimensions

| scalability mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | Reserved |  |
| 1 | Multiview | View Order Index |
| 2 | spatial/SNR scalability | DependencyId |
| 3-15 14 | Reserved |  |
| 15 | reordering | None |

NOTE 1 – It is anticipated that in future 3D extensions of this Specification, scalability mask index 0 will be used to indicate depth maps.

NOTE 3 – The reordering scalability dimension (the dimension with scalability_mask_index equal to 15) is not mapped to any ScalabilityId. The bits in the nuh_layer_id allocated for this scalability dimension can be used to fulfill the requirement for the values of layer_id_in_nuh that nuh_layer_id [i] > nuh_layer_id[i-1] for any value of i >0.

## Syntax variant 2 (implicit use of MSB to resolve the dependencies)

The same capabilities as with syntax variant 1 can be achieved by allowing the encoder to not assigning the most significant bits in nuh_layer_id to the scalability dimension. Then, these MSB can be used by an encoder to force the increasing values of nuh_layer_id. Therefore the most significant bits in the layer_id_in_nuh are implicitly used by the encoder as "reordering bits". The required syntax and semantics modification are quite small (actually, the semantics and syntax are simplified).

Video parameter set extension syntax

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| vps_vui_offset | u(16) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |

- Video parameter set extension semantics

splitting_flag equal to 1 indicates that the dimension_id[ i ][ j ] syntax elements are not present and that the binary representation of the nuh_layer_id value in the NAL unit header are split into NumScalabilityTypes segments with lengths, in bits, according to the values of dimension_id_len_minus1[ j ] and that the values of dimension_id[ LayerIdxInVps[ nuh_layer_id ] ][ j ] are inferred from the NumScalabilityTypes segments. splitting_flag equal to 0 indicates that the syntax elements dimension_id[ i ][ j ] are present.

NOTE 1 – When splitting_flag is equal to 1, scalable identifiers can be derived from the nuh_layer_id syntax element in the NAL unit header by a bit masked copy. The respective bit mask for the i-th scalable dimension is defined by the value of the dimension_id_len_minus1[ i ] syntax element and dimBitOffset[ i ] as specified in the semantics of dimension_id_len_minus1[ j ]. When the most significant bits in the nuh_layer_id syntax element are not assigned for derivation of any scalability identifiers, they can be used by the encoder to fulfil the constraint that nuh_layer_id [i] > nuh_layer_id[i-1] for any values of i >0.

scalability_mask_flag[ i ] equal to 1 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension in Table F-1 are present. scalability_mask_flag[ i ] equal to 0 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension are not present.

Table F–1 – Mapping of ScalabiltyId to scalability dimensions

| scalability mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | Reserved |  |
| 1 | Multiview | View Order Index |
| 2 | spatial/SNR scalability | DependencyId |
| 3-15 | Reserved |  |

NOTE 2 – It is anticipated that in future 3D extensions of this Specification, scalability mask index 0 will be used to indicate depth maps.

dimension_id_len_minus1[ j ] plus 1 specifies the length, in bits, of the dimension_id[ i ][ j ] syntax element.

When splitting_flag is equal to 1, the following applies:

–	The variable dimBitOffset[ 0 ] is set equal to 0 and for j in the range of 1 to  NumScalabilityTypes − 1 , inclusive, dimBitOffset[ j ] is derived as follows:

dimBitOffset[ j ]=dimIdx=0j−1dimension_id_len_minus1[ dimIdx ]+1	(F-1)

–	The value of dimension_id_len_minus1[ NumScalabilityTypes − 1 ] is inferred to be equal to 5 − dimBitOffset[ NumScalabilityTypes − 1 ].

–	The value of dimBitOffset[ NumScalabilityTypes ] is set equal to 6.

It is a requirement of bitstream conformance that when NumScalabilityTypes is greater than 0, dimBitOffset[ NumScalabilityTypes − 1 ] shall be less than or equal to 6.

## Syntax variant 3

During a discussion in the BoG, another variant of the semantics and syntax has been proposed, which does not change the current syntax. The text for proposed modifications to SHVC and MV-HEVC specifications (including the profiles section) is attached to the contribution as two separate files. The editing marks are used to highlight the changes.

# Proposed solution 2

Another solution that is capable changing the coding order from the one determined by the order of scalability dimensions in the scalability mask is explained in the following. This solution signals the order “fields” corresponding to different scalability dimensions in the nuh_layer_id. This solution uses the bits of the nuh_layer_id in the most efficient way (the number of bits spend on signaling the scalability dimensions does not change). However, it cannot solve the coding order dependencies, where the scalability dimensions order is flexibly changing, e.g. such as T0, D0, D1, T1 (where T means texture, D means depth and the numbers denote the ViewId). This solution can also be combined with solution 1, thus providing any coding order and efficient used on nuh_layer_id bits in most cases.

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| vps_vui_offset | u(16) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ){ |  |
| dimension_id_len_minus1[ j ] | u(3) |
| if (splitting_flag) |  |
| dimension_id_position[ j ] | u(v) |
| } |  |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if( !splitting_flag ) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |

          - Video parameter set extension semantics

splitting_flag equal to 1 indicates that the dimension_id[ i ][ j ] syntax elements are not present and that the binary representation of the nuh_layer_id value in the NAL unit header are split into NumScalabilityTypes segments with lengths, in bits, according to the values of dimension_id_len_minus1[ j ] and that the values of dimension_id[ LayerIdxInVps[ nuh_layer_id ] ][ j ] are inferred from the NumScalabilityTypes segments. splitting_flag equal to 0 indicates that the syntax elements dimension_id[ i ][ j ] are present.

NOTE 1 – When splitting_flag is equal to 1, scalable identifiers can be derived from the nuh_layer_id syntax element in the NAL unit header by a bit masked copy. The respective bit mask for the i-th scalable dimension is defined by the value of the dimension_id_len_minus1[ i ] syntax element and dimBitOffset[ i ] as specified in the semantics of dimension_id_len_minus1[ j ].

scalability_mask_flag[ i ] equal to 1 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension in Table F-1 are present. scalability_mask_flag[ i ] equal to 0 indicates that dimension_id syntax elements corresponding to the i-th scalability dimension are not present.

Table F–1 – Mapping of ScalabiltyId to scalability dimensions

| scalability mask index | Scalability dimension | ScalabilityId mapping |
| --- | --- | --- |
| 0 | Reserved |  |
| 1 | Multiview | View Order Index |
| 2 | spatial/SNR scalability | DependencyId |
| 3-15 | Reserved |  |

NOTE 2 – It is anticipated that in future 3D extensions of this Specification, scalability mask index 0 will be used to indicate depth maps.

dimension_id_position [ j ] specifies the position of the j-th signaled scalability dimension in the layer_id_in_nuh when the splitting_flag is used. The length of the dimension_id_position [ j ] element is Ceil( Log2( NumScalabilityTypes ) ) bits. It is a requirement of bitstream conformance that there are no identical values of dimension_id_position[j] for j = 0, ..., (NumScalabilityTypes − splitting_flag ).

dimension_id_len_minus1[ j ] plus 1 specifies the length, in bits, of the dimension_id[ i ][ j ] syntax element.

When splitting_flag is equal to 1, the following applies:

–	The variable dimBitOffset[ 0 ] is set equal to 0 and for j in the range of 1 to  NumScalabilityTypes − 1 , inclusive, dimBitOffset[ j ] is derived as follows:

dimBitOffset[ j ]=dimIdx=0j−1dimension_id_len_minus1[ dimIdx ]+1	(F-1)

–	The value of dimension_id_len_minus1[ NumScalabilityTypes − 1 ] is inferred to be equal to 5 − dimBitOffset[ NumScalabilityTypes − 1 ].

–	The value of dimBitOffset[ NumScalabilityTypes ] is set equal to 6.

It is a requirement of bitstream conformance that when NumScalabilityTypes is greater than 0, dimBitOffset[ NumScalabilityTypes − 1 ] shall be less than 6.

vps_nuh_layer_id_present_flag equal to 1 specifies that layer_id_in_nuh[ i ] for i from 0 to vps_max_layers_minus1, inclusive, are present. vps_nuh_layer_id_present_flag equal to 0 specifies that layer_id_in_nuh[ i ] for i from 0 to vps_max_layers_minus1, inclusive, are not present.

layer_id_in_nuh[ i ] specifies the value of the nuh_layer_id syntax element in VCL NAL units of the i-th layer. For i in the range of 0 to vps_max_layers_minus1, inclusive, when layer_id_in_nuh[ i ] is not present, the value is inferred to be equal to i.

When i is greater than 0, layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i – 1 ].

For i from 0 to vps_max_layers_minus1, inclusive, the variable LayerIdxInVps[ layer_id_in_nuh[ i ] ] is set equal to i.

dimension_id[ i ][ j ] specifies the identifier of the j-th present scalability dimension type or dimension_id_position[j]-th in case of splitting_flag of the i-th layer. The number of bits used for the representation of dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits.

If splitting_flag is equal to 1, for i from 0 to vps_max_layers_minus1, inclusive, and j from 0 to NumScalabilityTypes − 1, inclusive, dimension_id[ i ][ j ] is inferred to be equal to ( ( layer_id_in_nuh[ i ] & ( (1 << dimBitOffset[ j + 1 ] ) − 1) ) >> dimBitOffset[ j ] ).

Otherwise, for j from 0 to NumScalabilityTypes − 1, inclusive, dimension_id[ 0 ][ j ] is inferred to be equal to 0.

The variable ScalabilityId[ i ][ smIdx ] specifying the identifier of the smIdx-th scalability dimension type of the i-th layer, the variable ViewOrderIdx[ layer_id_in_nuh[ i ] ] specifying the view order index of the i-th layer, DependencyId[ layer_id_in_nuh[ i ] ] specifying the spatial/SNR scalability identifier of the i-th layer, and the variable ViewScalExtLayerFlag specifying whether the i-th layer is a view scalability extension layer are derived as follows:

NumViews = 1

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {

lId = layer_id_in_nuh[ i ]

for( smIdx= 0, j = 0; smIdx < 16; smIdx++ )

if( scalability_mask_flag[ smIdx ] )

if (splitting_flag)

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ dimension_id_position[j++] ]

else

ScalabilityId[ i ][ smIdx ] = dimension_id[ i ][ j++ ]

ViewOrderIdx[ lId ] = ScalabilityId[ i ][ 1 ]

DependencyId [ lId  ] = ScalabilityId[ i ][ 2 ]

if( i > 0  &&  ( ViewOrderIdx[ lId ]  !=  ScalabilityId[ i – 1][ 1 ] ) )

NumViews++

ViewScalExtLayerFlag[ lId ] = ( ViewOrderIdx[ lId ] > 0 )

}

# Conclusions

Current syntax associated with splitting_flag does not let using the splitting_flag with the coding order of layers different from the order of scalability dimensions in the in scalability mask. Two solutions to this problem are provided. Solution 1 enables use of splitting_flag with any coding order of scalability dimensions (also with future extensions) while at the same time simplifying syntax and semantics of the working draft (see solution 1, syntax variant 2). Solution 2 enables more efficient use of nuh_layer_id bits than the Solution 1 in some cases. However, Solution 2 does not cover all cases of scalability dimensions coding order.

It is proposed to adopt the Solution 1 (which supports any coding order and simplifies the syntax and semantics of the working draft). Alternatively, a combination of Solution 1 and Solution 2 can be adopted as capable of more efficient use of nuh_layer_id bits in certain cases. It is proposed to adopt the proposal to SHVC, MV-HEVC and 3D-HEVC working drafts and the reference software.

# References

- A. Norkin, T. Rusert, "MV-HEVC/SHVC HLS: On use of splitting_flag with flexible coding order", JCT3V-E0104, Vienna, Austria, July - Aug. 2013.

- J. Chen, J. Boyce, Y. Ye, M. M. Hannuksela, " Scalable High Efficiency Video Coding Draft 3", JCTVC-N1008, Vienna, Austria, July - Aug. 2013.

- G. Tech, K. Wegner, Y. Chen, M. Hannuksela, J. Boyce, "MV-HEVC Draft Text 5", JCT3V-E1004, Vienna, Austria, July - Aug. 2013

- L. Zhang, G. Tech, K. Wegner, S. Yea, "3D-HEVC Test Model 5", JCT3V-E1005, Vienna, Austria, July - Aug. 2013

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).