| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0126 |
| --- | --- |

| Title: | Request for an 8 bit 4:4:4 profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Fuldseth, Cisco Systems G. Bjøntegaard, Cisco Systems | Tel: Email: Tel: Email: | +47 92414082 arild.fuldseth@cisco.com gbjonteg@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This document proposes to include an 8 bit 4:4:4 profile (Main 4:4:4 8) in the next version of the HEVC standard. It is proposed that this profile shall include the same coding tools as the Main 4:4:4 10 and Main 4:4:4 12 profiles as currently specified in the draft text.

# Request for an 8 bit 4:4:4 profile

The next version of the HEVC standard is anticipated to include support for higher bit depths, 4:4:4 chroma format, and new coding tools. For many consumer applications handling screen content, we believe the 4:4:4 chroma format and new tools particularly useful for coding of screen content tools are desirable. On the other hand, the benefits of higher bit depths beyond 8 bits might not justify the additional implementation cost. A particularly large potential usage is wireless display applications, especially using lossless tools. Another important use case is video conferencing applications, in particular those using software implementations of HEVC.

Hence, we believe there is a need for a new 8 bit 4:4:4 profile:

- Bit depth equal to 8 (bit_depth_luma_minus8 and bit_depth_chroma_minus8 equal to 0).

- 4:4:4 chroma format (chroma_format_idc less than or equal to 3).

- Same coding tools as Main 4:4:4 10 and Main 4:4:4 12.

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix A

Following are proposed changes to the draft text.

A.3.5 Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 8, Main 4:4:4 10 and Main 4:4:4 12 profiles

Bitstreams conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 8, Main 4:4:4 10 and Main 4:4:4 12 profiles shall obey the following constraints:

- In bitstreams conforming to the Main 12, Main 4:2:2 10 or Main 4:2:2 12  profile, SPSs shall have chroma_format_idc constrained as follows:

–	In bitstreams conforming to the Main 12 profile, SPSs shall have chroma_format_idc equal to 0 or 1 only.

–	In bitstreams conforming to the Main 4:2:2 10 or Main 4:2:2 12 profile, SPSs shall have chroma_format_idc equal to 0, 1 or 2 only.

- SPSs shall have bit_depth_luma_minus8 and bit_depth_chroma_minus8 constrained as follows:

- In bitstreams conforming to the Main 4:4:4 8 profile, SPSs shall have bit_depth_luma_minus8 and bit_depth_chroma_minus8 equal to 0.

–	In bitstreams conforming to the Main 4:2:2 10 or Main 4:4:4 10 profile, SPSs shall have bit_depth_luma_minus8 and bit_depth_chroma_minus8 in the range of 0 to 2, inclusive.

–	In bitstreams conforming to the Main 12, Main 4:2:2 12, or Main 4:4:4 12 profile, SPSs shall have bit_depth_luma_minus8 and bit_depth_chroma_minus8 in the range of 0 to 4, inclusive.

- In bitstreams conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 8, Main 4:4:4 10 and Main 4:4:4 12 profiles,

- SPSs shall have separate_colour_plane_flag, when present, equal to 0 only.

- SPSs shall have extended_precision_processing_flag, when present, equal to 0 only.

- CtbLog2SizeY shall be in the range of 4 to 6, inclusive.

- When a PPS has tiles_enabled_flag is equal to 1, it shall have entropy_coding_sync_enabled_flag equal to 0.

- When a PPS has tiles_enabled_flag is equal to 1, ColumnWidthInLumaSamples[ i ] shall be greater than or equal to 256 for all values of i in the range of 0 to num_tile_columns_minus1, inclusive, and RowHeightInLumaSamples[ j ] shall be greater than or equal to 64 for all values of j in the range of 0 to num_tile_rows_minus1, inclusive.

- The number of times read_bits( 1 ) is called in subclauses Error: Reference source not found and Error: Reference source not found when parsing coding_tree_unit( ) data for any coding tree unit shall be less than or equal to 5 * RawCtuBits / 3.

- general_level_idc shall not be equal to 255 (which indicates level 8.5).

- The level constraints specified for the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 8, Main 4:4:4 10 or Main 4:4:4 12 profile in in subclause Error: Reference source not found, as applicable, shall be fulfilled.

Conformance of a bitstream to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 8, Main 4:4:4 10 or Main 4:4:4 12 profile is indicated by general_profile_idc being equal to 4 or general_profile_compatibility_flag[ 4 ] being equal to 1 with the following additional indications as applicable:

- Conformance of the bitstream to the Main 12 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 1, and general_max_420chroma_constraint_flag equal to 1.

- Conformance of the bitstream to the Main 4:2:2 10 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 1, general_max_422chroma_constraint_flag equal to 1, and general_max_420chroma_constraint_flag equal to 1.

- Conformance of the bitstream to the Main 4:2:2 12 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 1, and general_max_420chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the Main 4:4:4 8 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 1, general_max_422chroma_constraint_flag equal to 0, and general_max_420chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the Main 4:4:4 10 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 1, general_max_422chroma_constraint_flag equal to 0, and general_max_420chroma_constraint_flag equal to 0.

- Conformance of the bitstream to the Main 4:4:4 12 profile is indicated by general_max_12bit_constraint_flag equal to 1, general_max_10bit_constraint_flag equal to 0, general_max_422chroma_constraint_flag equal to 0, and general_max_420chroma_constraint_flag equal to 0.

All other combinations of general_max_12bit_constraint_flag, general_max_10bit_constraint_flag, general_max_422chroma_constraint_flag, and general_max_420chroma_constraint_flag with general_profile_idc equal to 4 or general_profile_compatibility_flag[ 4 ] equal to 1 are reserved for future use by ITU-T | ISO/IEC. Such combinations shall not be present in bitstreams conforming to this specification. However, decoders conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 10 or Main 4:4:4 12 profile shall allow other combinations as specified below in this subclause to occur in the bitstream.

Decoders conforming to the Main 12, Main 4:2:2 10, Main 4:2:2 12, Main 4:4:4 8, Main 4:4:4 10 or Main 4:4:4 12 profile at a specific level (identified by a specific value of general_level_idc) shall be capable of decoding all bitstreams for which all of the following conditions apply:

- One or both of the following conditions apply:

–	general_profile_idc is equal to 1 or 2, general_profile_compatibility_flag[ 1 ] is equal to 1, general_profile_compatibility_flag[ 2 ] is equal to 1.

–	general_profile_idc is equal to 4 or general_profile_compatibility_flag[ 4 ] is equal to 1, and any of the following conditions apply:

–	The decoder conforms to the Main 12 profile, general_max_12bit_constraint_flag is equal to 1, and general_max_420chroma_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:2:2 10 profile, general_max_10bit_constraint_flag is equal to 1, and general_max_422chroma_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:2:2 12 profile, general_max_12bit_constraint_flag is equal to 1, and general_max_422chroma_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:4:4 10 profile and general_max_10bit_constraint_flag is equal to 1.

–	The decoder conforms to the Main 4:2:2 12 profile and general_max_12bit_constraint_flag is equal to 1.

- general_level_idc is not equal to 255 and represents a level lower than or equal to the specified level.

general_tier_flag represents a tier lower than or equal to the specified tier.