| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0127 |
| --- | --- |

| Title: | Transcoder-friendly scalable coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Thomas Rusert, Rickard Sjöberg, Jonatan Samuelsson Färögatan 6 16480 Stockholm | Tel: Email: | +46 10 7141538 kenneth r andersson @ ericsson com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

To be able to provide scalability with high coding efficiency both from an encoding device to a network node (uplink) and from the network node to the end user device (downlink) this contribution proposes to add new functionality to SHVC. It is proposed that the transform coefficients of a low fidelity layer can be derived from transform and quantization of the difference between the reconstructed pixel values of the high fidelity base layer and the prediction from the low fidelity layer. This enables BDR gains compared to simulcast of 12.1% for SNR scalability at the same time as the high fidelity base layer can be represented without any loss compared to single layer HEVC and also be compatible with HEVC version 1 in the down link to the end user device.  The BD rate of the dependent layer is reduced by 35.7% compared to having the layer independently coded. This contribution proposes to add an additional decoding process before the inverse quantization and inverse transformation for the dependent layers. The use of the proposed decoding process is indicated in the VPS.

It is assessed that this new functionality can be used by a transcoder to get a HEVC version 1 high-fidelity bitstream by only removing higher layer NAL units and that Transcoding of the scalable bitstream to a low fidelity HEVC version 1 bitstream can be done without computationally demanding mode decisions and motion estimation.

# Background

One important use case for HEVC version 1 is IP based video delivery providing a coding efficient bitstream from the encoding device(s) to a network node (uplink/UL) and depending on the network characteristics from the network node to the end device (downlink/DL) provide a coding efficient HEVC version 1 bitstream suitable to the available bandwidth. In the best of worlds both the uplink and the downlink should have same coding efficiency as encoding for a single representation.

One approach to support this use case is to simulcast bitstreams, e.g. encode several fidelities independently and send them to a network node (UL). The network node can then forward the most suitable bitstream to the end device (DL). The nice feature of simulcasting HEVC version 1 bitstreams is that each bitstream can be coded very efficiently for the DL and thus require minimal size for respective quality. The draw-back of simulcast is the redundancy of having several independent encodings of similar content in the UL. Below are alternatives to simulcast described and it is considered if they can fulfil this use case and how their performance compares to simulcast.

## Scalable coding with SHVC

Scalable coding has the capability to represent several fidelities by the use of layered coding. The higher fidelities depends on the lower fidelities to reduce the overhead for representing them. Scalable coding can thus reduce the redundancy of independent coding of several fidelities as in simulcast. SHVC can reduce the overhead for coding several fidelities in UL compared to simulcast by 21% for random access and 12% for low delay B with SNR scalability common conditions. The low fidelity base layer can be coded very efficient and thus require minimal size for its use in the DL and it can be decoded by an HEVC version 1 decoder. The higher fidelities get an overhead due to the dependency to the lower fidelities for its use in the DL. SHVC has an overhead of the highest fidelity of 14% for random access and 24% for low delay B with SNR scalability common conditions compared to HEVC version 1 and cannot be decoded with a HEVC version 1 decoder so it fails to fulfill the use case.

Below the decoding and encoding processes for a higher layer of SHVC is shown, here layer 1, excluding the loop filter (de-blocking and SAO) which is after the clipping.

Figure 1 Decoding process for a dependent layer of SHVC, where Q is quantization, T is transformation and P is prediction, D is delay, R is the reconstructed pixel values.

Figure 2Encoding process for a dependent layer of SHVC, where RDO is rate distortion optimization and O is the original video source.

## Transcoding HEVC bitstream

Transcoding has the capability to provide several lower fidelities by re-encoding a higher fidelity HEVC version 1 bitstream.. The lower fidelity bitstream will have worse coding efficiency than a low fidelity bitstream in a simulcast scenario since it has to rely on the quality of the reconstructed high fidelity video for its encoding. To achieve an efficient representation of the low fidelity bitstream it is required to spend a significant amount of computations for estimation of coding parameters (mode and motion estimation). A transcoder can take short cuts to speed up the transcoding, e.g. re-using mode and motion information from the high-quality bitstream, but that comes at the cost of worse coding efficiency of the low fidelity bitstream. A transcoder has potential to accomplish the use case but to do that it requires a large amount of computations. Below is full transcoding shown where only pixel values from the decoding are used.

Figure 3 Full transcoding

# Proposal

To support the use case of a coding efficient HEVC version 1 bitstream for the end points in DL we propose to extend SHVC with new functionality so that it has the ability to provide the highest fidelity in the base layer and have the lower fidelities coded with dependency to the highest fidelity to get an efficient representation in UL. This would then enable HEVC version 1 decoding in DL of the high fidelity base layer by just removing higher layer NALs. To obtain lower fidelity version 1 compatible HEVC  bitstreams the SHVC bitstream needs to be transcoded.

Since a large portion of the bits comes from the transform coefficients those are proposed to be re-generate in the decoder for higher layers and thus reduce the overhead in the uplink compared to simulcast. Other coding parameters like, CU-, PU-, and TU partitioning, transform skip flag and motion and intra prediction information are unchanged. A block diagram of the decoding process of the proposed approach is shown in below figure.

Figure 4 Decoding process for proposed functionality for SHVC. The proposed additional process is within the dashed box.

As shown in the figure above the transform coefficients is re-generated by computing the difference between the reconstruction (after loop filtering) from layer 0 and the prediction in layer 1. Then transform and quantization are performed before the normal decoding process.

The corresponding encoding process is shown below.

Figure 5 Encoding process for proposed functionality to SHVC.

As can be seen in the figure of the encoding process for the proposed functionality the main differences compared to SHVC is the separation of used and sent transform parameters and that the prediction error is coded against the output from layer 0. The RDO is made with reference to the original, as for SHVC. With this approach the encoder can potentially optimize the coding parameters used both in the uplink and the downlink with knowledge of the original video source. This is not exploited in the contribution which only optimize for DL. The transform partitioning and the transform skip flag indication is same in the sent and the used transform parameters. The only difference is that the transform coefficients sent in the uplink are mostly zeros to achieve little overhead.

A transcoder can use the decoded coding parameters from layer 1 and the re-generated transform coefficients for encoding a HEVC version 1 low-fidelity bitstream without performing any mode decisions and motion estimation.

To enable a decoder to know when it shall do the proposed decoding process we suggest to provide one new dependency_type in VPS in parallel to existing inter-layer motion and sample based dependency types. Thus an SHVC encoder could use the existing dependency types to further improve the bitrate reduction in the uplink compared to simulcast.

To have a minimal and clean design we suggest to use the transpose of the inverse transform as the transform to be used and that the quantization is done without RDOQ, sign bit hiding and using flat scaling matrices.

Below sections disclose changes, high-lighted in yellow, to the SHVC draft [1]. It can be noted that there are no changes to the syntax below VPS.

## VPS extension

To indicate the deployment of the proposed functionality in the video bitstream we propose following modifications.

F.7.4.3.1.1  Video parameter set extension semantics

direct_dep_type_len_minus2 plus 2 specifies the number of bits of the direct_dependency_type[ i ][ j ] syntax element. In bitstreams conforming to this version of this Specification the value of direct_dep_type_len_minus2 shall be equal 01. Although the value of direct_dep_type_len_minus2 shall be equal to 01 in this version of this Specification, decoders shall allow other values of direct_dep_type_len_minus2 in the range of 0 to 30, inclusive, to appear in the syntax.

direct_dependency_type[ i ][ j ] indicates the type of dependency between the layer with nuh_layer_id equal layer_id_in_nuh[ i ] and the layer with nuh_layer_id equal to layer_id_in_nuh[ j ]. direct_dependency_type[ i ][ j ] equal to 0, 2, 4 or 6 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer sample prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ] equal to 1, 2, 5 or 6  indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ] equal to 3, 4, 5 or 6 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for residual re-generation of the layer with nuh_layer_id equal layer_id_in_nuh[ i ].  Although the value of direct_dependency_type[ i ][ j ] shall be in the range of 0 to 26, inclusive, in this version of this Specification, decoders shall allow values of direct_dependency_type[ i ][ j ] in the range of 37 to 232 − 2, inclusive, to appear in the syntax

The variables NumDirectRefLayers[ i ], and RefLayerId[ i ][ j ] SamplePredEnabledFlag[ i ][ j ], MotionPredEnabledFlag[ i ][ j ], ResidualReGenerationEnabledFlag[i][j] and DirectRefLayerIdx[ i ][ j ] are derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {

iNuhLId = layer_id_in_nuh[ i ]

NumDirectRefLayers[ iNuhLId ] = 0

for( j = 0; j < i; j++ )

if( direct_dependency_flag[ i ][ j ] ) {

RefLayerId[ iNuhLId ][ NumDirectRefLayers[ iNuhLId ]++ ] = layer_id_in_nuh[ j ]

SamplePredEnabledFlag[ iNuhLId ][ j ] = ( ( direct_dependency_type[ i ][ j ] + 1 ) & 1 )

MotionPredEnabledFlag[ iNuhLId ][ j ] = ( ( ( direct_dependency_type[ i ][ j ] + 1 ) & 2 ) >> 1 )

ResidualReGenerationEnabledFlag[ iNuhLId ][ j ] = ( ( ( direct_dependency_type[ i ][ j ] + 1 ) & 4 ) >> 2 )

DirectRefLayerIdx[ iNuhLid ][ layer_id_in_nuh[ j ] ] = NumDirectRefLayers[ iNuhLId ] – 1		}

}

## Decoding process for residual dependent layers

We define an additional decoding process for SHVC in addition to existing ones to re-generate the correct transform coefficient levels  instead of using the transform coefficient levels parsed in the bitstream (TransCoeffLevels) when both the new ResidualReGenerationEnabledFlag and the existing inter_layer_pred_enable_flag is equal to 1.

H.8.4 Decoding process for coding units coded in intra prediction mode

If both ResidualReGenerationEnabledFlag[currLayerId][rLId] and inter_layer_pred_enable_flag  is equal to 1 the specifications in subclause 8.4 apply with the exception that prior to 8.6.2 the transform coefficient levels TransCoeffLevel are modified as specified in H 8.6.2. Otherwise the specification in subclause 8.4 apply.

H 8.5.4	Decoding process for the residual signal of coding units coded in inter prediction mode

If both ResidualReGenerationEnabledFlag[currLayerId][rLId] and inter_layer_pred_enable_flag  is equal to 1 the specifications in subclause 8.5.4 apply with the exception that prior to 8.6.2 the transform coefficient levels TransCoeffLevel are modified as specified in H 8.6.2. Otherwise the specification in subclause 8.5.4 apply.

H.8.6	Scaling, transformation and array construction process prior to deblocking filter process

If both ResidualReGenerationEnabledFlag[currLayerId][rLId] and inter_layer_pred_enable_flag  is equal to 1 the specifications in subclause 8.6 apply with the exception that prior to 8.6.2 (Scaling and transformation process) the transform coefficient levels TransCoeffLevel  are modified as specified in H 8.6.2. Otherwise the specification on subclause 8.6 apply.

NOTE: A transcoder can re-use most layer specific syntax from the current layer including the modified TransCoeffLevel when the ResidualReGenerationEnabledFlag[currLayerId][rLId] is equal to 1 or inter_layer_pred_enable_flag  is equal to 0 to encode a HEVC version 1 compatible bitstream.

H 8.6.2	Scaling and transformation process

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nTbS specifying the size of the current transform block.

–	an array of reconstructed samples from the inter-layer reference picture (after de-block and sao) rsPic

–	an array of prediction samples predSamples

Output of this process is the (nTbS)x(nTbS) array of transform coefficient levels with elements TransCoeffLevel [ xTbY ][ yTbY ][ cIdx ][ x ][ y ].

First an (nTbS)x(nTbS) array of temporary residual samples (rT) is obtained by taking the difference between rlPicSample and predSamples:

rT[x][y] = rlPicSample[x][y]-predSamples[x][y]

The (nTbS)x(nTbS) array of levels l is derived as follows:

–	If cu_transquant_bypass_flag is equal to 1, the (nTbS)x(nTbS) array of levels l is set equal to the (nTbS)x(nTbS) array of temporary residual samples rT

l[x][y] =rT[x][y] .

–	Otherwise, the following ordered steps apply:

1.	The forward transform as specified in clause H 8.6.4 is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, the quantization parameter qP, and the array of temporary residual samples rT as inputs, and the output is an (nTbS)x(nTbS) array of forward transform coefficients d.

2.	The (nTbS)x(nTbS) array of transform coefficients TransCoeffLevel is modified as follows:

-	If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the sample array values TransCoeffLevel [xTbY][yTbY][cIdx][ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

TransCoeffLevel [xTbY][yTbY][cIdx] [ x ][ y ] = ( l[ x ][ y ]  <<  (15-bitDepth-log2(nTbS) )	(H 8-267)

Where bitDepth is equal to bitDepthY when cIdx is equal to 0 and otherwise equal to bitDepthC.

-	Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the forward scaling process for transform coefficients as specified in clause H 8.6.3 is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of forward transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of transform coefficients TransCoeffLevel.

H 8.6.3	Forward scaling process for forward transform coefficients

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top left luma sample of the current picture,

–	a variable nTbS specifying the size of the current transform block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable qP specifying the quantization parameter.

–	an array of forward transform coefficients d with elements d[x][y]

Output of this process is the (nTbS)x(nTbS) array TransCoeffLevel of transform coefficients with elements TransCoeffLevel[xTbY][yTbY][cIdx] [ x ][ y ].

The transform coefficients are modified by the following ordered steps:

- intLevel[x,y] is set equal to d[x,y]

- intSign[x,y] is set equal to -1 if intLevel[x,y] is less than 0 otherwise it is set to 1.

- fwdScaleFactor = fwdLevelScale [Qp % 6] where  fwdLevelScale= {26214, 23302, 20560, 18396, 16384, 14564}

- int64Level[x,y] is set equal to abs(intLevel[x,y]) * fwdScaleFactor

- intLevel[x,y] is set equal to (int64Level[x,y]+intAdd) >> intQBits

where intQBits is equal to 29+Qp/6-bitDepth-log2(nTbS) ,

bitDepth is equal to bitDepthY when cIdx is equal to 0 and otherwise equal to bitDepthC ,

intAdd is equal to 171 << (intQBits-9) if slice_type is equal to I or the inter-layer picture rsPic is an IRAP, otherwise intAdd is equal to 85 << (intQBits-9)

- intLevel[x,y] is set equal to intLevel[x,y]*intSign[x,y]

- TransCoeffLevel[xTbY][yTbY][cIdx] [x][y]  is set equal to  Clip3( -32768, 32767, intLevel[x,y] )

H 8.6.4	Transformation process for temporary residual samples

H 8.6.4.1	General

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top left luma sample of the current picture,

–	a variable nTbS specifying the size of the current transform block,

–	a variable cIdx specifying the colour component of the current block,

–	an (nTbS)x(nTbS) array rT of temporary residual samples with elements rT[ x ][ y ].

Output of this process is the (nTbS)x(nTbS) array d of forward transform coefficients with elements d[ x ][ y ].

Depending on the values of CuPredMode[ xTbY ][ yTbY ], nTbS, and cIdx, the variable trType is derived as follows:

–	If CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA, nTbS is equal to 4, and cIdx is equal to 0, trType is set equal to 1.

–	Otherwise, trType is set equal to 0.

The (nTbS)x(nTbS) array d of transform coefficients is derived as follows:

1.	Each (vertical) column of temporary residual samples rT[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 is transformed to e[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 by invoking the one-dimensional transformation process as specified in clause H 8.6.4.2 for each column x = 0..nTbS − 1 with the size of the transform block nTbS, the list r[ x ][ y ] with y = 0..nTbS − 1, and the transform type variable trType as inputs, and the output is the list e[ x ][ y ] with y = 0..nTbS − 1.

2.	The intermediate sample values g[ x ][ y ] with  x = 0..nTbS − 1, y = 0..nTbS − 1  are derived as follows:

shift1 is equal to  log2(nTbS) - 1 + bitDepth – 8, where bitDepth is equal to bitDepthY when cIdx is equal to 0 and otherwise equal to bitDepthC

g[ x ][ y ] = (e[ x ][ y ] + (1<<(shift1-1) )  >>  shift1	(H 8.275)

3.	Each (horizontal) row of the resulting array g[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 is transformed to h[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 by invoking the one-dimensional transformation process as specified in clause H 8.6.4.2 for each row y = 0..nTbS − 1 with the size of the transform block nTbS, the list g[ x ][ y ] with x = 0..nTbS − 1, and the transform type variable trType as inputs, and the output is the list h[ x ][ y ] with x = 0..nTbS − 1.

4. The forward transform coefficients d are derived as follows:

shift2 is equal to log2(nTbS) + 8

d[ x ][ y ] = (h[ x ][ y ] + (1<<(shift2-1)) )  >>  shift2

H 8.6.4.2	Transformation process

Inputs to this process are:

–	a variable nTbS specifying the sample size of the residual samples,

–	a list of transform coefficients x with elements x[ j ], with j = 0..nTbS − 1.

–	a transform type variable trType

Output of this process is the list of transformed samples y with elements y[ i ], with i = 0..nTbS − 1.

Depending on the value of trType, the following applies:

–	If trType is equal to 1, the following transform matrix multiplication applies:

y[ i ] = j=0nTbS−1transMatrix[j][i]∗x[j] with i = 0..nTbS – 1       (H 8-276)

where transform coefficient array transMatix is defined in 8-277.

–	Otherwise (trType is equal to 0), the following transform matrix multiplication applies:

y[ i ] = j=0nTbS−1transMatrix[j∗25−Log2(nTbS)][i]∗x[j] with i = 0..nTbS − 1,y[ i ] =   with i = 0..nTbS − 1,	(H 8-278)

where the transform coefficient array transMatrix is specified in 8-279 and 8-281.

# Results

In Table 1 the proposal is compared to simulcast coding using common test conditions [2] for SNR scalability. Simulcast is encoded with SHVC without any dependency between the two layers and it uses RDOQ and sign data hiding. The base layer contains the high fidelity and layer 1 contains the low fidelity. The proposal is coded without motion or sample based dependency only using the proposed residual dependency. The base layer of the proposal uses RDOQ and sign data hiding but those are not used for layer 1. Encoding and decoding times are computed on a linux cluster and are unreliable.

Table 1Proposal compared to simulcast using SHVC

|  | RA HEVC SNR |  |  | LD-B HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -12,2% | -12,3% | -12,2% | -10,4% | -10,5% | -10,5% |
| Class B | -13,6% | -13,7% | -13,8% | -12,3% | -12,5% | -12,6% |
| Overall (Test vs Ref) | -13,2% | -13,3% | -13,4% | -11,8% | -11,9% | -12,0% |
| Overall (Test vs single layer) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall (Ref vs single layer) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| EL only (Test vs Ref) | -38,4% | -33,1% | -32,9% | -34,9% | -29,6% | -29,3% |
| Enc Time[%] | 97,8% |  |  | 101,5% |  |  |
| Dec Time[%] | 111,9% |  |  | 116,4% |  |  |
|  | LD-P HEVC SNR |  |  |  |  |  |
|  | Y | U | V |  |  |  |
| Class A | -9,9% | -10,1% | -10,0% |  |  |  |
| Class B | -11,8% | -11,9% | -12,0% |  |  |  |
| Overall (Test vs Ref) | -11,3% | -11,4% | -11,4% |  |  |  |
| Overall (Test vs single layer) | 0,0% | 0,0% | 0,0% |  |  |  |
| Overall (Ref vs single layer) | 0,0% | 0,0% | 0,0% |  |  |  |
| EL only (Test vs Ref) | -33,9% | -28,4% | -28,4% |  |  |  |
| Enc Time[%] | 94,7% |  |  |  |  |  |
| Dec Time[%] | 104,7% |  |  |  |  |  |

As shown in Table 1 the proposal reduce the average BD rate by 12.1% compared to simulcast while having no loss compared to single layer coding for coding of the high-fidelity base layer since a node in the network can remove higher scalable layers. The BD rate can be reduced by 35.7% on average for coding of layer 1 compared to simulcast.

In Table 2 the proposal is compared to simulcast when the coefficients of layer 1 in the proposal are kept unchanged and not re-generated in the decoder as in Table 1.

Table 2 Proposal (when coefficients are kept unchanged in layer 1) compared to simulcast using SHVC.

|  | RA HEVC SNR |  |  | LD-B HEVC SNR |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0,1% | -0,1% | -0,1% | -0,5% | -0,5% | -0,5% |
| Class B | -0,2% | -0,2% | -0,1% | -0,4% | -0,4% | -0,3% |
| Overall (Test vs Ref) | -0,2% | -0,1% | -0,1% | -0,4% | -0,4% | -0,4% |
| Overall (Test vs single layer) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| Overall (Ref vs single layer) | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% | 0,0% |
| EL only (Test vs Ref) | 10,5% | 23,5% | 24,3% | 9,0% | 21,6% | 22,0% |
| Enc Time[%] | 96,4% |  |  | 98,6% |  |  |
| Dec Time[%] | 109,1% |  |  | 108,5% |  |  |
|  | LD-P HEVC SNR |  |  |  |  |  |
|  | Y | U | V |  |  |  |
| Class A | -0,5% | -0,5% | -0,5% |  |  |  |
| Class B | -0,5% | -0,5% | -0,4% |  |  |  |
| Overall (Test vs Ref) | -0,5% | -0,5% | -0,5% |  |  |  |
| Overall (Test vs single layer) | 0,0% | 0,0% | 0,0% |  |  |  |
| Overall (Ref vs single layer) | 0,0% | 0,0% | 0,0% |  |  |  |
| EL only (Test vs Ref) | 7,8% | 20,7% | 20,5% |  |  |  |
| Enc Time[%] | 96,4% |  |  |  |  |  |
| Dec Time[%] | 103,8% |  |  |  |  |  |

As shown in Table 2 the proposal give a slight reduction in BD rate compared to simulcast while it has no loss compared to single layer coding for coding of the high-fidelity base layer since a node in the network can remove higher scalable layers. The BD rate increase for layer 1 compared to layer 1 of simulcast is on average 9.1%.

# Conclusion

We have shown how SHVC can provide a high fidelity bitstream with same performance as single layer HEVC and also be compliant with HEVC version 1. Low fidelity layers make use of the high fidelity base layer to reduce overhead compared to simulcast. To reconstruct a low fidelity layer the residual is re-generated from the high fidelity layer. A transcoder can use this SHVC bitstream to derive a low fidelity bitstream without performing mode decisions and motion estimation. There are no syntax changes below VPS.

We propose that this new functionality is adopted to the SHVC draft.

# References

- “High efficiency video coding (HEVC) scalable extension draft 3”,  JCTVC-N1008_v3, July, 2013

- X. Li, J. Boyce, P. Onno, Y. Ye, “Common SHM test conditions and software reference configurations”, JCTVC-N1009, July, 2013

# Patent rights declaration(s)

Ericsson AB may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).