| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0133-wd |
| --- | --- |

| Title: | Text fixes for inter RDPCM in HEVC-RExt (text changes) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak Centre House, Block D 56 Wood Lane, W12 7SB London | Tel: Email: | +44 (0)303 0409640 matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

This document complements JCTVC-O0133 and contains the text changes required to align the HEVC-RExt text specification to the inter RDPCM implementation in the reference software. Text changes are provided with respect to JCTVC-N1005_v3 and using the MS Word track changes feature. Changes with respect to the first submission of O0133 are highlighted in yellow.

# Changes to residual coding syntax and semantics

7.3.8.11		Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize <= Log2MaxTransformSkipSize ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTER && residual_dpcm_inter_enabled_flag && ( transform_skip_flag[ x0 ][ y0 ][ cIdx ] ¦ ¦ cu_transquant_bypass_flag ) ) { |  |
| inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| inter_rpdcm_dir_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |
| ... | ae(v) |
| } |  |

# Changes to general decoding process for intra blocks

Inputs to this process are:

–	a sample location ( xTb0, yTb0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	a variable log2TrafoSize specifying the size of the current transform block,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable predModeIntra specifying the intra prediction mode,

–	a variable predModeIntraBc specifying the intra block copying mode,

–	a variable bvIntra specifying the intra block copying vector,

–	a variable cIdx specifying the colour component of the current block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The luma sample location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture is derived as follows:

( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTb0, yTb0 ) : ( xTb0 * SubWidthC, yTb0 * SubHeightC )	(8-1)

The variable splitFlag is derived as follows:

–	If cIdx is equal to 0, splitFlag is set equal to split_transform_flag[ xTbY ][ yTbY ][ trafoDepth ].

–	Otherwise, if all of the following conditions are true, splitFlag is set equal to 1.

- cIdx is greater than 0

- split_transform_flag[ xTbY ][ yTbY ][ trafoDepth ] is equal to 1

- log2TrafoSize is greater than 2

–	Otherwise, splitFlag is set equal to 0.

Depending on the value of splitFlag, the following applies:

–	If splitFlag is equal to 1, the following ordered steps apply:

- The variables xTb1 and yTb1 are derived as follows:

  - The variable xTb1 is set equal to xTb0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

  - The variable yTb1 is set equal to yTb0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb0, yTb0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb1, yTb0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb0, yTb1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb1, yTb1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (splitFlag is equal to 0), for the variable blkIdx proceeding over the values 0..( cIdx > 0  &&  ChromaArrayType  = =  2 ? 1 : 0 ), the following ordered steps apply:

- The variable nTbS is set equal to 1  <<  log2TrafoSize.

- The variable yTbOffset is set equal to blkIdx * nTbS.

- The variable residualDpcm is derived as follows:

  - If residual_dpcm_intra_enabled_flag is equal to 1 and one or more of the following conditions are true, residualDpcm is set equal to 1.

    - cu_transquant_bypass_flag is equal to 1

    - transform_skip_flag[ xTb0 ][ yTb0 + yTbOffset ][ cIdx ] is equal to 1

  - Otherwise, residualDpcm is set equal to 0.

- Depending upon the value of predModeIntraBc, the following applies

–	When predModeIntraBc is equal to 0, the general intra sample prediction process as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the intra prediction mode predModeIntra, the transform block size nTbS, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

–	Otherwise (predModeIntraBc is equal to 1), the intra block copying process as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the transform block size nTbS, the variable bvIntra, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

- The scaling and transformation process as specified in subclause Error: Reference source not found is invoked with the luma location ( xTbY, yTbY + yTbOffset * SubHeightC ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nTbS as inputs, and the output is an (nTbS)x(nTbS) array resSamples.

- When residualDpcm is equal to 1 and either predModeIntra is equal to 10, or predModeIntra is equal to 26, the directional residual modification process for blocks using a transform bypass as specified in subclause 5 is invoked with the variable mDir set equal to predModeIntra / 26, the variable nTbS, the variable cIdx, the location ( xTb0, yTb0 ), and the (nTbS)x(nTbS) array r set equal to the array resSamples as inputs, and the output is a modified (nTbS)x(nTbS) array resSamples.

- The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the variables nCurrSw and nCurrSh both set equal to nTbS, the variable cIdx, the (nTbS)x(nTbS) array predSamples, and the (nTbS)x(nTbS) array resSamples as inputs.

# Changes to decoding process for coding units coded in inter prediction mode

8.5.4.2	Decoding process for luma residual blocks

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xB0, yB0 ) specifying the top-left sample of the current luma block relative to the top-left sample of the current luma coding block,

–	a variable log2TrafoSize specifying the size of the current luma block,

–	a variable trafoDepth specifying the hierarchy depth of the current luma block relative to the luma coding block,

–	a variable nCbS specifying the size of the current luma coding block,

–	an (nCbS)x(nCbS) array resSamples of luma residual samples.

Output of this process is a modified version of the (nCbS)x(nCbS) array of luma residual samples.

Depending on the value of split_transform_flag[ xCb + xB0 ][ yCb + yB0 ][ trafoDepth ], the following applies:

–	If split_transform_flag[ xCb + xB0 ][ yCb + yB0 ][ trafoDepth ] is equal to 1, the following ordered steps apply:

- The variables xB1 and yB1 are derived as follows:

–	The variable xB1 is set equal to xB0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

–	The variable yB1 is set equal to yB0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

- The decoding process for luma residual blocks as specified in this subclause is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable nCbS, and the (nCbS)x(nCbS) array resSamples as inputs, and the output is a modified version of the (nCbS)x(nCbS) array resSamples.

- The decoding process for luma residual blocks as specified in this subclause is invoked with the luma location ( xCb, yCb ), the luma location ( xB1, yB0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable nCbS, and the (nCbS)x(nCbS) array resSamples as inputs, and the output is a modified version of the (nCbS)x(nCbS) array resSamples.

- The decoding process for luma residual blocks as specified in this subclause is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable nCbS, and the (nCbS)x(nCbS) array resSamples as inputs, and the output is a modified version of the (nCbS)x(nCbS) array resSamples.

- The decoding process for luma residual blocks as specified in this subclause is invoked with the luma location ( xCb, yCb ), the luma location ( xB1, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable nCbS, and the (nCbS)x(nCbS) array resSamples as inputs, and the output is a modified version of the (nCbS)x(nCbS) array resSamples.

–	Otherwise (split_transform_flag[ xCb + xB0 ][ yCb + yB0 ][ trafoDepth ] is equal to 0), the following ordered steps apply:

- The variable nTbS is set equal to 1  <<  log2TrafoSize.

- The scaling and transformation process as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb + xB0, yCb + yB0 ), the variable trafoDepth, the variable cIdx set equal to 0, and the transform size trafoSize set equal to nTbS as inputs, and the output is an (nTbS)x(nTbS) array transformBlock.

- When residual_dpcm_inter_enabled_flag is equal to 1 and inter_rdpcm_flag[ xCb + xB0 ][ yCb + yB0 ][ 0 ] is equal to 1, the directional residual modification process for blocks using a transform bypass as specified in subclause 5 is invoked with the variable mDir set equal to inter_rdpcm_dir_flag[ xCb + xB0 ][ yCb + yB0 ][ 0 ], the variable nTbS, the variable cIdx set equal to 0, the location ( xCb + xB0, yCb + yB0 ), and the (nTbS)x(nTbS) array r set equal to the array transformBlock as inputs, and the output is a modified (nTbS)x(nTbS) array transformBlock.

- The (nCbS)x(nCbS) residual sample array of the current coding block resSamples is modified as follows:

resSamples[ xB0 + i, yB0 + j ] = transformBlock[ i, j ], with i = 0..nTbS − 1, j = 0..nTbS − 1	(8-2)

8.5.4.3	Decoding process for chroma residual blocks

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

–	a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

–	a luma location ( xB0, yB0 ) specifying the top-left luma sample of the current chroma block relative to the top-left sample of the current luma coding block,

–	a variable log2TrafoSize specifying the size of the current chroma block in luma samples,

–	a variable trafoDepth specifying the hierarchy depth of the current chroma block relative to the chroma coding block,

–	a variable cIdx specifying the chroma component of the current block,

–	the variables nCbSw and nCbSh specifying the width and height respectively of the current chroma coding block,

–	an (nCbSw)x(nCbSh) array resSamples of chroma residual samples.

Output of this process is a modified version of the (nCbSw)x(nCbSh) array of chroma residual samples.

The variable splitChromaFlag is derived as follows:

–	If split_transform_flag[ xCb + xB0 ][ yCb + yB0 ][ trafoDepth ] is equal to 1 and log2TrafoSize is greater than 3, splitChromaFlag is set equal to 1.

–	Otherwise (split_transform_flag[ xCb + xB0 ][ yCb + yB0 ][ trafoDepth ] is equal to 0 or log2TrafoSize is equal to 3), splitChromaFlag is set equal to 0.

Depending on the value of splitChromaFlag, the following applies:

–	If splitChromaFlag is equal to 1, the following ordered steps apply:

- The variables xB1 and yB1 are derived as follows:

–	The variable xB1 is set equal to xB0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

–	The variable yB1 is set equal to yB0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCbSw, the variable nCbSh, and the (nCbSw)x(nCbSh) array resSamples as inputs, and the output is a modified version of the (nCbSw)x(nCbSh) array resSamples.

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xCb, yCb ), the luma location ( xB1, yB0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCbSw, the variable nCbSh, and the (nCbSw)x(nCbSh) array resSamples as inputs, and the output is a modified version of the (nCbSw)x(nCbSh) array resSamples.

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xCb, yCb ), the luma location ( xB0, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCbSw, the variable nCbSh, and the (nCbSw)x(nCbSh) array resSamples as inputs, and the output is a modified version of the (nCbSw)x(nCbSh) array resSamples.

- The decoding process for residual chroma blocks as specified in this subclause is invoked with the luma location ( xCb, yCb ), the luma location ( xB1, yB1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the variable cIdx, the variable nCbSw, the variable nCbSh, and the (nCbSw)x(nCbSh) array resSamples as inputs, and the output is a modified version of the (nCbSw)x(nCbSh) array resSamples.

–	Otherwise (splitChromaFlag is equal to 0), for the variable blkIdx proceeding over the values 0..( ChromaArrayType  = =  2 ? 1 : 0 ), the following ordered steps apply:

- The variable nTbS is set equal to ( 1  <<  log2TrafoSize ) / SubWidthC.

- The variable yBN is set equal to yB0 + blkIdx * nTbS * SubHeightC.

- The scaling and transformation process as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb + xB0, yCb + yBN ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nTbS as inputs, and the output is an (nTbS)x(nTbS) array transformBlock.

- When residual_dpcm_inter_enabled_flag is equal to 1 and inter_rdpcm_flag[ xCb + xBN ][ yCb + yBN ][ cIdx ], the directional residual modification process for blocks using a transform bypass as specified in subclause 5 is invoked with the variable mDir set equal to inter_rdpcm_dir_flag[ xCb + xBN ][ yCb + yBN ][ cIdx ], the variable nTbS, the variable cIdx, the location ( xCb + xB0, yCb + yBN ), and the (nTbS)x(nTbS) array r set equal to the array transformBlock as inputs, and the output is a modified (nTbS)x(nTbS) array transformBlock.

- The (nCbS)x(nCbS) residual sample array of the current coding block resSamples is modified as follows, for i = 0..nTbS − 1, j = 0..nTbS − 1:

resSamples[ ( xCb + xB0 ) / SubWidthC + i, ( yCb + yBN ) / SubHeightC + j ] = transformBlock[ i, j ]	(8-3)

# Changes to scaling, transformation and array construction process prior to deblocking filter process

8.6.2	Scaling and transformation process

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nTbS specifying the size of the current transform block.

Output of this process is the (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = Qp′Y			(8-4)

–	Otherwise, if cIdx is equal to 1,

qP = Qp′Cb		(8-5)

–	Otherwise (cIdx is equal to 2),

qP = Qp′Cr			(8-6)

–	The variable bdShift is derived as follows:

bdShift = ( cIdx  = =  0 ) ? 20 − BitDepthY : 20 − BitDepthC	(8-7)

The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If cu_transquant_bypass_flag is equal to 1, the (nTbS)x(nTbS) array of residual samples r is derived as follows:

–	If transform_skip_rotation_enabled_flag is equal to 1 and nTbS is equal to 4, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( TransCoeffLevel[ nTbS − x −1 ][ nTbS − y − 1 ]  <<  tsShift )	(8-8)

–	Otherwise, the (nTbS)x(nTbS) array r is set equal to the (nTbS)x(nTbS) array of transform coefficients TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the quantization parameter qP as inputs, and the output is an (nTbS)x(nTbS) array of scaled transform coefficients d.

- The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

–	The variable tsShift is derived as follows:

tsShift = extended_precision_processing_flag ? Max( 7, bdShift ) : 7	(8-9)

–	If transform_skip_rotation_enabled_flag is equal to 1 and nTbS is equal to 4,

–	If inter_rdpcm_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1,

r[ x ][ y ] = d[ nTbS − x −1 ][ nTbS − y − 1 ]	(8-10)

–	Otherwise,

r[ x ][ y ] = ( d[ nTbS − x −1 ][ nTbS − y − 1 ]  <<  tsShift )	(8-114)

–	Otherwise,

–	If inter_rdpcm_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1,

r[ x ][ y ] = d[ x ][ y ]		(8-126)

–	Otherwise,

r[ x ][ y ] = ( d[ x ][ y ]  <<  tsShift )	(8-137)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

- The residual sample values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are modified as follows:

r[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-148)

# Changes to residual modification process for blocks using a transform bypass

Inputs to this process are:

–	a variable mDir specifying the residual modification direction,

–	a variable nTbS specifying the transform block size,

–	a variable cIdx specifying the colour component of the current block,

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture

–	an (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

Output of this process is the modified (nTbS)x(nTbS) array of residual samples .

Depending upon the value of mDir, the (nTbS)x(nTbS) array of samples r is modified as follows:

–	If mDir is equal to 0 (horizontal direction), the array values r[ x ][ y ], for x = 1..nTbS − 1, y = 0..nTbS − 1, are modified as follows:

r[ x ][ y ] += r[ x − 1 ][ y ]	(8-15)

–	Otherwise (vertical direction), the array values r[ x ][ y ], for x = 0..nTbS − 1, y = 1..nTbS − 1, are modified as follows:

r[ x ][ y ] += r[ x ][ y − 1 ]	(8-16)

If cu_transquant_bypass_flag is equal to zero and inter_rdpcm_flag[ xTbY ][ yTbY ][ cIdx ] is equal to one, the (nTbS)x(nTbS) array of samples r is modified as follows:

–	The variables bdShift and tsShift are derived as follows:

bdShift = ( cIdx  = =  0 ) ? 20 − BitDepthY : 20 − BitDepthC

tsShift = extended_precision_processing_flag ? Max( 7, bdShift ) : 7

–	The array values r[ x ][ y ], for x = 0..nTbS − 1, y = 1..nTbS − 1, are modified as follows:

r[ x ][ y ] = ( r[ x ][ y ]  <<  tsShift )