| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0134-wd |
| --- | --- |

| Title: | On residual DPCM design unification for lossy coding in HEVC-RExt (text changes) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak Centre House, Block D 56 Wood Lane, W12 7SB London | Tel: Email: | +44 (0)303 0409640 matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

This document complements JCTVC-O0134 and contains the text changes required to align the intra RDPCM design to the inter one. Text changes are provided with respect to JCTVC-N1005_v3 with the text fixes suggested in Document JCTVC-O0133. Text additions and modifications are highlighted in yellow.

# Changes to scaling, transformation and array construction process prior to deblocking filter process

8.6.2	Scaling and transformation process

Inputs to this process are:

–	a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding block,

–	a variable cIdx specifying the colour component of the current block,

–	a variable nTbS specifying the size of the current transform block.

Output of this process is the (nTbS)x(nTbS) array of residual samples r with elements r[ x ][ y ].

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = Qp′Y			(8-1)

–	Otherwise, if cIdx is equal to 1,

qP = Qp′Cb		(8-2)

–	Otherwise (cIdx is equal to 2),

qP = Qp′Cr			(8-3)

–	The variable bdShift is derived as follows:

bdShift = ( cIdx  = =  0 ) ? 20 − BitDepthY : 20 − BitDepthC	(8-4)

The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If cu_transquant_bypass_flag is equal to 1, the (nTbS)x(nTbS) array of residual samples r is derived as follows:

–	If transform_skip_rotation_enabled_flag is equal to 1 and nTbS is equal to 4, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( TransCoeffLevel[ nTbS − x −1 ][ nTbS − y − 1 ]  <<  tsShift )	(8-5)

–	Otherwise, the (nTbS)x(nTbS) array r is set equal to the (nTbS)x(nTbS) array of transform coefficients TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the quantization parameter qP as inputs, and the output is an (nTbS)x(nTbS) array of scaled transform coefficients d.

- The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

–	The variable tsShift is derived as follows:

tsShift = extended_precision_processing_flag ? Max( 7, bdShift ) : 7	(8-6)

–	If transform_skip_rotation_enabled_flag is equal to 1 and nTbS is equal to 4,

–	If inter_rdpcm_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1 or predModeIntra is equal to 10 or predModeIntra is equal to 26,

r[ x ][ y ] = d[ nTbS − x −1 ][ nTbS − y − 1 ]	(8-7)

–	Otherwise,

r[ x ][ y ] = ( d[ nTbS − x −1 ][ nTbS − y − 1 ]  <<  tsShift )	(8-84)

–	Otherwise,

–	If inter_rdpcm_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1 or predModeIntra is equal to 10 or predModeIntra is equal to 26,

r[ x ][ y ] = d[ x ][ y ]		(8-96)

–	Otherwise,

r[ x ][ y ] = ( d[ x ][ y ]  <<  tsShift )	(8-107)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

- The residual sample values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are modified as follows:

r[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-118)