| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0134 |
| --- | --- |

| Title: | On residual DPCM design unification for lossy coding in HEVC-RExt |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Marta Mrak Centre House, Block D 56 Wood Lane, W12 7SB London | Tel: Email: | +44 (0)303 0409640 matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

Residual DPCM (RDPCM) is adopted for inter and intra coding in the HEVC-RExt specification text Draft 4. Both in intra and inter coding, RDPCM performs reconstruction of residuals after inverse quantisation but inter RDPCM reconstructs the residuals before the inverse shift operated when transform skip flag is equal to one while intra RDPCM performs the reconstruction after the shift down. This documents analyses a design unification approach and reports both on the performance as well as on the required text changes. Under RCE3 common test conditions it is reported that the proposed design unification does not bring losses when compared to the anchors and, for some sequence classes, BD-rate gains are registered.

# Introduction

RDPCM has been initially adopted for intra prediction and lossless coding at the 13th meeting in Incheon when it was presented in Document JCTVC-M0056. At the 14th meeting in Vienna, RDPCM was extended for inter prediction both in lossless and lossy as proposed in Document JCTVC-N0074. Moreover, RDPCM for intra prediction was extended to lossy coding as proposed in JCTVC-N0052. In lossy coding, RDPCM is applied only on those Transform Units (TUs) where the transform skip flag is equal to one. In fact, in these TUs the spatial redundancy among residuals can be exploited by performing sample-based prediction using dequantised samples to avoid any drift between the encoder and the decoder. At the decoder, after inverse quantisation, the residuals can be reconstructed by simple adding along the RDPCM prediction direction which can be either horizontal or vertical. The reconstruction of residuals can be done either before the shift down performed when transform skip flag is one or after. In the current reference software version for HEVC-RExt (4.1) the residuals reconstruction is carried at different stages of the decoding process depending on whether intra or inter coding is selected. More specifically:

- Intra prediction: residual reconstruction done after shift down operated when transform skip flag flag is one (see Figure 1)

- Inter prediction: residual reconstruction is done before the shift down operated when transform skip flag is one (see Figure 2)

This difference between intra and inter coding may be problematic for some practical implementations and does not provide a clear design of the standard specification text.

This document therefore discusses a design unification approach and presents the performance obtained as well as the required text changes.

Figure 1: Decoding execution flow followed when RDPCM is applied to intra coded residuals.

Figure 2: Decoding execution flow followed when RDPCM is applied to inter coded residuals.

# Proposed RDPCM design unification

Given the differences between intra and inter RDPCM design mentioned in Section 1, it is proposed to align the intra RDPCM design to the inter one. Therefore at the decoder side for both intra and inter codin, after inverse quantisation, residual reconstruction takes place before the final shift down required by the transform skip operation (see Figure 2). The performance and the text changes required by the proposed design unification are now discussed in the following.

## Performance results

The design unification proposed in this contribution has been implemented in the HEVC-RExt software version 4.1 and tested under the RCE3 test conditions described in Document JCTVC-N1123. The BD-rates are measured using the RCE3 anchors and reported in Table 1. For this design unification there are no BD-rate losses (beside Class F and the V component which may suffer from some bits distribution between the U component where gains are showed). Moreover, as listed in Table 1 some gains are reported; these gains come from the higher precision, i.e. after the shift up operated by the encoder when transform skip flag is equal to 1, where the RDPCM process is performed.

Table 1: BD-rates between RCE3 anchors and the HM-RExt-4.1 modified to align intra RDPCM design towards inter.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.4% | -0.2% | -0.2% |
| SC(444) YUV Optional | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.2% | -0.1% | -0.2% | -0.2% |
| Enc Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| SC RGB 444 | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| SC(444) YUV Optional | -0.2% | -0.2% | -0.3% | -0.1% | -0.1% | -0.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.3% | 0.4% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| Animation RGB 444 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC YUV 444 | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% |  |  |  |
| Animation YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -0.3% | -0.1% | -0.3% | -0.5% | -0.4% | -0.7% |  |  |  |
| SC(444) YUV Optional | -0.7% | -0.4% | -0.7% | -0.3% | -0.3% | -0.5% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

## Proposed text changes

The text changes related to this design unification are reported in the document attached to this proposal.

# Conclusions

Residuals DPCM as currently implemented in both the reference software and the specification text implies a different design for intra and inter coding. This contribution presented a designed unification approach and showed that aligning intra RDPCM design to the inter one provides no losses and in some cases also gains are reported. Moreover, the required text changes for this design unification are minimal.

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).