| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0139r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0052r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Layer-wise initialization and parameter set activation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Several improvements on layer-wise start-up, parameter set activation, and IRAP alignment are proposed. The proposed modifications are summarized below.

- A new VPS can be activated with a CRA with HandleCraAsBlaFlag equal to 1, when nuh_layer_id equal to 0.

- The SPS syntax elements related to the representation format (width, height, bit-depth) shall remain unchanged within a sequence of activated SPS RBSPs, until a new VPS can be activated.

- A layer can be initialized when the current picture is an IRAP picture with NoRaslOutputFlag equal to 1.

- A picture that is an IRAP picture with NoRaslOutputFlag equal to 1 or that is contained in an initial IRAP access unit can be an LIP picture of a layer, when all dependent layers of the layer are initialized.

- Semantics changes on poc_reset_flag and cross_layer_irap_aligned_flag.

# Introduction

A layer-wise start-up and its related process were adopted in the last Vienna meeting. In this contribution, the improvements of the process related to the layer-wise start-up and the conditions of the parameter set activation are proposed.

# On video parameter set activation

When a CVS is layer-wisely initiated, the activation condition of video parameter set (VPS) is not clear. Of course, the following conditions are described in the current specification text. However, we raise the issue whether those conditions are fully discussed and agreed at the last meeting.

- F.7.4.2.4.2 Order of VPS, SPS and PPS RBSPs and their activation The specifications in subclause 7.4.2.4.2 apply with the following additions. An activated VPS RBSP shall remain active until the end of the bitstream or until it is deactivated by another VPS RBSP in an access unit in which at least one of the following conditions is true: - The access unit includes a picture for each nuh_layer_id value in TargetDecLayerIdList and each picture in the access unit is an IDR picture. The access unit includes an IRAP picture with nuh_layer_id equal to 0 for which NoClrasOutputFlag is equal to 1. 8.1 General decoding process When the current picture is an IRAP picture and has nuh_layer_id equal to 0, the following applies: – The variable NoClrasOutputFlag is specified as follows: – If the current picture is the first picture in the bitstream, NoClrasOutputFlag is set equal to 1. – Otherwise, if the current picture is a BLA picture, NoClrasOutputFlag is set equal to 1. – Otherwise, if some external means not specified in this Specification is available to set NoClrasOutputFlag, NoClrasOutputFlag is set by the external means. – Otherwise, NoClrasOutputFlag is set equal to 0.

We summarize the above description as follows:

- A VPS remains active until a new VPS is activated or until the end of the bitstream.

- A new VPS is activated at an access unit in which all pictures are IDRs.

- A new VPS is activated at an access unit in which a picture with nuh_layer_id equal to 0 is a BLA picture.

Unlike HEVC version 1, the scope of an active VPS RBSP is not within a CVS, but within a bitstream, and a new VPS is activated when all IDRs are aligned except for spliced points.

Even though it is not sure that those conditions were fully studied and agreed in the last meeting, we agreed that those conditions are applicable for the layer-wise initialization.

In this section, we add a minor improvement on top of those conditions for VPS activation, as follows:

- F.7.4.2.4.2 Order of VPS, SPS and PPS RBSPs and their activation The specifications in subclause 7.4.2.4.2 apply with the following additions. An activated VPS RBSP shall remain active until the end of the bitstream or until it is deactivated by another VPS RBSP in an access unit in which at least one of the following conditions is true: - The access unit includes a picture for each nuh_layer_id value in TargetDecLayerIdList and each picture in the access unit is an IDR picture. The access unit includes an IRAP picture with nuh_layer_id equal to 0 for which NoClrasOutputFlag is equal to 1. 8.1 General decoding process When the current picture is an IRAP picture and has nuh_layer_id equal to 0, the following applies: – The variable NoClrasOutputFlag is specified as follows: – If the current picture is the first picture in the bitstream, NoClrasOutputFlag is set equal to 1. – Otherwise, if the current picture is a BLA picture or a CRA with HandleCraAsBlaFlag equal to 1, NoClrasOutputFlag is set equal to 1. – Otherwise, if some external means not specified in this Specification is available to set NoClrasOutputFlag, NoClrasOutputFlag is set by the external means. – Otherwise, NoClrasOutputFlag is set equal to 0.

Hence, a new VPS is also activated at an access unit in which a picture with nuh_layer_id equal to 0 is a CRA picture with HandleCraAsBlaFlag equal to 1.

# On sequence parameter set activation

A condition for changing the SPS syntax elements related to the representation format is proposed.

If layers have different IRAP periods, a non-IRAP picture with nuh_layer_id greater than 0 can be an LIP in an initial IRAP access unit. Then, the pictures with the same nuh_layer_id following the LIP in decoding order may or may not refer to the decoded picture belonging to the previous CVS as references. If the picture size or the bit-depth is changed by activating a new SPS in the following CVS, the pictures in the following CVS may refer to the decoded picture with different formats. That is not allowed in the current SHVC and MV-HEVC. Therefore, we propose the following condition for changing the SPS syntax elements related to the representation format.

The SPS syntax elements pic_width_in_luma_samples, pic_height_in_luma_samples, bit_depth_luma_minus8, bit_depth_chroma_minus8, and chroma_format_idc shall remain unchanged within a sequence of activated SPS RBSPs, in their activation order, from any activated SPS RBSP until the end of the bitstream or up to but excluding an SPS RBSP that is activated within the next access unit in which at least one of the following conditions is true: - The access unit includes a picture for each nuh_layer_id value in TargetDecLayerIdList and each picture in the access unit is an IDR picture. - The access unit includes an IRAP picture with nuh_layer_id equal to 0 for which NoClrasOutputFlag is equal to 1.

# On LayerInitialisedFlag

In the current specification text, when a layer with nuh_layer_id greater than 0 is initialized at an IRAP picture, the RASL pictures associated with the IRAP picture of the layer cannot be correctly decoded. Therefore, we believe that each layer can be initialized when the current picture is an IRAP picture with NoRaslOutputFlag equal to 1. Adding the phrase highlighted in yellow is proposed in the below specification text.

# Decoding process ## General decoding process … When the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and one of the following conditions is true, LayerInitialisedFlag[ nuh_layer_id ] is set equal to 1: – nuh_layer_id is equal to 0. – LayerInitialisedFlag[ nuh_layer_id ] is equal to 0 and LayerInitialisedFlag[ refLayerId ] is equal to 1 for all values of refLayerId equal to RefLayerId[ nuh_layer_id ][ j ], where j is in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] – 1, inclusive.

# On definition of layer initialisation picture

Each layer can be initialized when the current picture of the layer is an LIP picture. The LIP picture may activate a new SPS for each layer. Specifically, an activated SPS RBSP for a particular nuh_layer_id value shall remain active for a sequence of pictures in decoding order with that nuh_layer_id value starting from a LIP picture having that nuh_layer_id value, inclusive, until either the next LIP picture with that nuh_layer_id value, exclusive, or the end of the CVS, whichever is earlier.

The current definition of LIP picture is as follows:

layer initialisation picture (LIP): A picture that is an IRAP picture with NoRaslOutputFlag equal to 1 or that is contained in an initial IRAP access unit.

Based on the above definition of LIP, however, a layer cannot be correctly initialized and all the following pictures from the LIP cannot be correctly decoded in cases that the LayerInitialisedFlag[i] of a directly dependent layer of the LIP is not equal to 1. This is because the successful decoding of an LIP of the current layer is possible when all dependent layers of the current layer are initialized.

Therefore, a new definition of LIP is proposed as follows:

layer initialisation picture (LIP): A picture that is an IRAP picture with NoRaslOutputFlag equal to 1 or that is contained in an initial IRAP access unit, of which LayerInitialisedFlag[ refLayerId ] is equal to 1 for all values of refLayerId equal to RefLayerId[ nuh_layer_id ][ j ], where j is in the range of 0 to NumDirectRefLayers[ nuh_layer_id ] – 1, inclusive.

# On cross_layer_irap_aligned_flag

We propose to modify the semantics of cross_layer_irap_aligned_flag, or the semantics of poc_reset_flag related to cross_layer_irap_aligned_flag. This modification is needed to resolve the confliction between cross_layer_irap_aligned_flag and poc_reset_flag, on the constraint that the POC value shall be unchanged within an access unit.

The cross_layer_irap_aligned_flag ensures that when a picture pictureA of a layer layerA in an access unit is an IRAP picture, each picture pictureB in the same access unit that belongs to a direct reference layer of layerA or that belongs to a layer for which layerA is a direct reference layer of that layer is an IRAP picture and the VCL NAL units of pictureB have the same value of nal_unit_type as that of pictureA.

Hence, the cross_layer_irap_aligned_flag cannot ensure the IRAP alignment for all layers. If two pictures in an access unit have no dependency to each other, two pictures may have different nal_unit_type values. If poc_reset_flag is not set equal to 1, the POC values of pictures may or may not be the same. However, it is a requirement of bitstream conformance that the value of poc_reset_flag shall be equal to 0 when cross_layer_irap_aligned_flag is equal to 1, in the current SHVC and MV-HEVC specification text.

In conclusion, if cross_layer_irap_aligned_flag is equal to 1 and two pictures having no dependency to each other in an access unit have different nal_unit_type values, the POC value alignment cannot be guaranteed.

In order to resolve this conflict, we propose to change the semantics of cross_layer_irap_aligned_flag, as follows:

cross_layer_irap_aligned_flag equal to 1 specifies that IRAP pictures in the CVS are cross-layer aligned, i.e. when a picture pictureA of a layer layerA in an access unit is an IRAP picture, each picture pictureB in the same access unit that belongs to a direct reference layer of layerA or that belongs to a layer for which layerA is a direct reference layer of that layer is an IRAP picture and the VCL NAL units of pictureB have the same value of nal_unit_type as that of pictureA. cross_layer_irap_aligned_flag equal to 0 specifies that the above restriction may or may not apply.

->

cross_layer_irap_aligned_flag equal to 1 specifies that IRAP pictures in the CVS are cross-layer aligned, i.e. when a picture pictureA of a layer layerA in an access unit is an IRAP picture, each picture pictureB in the same access unit have the same value of nal_unit_type as that of pictureA. cross_layer_irap_aligned_flag equal to 0 specifies that the above restriction may or may not apply.

Or alternatively, we propose to change the semantics of poc_reset_flag, as follows:

poc_reset_flag equal to 1 specifies that the derived picture order count for the current picture is equal to 0. poc_reset_flag equal to 0 specifies that the derived picture order count for the current picture may or may not be equal to 0. It is a requirement of bitstream conformance that when cross_layer_irap_aligned_flag is equal to 1, the value of poc_reset_flag shall be equal to 0. When not present, the value of poc_reset_flag is inferred to be equal to 0.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).