| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0140r1 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0053r1 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Alignment of picture order counts |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In order to improve the existing POC value alignment method with poc_reset_flag, a modification is proposed.

- On POC reset

  - poc_reset_flag is replaced with poc_msb_reset_flag.

# Introduction

The derivation methods of picture order count (POC) values for each layer are proposed to ensure the same POC value across layers for HEVC multi-layered extension. The POC value shall be unchanged across layers within an access unit. In SHVC or MV-HEVC, there is no restriction on the alignment of IRAP (Random access point) pictures across layers. Hence, different IRAP pictures and non-IRAP pictures can be present together in an access unit. Then, keeping the same POC value for different picture types of layers forces to change the POC value derivation process other than HEVC single layer coding. We propose an improvement for POC value alignment across layers.

# On POC reset

The flag poc_reset_flag was adopted, which was included in the slice header to specify whether the POC value needs to be reset for the picture. When POC misalignment can possibly happen, encoders are required to put the value of the flag to be equal to 1 to ensure that POC values are cross-layer aligned.

When this flag is equal to 1, the POC value of the current picture is set equal to 0, in addition, the POC values of the pictures in DPB belonging to the same layer are changed accordingly to maintain the POC differences to the current picture.

slice_segment_header( ) {; Descriptor. .... if( !dependent_slice_segment_flag ) {. poc_reset_flag; u(1). for ( i =10; i < num_extra_slice_header_bits; i++ ). slice_reserved_flag[ i ]; u(1). .... }. poc_reset_flag equal to 1 specifies that the derived picture order count for the current picture is equal to 0. poc_reset_flag equal to 0 specifies that the derived picture order count for the current picture may or may not be equal to 0.

With poc_reset_flag, the POC value alignment across layers can be guaranteed, when IDR pictures and non-IRAP pictures are present in an access unit.

However, when a bitstream is spliced and a CRA picture type is changed to a BLA picture type at that point, the POC value alignment cannot be ensured. When an each layer starts up with an IRAP picture with NoRaslOutputFlag equal to 1 gradually from the spliced point of a bitstream, if an IRAP picture of non-base layer is equal to a BLA picture or a CRA with HandleCraAsBlaFlag equal to 1, that picture may or may not have the same POC value with the base layer because the MSB value is reset equal to 0.

- In addition, a HEVC version 1 decoder cannot recognize the POC reset indication, which may result in a situation for which there appears to be no appropriate value for the POC LSBs to send in the base layer because the appropriate LSB values to send should be different depending on whether the POC has been reset or not.

In this proposal (JCTVC-O0140 revision 1), we propose that the poc_reset_flag is replaced with poc_msb_reset_flag. POC LSB values will be reset only at IDR pictures in base layers. If a base layer picture is IDR, slice_pic_order_cnt_lsb equal to 0 will be signaled in slice headers of non-base layers. Otherwise, POC LSB will not be reset to 0. Therefore, the indication of POC LSB reset is not needed for both a normal CVS start-up with IDR alignment and a layer-wise start-up. The main problem with POC value alignment is POC MSB reset, because POC MSB values are derived from the previous picture without explicit signaling in any slice header. When a picture in a layer is IDR or BLA, other pictures need a POC MSB reset indication for POC value alignment. Also, when a layer is initiated in a layer-wise start-up, POC MSB reset indication is useful. Another benefit of poc_msb_reset_flag is the compatibility with HEVC version 1 decoders. If only POC MSB values are reset, there is no POC LSB mismatch problem.

For error resilience aspects, POC MSB reset can be indicated in successive pictures for improved error robustness. POC reset (all reset of MSB & LSB) can be indicated only at specific pictures in which POC values is forced to be equal to 0. However, POC MSB reset can be indicated in success pictures from POC reset points to POC MSB increment points. This feature can improve the error robustness for POC value alignments.

slice_segment_header( ) {; Descriptor. .... if( !dependent_slice_segment_flag ) {. i = 0. if( num_extra_slice_header_bits > i ) {. i++. poc_msb_reset_flag; u(1). }. if( num_extra_slice_header_bits > i ) {. i++. poc_reset_flag; u(1). }. if( num_extra_slice_header_bits > i ) {. i++. discardable_flag; u(1). }. for( i = 1; i < num_extra_slice_header_bits; i++ ). slice_reserved_flag[ i ]; u(1). .... }. poc_msb_reset_flag equal to 1 specifies that the PicOrderCntMsb for the current picture is equal to 0. poc_msb_reset_flag equal to 0 specifies that the derived PicOrderCntMsb for the current picture may or may not be equal to 0. It is a requirement of bitstream conformance that when cross_layer_irap_aligned_flag is equal to 1, the value of poc_msb_reset_flag shall be equal to 0. When not present, the value of poc_msb_reset_flag is inferred to be equal to 0. poc_reset_flag equal to 1 specifies that the derived picture order count for the current picture is equal to 0. poc_reset_flag equal to 0 specifies that the derived picture order count for the current picture may or may not be equal to 0. It is a requirement of bitstream conformance that when cross_layer_irap_aligned_flag is equal to 1, the value of poc_reset_flag shall be equal to 0. When not present, the value of poc_reset_flag is inferred to be equal to 0.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).