| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0141 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0054 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: Clean-ups of parameter sets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Yongjin Cho Min Woo Park Jin Young Lee Hochun Wey Chanyul Kim | Email: | b.d.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

Several clean-ups and improvements of syntax elements in parameter sets are proposed. The proposed modifications are summarized below.

- all_layer_independent_flag is proposed.

- direct_dep_type_present_flag is proposed.

- When splitting_flag is equal to 1, vps_nuh_layer_id_present_flag shall be equal to 1.

- When i is greater than 0 and splitting_flag is equal to 0, layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i – 1 ].

- view_id_present_flag is proposed.

- vps_profile_tier_level_present_flag is proposed.

- If NumScalabilityTypes is equal to 1, dimension_id[ i ][ j ] is not present and dimension_id[ i ][ 0 ] is inferred to be equal to layer_id_in_nuh[ i ].

- If NumScalabilityTypes is equal to 1, dimension_id_len_minus1[ j ]is not present and dimension_id_len_minus1[ 0 ] is inferred to be equal to 6.

- If nuh_layer_id is greater than 0,  sps_max_num_reorder_pics[ i ] and sps_max_latency_increase_plus1[ i ] are not present and inferred to be equal to vps_max_num_reorder_pics[ i ] and vps_max_latency_increase_plus1[ i ].

# Introduction

In the current VPS extension, a number of syntax elements are signalled to carry layer-dependencies, scalability dimension identifiers, and other information to be helpful for session negotiation. As reported in JCTVC-O109, the total bits of VPS NAL units are not negligible compared to the VCL NAL units, when each VPS contains full representations on all layers. Even if the various syntax elements in VPS extension derive high flexibilities to construct various cross-layer dependencies and provide plentiful information for session negotiation, a simple representation will be enough for some bitstreams.

Therefore, we assert that it is desirable to have an option to skip most syntax elements and to set them equal to their default values that are mostly used in VPS extension. In this contribution, several clean-ups and improvements of syntax elements in parameter sets, especially VPS extension, are proposed.

# On video parameter set extension

## On direct_dependency_flag

- all_layer_independent_flag is proposed.
- all_layer_independent_flag equal to 1 specifies that direct_dependency_flag[ i ][ j ] is not present and is inferred to be equal to 0. all_layer_independent_flag equal to 0 specifies that direct_dependency_flag[ i ][ j ] is present.

## On direct_dependency_type

- direct_dep_type_present_flag is proposed.
- direct_dep_type_present_flag equal to 1 indicates that direct_dep_type_len_minus2 and direct_dependency_type[ i ][ j ] are present. direct_dep_type_present_flag equal to 0 indicates that direct_dep_type_len_minus2 and direct_dependency_type[ i ][ j ] are not present. When direct_dep_type_present_flag is equal to 0, direct_dep_type_len_minus2 is inferred to be equal to 0, and direct_dependency_type[i][j] is inferred to be equal to 1. When not present, direct_dep_type_present_flag is inferred to be equal to 0.

## On splitting_flag

- When splitting_flag is equal to 1, vps_nuh_layer_id_present_flag shall be equal to 1.
- To enable the sparse distribution of nuh_layer_id, layer_id_in_nuh[ i ] shall be signaled when splitting_flag is equal to 1.

- When i is greater than 0 and splitting_flag is equal to 0, layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i – 1 ].
- When splitting_flag is equal to 1 and multiple scalability dimensions are used, it is difficult to align the decoding order and the order of layer_id_in_nuh[ i ] values. Therefore, the constraint that layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i – 1 ] shall be available only when splitting_flag is equal to 0.

## On view_id_val

- view_id_present_flag is proposed.
- view_id_present_flag equal to 0 indicates that view_id_val[ i ] is not present and is inferred to be equal to ViewOrderIdx[ layer_id_in_nuh[ i ] ]. view_id_present_flag equal to 0 indicates that view_id_val[ i ] is present.

## On signalling of profile, tier, and level information

- vps_profile_tier_level_present_flag is proposed.
- vps_profile_tier_level_present_flag equal to 1 indicates that profile, tier, and level information for each output layer set is signalled. vps_profile_tier_level_present_flag equal to 0 indicates that profile, tier, and level information for each output layer set are not signalled.

## On single scalability type

- If NumScalabilityTypes is equal to 1, dimension_id[ i ][ j ] is not present and dimension_id[ i ][ 0 ] is inferred to be equal to layer_id_in_nuh[ i ].
- When only one dimension_id[ i ][ 0 ] is used, there is no reason to re-map layer_id_in_nuh[ i ] to dimension_id[ i ][ 0 ].

- If NumScalabilityTypes is equal to 1, dimension_id_len_minus1[ j ]is not present and dimension_id_len_minus1[ 0 ] is inferred to be equal to 6.
- When only one dimension_id[ i ][ 0 ] is used, there is no reason to reset the length of dimension_id[ i ][ 0 ] to be equal to 6.

# On sequence parameter set

- If nuh_layer_id is greater than 0,  sps_max_num_reorder_pics[ i ] and sps_max_latency_increase_plus1[ i ] are not present and inferred to be equal to vps_max_num_reorder_pics[ i ] and vps_max_latency_increase_plus1[ i ].

# Proposed specification text

| vps_extension( ) { | Descriptor |
| --- | --- |
| avc_base_layer_flag | u(1) |
| vps_vui_offset | u(16) |
| splitting_flag | u(1) |
| for( i = 0, NumScalabilityTypes = 0; i < 16; i++ ) { |  |
| scalability_mask_flag[ i ] | u(1) |
| NumScalabilityTypes += scalability_mask_flag[ i ] |  |
| } |  |
| if( NumScalabilityTypes > 1 ) |  |
| for( j = 0; j < ( NumScalabilityTypes − splitting_flag ); j++ ) |  |
| dimension_id_len_minus1[ j ] | u(3) |
| if(!splitting_flag) |  |
| vps_nuh_layer_id_present_flag | u(1) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) { |  |
| if( vps_nuh_layer_id_present_flag ) |  |
| layer_id_in_nuh[ i ] | u(6) |
| if(NumScalabilityTypes > 1 && !splitting_flag) |  |
| for( j = 0; j < NumScalabilityTypes; j++ ) |  |
| dimension_id[ i ][ j ] | u(v) |
| } |  |
| view_id_present_flag | u( 1 ) |
| if( view_id_present_flag ) { |  |
| if( NumViews > 1 ) |  |
| view_id_len_minus1 | u(4) |
| for( i = 0; i < NumViews; i++ ) |  |
| view_id_val[ i ] | u(v) |
| } |  |
| all_layer_independent_flag | u( 1 ) |
| if( !all_layer_independent_flag ) { |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| direct_dependency_flag[ i ][ j ] | u(1) |
| } |  |
| max_tid_ref_present_flag | u(1) |
| if( max_tid_ref_present_flag ) |  |
| for( i = 0; i < vps_max_layers_minus1; i++ ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| all_ref_layers_active_flag | u(1) |
| vps_number_layer_sets_minus1 | u(10) |
| vps_profile_tier_level_present_flag | u(1) |
| if( vps_profile_tier_level_present_flag ) { |  |
| vps_num_profile_tier_level_minus1 | u(6) |
| for( i = 1; i <= vps_num_profile_tier_level_minus1; i ++ ) { |  |
| vps_profile_present_flag[ i ] | u(1) |
| if( !vps_profile_present_flag[ i ] ) |  |
| profile_ref_minus1[ i ] | u(6) |
| profile_tier_level( vps_profile_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| } |  |
| numOutputLayerSets = vps_number_layer_sets_minus1 + 1 |  |
| more_output_layer_sets_than_default_flag | u(1) |
| if( more_output_layer_sets_than_default_flag ) { |  |
| num_add_output_layer_sets_minus1 | u(10) |
| numOutputLayerSets += num_add_output_layer_sets_minus1 + 1 |  |
| } |  |
| if( numOutputLayerSets > 1 ) |  |
| default_one_target_output_layer_flag | u(1) |
| for( i = 1; i <numOutputLayerSets; i++ ) { |  |
| if( i > vps_number_layer_sets_minus1 ) { |  |
| output_layer_set_idx_minus1[ i ] | u(v) |
| lsIdx = output_layer_set_idx_minus1[ i ] + 1 |  |
| for( j = 0 ; j < NumLayersInIdList[ lsIdx ] − 1; j++) |  |
| output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| if( vps_profile_tier_level_present_flag ) |  |
| profile_level_tier_idx[ i ] | u(v) |
| } |  |
| rep_format_idx_present_flag | u(1) |
| if( rep_format_idx_present_flag ) |  |
| vps_num_rep_formats_minus1 | u(4) |
| for( i = 0; i <= vps_num_rep_formats_minus1; i++ ) |  |
| rep_format( ) |  |
| if( rep_format_idx_present_flag ) |  |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| if( vps_num_rep_formats_minus1 > 0 ) |  |
| vps_rep_format_idx[ i ] | u(4) |
| max_one_active_ref_layer_flag | u(1) |
| cross_layer_irap_aligned_flag | u(1) |
| if( !all_layer_independent_flag ) { |  |
| direct_dep_type_present_flag | u(1) |
| if( direct_dep_type_present_flag ) { |  |
| direct_dep_type_len_minus2 | ue(v) |
| for( i = 1; i <= vps_max_layers_minus1; i++ ) |  |
| for( j = 0; j < i; j++ ) |  |
| if( direct_dependency_flag[ i ][ j ] ) |  |
| direct_dependency_type[ i ][ j ] | u(v) |
| } |  |
| } |  |
| single_layer_for_non_irap_flag | u(1) |
| vps_vui_present_flag | u(1) |
| if( vps_vui_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| vps_vui_alignment_bit_equal_to_one | u(1) |
| vps_vui( ) |  |
| } |  |
| } |  |

vps_nuh_layer_id_present_flag equal to 1specifies that layer_id_in_nuh[ i ] for i from 0 to vps_max_layers_minus1, inclusive, are present. vps_nuh_layer_id_present_flag equal to 0 specifies that layer_id_in_nuh[ i ] for i from 0 to vps_max_layers_minus1, inclusive, are not present. When not present, vps_nuh_layer_id_present is inferred to be equal to 0.

layer_id_in_nuh[ i ] specifies the value of the nuh_layer_id syntax element in VCL NAL units of the i-th layer. For i in the range of 0 to vps_max_layers_minus1, inclusive, when layer_id_in_nuh[ i ] is not present, the value is inferred to be equal to i.

When i is greater than 0 and splitting_flag is equal to 0, layer_id_in_nuh[ i ] shall be greater than layer_id_in_nuh[ i – 1 ].

For i from 0 to vps_max_layers_minus1, inclusive, the variable LayerIdxInVps[ layer_id_in_nuh[ i ] ] is set equal to i.

all_layer_independent_flag equal to 1 specifies that direct_dependency_flag[ i ][ j ] is not present and is inferred to be equal to 0. all_layer_independent_flag equal to 0 specifies that direct_dependency_flag[ i ][ j ] is present.

direct_dep_type_present_flag equal to 1 indicates that direct_dep_type_len_minus2 and direct_dependency_type[ i ][ j ] are present. direct_dep_type_present_flag equal to 0 indicates that direct_dep_type_len_minus2 and direct_dependency_type[ i ][ j ] are not present. When direct_dep_type_present_flag is equal to 0, direct_dep_type_len_minus2 is inferred to be equal to 0, and direct_dependency_type[i][j] is inferred to be equal to 1. When not present, direct_dep_type_present_flag is inferred to be equal to 0.

direct_dep_type_present_flag equal to 1 indicates that direct_dep_type_len_minus2 and direct_dependency_type[ i ][ j ] are present. direct_dep_type_present_flag equal to 0 indicates that direct_dep_type_len_minus2 and direct_dependency_type[ i ][ j ] are not present. When direct_dep_type_present_flag is equal to 0, direct_dep_type_len_minus2 is inferred to be equal to 0, and direct_dependency_type[i][j] is inferred to be equal to 1. When not present, direct_dep_type_present_flag is inferred to be equal to 0.

direct_dep_type_len_minus2 plus 2 specifies the number of bits of the direct_dependency_type[ i ][ j ] syntax element. In bitstreams conforming to this version of this Specification the value of direct_dep_type_len_minus2 shall be equal 0. Although the value of direct_dep_type_len_minus2 shall be equal to 0 in this version of this Specification, decoders shall allow other values of direct_dep_type_len_minus2 in the range of 0 to 30, inclusive, to appear in the syntax. When not present, direct_dep_type_len_minus2 is inferred to be equal to 0.

direct_dependency_type[ i ][ j ] indicates the type of dependency between the layer with nuh_layer_id equal layer_id_in_nuh[ i ] and the layer with nuh_layer_id equal to layer_id_in_nuh[ j ]. direct_dependency_type[ i ][ j ] equal to 0 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer sample prediction but not for inter-layer motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ] equal to 1 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for inter-layer motion prediction but not for inter-layer sample prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. direct_dependency_type[ i ][ j ] equal to 2 indicates that the layer with nuh_layer_id equal to layer_id_in_nuh[ j ] is used for both inter-layer sample motion prediction and  inter-layer  motion prediction of the layer with nuh_layer_id equal layer_id_in_nuh[ i ]. Although the value of direct_dependency_type[ i ][ j ] shall be in the range of 0 to 2, inclusive, in this version of this Specification, decoders shall allow values of direct_dependency_type[ i ][ j ] in the range of 3 to 232 − 2, inclusive, to appear in the syntax. When not present, direct_dependency_type[ i ][ j ] is inferred to be equal to 1.

view_id_present_flag equal to 0 indicates that view_id_val[ i ] is not present and is inferred to be equal to ViewOrderIdx[ layer_id_in_nuh[ i ] ]. view_id_present_flag equal to 0 indicates that view_id_val[ i ] is present.

dimension_id[ i ][ j ] specifies the identifier of the j-th present scalability dimension type of the i-th layer. The number of bits used for the representation of dimension_id[ i ][ j ] is dimension_id_len_minus1[ j ] + 1 bits.

If NumScalabilityTypes is equal to 1, dimension_id[ i ][ 0 ] is inferred to be equal to layer_id_in_nuh[ i ].

If splitting_flag is equal to 1, for i from 0 to vps_max_layers_minus1, inclusive, and j from 0 to NumScalabilityTypes − 1, inclusive, dimension_id[ i ][ j ] is inferred to be equal to ( ( layer_id_in_nuh[ i ] & ( (1 << dimBitOffset[ j + 1 ] ) − 1) ) >> dimBitOffset[ j ] ).

Otherwise, for j from 0 to NumScalabilityTypes − 1, inclusive, dimension_id[ 0 ][ j ] is inferred to be equal to 0.

dimension_id_len_minus1[ j ] plus 1 specifies the length, in bits, of the dimension_id[ i ][ j ] syntax element.

If NumScalabilityTypes is equal to 1, dimension_id_len_minus1[ 0 ] is inferred to be equal to 6.

When splitting_flag is equal to 1, the following applies:

–	The variable dimBitOffset[ 0 ] is set equal to 0 and for j in the range of 1 to  NumScalabilityTypes − 1 , inclusive, dimBitOffset[ j ] is derived as follows:

dimBitOffset[ j ]=dimIdx=0j−1dimension_id_len_minus1[ dimIdx ]+1	(F-1)

–	The value of dimension_id_len_minus1[ NumScalabilityTypes − 1 ] is inferred to be equal to 5 − dimBitOffset[ NumScalabilityTypes − 1 ].

–	The value of dimBitOffset[ NumScalabilityTypes ] is set equal to 6.

It is a requirement of bitstream conformance that when NumScalabilityTypes is greater than 0, dimBitOffset[ NumScalabilityTypes − 1 ] shall be less than 6.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| if( nuh_layer_id = = 0 ) { |  |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| } |  |
| sps_seq_parameter_set_id | ue(v) |
| if( nuh_layer_id > 0 ) |  |
| update_rep_format_flag | u(1) |
| if( update_rep_format_flag ) { |  |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| } |  |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| if( update_rep_format_flag ) { |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| } |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| if( nuh_layer_id == 0) { |  |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enabled_flag ) { |  |
| if( nuh_layer_id > 0 ) |  |
| sps_infer_scaling_list_flag | u(1) |
| if( sps_infer_scaling_list_flag ) |  |
| sps_scaling_list_ref_layer_id | u(6) |
| else { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| short_term_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| strong_intra_smoothing_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) { |  |
| sps_extension( ) |  |
| sps_extension2_flag | u(1) |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

- sps_max_num_reorder_pics[ i ] indicates the maximum allowed number of pictures that can precede any picture in the CVS in decoding order and follow that picture in output order when HighestTid is equal to i. The value of sps_max_num_reorder_pics[ i ] shall be in the range of 0 to sps_max_dec_pic_buffering_minus1[ i ], inclusive. When i is greater than 0, sps_max_num_reorder_pics[ i ] shall be greater than or equal to sps_max_num_reorder_pics[ i − 1 ]. The value of sps_max_num_reorder_pics[ i ] shall be less than or equal to vps_max_num_reorder_pics[ i ] for each value of i. When sps_max_num_reorder_pics[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_num_reorder_pics[ sps_max_sub_layers_minus1 ]. When sps_max_num_reorder_pics[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1, inclusive, due to nuh_layer_id greater than 0, it is inferred to be equal to vps_max_num_reorder_pics[ i ].

sps_max_latency_increase_plus1[ i ] not equal to 0 is used to compute the value of SpsMaxLatencyPictures[ i ], which specifies the maximum number of pictures that can precede any picture in the CVS in output order and follow that picture in decoding order when HighestTid is equal to i.

- When sps_max_latency_increase_plus1[ i ] is not equal to 0, the value of SpsMaxLatencyPictures[ i ] is specified as follows:

SpsMaxLatencyPictures[ i ] = sps_max_num_reorder_pics[ i ] + 	(7-2)		sps_max_latency_increase_plus1[ i ] − 1

When sps_max_latency_increase_plus1[ i ] is equal to 0, no corresponding limit is expressed.

The value of sps_max_latency_increase_plus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. When vps_max_latency_increase_plus1[ i ] is not equal to 0, the value of sps_max_latency_increase_plus1[ i ] shall not be equal to 0 and shall be less than or equal to vps_max_latency_increase_plus1[ i ] for each value of i. When sps_max_latency_increase_plus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1 − 1, inclusive, due to sps_sub_layer_ordering_info_present_flag being equal to 0, it is inferred to be equal to sps_max_latency_increase_plus1[ sps_max_sub_layers_minus1 ]. When sps_max_latency_increase_plus1[ i ] is not present for i in the range of 0 to sps_max_sub_layers_minus1, inclusive, due to nuh_layer_id greater than 0, it is inferred to be equal to vps_max_latency_increase_plus1[ sps_max_sub_layers_minus1 ].

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).