| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0142 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0055 |
| --- | --- |

| Title: | Conditional SPS extension syntax for RExt, SHVC, and MV-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce 433 Hackensack Ave. Hackensack, NJ 07601, USA | Tel: Email: | jill@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

Multiple extensions to HEVC (RExt, MV-HEVC, SHVC) are currently being developed in parallel. It is asserted to be desirable to be able to implement a codec for a particular extension without having to be aware of the other extensions. It is proposed to add flags in the SPS extension to indicate conditional inclusion of SPS extension syntax for particular extensions, so that decoders are not required to parse SPS extension syntax elements related to unsupported extensions.  The proposed syntax enables future profile definitions which enable combinations of extension types,

It is also asserted that the recent drafts of the RExt (JCTVC-N1005_v3) and MV-HEVC (JCT3V-E1004) extensions have incompatible SPS extension syntax.  The proposed syntax addresses this incompatibility between extensions.

# Proposal

The current drafts of the RExt (JCTVC-N1005) and MV-HEVC (JCT3V-E1004_v6) extensions have incompatible SPS extension syntax.  A solution is needed to address this incompatibility.  Two options to address handling of multiple extensions are discussed.  The first option is considered to be undesirable, but is described for comparison purposes.  The second option is proposed.

It is proposed that when the sps_extension_present_flag is equal to 1, to send 8 additional flags, 1 per extension type, and conditionally include separate extension data for each indicated extension type.  The 8th flag could be used to indicate an additional extension. This approach is similar to the previous JCTVC-M0045 proposal #4.

## Undesirable option: chained extension flags

In this undesirable option, which is described only for comparison purposes, extension flags are chained.  The first standardized extension uses the first extension flag to indicate the presence of syntax elements related to the first extension, and a second extension flag is included.  The second standardized extension then uses the second extension flag to indicate the presence of syntax elements related to the second extension, and a third extension flag is included.

Bitstreams for the second standardized extension must include syntax elements related to the first standardized extension, even if the profile used is not one of those defined in the first standardized extension.

| … |  |
| --- | --- |
| sps_extension_flag0 | u(1) |
| if( sps_extension_flag0 ) { |  |
| sps_extension0( ) |  |
| sps_extension_flag1 | u(1) |
| if( sps_extension_flag1 ) { |  |
| sps_extension1( ) |  |
| sps_extension_flag2 | u(1) |
| if( sps_extension_flag2 ) { |  |
| sps_extension2( ) |  |
| sps_extension_flag3 | u(1) |
| if( sps_extension_flag3 ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

## Proposed option: conditional extension syntax

In the proposed option, when the sps_extension_present_flag is equal to 1, an additional 8 flags are present.  Each flag can be used to individually control whether syntax elements related to a particular extension type are present.  The last of the 8 additional flags can be used to indicate a further extension.

| … |  |
| --- | --- |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| for ( i = 0; i < 8; i++ ) |  |
| sps_extension_flag[ i ] | u(1) |
| if( sps_extension_flag[ 0 ] ) { |  |
| sps_extension0( ) |  |
| if( sps_extension_flag[ 1 ] ) { |  |
| sps_extension1( ) |  |
| if( sps_extension_flag[ 2 ] ) { |  |
| sps_extension2( ) |  |
| … |  |
| if( sps_extension_flag[ 7 ] ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

sps_extension_present_flag specifies that sps_extension_flag[ i ] for i in 0 .. 7 are present.

sps_extension_flag[ 0 ] equal to 1 specifies that the sps_extension0 syntax structure is present. sps_extension_flag[ 0 ] equal to 0 specifies that the sps_extension0 syntax structure is not present.

sps_extension_flag[ 1 ] equal to 1 specifies that the sps_extension1 syntax structure is present. sps_extension_flag[ 1 ] equal to 0 specifies that the sps_extension1 syntax structure is not present.

sps_extension_flag[ 2 ] equal to 1 specifies that the sps_extension2 syntax structure is present. sps_extension_flag[ 2 ] equal to 0 specifies that the sps_extension2 syntax structure is not present.

sps_extension_flag[ i ] shall be equal to 0, for i in the range of 3 .. 6, in bitstreams conforming to this version of this Specification. The value of 1 for sps_extension_flag[ i ] is reserved for future use by ITU-T | ISO/IEC.

sps_extension_flag[ 7 ] equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the SPS RBSP syntax structure. sps_extension_flag[ 7 ] shall be equal to 0 in bitstreams conforming to this version of this Specification. The value of 1 for sps_extension_flag[ 7 ] is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all sps_extension_data_flag syntax elements that follow the value 1 for sps_extension_flag[ 7 ] in an SPS NAL unit.

| Extension flag number | Extension |
| --- | --- |
| 0 | RExt |
| 1 | MV-HEVC |
| 2 | SHVC |
| 3-6 | Reserved for new extension syntax structure |
| 7 | Reserved for arbitrary extension |

## Discussion

The proposed approach allows codecs to avoid using syntax elements related to unsupported extensions.  However, it also allows future combinations of extension types, in which more than one extension flag is set.  A profile defined in an extension may require that a particular extension flag be set, and hence a particular extension be present, if desirable. However, profiles may be defined which require that the unrelated extension types not be present in the SPS of a layer which conforms to a particular profile.

So, for example, stereo main profile could require that sps_extension_flag[ 0 ] and sps_extension_flag[ 2 ] shall be equal to 0.  However, a hypothetical future stereo main 4:2:2 10 profile may allow sps_extension_flag[ 0 ] to be equal to 1.

The same approach could be applied to the PPS.  However, the current MV-HEVC, SHVC, and 3D-HEVC drafts do not use the PPS extension flag.

# Existing SPS extension syntax in draft specifications

For reference, the existing SPS extension syntax for the extensions are copied in this section.

## RExt syntax in JCTVC-N1005

| … |  |
| --- | --- |
| sps_extension1_flag | u(1) |
| if( sps_extension1_flag ) { |  |
| transform_skip_rotation_enabled_flag | u(1) |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| residual_dpcm_intra_enabled_flag | u(1) |
| residual_dpcm_inter_enabled_flag | u(1) |
| extended_precision_processing_flag | u(1) |
| intra_smoothing_disabled_flag | u(1) |
| sps_extension2_flag | u(1) |
| } |  |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |

## MV-HEVC Syntax in JCT3V-E1004

| sps_extension_flag | u(1) |
| --- | --- |
| if( sps_extension_flag ) { |  |
| sps_extension( ) |  |
| sps_extension2_flag | u(1) |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| sps_shvc_reserved_zero_idc | ue(v) |
| } |  |

## SHVC Syntax in JCTVC-N1008

| sps_extension_flag | u(1) |
| --- | --- |
| if( sps_extension_flag ) { |  |
| sps_extension( ) |  |
| sps_extension2_flag | u(1) |
| if( sps_extension2_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| } |  |

| sps_extension( ) { | Descriptor |
| --- | --- |
| inter_view_mv_vert_constraint_flag | u(1) |
| num_scaled_ref_layer_offsets | ue(v) |
| for( i = 0; i < num_scaled_ref_layer_offsets; i++) { |  |
| scaled_ref_layer_left_offset[ i ] | se(v) |
| scaled_ref_layer_top_offset[ i ] | se(v) |
| scaled_ref_layer_right_offset[ i ] | se(v) |
| scaled_ref_layer_bottom_offset[ i ] | se(v) |
| } |  |
| } |  |

# Patent rights declaration(s)

Vidyo, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).