| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0144_v2 |
| --- | --- |

| Title: | AHG5 and AHG8: Recommended profiling of range extension coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunil Lee Elena Alshina Chanyul Kim Ken McCann | Email: | sunil.lee@samsung.com elena_a.alshina@samsung.com dionism@samsung.com ken@zetacast.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution provides recommendations on the profiling of range extension coding tools. It is suggested that the decision on profiling for the range extension should follow a similar process to that used for HEVC v1, i.e. it should be based on an assessment of the trade-off between coding gain and complexity of the additional range extension coding tools, evaluated under the common test conditions of HEVC range extensions. It is asserted that under these conditions the coding gain of the five tools tested in this contribution is so negligible that the trade-off is not beneficial.

Considering that the stated purpose of the HEVC range extension is to support high fidelity video signals in the high end consumer and professional environment, it is suggested not to include additional coding tools in the set of generic range extension profiles. It is also suggested that a test model for Screen Content (SC) 4:4:4 Profiles, including all of the currently adopted range extension coding tools, should be developed, in coordination with the the call for proposals for coding of screen content and medical visual content planned for January 2014.

# Introduction

At the last JCT-VC meeting in Vienna, five HEVC range extension profiles were defined: Main 12, Main 4:2:2 10/12, and Main 4:4:4 10/12. However, it was not yet determined which range extension coding tools were to be included in each profile.

In addition to the design improvements to properly support high bit-depth and extended colour format, the range extension coding tools which have been adopted so far include:

- intra residual DPCM (RDPCM) [1][2]

- inter RDPCM [3]

- residual rotation [4]

- single context model for significance map coding in transform-skipped (TS) block [5]

- intra block copy [6]

In this contribution, the coding gain and complexity of these additional coding tools are evaluated under the common test condition of HEVC range extensions, and a profiling of the coding tools is recommended based on the test results.

# Test results

The coding gain and complexity of the additional coding tools are evaluated under the common test condition of HEVC range extensions [7]. In all the tests, the anchor is “coding tool off” and the tested condition is “coding tool on”, i.e. the tested results are always the HM12.0-RExt4.1 anchors. The test results show how much coding gain can be provided by the coding tools when they are enabled, and how encoding and decoding time change. Tables 1, 2, 3, 4, and 5 summarize the test results for intra RDPCM, inter RDPCM, residual rotation, single context model for significance map coding in TS block, and intra block copy, respectively.

The results show that the coding gain provided by the additional range extension coding tools is not significant for the test sequences in the common test condition. This has been already confirmed when those coding tools were reviewed and adopted, since they were mainly for screen content (SC) coding. The complexity of the coding tools measured in terms of encoding and decoding time was not significant for intra/inter RDPCM, residual rotation, and single context model for significance map coding in TS block. However, intra block copy significantly increases the encoding time. Intra block copy is also known to increase the cost of practical implementations, since the samples within the search range (currently two CTUs) should be kept in intra slices.

Table 1. Performance of intra RDPCM on vs. off

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| YCbCr 4:2:2 | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 101% | 100% | 101% | 101% | 101% | 100% |
| Dec Time[%] | 100% | 100% | 100% | 100% | 100% | 99% | 98% |

Table 2. Performance of inter RDPCM on vs. off

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| YCbCr 4:2:2 | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% | 101% | 100% | 103% | 103% | 102% | 102% |
| Dec Time[%] | 101% | 101% | 101% | 98% | 97% | 96% | 97% |

Table 3. Performance of residual rotation on vs. off

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| YCbCr 4:4:4 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |
| YCbCr 4:2:2 | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| Overall | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% | 101% | 101% | 100% | 100% | 100% | 100% |
| Dec Time[%] | 99% | 100% | 100% | 96% | 97% | 96% | 97% |

Table 4. Performance of single context model for significance map coding in TS block on vs. off

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |
| YCbCr 4:4:4 | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| YCbCr 4:2:2 | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
| Dec Time[%] | 100% | 100% | 100% | 99% | 99% | 99% | 100% |

Table 5. Performance of intra block copy on vs. off

| BD-rate Y | AI-MT | AI-HT | AI-SHT | RA-MT | RA-HT | LB-MT | LB-HT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 | -0.7% | -0.3% | -0.2% | -0.4% | -0.1% | -0.2% | -0.1% |
| YCbCr 4:4:4 | -0.3% | -0.3% | -0.2% | -0.1% | -0.1% | 0.0% | 0.0% |
| YCbCr 4:2:2 | -0.3% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% |
| Overall | -0.4% | -0.3% | -0.1% | -0.2% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 158% | 155% | 152% | 119% | 117% | 114% | 112% |
| Dec Time[%] | 99% | 99% | 100% | 99% | 98% | 98% | 99% |

# Recommended profiling of range extension coding tools

The profiling of the range extension coding tools is recommended to follow a similar process to that used for HEVC v1. As shown in the test results, the coding gain of all the additional tools is minor for natural video sequences.

However, there remains a market requirement for HEVC range extensions which fulfill the planned purpose of HEVC Amd 1, within the planned timescale of Amd 1. The target date for FDAM is January 2014 and the rationale is specified [8] as follows:

“The purpose of this amendment of ISO/IEC 23008-2 is to provide the capability to support high fidelity video signals. In high end consumer and professional environment, larger sample accuracy and alternative chroma sampling structures such as 4:4:4 and 4:2:2 are used. This amendment is intended to extend the capability of ISO/IEC 23008-2 for those applications.”

Considering that the primary goal of HEVC range extensions is to support high bit-depth and extended colour format, it is suggested not to include any additional coding tools in the set of generic HEVC range extension profiles. Instead, it is suggested to develop Screen Content (SC) 4:4:4 Profiles from a Test Model which includes all of the currently adopted range extension coding tools, in coordination with the call for proposals for coding of screen content and medical visual content planned for January 2014.

The recommended mapping between profiles and coding tools is summarized in Table 6.

Table 6. HEVC range extension profiles and supported coding tools

| Range extension coding tools | Main 12 | Main 4:2:2 |  | Main 4:4:4 |  | SC 4:4:4 TM1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 | 12 | 10 | 12 | 10 | 12 |
| Intra RDPCM | X | X | X | X | X | O | O |
| Inter RDPCM | X | X | X | X | X | O | O |
| Residual rotation | X | X | X | X | X | O | O |
| Single ctx for TS | X | X | X | X | X | O | O |
| Intra block copy | X | X | X | X | X | O | O |

# Conclusion

In this contribution, it is suggested that the decision process for profiling of range extension coding tools should follow a similar process to that used for HEVC v1.

Based on the coding gain and complexity trade-off for the additional range extension coding tools, evaluated under HEVC range extension common test condition, it is suggested that the five additional coding tools considered in this proposal are not included in any of the five currently defined generic profiles. It is suggested that this proposed mapping between HEVC range extension profiles and coding tools is adopted into the next version of HEVC range extension text and reference software.

It is also suggested that SC 4:4:4 Profiles, which support all of the currently adopted range extension coding tools, should be developed, in coordination with the call for proposals for coding of screen content and medical visual content planned for January 2014. It may be appropriate to include the five additional coding tools considered in this proposal in the initial test model for this new activity.

# References

- M. Zhou, M. Budagavi, “RCE2: Experimental results on Test 3 and Test 4,” Document of Joint Collaborative Team on Video Coding, JCTVC-M0056, Apr. 2013.

- R. Joshi, J. Sole, M. Karczewicz, “RCE2 subtest C.2: Extension of residual DPCM to lossy coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0052, Jul. 2013.

- M. Naccari, M. Mrak, “RCE2: Experimental results for Test C.1,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0074, Jul. 2013.

- D. He, J. Wang, G. Martin-Cocher, “Rotation of Residual Block for Transform Skipping,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0093, Jun. 2012.

- T. Tsukuba, T. Yamamoto, “Constant coefficient context for intra transform skipping,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0069, Jun. 2012.

- C. Pang, J. Sole, L. Guo, M. Karczewicz, R. Joshi, “Non-RCE3: Intra Motion Compensation with 2-D MVs,” Document of Joint Collaborative Team on Video Coding, JCTVC-N0256, Jul. 2013.

- D. Flynn, K. Sharman, C. Rosewarne, “Common test conditions and software reference configurations for HEVC range extensions,” Document of Joint Collaborative Team on Video Coding, JCTVC-N1006, Aug. 2013.

- MPEG Video and JCT-VC, “Request for ISO/IEC 23008-2:201X/Amd.1”, ISO/IEC document JTC1/SC29/WG11/N13344, Jan. 2013

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).