| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0145 |
| --- | --- |

| Title: | [SCE2] Key picture concept and single loop decoding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Christian Feldmann Mathias Wien Institut für Nachrichtentechnik RWTH Aachen University 52056 Aachen, Germany | Email: Email: | feldmann@ient.rwth-aachen.de wien@ient.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

In this document a performance and complexity analysis of single loop decoding in SHVC using the key picture concept is presented in response to SCE2 test 2.3. The scheme has similarity with MGS as used in SVC. The enhancement layer rate-distortion performance is reported to be in the range of 1.2 % BD delta rate comparable to the multi-loop SHM CTC configuration while the reconstruction of the base layer only is reportedly reduced by 0.20 dB BD-PSNR for RA and 0.47 dB BD-PSNR for the LDB configuration. The AHG17 measurements of the presented single loop decoding scheme are reported to be in the range of 59-73 % of the memory access of SHM3.0.1 under CTC.

Additionally, results for a modified scheme are presented where the deblocking filter and SAO are enabled for the base layer reconstruction of the key pictures.  The results show comparable performance to the multi-loop SHM CTC configuration for the full reconstruction while the drift for reconstruction of the base layer only is reported to be in the range of 0.19 dB BD-PSNR for RA and 0.47 dB BD-PSNR for the LDB configuration. The AHG17 measurements are reported to be in the range of 59-69 % of the memory access of SHM3.0.1 under CTC.

# Introduction

In this document the integration of the key picture concept as presented in JCTVC-N0202 into the SHM 3.0.1 reference software is presented. This modification of the prediction scheme only requires minimal changes to the reference decoder. The scheme has some similarity with MGS as used in SVC.

Some restrictions for the base layer have been applied in order to allow for single loop decoding of the enhancement layer. However, the conventional multi-loop based approach can also be used for decoding.

In conformance with SCE2, PSNR results are presented for the Random Access (RA) and the Low Delay B (LDB) configuration. In addition the complexity of the single loop decoding approach is reported in accord with the AHG17 complexity assessment and pixel level complexity measurements are provided.

Also a modification to the integration of the key picture concept as presented in JCTVC-N0202 is presented and evaluated in this document.

# Single loop decoding

## Key picture concept

### Configuration for Common Testing Conditions

For the CTC random access configuration every first frame of the GOP in the base layer (0, 8, 16…) is a key-picture. For the low-delay B configuration every fourth frame (0, 4, 8 …) is a key-picture. These base layer key-pictures only use other base layer key-pictures as reference for inter prediction. All non-key-pictures in the base layer use the reconstructed enhancement layer pictures for prediction. See Table 1 and Table 2 for a list of references used in the base layer.

For all base layer frames, the temporal motion vector predictor candidate is always derived from the base layer motion. This way it can be ensured that the correct motion is decoded when only decoding the base layer.

| POC | Base layer references | Enhancement layer references |
| --- | --- | --- |
| 0 | -8, -16 |  |
| 8 | 0, -8 |  |
| 4 |  | 0, -2, 8 |
| 2 |  | 0, -2, 4, 8 |
| 1 |  | 0, 2, 4 |
| 3 |  | 2, 0, 4, 8 |
| 6 |  | 4, 2, 8 |
| 5 |  | 4, 0, 6, 8 |
| 7 |  | 6, 4, 8 |

Table 1 - Used references in the base layer for RA configuration

| POC | Base layer references | Enhancement layer references |
| --- | --- | --- |
| 0 | -4, -8, -12, -16 |  |
| 1 |  | 0, -4, -8, -12 |
| 2 |  | 1, 0, -4, -8 |
| 3 |  | 2, 0, -4, -8 |

Table 2 - Used references in the base layer for LDB configuration

### Configuration for Temporal Scalability

Table 3 and Table 4 show the modified list of references in the base layer when temporal scalability is enabled.

| POC | Base layer references | Enhancement layer references |
| --- | --- | --- |
| 0 | -8, -16 |  |
| 8 | 0, -8 |  |
| 4 |  | 0, -4, 8 |
| 2 |  | 0, -4, 4, 8 |
| 1 |  | 0, 2, 4 |
| 3 |  | 2, 0, 4, 8 |
| 6 |  | 4, 0, 8 |
| 5 |  | 4, 0, 6, 8 |
| 7 |  | 6, 4, 8 |

Table 3 - Used references in the base layer for RA configuration and temporal scalability

| POC | Base layer references | Enhancement layer references |
| --- | --- | --- |
| 0 | -4, -8, -12, -16 |  |
| 1 |  | 0, -4, -8, -12 |
| 2 |  | 1, 0, -4, -8 |
| 3 |  | 2, 0, -4, -8 |

Table 4 - Used references in the base layer for LDB configuration and temporal scalability

## Base layer restrictions

In order to allow for single loop decoding, the coding scheme is modified:

- The enhancement layer uses the unfiltered lower layer reference for the refIdx based prediction

- Constrained intra prediction is enabled in the base layer

- Only zero motion is allowed for prediction from the base layer reconstruction

These modifications make it possible to decode the enhancement layer without full reconstruction of the base layer. In the base layer, the key-pictures have to be fully reconstructed. In non-key-pictures, only the intra coded CUs have to be reconstructed. Since no deblocking or SAO is performed in the base layer and constrained intra prediction is used, motion compensation does not need to be performed in the base layer if the full stream is decoded.

In the enhancement layer conventional intra or inter prediction can be used as well as SHM like inter layer prediction. If the base layer information is used, the enhancement layer decoder has to perform motion compensation with the motion vector and reference from the base layer and reconstruct the residual signal from the base layer. This is possible since base and enhancement layer use the same references for inter prediction. The enhancement layer can then add another residual signal to the reconstruction.

The proposed modifications make the scheme similar to the medium grain SNR scalability approach followed in SVC.

## Single loop decoding with EL prediction restriction

In the described coding scheme the situation can arise where 3 motion compensation operations are required for the single loop decoding operation in the enhancement layer (bi-prediction in the BL and bi-prediction in the EL using the base layer as a reference). In order to avoid this situation, the enhancement layer is forced to only use uni-prediction if it references a BL block that already uses bi-prediction.

## Single loop decoding with prediction from filtered key pictures

According to proposal JCTVC-N0202 the enhancement layer prediction in key pictures uses the unfiltered lower layer reconstruction picture for inter layer prediction. For single loop decoding, the lower layer key pictures are filtered for prediction of the other base layer key pictures. Here, the proposed scheme is modified to use the filtered lower layer key pictures for prediction in the enhancement layer.

# Results

## PSNR Results

BD-Rate and BD-PSNR results are reported for all tested configurations. As the reconstructed BL differs from the SHM 3.0.1 reconstructed BL, BD measurements are provided for both, the enhancement layer and the base layer. The BD deltas reported for in the BL ONLY tables are induced by two factors:

- The modifications relative to the CTC anchor configuration, i.e. the restriction to constrained intra prediction and the modified prediction references,

- Drift which is induced by the proposed scheme when decoding the base layer only.

### Single loop decoding

|  | BD-Rate |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA HEVC SNR |  |  |  | RA HEVC SNR BL ONLY |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A | 1,6% | -7,0% | -7,1% |  | 6,7% | 3,5% | 3,6% |
| Class B | 1,1% | -5,4% | -7,7% |  | 6,7% | 4,7% | 4,6% |
| Overall (Test vs Ref) | 1,2% | -5,9% | -7,5% |  | 6,7% | 4,4% | 4,3% |
| Overall (Test vs single layer) | 15,8% | 23,4% | 23,1% |  |  |  |  |
| Overall (Ref vs single layer) | 14,4% | 32,1% | 34,2% |  |  |  |  |
| EL only (Test vs Ref) | 1,2% | -6,5% | -8,3% |  |  |  |  |

Table 5 - BD-Rate results for the proposed single loop decoding scheme using the RA configuration

|  | BD-PSNR (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA HEVC SNR |  |  |  | RA HEVC SNR BL ONLY |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A | -0,0703 | 0,1842 | 0,1464 |  | -0,2839 | -0,0748 | -0,0674 |
| Class B | -0,0261 | 0,0704 | 0,1206 |  | -0,1672 | -0,0630 | -0,0841 |
| Overall (Test vs Ref) | -0,0387 | 0,1029 | 0,1280 |  | -0,2006 | -0,0664 | -0,0794 |

Table 6 - BD-PSNR results for the proposed single loop decoding scheme using the RA configuration

|  | BD-Rate |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | LD-B HEVC SNR |  |  |  | LD-B HEVC SNR BL ONLY |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A | 1,8% | -5,2% | -5,8% |  | 15,3% | 5,0% | 3,5% |
| Class B | 1,1% | -3,7% | -5,5% |  | 15,6% | 6,6% | 5,5% |
| Overall (Test vs Ref) | 1,3% | -4,1% | -5,6% |  | 15,5% | 6,1% | 4,9% |
| Overall (Test vs single layer) | 25,9% | 28,5% | 30,9% |  |  |  |  |
| Overall (Ref vs single layer) | 24,3% | 34,7% | 39,5% |  |  |  |  |
| EL only (Test vs Ref) | -1,1% | -6,7% | -8,3% |  |  |  |  |

Table 7 - BD-Rate results for the proposed single loop decoding scheme using the LDB configuration

|  | BD-PSNR (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | LD-B HEVC SNR |  |  |  | LD-B HEVC SNR BL ONLY |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A | -0,1009 | 0,1259 | 0,1178 |  | -0,6292 | -0,1031 | -0,0618 |
| Class B | -0,0262 | 0,0467 | 0,0758 |  | -0,4130 | -0,0873 | -0,1176 |
| Overall (Test vs Ref) | -0,0475 | 0,0694 | 0,0878 |  | -0,4747 | -0,0918 | -0,1017 |

Table 8 - BD-PSNR results for the proposed single loop decoding scheme using the LDB configuration

The reference results were created following the random access configuration and the common test conditions [1]. For the key picture concept the prediction structure was changed as described in (2.1).

Table 5 shows the BD-rate results for the implemented key picture concept with single loop decoding compared to the unmodified SHM3.0.1 reference using the RA configuration. On the right (“RA HEVC SNR BL ONLY”) the BD-rate results when decoding only the base layer are shown. Table 6 shows the corresponding delta PSNR results. Table 7 and Table 8 show the corresponding results for the LDB configuration.

The full table is available in the accompanying data sheet [JCTVC-O0145-RWTH-PSNR-SingleLoopDecoding.xls]

As reported in [2], the introduction of the key picture concept in a multi-loop configuration can improve the RD performance when decoding both, the base and the enhancement layer. With the restriction to the single-loop configuration, a slight loss compared to the CTC anchor is observed. The complexity impact of this restriction to the single-loop configuration is reported in Sections 3.2 and 3.3.

### Single loop decoding with EL prediction restriction

|  | BD-Rate |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA HEVC SNR |  |  |  | RA HEVC SNR BL ONLY |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A | 2,1% | -6,6% | -6,9% |  | 10,0% | 3,8% | 4,0% |
| Class B | 1,9% | -5,3% | -7,8% |  | 8,8% | 5,2% | 5,1% |
| Overall (Test vs Ref) | 2,0% | -5,7% | -7,6% |  | 9,1% | 4,8% | 4,8% |
| Overall (Test vs single layer) | 16,6% | 23,7% | 23,1% |  |  |  |  |
| Overall (Ref vs single layer) | 14,4% | 32,1% | 34,2% |  |  |  |  |
| EL only (Test vs Ref) | 2,1% | -6,2% | -8,3% |  |  |  |  |

Table 9 - BD-Rate results for the proposed single loop decoding scheme using the RA configuration and the EL prediction restriction

|  | BD-PSNR (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA HEVC SNR |  |  |  | RA HEVC SNR BL ONLY |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A | -0,0872 | 0,1749 | 0,1417 |  | -0,4142 | -0,0829 | -0,0747 |
| Class B | -0,0418 | 0,0674 | 0,1190 |  | -0,2258 | -0,0694 | -0,0938 |
| Overall (Test vs Ref) | -0,0548 | 0,0981 | 0,1255 |  | -0,2796 | -0,0732 | -0,0883 |

Table 10 - BD-PSNR results for the proposed single loop decoding scheme using the RA configuration and the EL prediction restriction

|  | BD-Rate |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | LD-B HEVC SNR |  |  |  | LD-B HEVC SNR BL ONLY |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A | 2,7% | -4,6% | -5,4% |  | 15,3% | 5,0% | 3,5% |
| Class B | 2,4% | -3,2% | -5,5% |  | 15,5% | 6,5% | 5,6% |
| Overall (Test vs Ref) | 2,5% | -3,6% | -5,5% |  | 15,5% | 6,1% | 5,0% |
| Overall (Test vs single layer) | 27,3% | 29,2% | 31,0% |  |  |  |  |
| Overall (Ref vs single layer) | 24,3% | 34,7% | 39,5% |  |  |  |  |
| EL only (Test vs Ref) | 0,5% | -5,8% | -7,7% |  |  |  |  |

Table 11 - BD-Rate results for the proposed single loop decoding scheme using the LDB configuration and the EL prediction restriction

|  | BD-PSNR (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | LD-B HEVC SNR |  |  |  | LD-B HEVC SNR BL ONLY |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A | -0,1325 | 0,1117 | 0,1097 |  | -0,6290 | -0,1026 | -0,0632 |
| Class B | -0,0562 | 0,0378 | 0,0707 |  | -0,4117 | -0,0871 | -0,1174 |
| Overall (Test vs Ref) | -0,0780 | 0,0589 | 0,0818 |  | -0,4737 | -0,0915 | -0,1019 |

Table 12 - BD-PSNR results for the proposed single loop decoding scheme using the LDB configuration and the EL prediction restriction

Table 9 and Table 10 shws the BD-rate and BD-PSNR results for the implemented key picture concept with single loop decoding using the EL prediction restriction as described in 2.3. The unmodified SHM3.0.1 is used as a reference.

Table 9 and Table 10 provide results for the RA configuration while Table 11 and Table 12 provide the results for the LDB configuration.

The full table is available in the accompanying data sheet [JCTVC-O0145-RWTH-PSNR-SingleLoopDecoding_ELRestriction.xls]

The results reveal a performance drop of about 1 % point relative to the results in Section 3.1.1 when restricting the EL prediction.  For the RA test case, the drift when decoding only the BL is increased by about 3 % points. The drift in the LDB configuration is comparable for both settings.

### Single loop decoding with prediction from filtered key pictures

|  | BD-Rate |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA HEVC SNR |  |  |  | RA HEVC SNR BL ONLY |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A | -0,7% | -8,2% | -8,1% |  | 6,5% | 3,3% | 3,5% |
| Class B | -0,7% | -6,3% | -8,1% |  | 6,5% | 4,6% | 4,5% |
| Overall (Test vs Ref) | -0,7% | -6,8% | -8,1% |  | 6,5% | 4,2% | 4,2% |
| Overall (Test vs single layer) | 13,5% | 22,4% | 22,5% |  |  |  |  |
| Overall (Ref vs single layer) | 14,4% | 32,1% | 34,2% |  |  |  |  |
| EL only (Test vs Ref) | -2,8% | -9,2% | -10,7% |  |  |  |  |

Table 13 - BD-Rate results for the proposed single loop decoding scheme using the RA configuration and prediction from filtered key pictures

|  | BD-PSNR (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA HEVC SNR |  |  |  | RA HEVC SNR BL ONLY |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A | 0,0201 | 0,2068 | 0,1673 |  | -0,2748 | -0,0723 | -0,0658 |
| Class B | 0,0188 | 0,0860 | 0,1357 |  | -0,1632 | -0,0613 | -0,0816 |
| Overall (Test vs Ref) | 0,0192 | 0,1205 | 0,1447 |  | -0,1951 | -0,0644 | -0,0771 |

Table 14 - BD-PSNR results for the proposed single loop decoding scheme using the RA configuration and prediction from filtered key pictures

|  | BD-Rate |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | LD-B HEVC SNR |  |  |  | LD-B HEVC SNR BL ONLY |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A | 0,3% | -5,7% | -6,4% |  | 15,1% | 4,9% | 3,4% |
| Class B | -0,1% | -4,3% | -5,9% |  | 15,5% | 6,5% | 5,6% |
| Overall (Test vs Ref) | 0,0% | -4,7% | -6,0% |  | 15,4% | 6,1% | 5,0% |
| Overall (Test vs single layer) | 24,4% | 27,7% | 30,4% |  |  |  |  |
| Overall (Ref vs single layer) | 24,3% | 34,7% | 39,5% |  |  |  |  |
| EL only (Test vs Ref) | -3,5% | -8,3% | -9,7% |  |  |  |  |

Table 15 - BD-Rate results for the proposed single loop decoding scheme using the LDB configuration and EL prediction from filtered key pictures

|  | BD-PSNR (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | LD-B HEVC SNR |  |  |  | LD-B HEVC SNR BL ONLY |  |  |
|  | Y | U | V |  | Y | U | V |
| Class A | -0,0340 | 0,1372 | 0,1299 |  | -0,6241 | -0,1018 | -0,0615 |
| Class B | 0,0035 | 0,0586 | 0,0876 |  | -0,4106 | -0,0868 | -0,1170 |
| Overall (Test vs Ref) | -0,0072 | 0,0811 | 0,0997 |  | -0,4716 | -0,0911 | -0,1011 |

Table 16 - BD-PSNR results for the proposed single loop decoding scheme using the LDB configuration and prediction from filtered key pictures

Table 13 and Table 14 show the BD-rate and BD-PSNR results for the implemented key picture concept with single loop decoding using prediction from filtered key pictures as described in 2.4. The unmodified SHM3.0.1 is used as a reference.

Table 13 and Table 14 provide results for the RA configuration while Table 15 and Table 16 provide the results for the LDB configuration.

The full table is available in the accompanying data sheet [JCTVC-O0145-RWTH-PSNR-SingleLoopDecoding_KeyPicFiltered.xls]

The results reveal no overall performance impact relative to SHM3.0.1 for the RA and LDB configuration. A base layer drift of 0.19dB and 0.47dB for the RA and LDB configuration is observed respectively. The proposed scheme also allows for single loop decoding. The decoder complexity results are reported in 3.2.3 and 3.3.3.

## AHG17 Decoder complexity assessment

### Single loop decoding

|  | RA HEVC SNR |  |  |  |  | LD-B HEVC SNR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Mults |  | Pure | DDR2 | DDR3 | Mults |
| Class A | 61% | 63% | 66% | 68% |  | 65% | 67% | 71% | 77% |
| Class B | 61% | 64% | 68% | 67% |  | 66% | 70% | 75% | 75% |
| Overall | 61% | 64% | 67% | 68% |  | 66% | 68% | 73% | 76% |

Table 17 - AHG 17 Complexity results for RA and LDB configuration

Table 17 shows the AHG17 complexity results for the proposed single loop scheme relative to the SHM3.0.1 anchor.

The full table is available in the accompanying data sheet [JCTVC-O0145-RWTH-AHG17-SingleLoopDecoding.xls]

The AHG17 complexity measurements reveal that the single-loop configuration reduces the required memory access of the scalable coding scheme to about 62 – 66 % of the SHM3.0.1 anchor.

### Single loop decoding with EL prediction restriction

|  | RA HEVC SNR |  |  |  |  | LD-B HEVC SNR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Mults |  | Pure | DDR2 | DDR3 | Mults |
| Class A | 59% | 61% | 64% | 66% |  | 64% | 65% | 68% | 75% |
| Class B | 59% | 62% | 66% | 64% |  | 64% | 67% | 71% | 72% |
| Overall | 59% | 62% | 65% | 65% |  | 64% | 66% | 70% | 73% |

Table 18 - AHG 17 Complexity results for RA and LDB configuration with EL prediction restriction

Table 18 shows the AHG17 complexity results for the proposed single loop scheme with EL prediction restriction as described in 2.3 relative to the SHM3.0.1 anchor.

The full table is available in the accompanying data sheet [JCTVC-O0145-RWTH-AHG17-SingleLoopDecoding_ELRestriction.xls]

The AHG17 complexity measurements reveal that the single-loop configuration including the EL prediction restriction as described in Section 2.3 further reduces the required memory access of the scalable coding scheme. The memory access is reduced to about 60 – 65 % of the SHM3.0.1 anchor.

### Single loop decoding with filtered key pictures

|  | RA HEVC SNR |  |  |  |  | LD-B HEVC SNR |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pure | DDR2 | DDR3 | Mults |  | Pure | DDR2 | DDR3 | Mults |
| Class A | 59% | 60% | 61% | 68% |  | 62% | 61% | 62% | 76% |
| Class B | 59% | 60% | 62% | 67% |  | 63% | 63% | 64% | 75% |
| Overall | 59% | 60% | 62% | 68% |  | 62% | 62% | 63% | 75% |

Table 19 - AHG 17 Complexity results for RA and LDB configuration with filtered key pictures

Table 19 shows the AHG17 complexity results for the proposed single loop scheme with prediction from filtered key pictures as described in 2.4 relative to the SHM3.0.1 anchor.

The full table is available in the accompanying data sheet [JCTVC-O0145-RWTH-AHG17-SingleLoopDecoding_KeyPicFiltered.xls]

The results are comparable to the results in 3.2.1. This is to be expected since no additional complexity is necessary for the prediction from the filtered key pictures.

## Pixel Level Complexity

On the decoder side the following numbers have been counted to be able to further evaluate the decoder complexity:

- relative number of pixels that are reconstructed using Inter/Intra prediction (“Pix Inter”, “Pix Intra”)

- relative number of pixels that use Uni/Bi-prediction with Full/Half/Quarter precision(“Pix MV Full Sum”, “Pix MV Half” “Pix MV Quarter”)

- relative number of pixels that are predicted using inter-layer prediction (“Pix MV Full BL”). For the single loop implementation this only includes the pixels that are intra coded in the base layer and are referenced in the enhancement layer.

- relative number of pixels that require inverse transform (“Pix Inv Transf Y”)

- relative Number of pixels that are modified by Deblocking/SAO(“Pix Deblocking Y”, “Pix SAO Y”)

### Single loop decoding

|  | BL Decoding |  |  |  | EL decoding (including required BL operations) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SHM vs HM | Prop vs HM | Prop vs SHM |  | SHM vs HM | Prop vs HM | Prop vs SHM |
| Pix Inter | 100,29% | 102,09% | 101,79% |  | 209,98% | 172,38% | 82,09% |
| Pix Intra | 95,89% | 70,48% | 73,50% |  | 86,83% | 73,48% | 84,62% |
| Pix MV Full Sum | 100,69% | 100,34% | 99,65% |  | 237,55% | 121,39% | 51,10% |
| Pix MV Half | 100,06% | 102,65% | 102,59% |  | 188,44% | 121,39% | 64,42% |
| Pix MV Quarter | 100,08% | 104,98% | 104,90% |  | 186,55% | 122,66% | 65,75% |
| Pix MV Full BL |  |  |  |  |  |  | 5,17% |
| Pix Inv Transf Y | 98,72% | 95,54% | 96,78% |  | 163,17% | 160,58% | 98,41% |
| Pix Deblocking Y | 99,34% | 95,65% | 96,28% |  | 195,09% | 128,09% | 65,66% |
| Pix SAO Y | 97,35% | 145,43% | 149,39% |  | 143,96% | 147,14% | 102,21% |

Table 20 - Pixel level decoder complexity results for the RA configuration

|  | BL Decoding |  |  |  | EL decoding (including required BL operations) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SHM vs HM | Prop vs HM | Prop vs SHM |  | SHM vs HM | Prop vs HM | Prop vs SHM |
| Pix Inter | 100,00% | 101,14% | 101,14% |  | 204,31% | 174,22% | 85,27% |
| Pix Intra | 100,00% | 61,90% | 61,90% |  | 90,58% | 71,03% | 78,42% |
| Pix MV Full Sum | 100,00% | 96,65% | 96,65% |  | 229,01% | 128,68% | 56,19% |
| Pix MV Half | 100,00% | 110,72% | 110,72% |  | 163,93% | 128,73% | 78,53% |
| Pix MV Quarter | 100,00% | 106,50% | 106,50% |  | 168,96% | 126,45% | 74,84% |
| Pix MV Full BL |  |  |  |  |  |  | 2,15% |
| Pix Inv Transf Y | 100,00% | 89,54% | 89,54% |  | 159,17% | 150,89% | 94,95% |
| Pix Deblocking Y | 100,00% | 92,10% | 92,10% |  | 191,84% | 127,85% | 67,30% |
| Pix SAO Y | 100,00% | 125,22% | 125,22% |  | 145,46% | 149,85% | 99,02% |

Table 21 - Pixel level decoder complexity results for the LDB configuration

Table 20 and Table 21 show the complexity results measured using the common testing conditions. [1] The results reveal that there are significant savings in required motion compensation operations as well as deblocking operations when using the proposed single loop decoding scheme compared to the current multi loop approach of SHM3.0.1.

The full table is available in the accompanying data sheet [JCTVC-O0145-RWTH-PixelLevelComplexity-SingleLoopDecoding.xls]

### Single loop decoding with EL prediction restriction

|  | BL Decoding |  |  |  | EL decoding (including required BL operations) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SHM vs HM | Prop vs HM | Prop vs SHM |  | SHM vs HM | Prop vs HM | Prop vs SHM |
| Pix Inter | 100,29% | 102,09% | 101,79% |  | 209,98% | 165,84% | 78,98% |
| Pix Intra | 95,89% | 70,50% | 73,52% |  | 86,83% | 74,71% | 86,04% |
| Pix MV Full Sum | 100,69% | 101,04% | 100,36% |  | 237,55% | 113,08% | 47,60% |
| Pix MV Half | 100,06% | 102,80% | 102,74% |  | 188,44% | 114,58% | 60,80% |
| Pix MV Quarter | 100,08% | 104,84% | 104,76% |  | 186,55% | 114,92% | 61,60% |
| Pix MV Full BL |  |  |  |  |  |  | 5,16% |
| Pix Inv Transf Y | 98,72% | 95,57% | 96,82% |  | 163,17% | 161,15% | 98,76% |
| Pix Deblocking Y | 99,34% | 95,67% | 96,30% |  | 195,09% | 125,26% | 64,21% |
| Pix SAO Y | 97,35% | 146,37% | 150,35% |  | 143,96% | 149,87% | 104,11% |

Table 22 - Pixel level decoder complexity results for the RA configuration and the EL prediction restriction

|  | BL Decoding |  |  |  | EL decoding (including required BL operations) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SHM vs HM | Prop vs HM | Prop vs SHM |  | SHM vs HM | Prop vs HM | Prop vs SHM |
| Pix Inter | 100,00% | 101,14% | 101,14% |  | 204,31% | 170,86% | 83,63% |
| Pix Intra | 100,00% | 61,98% | 61,98% |  | 90,58% | 74,44% | 82,18% |
| Pix MV Full Sum | 100,00% | 96,97% | 96,97% |  | 229,01% | 123,37% | 53,87% |
| Pix MV Half | 100,00% | 110,82% | 110,82% |  | 163,93% | 121,87% | 74,34% |
| Pix MV Quarter | 100,00% | 106,97% | 106,97% |  | 168,96% | 120,09% | 71,08% |
| Pix MV Full BL |  |  |  |  |  |  | 2,10% |
| Pix Inv Transf Y | 100,00% | 89,49% | 89,49% |  | 159,17% | 150,89% | 94,80% |
| Pix Deblocking Y | 100,00% | 92,27% | 92,27% |  | 191,84% | 127,85% | 66,64% |
| Pix SAO Y | 100,00% | 126,49% | 126,49% |  | 145,46% | 149,85% | 103,02% |

Table 23 - Pixel level decoder complexity results for the LDB configuration and the EL prediction restriction

Table 22 and Table 23 show the complexity results measured using the common testing conditions with EL prediction restriction as described in 2.3. The results reveal that there are significant savings in required motion compensation operations as well as deblocking operations when using the proposed single loop decoding scheme compared to the current multi loop approach of SHM3.0.1

The full table is available in the accompanying data sheet [JCTVC-O0145-RWTH-PixelLevelComplexity-SingleLoopDecoding_ELRestriction.xls]

### Single loop decoding with prediction from filtered key pictures

|  | BL Decoding |  |  |  | EL decoding (including required BL operations) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SHM vs HM | Prop vs HM | Prop vs SHM |  | SHM vs HM | Prop vs HM | Prop vs SHM |
| Pix Inter | 100,29% | 102,09% | 101,80% |  | 209,98% | 166,11% | 79,11% |
| Pix Intra | 95,89% | 70,43% | 73,45% |  | 86,83% | 72,28% | 83,25% |
| Pix MV Full Sum | 100,69% | 100,99% | 100,30% |  | 237,55% | 113,32% | 47,70% |
| Pix MV Half | 100,06% | 102,93% | 102,87% |  | 188,44% | 114,19% | 60,60% |
| Pix MV Quarter | 100,08% | 104,75% | 104,67% |  | 186,55% | 114,36% | 61,30% |
| Pix MV Full BL |  |  |  |  |  |  | 5,25% |
| Pix Inv Transf Y | 98,72% | 95,57% | 96,81% |  | 163,17% | 160,37% | 98,28% |
| Pix Deblocking Y | 99,34% | 95,67% | 96,30% |  | 195,09% | 123,56% | 63,33% |
| Pix SAO Y | 97,35% | 143,77% | 147,67% |  | 143,96% | 146,27% | 101,60% |

Table 24 - Pixel level decoder complexity results for the RA configuration

|  | BL Decoding |  |  |  | EL decoding (including required BL operations) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | SHM vs HM | Prop vs HM | Prop vs SHM |  | SHM vs HM | Prop vs HM | Prop vs SHM |
| Pix Inter | 100,00% | 101,14% | 101,14% |  | 204,31% | 174,34% | 85,33% |
| Pix Intra | 100,00% | 61,92% | 61,92% |  | 90,58% | 69,32% | 76,53% |
| Pix MV Full Sum | 100,00% | 96,52% | 96,52% |  | 229,01% | 127,88% | 55,84% |
| Pix MV Half | 100,00% | 110,59% | 110,59% |  | 163,93% | 127,65% | 77,87% |
| Pix MV Quarter | 100,00% | 106,55% | 106,55% |  | 168,96% | 125,40% | 74,22% |
| Pix MV Full BL |  |  |  |  |  |  | 2,11% |
| Pix Inv Transf Y | 100,00% | 89,53% | 89,53% |  | 159,17% | 150,68% | 94,67% |
| Pix Deblocking Y | 100,00% | 92,12% | 92,12% |  | 191,84% | 128,27% | 66,86% |
| Pix SAO Y | 100,00% | 125,62% | 125,62% |  | 145,46% | 139,06% | 95,60% |

Table 25 - Pixel level decoder complexity results for the LDB configuration

Table 24 and Table 25 show the complexity results measured using the common testing conditions with prediction from the filtered key pictures as described in 2.4. The results are comparable to the results presented in 3.3.1. This is to be expected since no additional complexity is necessary for the prediction from the filtered key pictures.

The full table is available in the accompanying data sheet [JCTVC-O0145-RWTH-PixelLevelComplexity-SingleLoopDecoding_ELRestriction.xls]

# Specification changes

See accompanying specification draft document [JCTVC-O0145-Delta-to-JCTVC-N1008-v3].

# Discussion

The suggested concepts only require high level changes to the decoding and encoding process. In combination with the suggested restriction to the encoder side, single loop decoding can be enabled. This way only the base layer key pictures have to be reconstructed completely while for all other base layer frames only the intra coded area requires reconstruction. Since both layers use the same pictures as reference, motion compensation only needs to be performed at the enhancement layer. Also the filtering operations (loop filter and SAO) of the non-key pictures is only required at the enhancement layer. The complexity measurements reveal a significant reduction of the required memory access, sub-pixel interpolation, and loop-filter operations compared to the multi-loop SHM3.0.1 anchor.  The complexity reduction is combined with a slight performance loss in the enhancement layer and some drift in the base layer.

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JCTVC-N1009, “Common SHM test conditions and software reference configurations” Vienna, AT, 25 Jul. – 2 Aug. 2013

- JCTVC-N0202, “AHG16: Key picture concept and single loop decoding” Vienna, AT, 25 Jul. – 2 Aug. 2013

- JCTVC-N1102, “HEVC Scalable Extension Core Experiment SCE2: Key pictures and single-loop decoding” Vienna, AT, 25 Jul. – 2 Aug. 2013