| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0147 |
| --- | --- |

| Title: | RExt: Proposed changes in the horizontal and vertical gradient filtering control for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Broadcom Corporation, USA | Email: | minhua@broadcom.com |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

In the current HEVC range extension design, the horizontal and vertical gradient filtering in intra prediction is conditioned on the cu_transquant_bypass_flag while it is unconditionally turned on in the HEVC version 1 design. This leads to a backward comptibility issue beween the HEVC range extension and the HEVC version 1 profiles. To reslove the problem, it is proposed to condition the intra horizontal and vertical gradient filtering on intra_smoothing_disabled_flag in the HEVC range extension. It is also recommended to rename intra_smoothing_disabled_flag to more properly refect the filtering process controlled by this flag after the proposed changes.

# Proposed changes

#### In 7.4.3.2 Sequence parameter set RBSP semantics

Replace

intra_smoothing_disabled_flag equal to 1 specifies that the filtering process of neighbouring samples is unconditionally disabled for intra prediction. intra_smoothing_disabled_flag equal to 0 specifies that the filtering process of neighbouring samples is not disabled.

With

intra_smoothing_disabled_flag equal to 1 specifies that the filtering process of neighbouring samples and the horizontal and vertical gradeint filtering process are unconditionally disabled for intra prediction. intra_smoothing_disabled_flag equal to 0 specifies that the filtering process of neighbouring samples and the horizontal and vertical gradeint filtering process are not disabled.

##### In 8.4.4.2.6 Specification of intra prediction mode in the range of INTRA_ANGULAR2.. INTRA_ANGULAR34

Replace

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

–	If predModeIntra is equal or greater than 18, the following ordered steps apply:

1.	The reference sample array ref[ x ] is specified as follows:

–	The following applies:

ref[ x ] = p[ −1 + x ][ −1 ], with x = 0..nTbS	(8 47)

–	If intraPredAngle is less than 0, the main reference sample array is extended as follows:

–	When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngle + 128 )  >>  8 ) ],

with x = −1..( nTbS * intraPredAngle )  >>  5	(8 48)

–	Otherwise,

ref[ x ] = p[ −1 + x ][ −1 ], with x = nTbS + 1..2 * nTbS	(8 49)

2.	The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

a.	The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngle )  >>  5		(8 50)

iFact = ( ( y + 1 ) * intraPredAngle ) & 31		(8 51)

b.	Depending on the value of iFact, the following applies:

–	If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] =

( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8 52)

–	Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8 53)

c.	When predModeIntra is equal to 26 (vertical), cIdx is equal to 0, nTbS is less than 32 and cu_transquant_bypass_flag is equal to 0, the following filtering applies with x = 0, y = 0..nTbS − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8 54)

–	Otherwise (predModeIntra is less than 18), the following ordered steps apply:

1.	The reference sample array ref[ x ] is specified as follows:

–	The following applies:

ref[ x ] = p[ −1 ][ −1 + x ], with x = 0..nTbS	(8 55)

–	If intraPredAngle is less than 0, the main reference sample array is extended as follows:

–	When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ],

with x = −1..( nTbS * intraPredAngle )  >>  5	(8 56)

–	Otherwise,

ref[ x ] = p[ −1 ][ −1 + x ], with x = nTbS + 1..2 * nTbS	(8 57)

2.	The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

a.	The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle )  >>  5		(8 58)

iFact = ( ( x + 1 ) * intraPredAngle ) & 31		(8 59)

b.	Depending on the value of iFact, the following applies:

–	If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] =

( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8 60)

–	Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(8 61)

c.	When predModeIntra is equal to 10 (horizontal), cIdx is equal to 0, nTbS is less than 32 and cu_transquant_bypass_flag is equal to 0, the following filtering applies with x = 0..nTbS − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8 62)

with

The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

–	If predModeIntra is equal or greater than 18, the following ordered steps apply:

1.	The reference sample array ref[ x ] is specified as follows:

–	The following applies:

ref[ x ] = p[ −1 + x ][ −1 ], with x = 0..nTbS	(8 47)

–	If intraPredAngle is less than 0, the main reference sample array is extended as follows:

–	When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 ][ −1 + ( ( x * invAngle + 128 )  >>  8 ) ],

with x = −1..( nTbS * intraPredAngle )  >>  5	(8 48)

–	Otherwise,

ref[ x ] = p[ −1 + x ][ −1 ], with x = nTbS + 1..2 * nTbS	(8 49)

2.	The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

a.	The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( y + 1 ) * intraPredAngle )  >>  5		(8 50)

iFact = ( ( y + 1 ) * intraPredAngle ) & 31		(8 51)

b.	Depending on the value of iFact, the following applies:

–	If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] =

( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8 52)

–	Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8 53)

c.	When predModeIntra is equal to 26 (vertical), cIdx is equal to 0, nTbS is less than 32 and intra_smoothing_disabled_flag is equal to 0, the following filtering applies with x = 0, y = 0..nTbS − 1:

predSamples[ x ][ y ] = Clip1Y( p[ x ][ −1 ] + ( ( p[ −1 ][ y ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8 54)

–	Otherwise (predModeIntra is less than 18), the following ordered steps apply:

1.	The reference sample array ref[ x ] is specified as follows:

–	The following applies:

ref[ x ] = p[ −1 ][ −1 + x ], with x = 0..nTbS	(8 55)

–	If intraPredAngle is less than 0, the main reference sample array is extended as follows:

–	When ( nTbS * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 ],

with x = −1..( nTbS * intraPredAngle )  >>  5	(8 56)

–	Otherwise,

ref[ x ] = p[ −1 ][ −1 + x ], with x = nTbS + 1..2 * nTbS	(8 57)

2.	The values of the prediction samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1 are derived as follows:

a.	The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( x + 1 ) * intraPredAngle )  >>  5		(8 58)

iFact = ( ( x + 1 ) * intraPredAngle ) & 31		(8 59)

b.	Depending on the value of iFact, the following applies:

–	If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] =

( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8 60)

–	Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(8 61)

c.	When predModeIntra is equal to 10 (horizontal), cIdx is equal to 0, nTbS is less than 32 and intra_smoothing_disabled_flag is equal to 0, the following filtering applies with x = 0..nTbS − 1, y = 0:

predSamples[ x ][ y ] = Clip1Y( p[ −1 ][ y ] + ( ( p[ x ][ −1 ] − p[ −1 ][ −1 ] )  >>  1 ) )	(8 62)

# References

[1] D. Flynn, J. Sole and T. Suzuki “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 4” JCTVC-N1005_v1, 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013

[2] B. Bross, G. Sullivan and Y.K. Wang “Editors' proposed corrections to HEVC version 1,” JCTVC-N0041_v1, 14th Meeting: Vienna, AT, 25 July – 2 Aug. 2013

# Patent rights declaration(s)

Broadcom Corporation does not have current or pending patent rights relating to the technology described in this contribution.