| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0150 |
| --- | --- |

| Title: | Non-RCE1: Extended Adaptive Inter-Component Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Khairat T. Nguyen M. Siekmann D. Marpe Einsteinufer 37 10587 Berlin Germany | Tel: Email: | +49 30 31002 230 ali.khairat@hhi-extern.fraunhofer.de |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute |  |  |

_____________________________

# Abstract

This document presents an extension of the inter-component prediction (ICP) scheme in RCE1. In the proposed extension, the second chroma residual block can be predicted either from the corresponding luma or the corresponding first chroma residual block. For that purpose, a flag is transmitted in the bit stream indicating whether the corresponding luma or the first chroma residual block is employed for ICP. Experimental results show an improvement in terms of BD-rate for YʹCbCr sequences relative to a version without ICP of about to -1.8% on average for the common test conditions all-intra configuration, -0.7% for the random access configuration, and -0.3% for the low delay configuration.

# Introduction

In [1], an inter-component residual prediction (ICP) method was proposed to predict the chroma residuals from luma residuals which is especially beneficial for RʹGʹBʹ content. This technique was later extended in [2] enabling improvement in terms of compression efficiency for YʹCbCr content. Particularly, the scheme in [2] introduces negative scale factors and allows more values for the gradient parameter of the linear predictor.

# Proposed Scheme

In this proposed extension, it is possible to use the luma or the first chroma residual block for the second chroma residual block. For each residual block of the second chroma component, i.e., the Cr component, a syntax element is transmitted in the bit stream and it signals whether the predictor is the correspondingereconstructed luma or the first reconstructed chroma residualtblock.

# Experimental Results

The experiments were performed based on the range extension reference software implementation in version 4.1 (RExt 4.1) and the common test conditions defined in [3] were used, which is basically the default configuration for RCE1.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | -18.0% | -16.5% | -17.9% | -14.1% | -13.5% | -14.3% | -10.7% | -10.4% | -11.1% |
| YCbCr 4:4:4 | -1.8% | -7.0% | -7.6% | -2.2% | -4.9% | -6.9% | -2.3% | -3.7% | -5.0% |
| Overall | -10.4% | -12.1% | -13.1% | -8.5% | -9.5% | -10.9% | -6.8% | -7.3% | -8.2% |
|  | -10.4% | -12.1% | -13.2% | -8.6% | -9.6% | -11.0% | -6.8% | -7.4% | -7.9% |
| Enc Time[%] | 136% |  |  | 139% |  |  | 143% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -13.7% | -11.7% | -15.0% | -10.7% | -8.7% | -12.1% |  |  |  |
| YCbCr 4:4:4 | -0.7% | -8.6% | -8.1% | -1.0% | -6.5% | -8.3% |  |  |  |
| Overall | -7.6% | -10.3% | -11.8% | -6.2% | -7.7% | -10.3% |  |  |  |
|  | -7.6% | -11.1% | -11.5% | -6.4% | -9.7% | -10.8% |  |  |  |
| Enc Time[%] | 130% |  |  | 136% |  |  |  |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| RGB 4:4:4 | -12.6% | -9.5% | -13.2% | -9.7% | -7.1% | -10.7% |  |  |  |
| YCbCr 4:4:4 | -0.3% | -6.4% | -6.1% | -0.7% | -4.5% | -6.6% |  |  |  |
| Overall | -6.8% | -8.1% | -9.9% | -5.5% | -5.9% | -8.8% |  |  |  |
|  | -6.7% | -7.6% | -9.2% | -5.7% | -6.7% | -8.8% |  |  |  |
| Enc Time[%] | 130% |  |  | 137% |  |  |  |  |  |
| Dec Time[%] | 105% |  |  | 105% |  |  |  |  |  |

# Sequence parameter set RBSP syntax and semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_extension1_flag ) { |  |
| if( chroma_format_idc = = 3 ) |  |
| inter_color_prediction_enabled_flag | u(1) |
| if( inter_color_prediction_enabled_flag ) { |  |
| inter_color_ prediction_extended_enabled_flag | u(1) |
| } |  |

inter_color_prediction_enabled_flag equal to 1 specifies that inter-color prediction is enabled. inter_color_prediction_enabled_flag equal to 0 specifies that inter color residual prediction is disabled. When inter_color_prediction_enabled_flag is not present, it is inferred to be equal to 0.

inter_color_prediction_extended_enabled_flag equal to 1 specifies that the first chroma component can be used as a predictor for the inter-color prediction. inter_color_prediction_extended_enabled_flag equal to 0 specifies that the first chroma component cannot be used as a predictor for the inter-color prediction. When inter_color_prediction_extended_flag is not present, it is inferred to be equal to be 0.

## Transform unit syntax and semantics

| transform_unit( x0, y0, xBase, yBase, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| … |  |
| if( log2TrafoSize > 2 ¦ ¦ ChromaArrayType = = 3 ) { |  |
| for( tIdx = 0; tIdx < ( ChromaArrayType = = 2 ? 2 : 1 ); tIdx++) { |  |
| if (cbf_luma[x0][y0][trafoDepth] && inter_color_prediction_enabled_flag) |  |
| if( ( cbf_luma[x0][y0][trafoDepth] && inter_color_prediction_enabled_flag ) ¦¦ |  |
| ( cbf_cb[x0][y0][trafoDepht] && inter_color_prediction_extended_enabled_flag ) ) |  |
| inter_color_pred(x0, y0 + ( tIdx << log2TrafoSizeC ), 0) |  |
| if( cbf_cb[ x0 ][ y0 + ( tIdx << log2TrafoSizeC ) ][ trafoDepth ] ) |  |
| residual_coding( x0, y0 + ( tIdx << log2TrafoSizeC ), log2TrafoSizeC, 1 ) |  |
| } |  |
| for( tIdx = 0; tIdx < ( ChromaArrayType = = 2 ? 2 : 1 ); tIdx++) { |  |
| if (cbf_luma[ x0 ][ y0 + ( tIdx << log2TrafoSizeC ) ][ trafoDepth ] && inter_color_prediction_enabled_flag) |  |
| if( ( cbf_luma[x0][y0][trafoDepth] && inter_color_prediction_enabled_flag ) ¦¦ |  |
| ( cbf_cb[x0][y0][trafoDepht] && inter_color_prediction_extended_enabled_flag ) ) |  |
| inter_color_pred (x0, y0 + ( tIdx << log2TrafoSizeC ), 1) |  |
| if( cbf_cr[ x0 ][ y0 + ( tIdx << log2TrafoSizeC ) ][ trafoDepth ] ) |  |
| residual_coding( x0, y0 + ( tIdx << log2TrafoSizeC ), log2TrafoSizeC, 2 ) |  |
| } |  |
| } else if( blkIdx = = 3 ) { |  |
| for( tIdx = 0; tIdx < ( ChromaArrayType = = 2 ? 2 : 1 ); tIdx++) { |  |
| if( cbf_luma[ xBase ][ yBase ][ trafoDepth ] && inter_color_prediction_enabled_flag) |  |
| if( ( cbf_luma[x0][y0][trafoDepth] && inter_color_prediction_enabled_flag ) ¦¦ |  |
| ( cbf_cb[x0][y0][trafoDepht] && inter_color_prediction_extended_enabled_flag ) ) |  |
| inter_color_pred ( xBase, yBase + ( tIdx << log2TrafoSize ), 0 ) |  |
| if( cbf_cb[ xBase ][ yBase ][ trafoDepth ] ) |  |
| residual_coding( xBase, yBase + ( tIdx << log2TrafoSize ), log2TrafoSize, 1 ) |  |
| } |  |
| for( tIdx = 0; tIdx < ( ChromaArrayType = = 2 ? 2 : 1 ); tIdx++) { |  |
| if( cbf_luma[ xBase ][ yBase ][ trafoDepth ] && inter_color_prediction_enabled_flag) |  |
| if( ( cbf_luma[x0][y0][trafoDepth] && inter_color_prediction_enabled_flag ) ¦¦ |  |
| ( cbf_cb[x0][y0][trafoDepht] && inter_color_prediction_extended_enabled_flag ) ) |  |
| inter_color_pred ( xBase, yBase + ( tIdx << log2TrafoSize ), 1 ) |  |
| if( cbf_cr[ xBase ][ yBase ][ trafoDepth ] ) |  |
| residual_coding( xBase, yBase + ( tIdx << log2TrafoSize ), log2TrafoSize, 2 ) |  |
| … |  |

## Inter-color prediction syntax and semantics

| inter_color_pred ( x0, y0, cIdx ) { | Descriptor |
| --- | --- |
| alpha_abs[ cIdx ][ x0 ][ y0 ] | ae(v) |
| if (alpha_abs) { |  |
| alpha_sign[ cIdx ][ x0 ][ y0 ] | ae(v) |
| if( cIdx = = 2 ) { |  |
| icp_from_cb[ cIdx ][ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |

alpha_ abs[ cIdx ][ x0 ][ y0 ] equal to 1 specifies the α  value used in inter-color prediction.

alpha_sign[ cIdx ][ x0 ][ y0 ] equal to 1 sepcifies that sign of alpha is positive. alpha_sign[ cIdx ][ x0 ][ y0 ] equal to 0 specifies that alpha is negative.

icp_from_cb[ cIdx ][ x0 ][ y0 ] equal to 1 sepcifies that the first chroma component is used for inter-color prediction. Otherwise, the luma component is used for the inter-color prediction for the specified transform block.

## Decoding process for alpha

The variable alpha[cIdx – 1][ x0 ][ y0 ] is derived as

- If alpha_abs[cIdx – 1][ x0 ][ y0 ] is equal to 0, alpha[ x0 ][ y0 ] is set equal to 0.

- Otherwise,
	alpha[cIdx – 1][ x0 ][ y0 ] = alpha_sign[cIdx – 1][ x0 ][ y0 ] * alpha_abs[cIdx – 1][ x0 ][ y0 ].

## Decoding process for chroma residual blocks

This process is only invoked when ChromaArrayType is not equal to 0.

Inputs to this process are:

…

– 	a variable alpha[cIdx – 1][xCb + xB0, yCb + yB0] specifying the inter color prediction coefficient,

–	an array resSamplesLuma of luma residual samples.

…

- The (nCbS)x(nCbS) residual sample array of the current coding block resSamples is modified as follows, for i = 0..nTbS − 1, j = 0..nTbS − 1:

If ChromaArrayType is equal to 3 and alpha[ xCb + xB0, yCb + yB0 ] is not equal to 0,

resSamples[ (xCb + xB0)/SubWidthC + i, (yCb + yBN)/SubHeightC + j ] =

transformBlock[ i, j ] + ( ( alpha[cIdx – 1][xCb + xB0, yCb + yB0] * resSamplesLuma[ xCb + xB0 + i, yCb + yBN + j ] ) >> 3 )

## CABAC parsing process for slice segment data

Table 9-1 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

| inter_color_pred( ) | alpha_abs[ ][ ][ ] | Table 9-xx | 0…2 | 3…5 | 6…8 |
| --- | --- | --- | --- | --- | --- |
|  | alpha_sign[ ][ ][ ] | Table 9-xx | 0 | 1 | 2 |
|  | icp_from_cb[ ][ ][ ] | Table 9-xx | 0 | 1 | 2 |

Table 9-xx – Values of initValue for ctxIdx of alpha_abs[ ][ ][ ]

| Initialization variable | ctxIdx of alpha_abs[ ][ ][ ] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 | 154 |

Table 9-xx – Values of initValue for ctxIdx of alpha_sign[ ][ ][ ]

| Initialization variable | ctxIdx of alpha_sign[ ][ ][ ] |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 |  | 1 | 2 |  |  |  |
| initValue | 154 |  | 154 | 154 |  |  |  |
| Table 9-2 – Syntax elements and associated binarizations |  |  |  |  |  |  |  |
| inter_color_pred( ) |  | alpha_abs[ ][ ][ ] |  |  |  | TR | cMax = 3, cRiceParam = 0 |
|  |  | alpha_sign[ ][ ][ ] |  |  |  | FL | cMax = 1 |
|  |  | icp_from_cb[ ][ ][ ] |  |  |  | FL | cMax = 1 |

| Table 9-3 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| alpha_abs[0][ ][ ][ ] alpha_abs[1][ ][ ][ ] | 0 3 | 1 4 | 2 5 | 2 5 | na | na |
| alpha_sign[0][ ][ ][ ] alpha_sign[1][ ][ ][ ] | 0 1 | na | na | na | na | na |
| icp_from_cb[1][ ][ ][ ] | 0 | na | na | na | na | na |

# References

- T. Nguyen, A. Khairat, and D. Marpe, “Non-RCE1/Non-RCE2/AHG5/AHG8: Adaptive Inter-Plane Prediction for RGB Content,” JCTVC-M0230, Incheon, Korea, Apr. 2013.

- W. Pu, W.-S. Kim, J. Chen, K. Rapaka, L. Guo, J. Sole, and M. Karczewicz, “Non-RCE1: Inter Color Component Residual Prediction,” JCTVC-N0266, Vienna, Austria, July 2013.

- D. Flynn, and K. Sharman, “Common Test Conditions and Software Reference,” JCTVC-L1006, Geneva, Switzerland, January 2013.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).