| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0153 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0058 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS: on output layer sets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The MV-HEVC/SHVC design allows access units in which the pictures may represent a subset of the layers present in the bitstream. In a typical SHVC use, only one picture per access unit (from the highest layer) is desired to be output. However, output layer sets are specified to be static for the duration that a VPS is active and the same output layer set is used for an entire coded video sequence. Hence, a mechanism is reportedly needed to specify which picture is to be output, if any, when no picture is present at an output layer.

It is proposed to specify a minimum alternative output layer in the VPS extension for each output layer. When a picture at the output layer is not present in an access unit, a candidate list of alternative output pictures includes pictures that are present in the access unit and have nuh_layer_id that is equal to a nuh_layer_id of a direct or indirect reference layer of the output layer and that is greater than or equal to nuh_layer_id of the minimum alternative output layer. Among the candidate list, the picture has the greatest nuh_layer_id value is output

# Introduction

In contrast to the MVC/SVC design, the MV-HEVC/SHVC design allows access units in which the pictures may represent a subset of the layers present in the bitstream. This characteristic can be used for example for the following purposes:

- Different picture rate in different layers, e.g. the base layer at a lower spatial resolution with 60 Hz and the enhancement layer at a higher spatial resolution with 30 Hz.

- Use SHVC for enabling adaptive resolution change of otherwise single-layer bitstream (see e.g. JCTVC-M0040)

- Layer-wise start-up, where the number of layers is step-wise increased after the start of the bitstream (see e.g. JCTVC-N0066)

The MV-HEVC/SHVC design includes output layer sets specified in the VPS extension, each of which specifies the layers that are output by the decoder when that output layer set is in use. The selection of the output layer set in use is intended to be performed by external means, similarly to selecting the target temporal level for example. In order to enable conformance checking in the HRD the selected output layer set must remain unchanged for the entire CVS.

The MV-HEVC/SHVC draft specification derives a default output layer set for each layer set specified in the VPS. The VPS extension flag default_one_target_output_layer_flag, when equal to 0, specifies that each layer is an output layer, and when equal to 1, specifies that only the highest layer is an output layer. In addition, to the default output layer sets, the VPS extension enables to specify additional output layer sets with selected layers indicated to be output layers.

While output layer sets suit well use cases and bitstreams where the highest layer stays unchanged in each access unit, they do not support use cases where the highest layer changes from one access unit to another. Referring to the first example above, the output layer set could be specified in either of the following ways: The output layer set containing the enhancement layer only would cause the decoder to output at 30-Hz picture rate. The output layer set containing both the base layer and the enhancement would cause the decoder to output to pictures per every other access unit, which might be difficult to handle in some standard APIs and by some renderers. Similar issues are faced in the other use cases too.

An approach to overcome the static nature of output layer sets could be to output a picture of the next lower present spatial or SNR scalable layer in the absence of a picture the specified output layer within an access unit. This approach is not proposed in this contribution due to the following reasons:

- It makes the MV-HEVC/SHVC design dependent on the current scalability types. If possible, the design should be independent of scalability types, which would enable the use of the approach for temporally varying output layers also for scalability types that are potentially specified in extensions beyond MV-HEVC/SHVC.

- It can be content-dependent whether the output of lower spatial and/or SNR layers is desired in the absence of the specified output layer. For example (referring to example 1 above), in some cases the content provider can intend the enhancement layer to be consumed at 30 Hz (not interleaving the base layer pictures into the output e.g. due to too heavy quality fluctuation), while in other cases the content provider can intend the enhancement layer decoding to produce a 60-Hz output.

# Overview of the Proposal

In the proposal each output layer is associated with a minimum alternative output layer. For the default output layer sets, the minimum output layer is the same as the output layer; i.e. when a default output layer set is in use, the decoder outputs only the specified output layers. For the additional output layer sets, the syntax element min_alt_output_layer_idx[ i ][ j ] is added to the VPS extension syntax for each output layer. It is used to specify the minimum nuh_layer_id value of a picture that can be output instead of the specified output layer, if a picture at the output layer is not present in an access unit. The first direct or indirect inter-layer reference picture present in the access unit in descending nuh_layer_id order down to the minimum value derived from min_alt_output_layer_idx[ i ][ j ] is output.

The detailed proposal is indicated with change marks in the accompanying specification text document. The proposed specification text changes can be summarized as follows:

- min_alt_output_layer_idx[ i ][ j ] is added to the VPS extension syntax for each output layer of the non-default output layer sets

- The variables NumRecursiveRefLayers[ i ] and RecursiveRefLayerFlag[ i ][ j ] are derived and indicate the number of direct and indirect reference layers and the direct and indicated reference layer mask, respectively, for layer i.

- The variable OutputLayerFlag[ i ][ j ] for output layer set with index i and for layer j is derived and indicates whether the layer is an output layer.

- The variable MinAltOutputLayerId[ i ][ j ] for output layer set with index i and for layer j is derived and indicates the minimum nuh_layer_id value of a picture that can be output instead of the specified output layer, if a picture at the output layer is not present in an access unit.

- The variable TargetOutputLayerSetIdx is set by external means, or (if not set by external means) by the HRD, or (if neither of the previous) set to 0.

- It is proposed that the derivation of PicOutputFlag stays unchanged when each picture is decoded. This process basically sets the value of PicOutputFlag as if the picture belonged to a layer that is output. When all pictures of an access unit have been decoded the values of PicOutputFlag are proposed to be updated depending on the output layers in the output layer set in use and if output layers are present in the access unit.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).