| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0154 |
| --- | --- |

| Title: | AhG5: Displacement vector signaling for intra block copying |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Liwei Guo, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Intra Block Copying was adopted as an intra coding tool in the HEVC Range Extensions draft specification. Currently, the displacement vector (DV) is signaled by sharing the motion vector difference coding method in HEVC. In this document, two DV signaling methods are proposed. The first method uses a vector predictor, while the second one does not. Reported experimental results demonstrate that up to 5.4% and 2.6% bit-rate reductions are obtained, respectively.

# Introduction

At the 14th JCT-VC meeting, Intra Block Copying (Intra BC) [1] was adopted as a new intra coding tool in HEVC Range Extensions draft specification. Intra BC is an efficient technique to predict repetitive patterns in a frame, which leads to high bit-rate savings for screen content. For Intra BC, currently the displacement (motion) vector (DV) is signaled by sharing the motion vector difference (MVD) coding method in HEVC.

In this document, two DV signaling methods are proposed to further improve the coding efficiency. A first method uses a prediction for the vector, while the second one has not predictor. The trade-off is that the first method has better coding performance than the second one.

# Proposed Methods

## Method 1

In method 1, the left or above DV is selected as predictor and the resulting displacement vector difference (DVD) is coded. First, one flag indicates whether the difference is zero. When DVD is not zero, exponential-Golomb codes with order  3 are used to encode the remaining absolute level of the DVD and a flag is used to code its sign.

In order to avoid extra line buffer to store the above MV, only the above MV inside current CTU is allowed to be used as the predictor.

## Method 2

There is no vector predictor in method 2. Instead, first one flag indicates whether the DV is zero. When the DV is not zero, exponential-Golomb codes of  order 4 are used to code the DV remaining absolute level and another flag is used to code its sign.

# Results

In this section, experimental results are presented for the proposed DV signaling methods. The methods are implemented in HM12.0_Rext4.1 and simulated under the RCE3 common test conditions [2]. The simulation platform is a homogenous LINUX cluster consisting of Intel(R) XEON CPUs. The performance is compared to the anchor for lossless as well as lossy configuration in terms of BD-rates.

- Results of method 1 for lossless (Table 1) and lossy (Table 2);

- Results of method 2 for lossless (Table 3) and lossy (Table 4);

Table 1. Lossless results for method 1

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 0.6% | 0.5% | 0.5% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.5% | 0.4% | 0.4% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.6% | 0.4% | 0.5% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 1.0% | 0.9% | 1.1% |
| SC YUV 444 Optional | 1.9% | 1.6% | 0.7% |
| Enc Time[%] | 98% | 98% | 97% |
| Dec Time[%] | 98% | 99% | 98% |

Table 2. Lossy results for method 1

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.3% | -1.2% | -1.2% | -0.9% | -1.0% | -0.9% | -0.7% | -0.7% | -0.7% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.8% | -1.8% | -1.9% | -1.4% | -1.5% | -1.5% | -1.2% | -1.2% | -1.1% |
| Animation RGB 444 | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| SC YUV 444 | -2.8% | -2.8% | -2.9% | -2.2% | -2.2% | -2.3% | -1.7% | -1.6% | -1.7% |
| Animation YUV 444 | -0.3% | -0.3% | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -5.4% | -5.6% | -5.6% | -5.1% | -5.1% | -5.1% | -4.5% | -4.2% | -4.2% |
| SC(444) YUV Optional | -4.8% | -4.9% | -4.9% | -4.0% | -4.1% | -4.2% | -3.3% | -3.4% | -3.3% |
| Enc Time[%] | 98% |  |  | 99% |  |  | 98% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  | 97% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.0% | -1.1% | -1.0% | -0.8% | -0.8% | -0.7% |  |  |  |
| Class B | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -1.4% | -1.4% | -1.5% | -1.1% | -1.1% | -1.1% |  |  |  |
| Animation RGB 444 | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |  |  |  |
| SC YUV 444 | -2.4% | -2.4% | -2.3% | -1.7% | -1.7% | -1.7% |  |  |  |
| Animation YUV 444 | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -5.1% | -5.1% | -5.2% | -4.6% | -4.7% | -4.6% |  |  |  |
| SC(444) YUV Optional | -4.8% | -4.8% | -5.0% | -4.1% | -4.0% | -4.2% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.8% | -1.4% | -1.2% | -0.6% | -0.8% | -0.8% |  |  |  |
| Class B | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.8% | -0.7% | -0.7% | -0.6% | -0.4% | -0.4% |  |  |  |
| Animation RGB 444 | -0.1% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| SC YUV 444 | -2.4% | -2.3% | -2.5% | -1.8% | -1.6% | -1.8% |  |  |  |
| Animation YUV 444 | -0.1% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | -3.6% | -3.3% | -3.4% | -3.6% | -3.3% | -3.3% |  |  |  |
| SC(444) YUV Optional | -4.7% | -4.3% | -4.7% | -3.2% | -2.7% | -3.1% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |

Table 3. Lossless results for method 2

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 0.2% | 0.2% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.3% | 0.2% | 0.2% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.3% | 0.3% | 0.2% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 0.5% | 0.3% | 1.4% |
| SC YUV 444 Optional | 0.8% | 0.8% | -0.6% |
| Enc Time[%] | 103% | 103% | 102% |
| Dec Time[%] | 99% | 101% | 100% |

Table 4. Lossy results for method 2

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.7% | -0.8% | -0.6% | -0.5% | -0.6% | -0.5% | -0.4% | -0.4% | -0.4% |
| Class B | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.8% | -0.8% | -0.8% | -0.6% | -0.6% | -0.6% | -0.5% | -0.5% | -0.5% |
| Animation RGB 444 | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -1.3% | -1.2% | -1.3% | -1.0% | -0.9% | -0.9% | -0.7% | -0.7% | -0.6% |
| Animation YUV 444 | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -0.9% | -1.1% | -1.0% | -0.7% | -1.0% | -0.9% | -0.7% | -0.8% | -0.7% |
| SC(444) YUV Optional | -1.8% | -1.8% | -1.8% | -1.5% | -1.5% | -1.5% | -1.3% | -1.4% | -1.3% |
| Enc Time[%] | 101% |  |  | 102% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | -0.6% | -0.5% | -0.4% | -0.4% | -0.3% |  |  |  |
| Class B | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -0.7% | -0.7% | -0.7% | -0.7% | -0.6% | -0.7% |  |  |  |
| Animation RGB 444 | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.9% | -1.0% | -1.0% | -0.6% | -0.7% | -0.7% |  |  |  |
| Animation YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -0.8% | -0.8% | -0.8% | -0.6% | -0.5% | -0.5% |  |  |  |
| SC(444) YUV Optional | -2.0% | -2.0% | -2.0% | -1.6% | -1.5% | -1.6% |  |  |  |
| Enc Time[%] | 95% |  |  | 96% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.3% | -0.7% | 0.0% | -0.2% | -0.3% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.7% | -0.4% | -0.5% | -0.5% | -0.3% | -0.4% |  |  |  |
| Animation RGB 444 | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -1.4% | -1.2% | -1.5% | -0.8% | -0.6% | -0.9% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | -0.4% | -0.3% | -0.2% | 0.2% | 0.4% | 0.3% |  |  |  |
| SC(444) YUV Optional | -2.6% | -2.4% | -2.4% | -1.6% | -1.4% | -1.6% |  |  |  |
| Enc Time[%] | 99% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

# References

[1] C. Pang, J. Sole, L. Guo, M. Karczewicz, and R. Joshi, “Non-RCE3: Intra Motion Compensation with 2-D MVs”, JCTVC-N0256, Vienna, AT, August 2013.

[2] A. Saxena, D. Kwon, M. Naccari, and C. Pang, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction Techniques”, JCTVC-N1123, Vienna, AT, August 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).