| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0155 |
| --- | --- |

| Title: | AhG5: Constrained intra prediction for intra block copying |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Jianle Chen, Joel Sole, Liwei Guo, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Intra Block Copying was adopted as an intra coding tool in the HEVC Range Extensions draft specification. Consequently, the existing constrained intra prediction (CIP) method does not provide the intended benefit. In this document, the CIP in HEVC Range Extensions is addressed. Three CIP methods are proposed to enable the CIP functionality.

# Introduction

During the 14th JCT-VC meeting, Intra Block Copying (Intra BC) [1] was adopted as a new intra coding tool in HEVC Range Extensions draft specification. Intra BC is an efficient technique to predict the repetitive patterns. It provides large coding efficiency gains for screen content.

However, after Intra BC is introduced, the existing CIP method is not valid any more. For example, the prediction samples for intra blocks in P- and B- slices can come from the Intra BC blocks of which the prediction might be from inter blocks, thus violating the design target of CIP.

# Technical description

In this proposal, three CIP methods are described that would enable efficient CIP functionality in HEVC Range Extensions with Intra BC.

- Method 1: when CIP is on, for P- and B- slices, Intra BC mode is disabled and the intra_bc_flag is not coded.

- Method 2: when CIP is on, for P- and B- slices, Intra BC mode is treated as an “inter” coding tool, and neither the neighboring samples from Intra BC blocks nor inter blocks are used for intra prediction.

- Method 3: when CIP is on, for both Intra BC blocks and intra blocks in I-, P- and B- slices, it is restricted that only the neighboring samples from Intra BC blocks and intra blocks can be used for prediction.

# Results

In this section, experimental results are presented for the three CIP methods, which are implemented in HM12.0_Rext4.1:

- CIP is turned on.

- The anchor has Intra BC disabled.

BD-rates are provided for lossless and lossy configurations:

- Results of method 1 for lossless (Table 1) and lossy (Table 2);

- Results of method 2 for lossless (Table 3) and lossy (Table 4);

- Results of method 3 for lossless (Table 5) and lossy (Table 6).

Table 1 Lossless results for method 1

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 4.5% | 2.4% | 0.4% |
| Class B | 0.1% | 0.0% | 0.0% |
| SC RGB 444 | 21.2% | 10.9% | 4.0% |
| Animation RGB 444 | 0.2% | 0.0% | 0.0% |
| SC YUV 444 | 20.7% | 11.1% | 4.2% |
| Animation YUV 444 | 0.1% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 37.2% | 24.1% | 7.6% |
| SC YUV 444 Optional | 41.5% | 27.8% | 7.9% |
| Enc Time[%] | 154% | 103% | 99% |
| Dec Time[%] | 100% | 99% | 100% |

Table 2. Lossy results for method 1.

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -9.3% | -9.2% | -9.3% | -8.2% | -8.4% | -8.4% | -7.1% | -7.3% | -7.3% |
| Class B | -0.2% | -0.3% | -0.4% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | -26.1% | -26.3% | -26.3% | -25.5% | -25.8% | -25.7% | -25.1% | -25.1% | -25.2% |
| Animation RGB 444 | -1.1% | -1.1% | -1.0% | -0.7% | -0.8% | -0.7% | -0.4% | -0.5% | -0.5% |
| SC YUV 444 | -25.3% | -24.9% | -25.1% | -25.3% | -24.9% | -25.0% | -25.0% | -24.7% | -24.5% |
| Animation YUV 444 | -0.9% | -1.1% | -1.1% | -0.7% | -0.8% | -0.9% | -0.5% | -0.6% | -0.7% |
| RangeExt | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -42.0% | -41.9% | -42.4% | -42.1% | -42.5% | -43.1% | -42.6% | -42.4% | -42.7% |
| SC(444) YUV Optional | -44.5% | -43.6% | -43.7% | -44.8% | -44.3% | -44.2% | -45.3% | -44.9% | -44.9% |
| Enc Time[%] | 144% |  |  | 142% |  |  | 140% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 102% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -6.2% | -6.7% | -6.7% | -5.6% | -6.0% | -6.0% |  |  |  |
| Class B | -0.1% | -0.4% | -0.3% | 0.0% | -0.2% | -0.2% |  |  |  |
| SC RGB 444 | -15.4% | -15.6% | -15.6% | -14.7% | -14.9% | -14.9% |  |  |  |
| Animation RGB 444 | -0.2% | -0.3% | -0.3% | -0.1% | -0.1% | -0.1% |  |  |  |
| SC YUV 444 | -15.3% | -15.5% | -15.8% | -15.3% | -15.0% | -15.3% |  |  |  |
| Animation YUV 444 | -0.3% | -0.4% | -0.4% | -0.2% | -0.2% | -0.2% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -24.6% | -24.9% | -25.4% | -25.3% | -25.8% | -26.4% |  |  |  |
| SC(444) YUV Optional | -27.7% | -27.6% | -28.0% | -28.0% | -27.9% | -28.2% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.9% | -2.6% | -1.7% | -1.9% | -2.2% | -1.9% |  |  |  |
| Class B | 0.0% | -0.1% | 0.2% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | -4.5% | -4.4% | -4.5% | -4.6% | -4.6% | -4.6% |  |  |  |
| Animation RGB 444 | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -4.2% | -4.2% | -4.2% | -4.6% | -4.4% | -4.4% |  |  |  |
| Animation YUV 444 | -0.1% | -0.2% | -0.2% | 0.0% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -7.8% | -7.8% | -7.9% | -7.6% | -7.7% | -7.7% |  |  |  |
| SC(444) YUV Optional | -6.6% | -6.6% | -6.6% | -7.8% | -7.7% | -7.6% |  |  |  |
| Enc Time[%] | 103% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |

Table 3 Lossless results for method 2

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 4.5% | 3.1% | 2.1% |
| Class B | 0.1% | 0.0% | 0.0% |
| SC RGB 444 | 21.2% | 15.3% | 12.3% |
| Animation RGB 444 | 0.2% | 0.0% | 0.0% |
| SC YUV 444 | 20.7% | 15.3% | 11.7% |
| Animation YUV 444 | 0.1% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 37.2% | 33.1% | 31.0% |
| SC YUV 444 Optional | 41.5% | 37.9% | 38.2% |
| Enc Time[%] | 154% | 113% | 107% |
| Dec Time[%] | 98% | 96% | 96% |

Table 4 Lossy results for method 2

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -9.3% | -9.2% | -9.3% | -8.2% | -8.4% | -8.4% | -7.1% | -7.3% | -7.3% |
| Class B | -0.2% | -0.3% | -0.4% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | -26.1% | -26.3% | -26.3% | -25.5% | -25.8% | -25.7% | -25.1% | -25.1% | -25.2% |
| Animation RGB 444 | -1.1% | -1.1% | -1.0% | -0.7% | -0.8% | -0.7% | -0.4% | -0.5% | -0.5% |
| SC YUV 444 | -25.3% | -24.9% | -25.1% | -25.3% | -24.9% | -25.0% | -25.0% | -24.7% | -24.5% |
| Animation YUV 444 | -0.9% | -1.1% | -1.1% | -0.7% | -0.8% | -0.9% | -0.5% | -0.6% | -0.7% |
| RangeExt | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -42.0% | -41.9% | -42.4% | -42.1% | -42.5% | -43.1% | -42.6% | -42.4% | -42.7% |
| SC(444) YUV Optional | -44.5% | -43.6% | -43.7% | -44.8% | -44.3% | -44.2% | -45.3% | -44.9% | -44.9% |
| Enc Time[%] | 145% |  |  | 142% |  |  | 140% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -6.9% | -7.4% | -7.4% | -6.2% | -6.7% | -6.6% |  |  |  |
| Class B | 0.0% | -0.4% | -0.4% | 0.0% | -0.2% | -0.2% |  |  |  |
| SC RGB 444 | -21.1% | -21.1% | -21.2% | -20.2% | -20.2% | -20.2% |  |  |  |
| Animation RGB 444 | -0.4% | -0.5% | -0.4% | -0.2% | -0.3% | -0.3% |  |  |  |
| SC YUV 444 | -20.2% | -20.2% | -20.6% | -20.3% | -19.9% | -20.2% |  |  |  |
| Animation YUV 444 | -0.2% | -0.5% | -0.5% | -0.1% | -0.3% | -0.3% |  |  |  |
| RangeExt | 0.0% | -0.2% | -0.2% | 0.1% | -0.1% | -0.1% |  |  |  |
| SC(444) GBR Optional | -37.6% | -37.1% | -37.5% | -37.9% | -37.5% | -38.0% |  |  |  |
| SC(444) YUV Optional | -39.5% | -38.6% | -39.1% | -40.4% | -39.5% | -39.9% |  |  |  |
| Enc Time[%] | 113% |  |  | 112% |  |  |  |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -4.3% | -4.8% | -4.1% | -4.0% | -4.3% | -4.0% |  |  |  |
| Class B | 0.0% | -0.2% | -0.4% | 0.0% | -0.1% | -0.3% |  |  |  |
| SC RGB 444 | -16.8% | -15.8% | -15.9% | -16.3% | -15.2% | -15.2% |  |  |  |
| Animation RGB 444 | -0.6% | -0.6% | -0.5% | -0.4% | -0.4% | -0.3% |  |  |  |
| SC YUV 444 | -14.7% | -14.0% | -14.3% | -15.2% | -13.9% | -14.1% |  |  |  |
| Animation YUV 444 | -0.2% | -0.4% | -0.5% | -0.2% | -0.3% | -0.3% |  |  |  |
| RangeExt | 0.0% | -0.1% | -0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC(444) GBR Optional | -32.2% | -30.6% | -30.6% | -32.2% | -30.3% | -30.5% |  |  |  |
| SC(444) YUV Optional | -35.8% | -34.4% | -34.6% | -37.3% | -35.5% | -35.9% |  |  |  |
| Enc Time[%] | 110% |  |  | 109% |  |  |  |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  |  |  |  |

Table 5 Lossless results for method 3

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 4.5% | 2.5% | 0.7% |
| Class B | 0.1% | 0.0% | 0.0% |
| SC RGB 444 | 21.2% | 13.0% | 7.9% |
| Animation RGB 444 | 0.2% | 0.0% | 0.0% |
| SC YUV 444 | 20.7% | 13.2% | 7.5% |
| Animation YUV 444 | 0.1% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 37.2% | 29.9% | 18.1% |
| SC YUV 444 Optional | 41.5% | 34.6% | 26.5% |
| Enc Time[%] | 153% | 99% | 97% |
| Dec Time[%] | 98% | 98% | 98% |

Table 6 Lossy results for method 3

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -9.3% | -9.2% | -9.3% | -8.2% | -8.4% | -8.4% | -7.1% | -7.3% | -7.3% |
| Class B | -0.2% | -0.3% | -0.4% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| SC RGB 444 | -26.1% | -26.3% | -26.3% | -25.5% | -25.8% | -25.7% | -25.1% | -25.1% | -25.2% |
| Animation RGB 444 | -1.1% | -1.1% | -1.0% | -0.7% | -0.8% | -0.7% | -0.4% | -0.5% | -0.5% |
| SC YUV 444 | -25.3% | -24.9% | -25.1% | -25.3% | -24.9% | -25.0% | -25.0% | -24.7% | -24.5% |
| Animation YUV 444 | -0.9% | -1.1% | -1.1% | -0.7% | -0.8% | -0.9% | -0.5% | -0.6% | -0.7% |
| RangeExt | -0.1% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -42.0% | -41.9% | -42.4% | -42.1% | -42.5% | -43.1% | -42.6% | -42.4% | -42.7% |
| SC(444) YUV Optional | -44.5% | -43.6% | -43.7% | -44.8% | -44.3% | -44.2% | -45.3% | -44.9% | -44.9% |
| Enc Time[%] | 142% |  |  | 140% |  |  | 137% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 101% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -6.4% | -6.9% | -6.9% | -5.8% | -6.3% | -6.3% |  |  |  |
| Class B | 0.0% | -0.3% | -0.3% | 0.0% | -0.2% | -0.2% |  |  |  |
| SC RGB 444 | -17.8% | -18.0% | -18.0% | -17.1% | -17.3% | -17.3% |  |  |  |
| Animation RGB 444 | -0.5% | -0.5% | -0.4% | -0.3% | -0.3% | -0.3% |  |  |  |
| SC YUV 444 | -17.2% | -17.4% | -17.7% | -17.2% | -16.9% | -17.2% |  |  |  |
| Animation YUV 444 | -0.3% | -0.6% | -0.6% | -0.3% | -0.4% | -0.4% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC(444) GBR Optional | -28.5% | -28.6% | -29.0% | -29.4% | -29.7% | -30.2% |  |  |  |
| SC(444) YUV Optional | -32.7% | -32.7% | -32.9% | -33.3% | -33.2% | -33.4% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -2.7% | -3.3% | -2.8% | -2.5% | -3.0% | -2.8% |  |  |  |
| Class B | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| SC RGB 444 | -9.5% | -9.2% | -9.3% | -9.5% | -9.3% | -9.4% |  |  |  |
| Animation RGB 444 | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -8.0% | -8.1% | -7.9% | -8.8% | -8.6% | -8.5% |  |  |  |
| Animation YUV 444 | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |  |  |  |
| RangeExt | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% |  |  |  |
| SC(444) GBR Optional | -14.8% | -14.6% | -14.6% | -15.5% | -15.3% | -15.6% |  |  |  |
| SC(444) YUV Optional | -17.5% | -17.4% | -17.4% | -20.0% | -19.6% | -19.8% |  |  |  |
| Enc Time[%] | 98% |  |  | 98% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

# Conclusions

In this document, three methods are presented to fix the CIP in HEVC Range Extensions.

# Draft Text Changes

## Method 1

if( intra_block_copy_enabled_flag && (!constrained_intra_pred_flag ¦¦ slice_type == I))

intra_bc_flag[ x0 ][ y0 ]

## Method 2

In 7.4.3.2

…

constrained_intra_pred_flag equal to 0 specifies that intra prediction allows usage of residual data and decoded samples of neighbouring coding blocks coded using either intra or inter prediction modes. constrained_intra_pred_flag equal to 1 specifies constrained intra prediction when intra_bc_flag equals to 0 and slice_type is not equal to I, in which case intra prediction only uses residual data and decoded samples from neighbouring coding blocks of which intra_bc_flag is equals to 0 and coded using intra prediction modes.

…

In 8.4.4.2.1

…

–	Each sample p[ x ][ y ] is derived as follows:

- If one or more of the following conditions are true, the sample p[ x ][ y ] is marked as "not available for intra prediction":

  - The variable availableN is equal to FALSE.

  - CuPredMode[ xNbY ][ yNbY ] is not equal to MODE_INTRA and constrained_intra_pred_flag is equal to 1.

  - When slice_type is not equal to I, CuPredMode[ xNbY ][ yNbY ] is equal to MODE_INTRA, and intra_bc_flag[ xNbY ][ yNbY ] is equal to 1, and constrained_intra_pred_flag is equal to 1.

…

## Method 3

In 7.4.9.9

…

The variable BvIntra[ x0 ][ y0 ][ compIdx ] specifies the vector used for the intra block copying prediction mode.  The value of BvIntra[ x0 ][ y0 ] shall be in the range of 0 to 128, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 1), MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 2), BvIntra[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

When constrained_intra_pred_flag is equal to 1, CuPredMode[ x ][ y ] shall be equal to MODE_INTRA, with x=x0 + BvIntra[ x0 ][ y0 ][ 0 ] .. x0+ BvIntra[ x0 ][ y0 ][ 0 ]+ (1 << log2CbSize) – 1, y=y0+ BvIntra[ x0 ][ y0 ][ 1 ].. y0+ BvIntra[ x0 ][ y0 ][ 1 ]+ (1 << log2CbSize) – 1 ;

# References

[1] C. Pang, J. Sole, L. Guo, M. Karczewicz, and R. Joshi, “Non-RCE3: Intra Motion Compensation with 2-D MVs”, JCTVC-N0256, Vienna, AT, August 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).