| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0157 |
| --- | --- |

| Title: | AhG5: Intra block copying with padding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Liwei Guo, Rajan Joshi, Marta Karczewicz Tao Lin, Shuhui Wang 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc., Tongji University |  |  |

_____________________________

# Abstract

Intra Block Copying (Intra BC) was adopted as an intra coding tool in the HEVC Range Extensions draft specification. For the CUs with Intra BC mode, all the samples in reference block need to be reconstructed (available). In this document, the intra block copying method is extended to the situation where the reference block is overlapped with current CU, and the reference block is completed by padding. The performance gains for the AhG8 test conditions are reported to be average BD-rate reduction of 0.8%, 0.9%, 3.2%, 1.9%, respectively for RGB SC, YUV SC, optional RGB SC, optional YUV SC sequences in AI HE Main-Tier configurations.

# Introduction

During the 14th JCT-VC meeting, Intra Block Copying (Intra BC) [1] was adopted as a new intra coding tool in HEVC Range Extensions draft specification. Intra BC is an efficient technique to predict repetitive patterns in the frame and leads to large bit-rate savings for screen content. Currently, for the CUs with Intra BC mode, all the samples in reference block need to be reconstructed samples. Because of this requirement, the reference block cannot overlap with current CU.

# Proposed Methods

In this document, we propose to extend current Intra BC to the following the situation where the reference block is overlapped with current CU.

Let DVx and DVy denote the horizontal and the vertical component of displacement vector (DV). Let W and H denote the width and the height of current CU. When the reference block is overlapped with current CU, or equivalently (DVx <= 0) && (DVx + W > 0) && (DVy <= 0) && (DVy + H > 0), the left and top parts of the reference block are available, as shown in Figure 1. The unavailable part is padded by horizontally copying the nearest available reconstructed sample in the reference block.

Figure 1. Illustration of Intra BC with padding

# Results

In this section, the experimental results comparing to anchor are presented for

- Test 1: The proposed method;

- Test 2: The proposed method with the search region being current CTU + left 4 columns;

- Test 3: Combination of proposed method and DV coding method 1 in [2];

BD-rates are provided for lossless and lossy configurations:

- Results of test 1 for lossless (Table 1) and lossy (Table 2);

- Results of test 2 for lossless (Table 5) and lossy (Table 6);

- Results of test 3 for lossless (Table 5) and lossy (Table 6);

Table 1 Lossless results for test 1

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 0.1% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.6% | 0.4% | 0.4% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.5% | 0.5% | 0.3% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 1.5% | 1.3% | 2.0% |
| SC YUV 444 Optional | 1.6% | 1.3% | 0.2% |
| Enc Time[%] | 103% | 102% | 100% |
| Dec Time[%] | 99% | 101% | 98% |

Table 2 Lossy results for test 1

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.5% | -0.5% | -0.5% | -0.5% | -0.4% | -0.4% | -0.4% | -0.3% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -0.8% | -0.8% | -0.9% | -0.9% | -0.8% | -0.8% | -0.8% | -0.7% | -0.7% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -0.9% | -0.8% | -0.9% | -0.8% | -0.8% | -0.7% | -0.8% | -0.7% | -0.6% |
| Animation YUV 444 | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -3.2% | -3.3% | -3.6% | -3.4% | -3.6% | -4.0% | -3.4% | -3.4% | -3.6% |
| SC(444) YUV Optional | -1.9% | -1.6% | -1.6% | -1.9% | -1.7% | -1.6% | -1.9% | -1.8% | -1.8% |
| Enc Time[%] | 102% |  |  | 102% |  |  | 100% |  |  |
| Dec Time[%] | 95% |  |  | 95% |  |  | 95% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.5% | -0.5% | -0.4% | -0.4% | -0.3% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.6% | -0.5% | -0.6% | -0.7% | -0.6% | -0.7% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.5% | -0.5% | -0.4% | -0.4% | -0.4% | -0.3% |  |  |  |
| Animation YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RangeExt | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | -2.3% | -2.4% | -2.6% | -2.4% | -2.5% | -3.0% |  |  |  |
| SC(444) YUV Optional | -2.0% | -1.8% | -2.0% | -1.9% | -1.8% | -1.9% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.4% | -0.7% | -0.6% | -0.3% | -0.3% | -0.3% |  |  |  |
| Class B | 0.0% | -0.2% | -0.1% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | -0.3% | -0.2% | -0.3% | -0.4% | -0.3% | -0.4% |  |  |  |
| Animation RGB 444 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC YUV 444 | -1.0% | -1.0% | -1.2% | -0.8% | -0.7% | -0.9% |  |  |  |
| Animation YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | -0.9% | -0.8% | -1.0% | -1.3% | -1.2% | -1.3% |  |  |  |
| SC(444) YUV Optional | -2.5% | -2.2% | -2.2% | -1.7% | -1.3% | -1.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |

Table 3 Lossless results for test 2

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 0.2% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 1.0% | 0.7% | 0.5% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.8% | 0.9% | 0.4% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 2.1% | 1.7% | 2.4% |
| SC YUV 444 Optional | 2.1% | 2.2% | 0.7% |
| Enc Time[%] | 101% | 103% | 100% |
| Dec Time[%] | 103% | 98% | 104% |

Table 4 Lossy results for test 2

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -0.7% | -0.7% | -0.7% | -0.6% | -0.5% | -0.5% | -0.5% | -0.4% | -0.4% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | -1.5% | -1.4% | -1.5% | -1.5% | -1.3% | -1.4% | -1.4% | -1.3% | -1.3% |
| Animation RGB 444 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | -1.5% | -1.3% | -1.4% | -1.4% | -1.2% | -1.3% | -1.4% | -1.2% | -1.2% |
| Animation YUV 444 | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -4.5% | -4.5% | -4.9% | -4.7% | -4.9% | -5.5% | -4.5% | -4.6% | -4.9% |
| SC(444) YUV Optional | -2.9% | -2.4% | -2.6% | -2.7% | -2.4% | -2.5% | -2.7% | -2.5% | -2.6% |
| Enc Time[%] | 104% |  |  | 103% |  |  | 104% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  | 96% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.6% | -0.5% | -0.5% | -0.5% | -0.4% | -0.4% |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| SC RGB 444 | -1.2% | -1.1% | -1.2% | -1.2% | -1.1% | -1.1% |  |  |  |
| Animation RGB 444 | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -1.3% | -1.1% | -1.2% | -1.3% | -1.1% | -1.1% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -3.5% | -3.5% | -3.8% | -3.6% | -3.6% | -4.2% |  |  |  |
| SC(444) YUV Optional | -2.8% | -2.5% | -2.5% | -2.7% | -2.4% | -2.4% |  |  |  |
| Enc Time[%] | 99% |  |  | 99% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -0.6% | -0.4% | -0.3% | -0.4% | -0.2% | -0.3% |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | -0.6% | -0.6% | -0.7% | -0.6% | -0.6% | -0.6% |  |  |  |
| Animation RGB 444 | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -1.1% | -0.9% | -1.1% | -0.7% | -0.5% | -0.5% |  |  |  |
| Animation YUV 444 | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% | 0.0% |  |  |  |
| RangeExt | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | -2.4% | -2.1% | -2.3% | -3.0% | -2.8% | -2.9% |  |  |  |
| SC(444) YUV Optional | -2.3% | -2.2% | -2.5% | -1.8% | -1.4% | -1.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |  |  |  |

Table 5 Lossless results for test 3

| Bit-rate saving (Average) | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 0.7% | 0.6% | 0.6% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 1.0% | 0.8% | 0.7% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 1.1% | 0.8% | 0.6% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 2.5% | 2.1% | 2.0% |
| SC YUV 444 Optional | 3.4% | 3.3% | 2.1% |
| Enc Time[%] | 103% | 97% | 99% |
| Dec Time[%] | 93% | 96% | 94% |

Table 6 Lossy results for test 3

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | -1.7% | -1.7% | -1.7% | -1.3% | -1.4% | -1.3% | -1.0% | -1.0% | -1.0% |
| Class B | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |
| SC RGB 444 | -2.5% | -2.6% | -2.7% | -2.2% | -2.3% | -2.2% | -1.9% | -1.9% | -1.8% |
| Animation RGB 444 | -0.2% | -0.3% | -0.3% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% |
| SC YUV 444 | -3.5% | -3.5% | -3.6% | -2.9% | -2.9% | -2.9% | -2.4% | -2.3% | -2.3% |
| Animation YUV 444 | -0.3% | -0.3% | -0.4% | -0.2% | -0.2% | -0.2% | -0.1% | -0.2% | -0.1% |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | -7.5% | -7.6% | -7.8% | -7.2% | -7.3% | -7.5% | -6.9% | -6.5% | -6.7% |
| SC(444) YUV Optional | -6.5% | -6.5% | -6.5% | -5.8% | -5.8% | -5.8% | -5.2% | -5.1% | -5.1% |
| Enc Time[%] | 104% |  |  | 104% |  |  | 102% |  |  |
| Dec Time[%] | 94% |  |  | 94% |  |  | 95% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.4% | -1.4% | -1.4% | -1.1% | -1.1% | -1.0% |  |  |  |
| Class B | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC RGB 444 | -2.0% | -2.0% | -2.0% | -1.9% | -1.8% | -1.8% |  |  |  |
| Animation RGB 444 | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| SC YUV 444 | -2.7% | -2.8% | -2.6% | -2.2% | -2.3% | -2.2% |  |  |  |
| Animation YUV 444 | -0.1% | -0.2% | -0.2% | -0.1% | -0.2% | -0.1% |  |  |  |
| RangeExt | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | -6.2% | -6.3% | -6.5% | -5.9% | -6.0% | -6.2% |  |  |  |
| SC(444) YUV Optional | -6.4% | -6.3% | -6.5% | -5.5% | -5.3% | -5.5% |  |  |  |
| Enc Time[%] | 104% |  |  | 103% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | -1.2% | -1.7% | -1.5% | -0.9% | -1.0% | -1.2% |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.2% |  |  |  |
| SC RGB 444 | -1.2% | -1.2% | -1.2% | -1.1% | -1.0% | -1.0% |  |  |  |
| Animation RGB 444 | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |  |  |  |
| SC YUV 444 | -3.0% | -3.0% | -3.1% | -2.3% | -2.2% | -2.5% |  |  |  |
| Animation YUV 444 | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% | -0.2% |  |  |  |
| SC(444) GBR Optional | -4.0% | -3.8% | -3.9% | -4.0% | -3.9% | -3.8% |  |  |  |
| SC(444) YUV Optional | -7.5% | -7.2% | -7.4% | -5.8% | -5.4% | -5.7% |  |  |  |
| Enc Time[%] | 103% |  |  | 102% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

# Draft text changes

In 8.4.4.2.7

….

The reference sample at location ( xTbCmp + x, yTbCmp + y), Ref[xTbCmp + x][ yTbCmp + y], with x, y = 0..nTbS − 1, is specified by

Ref[xTbCmp + x][ yTbCmp + y] = Ref[xTbCmp - 1][ yTbCmp + y]

The (nTbS)x(nTbS) array of predicted samples samples, with x, y = 0..nTbS − 1, are derived as follows:

…

# References

[1] C. Pang, J. Sole, L. Guo, M. Karczewicz, and R. Joshi, “Non-RCE3: Intra Motion Compensation with 2-D MVs”, JCTVC-N0256, Vienna, AT, August 2013.

[2] C. Pang, J. Sole, L. Guo, R. Joshi, and M. Karczewicz, “AhG5: Displacement vector signaling for intra block copying”, JCTVC-O0154, Geneva, CH, October 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).