| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0158 |
| --- | --- |

| Title: | AhG8: Context derivation method for intra_bc_flag coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Joel Sole, Liwei Guo, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Intra Block Copying (Intra BC) was adopted at the 14th JCT-VC meeting as an intra coding tool in the HEVC Range Extensions draft specification. Intra BC codes the intra_bc_flag to signal whether the method is applied for the CU. The context derivation for this flag needs a line buffer. Two methods are proposed in this contribution to remove the line buffer for the intra_bc_flag.

# Introduction

At the 14th JCT-VC meeting, Intra Block Copying (Intra BC) [1] was adopted as a new intra coding tool in HEVC Range Extensions draft specification. Intra BC is an efficient technique to remove repetitive patterns in a frame, which provides large bit-rate savings for screen content.

Currently the context for intra_bc_flag is derived as follows

context value = (above intra_bc_flag == 0) ? 0: 1 + (left intra_bc_flag == 0) ? 0: 1

Therefore, an extra line buffer is needed to store the above intra_bc_flag values.

# Proposed Methods

## Method 1

In order to avoid extra line buffer, only the above MV inside current CTU is allowed to be used. By modeling the context like this, the extra line buffer is avoided. The number of context is kept the same.

## Method 2

It is proposed to use single context for intra_bc_flag coding. By modeling the context like this, the extra line buffer is avoided. The number of context is reduced to 1.

# Results

In this section, experimental results are presented for the proposed methods. The methods are implemented in HM12.0_Rext4.1 and simulated under the RCE3 common test conditions [2]. The simulation platform is a homogenous LINUX cluster consisting of Intel(R) XEON CPUs. The performance is compared to the anchor for lossless as well as lossy configuration in terms of BD-rates.

- Results of method 1 for lossless (Table 1) and lossy (Table 2);

- Results of method 2 for lossless (Table 3) and lossy (Table 4);

Table 1. Lossless results for method 1

| Average bit-rate saving | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 0.0% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.0% | 0.0% | 0.0% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.0% | 0.1% | 0.0% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | 0.0% | 0.1% | 0.8% |
| SC YUV 444 Optional | 0.0% | 0.2% | -0.1% |
| Enc Time[%] | 100% | 99% | 99% |
| Dec Time[%] | 100% | 100% | 100% |

Table 2. Lossy results for method 1

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) YUV Optional | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Class B | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | 0.0% | 0.0% | 0.0% | -0.1% | -0.2% | -0.2% |  |  |  |
| Animation RGB 444 | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC(444) GBR Optional | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC(444) YUV Optional | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |  |  |  |
| Enc Time[%] | 101% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | -0.5% | -0.2% | 0.0% | -0.2% | -0.2% |  |  |  |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | 0.4% | 0.4% | 0.3% | 0.3% | 0.3% | 0.2% |  |  |  |
| Animation RGB 444 | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | -0.2% | -0.2% | -0.3% | -0.3% | -0.3% | -0.4% |  |  |  |
| Animation YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.2% |  |  |  |
| SC(444) GBR Optional | -0.2% | -0.4% | -0.2% | 0.2% | 0.1% | 0.2% |  |  |  |
| SC(444) YUV Optional | -0.4% | -0.2% | -0.3% | 0.1% | 0.1% | 0.0% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |  |  |  |

Table 3 Lossless results for method 2

| Average bit-rate saving | AI Main | RA Main | LB Main |
| --- | --- | --- | --- |
| Class F | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.0% | -0.1% | 0.0% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.0% | 0.0% | 0.0% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% |
| SC GBR 444 Optional | -0.1% | 0.0% | -0.2% |
| SC YUV 444 Optional | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 101% | 101% | 99% |
| Dec Time[%] | 99% | 105% | 100% |

Table 4 Lossy results for method 2

|  | All Intra HE Main-tier |  |  | All Intra HE High-tier |  |  | All Intra HE Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC YUV 444 | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% |
| Animation YUV 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SC(444) GBR Optional | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% |
| SC(444) YUV Optional | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 98% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  |
|  | Random Access HE Main-tier |  |  | Random Access HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC RGB 444 | 0.1% | 0.1% | 0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Animation RGB 444 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |  |  |  |
| Animation YUV 444 | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |  |  |  |
| SC(444) YUV Optional | -0.1% | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% |  |  |  |
| Enc Time[%] | 102% |  |  | 101% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  | 108% |  |  |  |  |  |
|  | Low delay B HE Main-tier |  |  | Low delay B HE High-tier |  |  |  |  |  |
|  | Y | U | V | Y | U | V |  |  |  |
| Class F | 0.1% | -0.1% | -0.2% | 0.1% | 0.0% | 0.1% |  |  |  |
| Class B | 0.1% | -0.1% | -0.1% | 0.0% | -0.1% | 0.1% |  |  |  |
| SC RGB 444 | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% |  |  |  |
| Animation RGB 444 | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |  |  |  |
| SC YUV 444 | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% |  |  |  |
| Animation YUV 444 | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |  |  |  |
| RangeExt | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |  |  |  |
| SC(444) GBR Optional | -0.3% | -0.2% | -0.3% | -0.1% | -0.1% | -0.3% |  |  |  |
| SC(444) YUV Optional | -0.2% | 0.1% | -0.1% | 0.7% | 1.1% | 0.7% |  |  |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |  |  |  |

# Text Modifications

## Method 2

Table 9-5 – Association of ctxIdx and syntax elements for each initializationType in the initialization process

…

| coding_unit( ) | cu_transquant_bypass_flag | Error: Reference source not found | 0 | 1 | 2 |
| --- | --- | --- | --- | --- | --- |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | intra_bc_flag[ ][ ] | Table 9-6 | 0 | 1 | 2 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 | 1..4 | 5..8 |
|  | prev_intra_luma_pred_flag[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | intra_chroma_pred_mode[ ][ ] | Error: Reference source not found | 0 | 1 | 2 |
|  | rqt_root_cbf | Error: Reference source not found |  | 0 | 1 |

Table 9-6 – Values of initValue for ctxIdx of intra_bc_flag

| Initialization variable | ctxIdx of intra_bc_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | 185 | 139 | 139 |

# References

[1] C. Pang, J. Sole, L. Guo, M. Karczewicz, and R. Joshi, “Non-RCE3: Intra Motion Compensation with 2-D MVs”, JCTVC-N0256, Vienna, AT, August 2013.

[2] A. Saxena, D. Kwon, M. Naccari, and C. Pang, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction Techniques”, JCTVC-N1123, Vienna, AT, August 2013.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).