| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0160 |
| --- | --- |

| Title: | Non-SCE4: Cross-check of Combined bit-depth and color gamut conversion with 3D LUT for SHVC color gamut scalability (JCTVC-O0161) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Philippe Salmon, Franck Hiron | Tel: Email: | +33.2.99.27.32.42 philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports cross-check results of JCTVC-O0161 on Combined bit-depth and color gamut conversion with 3D LUT for SHVC color gamut scalability. The color gamut scalability is based on 3D color look-up tables to predict the EL color samples from the reconstructed BL color samples. The source code has been checked to verify the consistency with the proposal, and the simulation results in this document match with those provided by the proponent.

# Introduction

At the last meeting, SCE4 was set up for testing color gamut and bit depth scalability. The proposed color gamut scalability is based on the use of 3D color look-up tables (LUT), in 8-bit or 10-bit precision, and bit depth scalability supports 8 to 10-bit scalability. 3 tests have been conducted:

- “shm3.0.1_scbd_cgs_InterDigital_r713_cgs0_vs_shm3.0.1_CGS_for_crosscheck_v0_LUT8B.xls” is the data sheet for the results of using 8 bit 3D LUT for base layer 8 bit coding test compared with SCE4 anchors.

- “shm3.0.1_scbd_cgs_InterDigital_r713_cgs0_vs_shm3.0.1_CGS_for_crosscheck_v0_LUT10B.xls” is the data sheet for the results of using 10 bit 3D LUT for base layer 8 bit coding test compared with SCE4 anchors.

- “shm3.0.1_scbd_cgs_InterDigital_r713_cgs0_vs_shm3.0.1_CGS_for_crosscheck_v0_LUT10B.xls” is the data sheet for the results of using 10 bit 3D LUT for base layer 8 bit coding test compared with using 8-bit 3D LUT.

The test conditions (test material, configuration parameters and anchors) are the same as described in SCE4 description [1].

# Simulation results

The experiments are conducted on the cluster platform of 64-bit Linux version. Table 1 represents the simulation results of the first test (8-bit base, 8-bit LUT),  represents the simulation results of the tests 2 and 3 (8-bit or 10-bit base, 10-bit LUT).

Table 1: Color gamut and bit depth scalability (8-bit LUT).

|  | AI HEVC 2x 8-bit base |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V |  |
| Class A+ | -13.2% | -13.2% | -18.9% |  |
| Overall (Test vs Ref) | -13.2% | -13.2% | -18.9% |  |
| Overall (Test vs single layer) | 12.6% | 14.7% | 7.2% |  |
| Overall (Ref vs single layer) | 29.8% | 32.1% | 32.1% |  |
| EL only (Test vs Ref) | -24.6% | -24.3% | -29.4% |  |
| Enc Time[%] | 97.1% |  |  |  |
| Dec Time[%] | 95.0% |  |  |  |
| BL Match | Matched |  |  |  |
|  | RA HEVC 2x 8-bit base |  |  |  |
|  | Y | U |  | V |
| Class A+ | -9.1% | -7.3% |  | -13.2% |
| Overall (Test vs Ref) | -9.1% | -7.3% |  | -13.2% |
| Overall (Test vs single layer) | 21.5% | 26.7% |  | 15.7% |
| Overall (Ref vs single layer) | 33.8% | 36.4% |  | 33.9% |
| EL only (Test vs Ref) | -16.8% | -14.4% |  | -19.9% |
| Enc Time[%] | 97.6% |  |  |  |
| Dec Time[%] | 99.7% |  |  |  |
| BL Match | Matched |  |  |  |

Table 2: Color gamut and bit depth scalability (10-bit LUT).

|  | AI HEVC 2x 10-bit base |  |  | AI HEVC 2x 8-bit base |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |  |
| Class A+ | -15.4% | -15.5% | -22.2% | -15.3% | -15.7% | -22.9% |  |
| Overall (Test vs Ref) | -15.4% | -15.5% | -22.2% | -15.3% | -15.7% | -22.9% |  |
| Overall (Test vs single layer) | 7.1% | 9.3% | 0.5% | 9.8% | 11.3% | 1.9% |  |
| Overall (Ref vs single layer) | 26.8% | 29.4% | 28.9% | 29.8% | 32.1% | 32.1% |  |
| EL only (Test vs Ref) | -28.2% | -27.8% | -33.7% | -28.3% | -28.3% | -34.6% |  |
| Enc Time[%] | 97.0% |  |  | 96.7% |  |  |  |
| Dec Time[%] | 98.7% |  |  | 92.3% |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |
|  | RA HEVC 2x 10-bit base |  |  | RA HEVC 2x 8-bit base |  |  |  |
|  | Y | U | V | Y | U |  | V |
| Class A+ | -10.5% | -9.1% | -17.2% | -10.0% | -8.7% |  | -16.6% |
| Overall (Test vs Ref) | -10.5% | -9.1% | -17.2% | -10.0% | -8.7% |  | -16.6% |
| Overall (Test vs single layer) | 18.3% | 23.3% | 9.6% | 20.3% | 24.7% |  | 11.2% |
| Overall (Ref vs single layer) | 32.3% | 35.5% | 33.0% | 33.8% | 36.4% |  | 33.9% |
| EL only (Test vs Ref) | -18.9% | -17.0% | -24.6% | -18.4% | -16.5% |  | -23.9% |
| Enc Time[%] | 96.7% |  |  | 115.7% |  |  |  |
| Dec Time[%] | 109.3% |  |  | 116.4% |  |  |  |
| BL Match | Matched |  |  | Matched |  |  |  |

# References

- Andrew Segall, Philippe Bordes, Cheung Auyeung, Xiang Li, Elena Alshina, Alberto Duenas, “Description of Core Experiment SCE4: Color Gamut and Bit-Depth Scalability”, JCTVC-N1104, 14th Meeting: Vienna, Austria, AT, July 29 – Aug 2, 2013.