| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0164v2 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0059v2 |
| --- | --- |

| Title: | MV-HEVC/SHVC HLS / JCT-VC AHG20: Multi-layer HRD operation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
|  | S. Hattori, K. Sato and T. Suzuki Sony City Osaki 9F 2-10-1 Osaki, Shinagawa, 141-8610 Tokyo, Japan |  | +81 50 3750 2741 shinobu.hattori@jp.sony.com teruhiko.suzuki@jp.sony.com |
|  | Ali Tabatabai 1730 N. First Street, MD:3NW San Jose, CA 95112 USA |  | 1-408-352-4715 ali.tabatabai@am.sony.com |
|  | Sam Narasimhan and Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 |  | 1-858-404-3985 sam.narasimhan@arrisi.com ajay.luthra@arrisi.com |
| Source: | Nokia, Sony Corp., Sony Electronics, Arris |  |  |

_____________________________

This document presents the specification text including the sequence-level HRD parameter signalling realized within the VPS VUI.

The change-marked text below indicates the proposed changes relative to the H.265/HEVC or the latest MV-HEVC/SHVC.

3.X	base bitstream partition: A bitstream partion that is also a conforming bitstream itself.

3.X	bitstream partition: A sequence of bits, in the form of a NAL unit stream or a byte stream, that is a subset of a bitstream according to a partitioning.

- Annex C

Hypothetical reference decoder

(This annex forms an integral part of this Recommendation | International Standard.)

  - General

This annex specifies the hypothetical reference decoder (HRD) and its use to check bitstream and decoder conformance.

Two types of bitstreams or bitstream subsets are subject to HRD conformance checking for this Specification. The first type, called a Type I bitstream, is a NAL unit stream containing only the VCL NAL units and NAL units with nal_unit_type equal to FD_NUT (filler data NAL units) for all access units in the bitstream. The second type, called a Type II bitstream, contains, in addition to the VCL NAL units and filler data NAL units for all access units in the bitstream, at least one of the following:

–	additional non-VCL NAL units other than filler data NAL units,

–	all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes, and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex Error: Reference source not found).

Figure C.1 shows the types of bitstream conformance points checked by the HRD.

Figure C.1 – Structure of byte streams and NAL unit streams for HRD conformance checks

The syntax elements of non-VCL NAL units (or their default values for some of the syntax elements), required for the HRD, are specified in the semantic clauses of clause Error: Reference source not found, Annexes Error: Reference source not found and Error: Reference source not found.

Two types of HRD parameter sets (NAL HRD parameters and VCL HRD parameters) are used. The HRD parameter sets are signalled through the hrd_parameters( ) syntax structure, which may be part of the SPS syntax structure or the VPS syntax structure.

Multiple tests may be needed for checking the conformance of a bitstream, which is referred to as the bitstream under test. For each test, the following steps apply in the order listed:

- An operation point under test, denoted as TargetOp, is selected. The layer identifier list OpLayerIdList of TargetOp consists of the list of nuh_layer_id values, in increasing order of nuh_layer_id values, present in the bitstream subset associated with TargetOp, which is a subset of the nuh_layer_id values present in the bitstream under test. The variable TargetLayerSetIdx is set equal to index i of the layer set specified in the active VPS that exactly consists of the layers in OpLayerIdList. The OpTid of TargetOp is equal to the highest TemporalId present in the bitstream subset associated with TargetOp.

- TargetDecLayerIdList is set equal to OpLayerIdList of TargetOp, HighestTid is set equal to OpTid of TargetOp, and the sub-bitstream extraction process as specified in clause  is invoked with the bitstream under test, HighestTid, and TargetDecLayerIdList as inputs, and the output is assigned to BitstreamToDecode.

- When num_bitstream_partitions_minus1 is greater than 0, bitstream-specific CPB operation or bitstream-partition-specific CPB operation is selected. Both CPB operations shall be tested for checking the conformance of a bitstream. When bitstream-specific CPB operation is tested, the subsequent steps apply for the bitstream under test. When bitstream-partition-specific CPB operation is tested, the subsequent steps apply to each bitstream partition of the bitstream under test, referred to as the bitstream partition under test. When bitstream-partition-specific CPB operation is tested and the input to the HRD is a bitstream, the bitstream partitions are derived with the demultiplexing process for deriving a bitstream partition in clause F.X.

- The hrd_parameters( ) syntax structure and the sub_layer_hrd_parameters( ) syntax structure applicable to TargetOp are selected as follows:

–	If bitstream-specific CPB operation is tested, the following applies:

–	If TargetDecLayerIdList contains all nuh_layer_id values present in the bitstream under test, the hrd_parameters( ) syntax structure in the active SPS (or provided through an external means not specified in this Specification) is selected.

–	Otherwise, the hrd_parameters( ) syntax structure in the active VPS (or provided through some external means not specified in this Specification) that applies to TargetOp is selected.

–	Otherwise, the hrd_parameters( ) syntax structure in the vps_vui_bsp_hrd( ) syntax structure in the active VPS (or provided through some external means not specified in this Specification) that applies to TargetOp and the bitstream partition under test is selected.

Within the selected hrd_parameters( ) syntax structure, if BitstreamToDecode is a Type I bitstream, the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( vcl_hrd_parameters_present_flag )" is selected and the variable NalHrdModeFlag is set equal to 0; otherwise (BitstreamToDecode is a Type II bitstream), the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows either the condition "if( vcl_hrd_parameters_present_flag )" (in this case the variable NalHrdModeFlag is set equal to 0) or the condition "if( nal_hrd_parameters_present_flag )" (in this case the variable NalHrdModeFlag is set equal to 1) is selected. When BitstreamToDecode is a Type II bitstream and NalHrdModeFlag is equal to 0, all non-VCL NAL units except filler data NAL units, and all  leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes, and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex Error: Reference source not found), when present, are discarded from BitstreamToDecode, and the remaining bitstream is assigned to BitstreamToDecode.

- An access unit associated with a buffering period SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) applicable to TargetOp is selected as the HRD initialization point and referred to as access unit 0. An applicable buffering period SEI message is selected from access unit 0 as follows:

–	If bitstream-specific CPB operation is tested, the following applies:

–	If TargetDecLayerIdList contains all nuh_layer_id values present in the bitstream under test, a non-nested buffering period SEI message is selected.

–	Otherwise, a buffering period SEI message included in the scalable nesting SEI message with bitstream_subset_flag equal to 1 and applicable to TargetOp is selected.

–	Otherwise, a buffering period SEI message included in the bitstream partition nesting SEI message applicable to the bitstream partition under test is selected.

- For each access unit in BitstreamToDecode starting from access unit 0, a buffering period SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the access unit and applies to TargetOp is selected, a picture timing SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the access unit and applies to TargetOp is selected, and when SubPicHrdFlag is equal to 1 and sub_pic_cpb_params_in_pic_timing_sei_flag is equal to 0, a decoding unit information SEI messages (present in BitstreamToDecode or available through external means not specified in this Specification) that are associated with decoding units in the access unit and apply to TargetOp are selected as follows.

–	If bitstream-specific CPB operation is tested, the following applies:

–	If TargetDecLayerIdList contains all nuh_layer_id values present in the bitstream under test, non-nested buffering period, picture timing and decoding unit information SEI messages are selected.

–	Otherwise, buffering period, picture timing and decoding unit information SEI messages included in the scalable nesting SEI message with bitstream_subset_flag equal to 1 and applicable to TargetOp are selected.

–	Otherwise, buffering period, picture timing and decoding unit information SEI messages included in the bitstream partition nesting SEI message and applicable to the bitstream partition under test are selected.

- A value of SchedSelIdx is selected as follows.

–	If bitstream-specific CPB operation is tested, the selected SchedSelIdx shall be in the range of 0 to cpb_cnt_minus1[ HighestTid ], inclusive, where cpb_cnt_minus1[ HighestTid ] is found in the sub_layer_hrd_parameters( HighestTid ) syntax structure as selected above.

–	Otherwise, a SchedSelCombIdx is selected for the bitstream under test and used for each bitstream partition under test. The selected SchedSelCombIdx shall be in the range of 0 to num_bsp_sched_combinations_minus1, inclusive. The selected SchedSelIdx shall be equal to bsp_comb_sched_idx[ TargetLayerSetIdx ][ SchedSelCombIdx ][ j ] where j is the index of the bitstream partition under test.

- When the coded picture in access unit 0 has nal_unit_type equal to CRA_NUT or BLA_W_LP, and irap_cpb_params_present_flag in the selected buffering period SEI message is equal to 1, either of the following applies for selection of the initial CPB removal delay and delay offset:

–	If NalHrdModeFlag is equal to 1, the default initial CPB removal delay and delay offset represented by nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. Otherwise, the default initial CPB removal delay and delay offset represented by vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. The variable DefaultInitCpbParamsFlag is set equal to 1.

–	If NalHrdModeFlag is equal to 1, the alternative initial CPB removal delay and delay offset represented by nal_initial_alt_cpb_removal_delay[ SchedSelIdx ] and nal_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. Otherwise, the alternative initial CPB removal delay and delay offset represented by vcl_initial_alt_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, in the selected buffering period SEI message are selected. The variable DefaultInitCpbParamsFlag is set equal to 0, and the RASL access units associated with access unit 0 are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

- For bitstream-partition-specific CPB operation, SubPicHrdFlag is set equal to 1. For bitstream-specific CPB operation, when sub_pic_hrd_params_present_flag in the selected hrd_parameters( ) syntax structure is equal to 1, the CPB is scheduled to operate either at the access unit level (in which case the variable SubPicHrdFlag is set equal to 0) or at the sub-picture level (in which case the variable SubPicHrdFlag is set equal to 1).

For each operation point under test, the number of bitstream conformance tests for bitstream-specific CPB operation to be performed is equal to n0 * n1 * ( n2 * 2 + n3 ) * n4, where the values of n0, n1, n2, n3, and n4 are specified as follows:

–	n0 is derived as follows:

–	If BitstreamToDecode is a Type I bitstream, n0 is equal to 1.

–	Otherwise (BitstreamToDecode is a Type II bitstream), n0 is equal to 2.

–	n1 is equal to cpb_cnt_minus1[ HighestTid ] + 1.

–	n2 is the number of access units in BitstreamToDecode that each is associated with a buffering period SEI message applicable to TargetOp and for each of which both of the following conditions are true:

–	nal_unit_type is equal to CRA_NUT or BLA_W_LP for the VCL NAL units;

–	The associated buffering period SEI message applicable to TargetOp has irap_cpb_params_present_flag equal to 1.

–	n3 is the number of access units in BitstreamToDecode BitstreamToDecode that each is associated with a buffering period SEI message applicable to TargetOp and for each of which one or both of the following conditions are true:

–	nal_unit_type is equal to neither CRA_NUT nor BLA_W_LP for the VCL NAL units;

–	The associated buffering period SEI message applicable to TargetOp has irap_cpb_params_present_flag equal to 0.

–	n4 is derived as follows:

–	If sub_pic_hrd_params_present_flag in the selected hrd_parameters( ) syntax structure is equal to 0, n4 is equal to 1;

–	Otherwise, n4 is equal to 2.

When BitstreamToDecode is a Type II bitstream, the following applies:

–	If the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( vcl_hrd_parameters_present_flag )" is selected, the test is conducted at the Type I conformance point shown in Figure C.1, and only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

–	Otherwise (the sub_layer_hrd_parameters( HighestTid ) syntax structure that immediately follows the condition "if( nal_hrd_parameters_present_flag )" is selected), the test is conducted at the Type II conformance point shown in Figure C.1, and all bytes of the Type II bitstream, which may be a NAL unit stream or a byte stream, are counted for the input bit rate and CPB storage.

NOTE 1 – NAL HRD parameters established by a value of SchedSelIdx for the Type II conformance point shown in Figure C.1 are sufficient to also establish VCL HRD conformance for the Type I conformance point shown in Figure C.1 for the same values of InitCpbRemovalDelay[ SchedSelIdx ], BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] for the VBR case (cbr_flag[ SchedSelIdx ] equal to 0). This is because the data flow into the Type I conformance point is a subset of the data flow into the Type II conformance point and because, for the VBR case, the CPB is allowed to become empty and stay empty until the time a next picture is scheduled to begin to arrive. For example, when decoding a CVS conforming to one or more of the profiles specified in Annex Error: Reference source not found using the decoding process specified in clauses Error: Reference source not found through , when NAL HRD parameters are provided for the Type II conformance point that not only fall within the bounds set for NAL HRD parameters for profile conformance in item Error: Reference source not found of clause Error: Reference source not found but also fall within the bounds set for VCL HRD parameters for profile conformance in item Error: Reference source not found of clause Error: Reference source not found, conformance of the VCL HRD for the Type I conformance point is also assured to fall within the bounds of item Error: Reference source not found of clause Error: Reference source not found.

All VPSs, SPSs and PPSs referred to in the VCL NAL units, and the corresponding buffering period, picture timing and decoding unit information SEI messages shall be conveyed to the HRD, in a timely manner, either in the bitstream (by non-VCL NAL units), or by other means not specified in this Specification.

In Annexes C, Error: Reference source not found, and Error: Reference source not found, the specification for "presence" of non-VCL NAL units that contain VPSs, SPSs, PPSs, buffering period SEI messages, picture timing SEI messages, or decoding unit information SEI messages is also satisfied when those NAL units (or just some of them) are conveyed to decoders (or to the HRD) by other means not specified in this Specification. For the purpose of counting bits, only the appropriate bits that are actually present in the bitstream are counted.

NOTE 2 – As an example, synchronization of such a non-VCL NAL unit, conveyed by means other than presence in the bitstream, with the NAL units that are present in the bitstream, can be achieved by indicating two points in the bitstream, between which the non-VCL NAL unit would have been present in the bitstream, had the encoder decided to convey it in the bitstream.

When the content of such a non-VCL NAL unit is conveyed for the application by some means other than presence within the bitstream, the representation of the content of the non-VCL NAL unit is not required to use the same syntax as specified in this Specification.

NOTE 3 – When HRD information is contained within the bitstream, it is possible to verify the conformance of a bitstream to the requirements of this clause based solely on information contained in the bitstream. When the HRD information is not present in the bitstream, as is the case for all "stand-alone" Type I bitstreams, conformance can only be verified when the HRD data are supplied by some other means not specified in this Specification.

For bitstream-specific CPB operation, the HRD contains a coded picture buffer (CPB), an instantaneous decoding process, a decoded picture buffer (DPB), and output cropping as shown in Figure C.2.

Figure C.2 – HRD buffer model

For bitstream-partition-specific CPB operation, the HRD contains a bitstream demultiplexer (optionally present), two or more bitstream partition buffers (BPB), two or more instantaneous decoding processes, a decoded picture buffer (DPB), and output cropping as shown in X.

Figure C.3 – Bitstream-partition-specific HRD buffer model

For each bitstream conformance test, the CPB size (number of bits) for bitstream-specific CPB operation and the BPB size for bitstream-partition-specific CPB operation is CpbSize[ SchedSelIdx ] as specified in clause Error: Reference source not found, where SchedSelIdx and the HRD parameters are specified above in this clause. The DPB size (number of picture storage buffers) is sps_max_dec_pic_buffering_minus1[ HighestTid ] + 1.

The variable SubPicHrdPreferredFlag is either specified by external means, or when not specified by external means, set equal to 0.

When the value of the variable SubPicHrdFlag has not been set by step 9 above in this clause, it is derived as follows:

SubPicHrdFlag = SubPicHrdPreferredFlag  &&  sub_pic_hrd_params_present_flag	(C-1)

If SubPicHrdFlag is equal to 0, the HRD operates at access unit level and each decoding unit is an access unit. Otherwise the HRD operates at sub-picture level and each decoding unit is a subset of an access unit.

NOTE 4 – If the HRD operates at access unit level, each time a decoding unit that is an entire access unit is removed from the CPB. Otherwise (the HRD operates at sub-picture level), each time a decoding unit that is a subset of an access unit is removed from the CPB. In both cases, each time an entire decoded picture is output from the DPB, though the picture output time is derived based on the differently derived CPB removal times and the differently signalled DPB output delays.

The following is specified for expressing the constraints in this annex:

–	Each access unit is referred to as access unit n, where the number n identifies the particular access unit. Access unit 0 is selected per step 5 above. The value of n is incremented by 1 for each subsequent access unit in decoding order.

–	Each decoding unit is referred to as decoding unit m, where the number m identifies the particular decoding unit. The first decoding unit in decoding order in access unit 0 is referred to as decoding unit 0. The value of m is incremented by 1 for each subsequent decoding unit in decoding order.

NOTE 5 – The numbering of decoding units is relative to the first decoding unit in access unit 0.

–	Picture n refers to the coded picture or the decoded picture of access unit n.

The HRD operates as follows:

–	The HRD is initialized at decoding unit 0, with the both the CPB and the DPB and each BPB being set to be empty (the DPB fullness is set equal to 0).

NOTE 6 – After initialization, the HRD is not initialized again by subsequent buffering period SEI messages.

–	For bitstream-specific CPB operation, data associated with decoding units that flow into the CPB according to a specified arrival schedule are delivered by the HSS. For bitstream-partition-specific CPB operation, data associated with decoding units that flow into the BPB according to a specified arrival schedule are delivered by an HBPS.

–	When bitstream-partition-specific CPB operation is used, each bitstream partition with index j is processed as specified in clause C.2 with SchedSelIdx equal to bsp_comb_sched_idx[ TargetLayerSetIdx ][ SchedSelCombIdx ][ j ] and the HSS replaced by the HPBS.

–	The data associated with each decoding unit are removed and decoded instantaneously by the instantaneous decoding process at the CPB removal time of the decoding unit.

–	Each decoded picture is placed in the DPB.

–	A decoded picture is removed from the DPB when it becomes no longer needed for inter prediction reference and no longer needed for output.

For each bitstream conformance test, the operation of the CPB and BPB is specified in clause C.2, the instantaneous decoder operation is specified in clauses Error: Reference source not found through , the operation of the DPB is specified in clause Error: Reference source not found, and the output cropping is specified in clause Error: Reference source not found and clause Error: Reference source not found.

HSS, HBPS and HRD information concerning the number of enumerated delivery schedules and their associated bit rates and buffer sizes is specified in clauses Error: Reference source not found and Error: Reference source not found. The HRD is initialized as specified by the buffering period SEI message specified in clauses Error: Reference source not found and Error: Reference source not found. The removal timing of decoding units from the CPB and output timing of decoded pictures from the DPB is specified using information in picture timing SEI messages (specified in clauses Error: Reference source not found and Error: Reference source not found) or in decoding unit information SEI messages (specified in clauses Error: Reference source not found and Error: Reference source not found). All timing information relating to a specific decoding unit shall arrive prior to the CPB removal time of the decoding unit.

The requirements for bitstream conformance are specified in clause Error: Reference source not found, and the HRD is used to check conformance of bitstreams as specified above in this clause and  to check conformance of decoders as specified in clause Error: Reference source not found.

NOTE 7 – While conformance is guaranteed under the assumption that all picture-rates and clocks used to generate the bitstream match exactly the values signalled in the bitstream, in a real system each of these may vary from the signalled or specified value.

All the arithmetic in this annex is performed with real values, so that no rounding errors can propagate. For example, the number of bits in a CPB just prior to or after removal of a decoding unit is not necessarily an integer.

The variable ClockTick is derived as follows and is called a clock tick:

ClockTick = vui_num_units_in_tick  vui_time_scale	(C-2)

The variable ClockSubTick is derived as follows and is called a clock sub-tick:

ClockSubTick = ClockTick  ( tick_divisor_minus2 + 2 )	(C-3)

  - Operation of coded picture buffer (CPB) and Bitstream Partition Buffer (BPB)

    - General

The specifications in this clause apply independently to each set of CPB parameters that is present and to both the Type I and Type II conformance points shown in Figure C.1, and the set of CPB parameters is selected as specified in clause C.1.

    - Timing of decoding unit arrival

- If SubPicHrdFlag is equal to 0, the variable subPicParamsFlag is set equal to 0, and the process in specified in the remainder of this clause is invoked with a decoding unit being considered as an access unit, for derivation of the initial and final CPB arrival times for access unit n.

- Otherwise (SubPicHrdFlag is equal to 1), the process in specified in the remainder of this clause is first invoked with the variable subPicParamsFlag set equal to 0 and a decoding unit being considered as an access unit, for derivation of the initial and final CPB arrival times for access unit n, and then invoked with subPicParamsFlag set equal to 1 and a decoding unit being considered as a subset of an access unit, for derivation of the initial and final CPB arrival times for the decoding units in access unit n.

The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are derived as follows:

–	If one or more of the following conditions are true, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_alt_cpb_removal_delay[ SchedSelIdx ] and nal_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_alt_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_alt_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message syntax elements are selected as specified in clause C.1:

–	Access unit 0 is a BLA access unit for which each coded picture has nal_unit_type equal to BLA_W_RADL or BLA_N_LP, and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1.

–	Access unit 0 is a BLA access unit for which each coded picture has nal_unit_type equal to BLA_W_LP or is a CRA access unit, and the value of irap_cpb_params_present_flag of the buffering period SEI message is equal to 1, and one or more of the following conditions are true:

–	UseAltCpbParamsFlag for access unit 0 is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	The value of subPicParamsFlag is equal to 1.

–	Otherwise, InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are set equal to the values of the buffering period SEI message syntax elements nal_initial_cpb_removal_delay[ SchedSelIdx ] and nal_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_cpb_removal_delay[ SchedSelIdx ] and vcl_initial_cpb_removal_offset[ SchedSelIdx ], respectively, when NalHrdModeFlag is equal to 0, where the buffering period SEI message syntax elements are selected as specified in clause C.1.

Variables BspSyncFlag[ bspIdx ][ m ], BspSyncBspIdx[ bspIdx ][ m ] and BspSyncDuIdx[ bspIdx ][ m ] are derived for bitstream-partition-specific CPB operation for bitstream partitions with index bspIdx greater than 0 as follows:

–	If du_based_bpb_sync_flag[ TargetLayerSetIdx ][ SchedSelCombIdx ] is equal to 1, the following applies:

–	If the DU preceding, in decoding order within the bitstream, the m-th DU of bitstream partion with index bspIdx belongs to to a bitstream partition with an index prevBspIdx less than bspIdx, BspSyncFlag[ bspIdx ][ m ] is set equal to 1, BspSyncBspIdx[ bspIdx ][ m ] is set equal to prevBspIdx, and BspSyncDuIdx[ bspIdx ][ m ] is set equal to the decoding unit index of the preceding DU within the bitstream partition with index prevBspIdx.

–	Otherwise, BspSyncFlag[ bspIdx ][ m ] is set equal to 0.

–	Otherwise (du_based_bpb_sync_flag[ TargetLayerSetIdx ][ SchedSelCombIdx ] is equal to 0), the following applies:

–	If the DU is the first DU, in decoding order, of a picture and any picture in the inter-layer reference picture set has a nuh_layer_id value that belongs to a bitstream partition with a smaller index than bspIdx, BspSyncFlag[ bspIdx ][ m ] is set equal to 1, BspSyncBspIdx[ bspIdx ][ m ] is the greatest bitstream partition index refBspIdx smaller than bspIdx associated with the nuh_layer_id value refLayerId of any picture in the inter-layer reference picture set, and BspSyncDuIdx[ bspIdx ][ m ] is set equal to the decoding unit index (within the bitstream partition with index refBspIdx) of the last decoding unit of the picture with nuh_layer_id equal to refLayerId in the same access unit as the m-th DU of bitstream partition with index bspIdx.

–	Otherwise, BspSyncFlag[ bspIdx ][ m ] is set equal to 0.

The time at which the first bit of decoding unit m begins to enter the CPB is referred to as the initial arrival time initArrivalTime[ m ].

If bitstream-specific CPB operation is used, decoding units are indexed in decoding order within the bitstream. Otherwise (bitstream-partition-specific CPB operation is used), decoding units are indexed in decoding order with each bitstream partition, and thus a decoding unit may be identified with the pair of the bitstream partition index bspIdx and the decoding unit index m within the bitstream partition.

The initial arrival time of decoding unit m is derived as follows:

–	If the decoding unit is decoding unit 0 (i.e., m = 0) and either bitstream-specific CPB operation is used or the decoding unit belongs to the base bitstream partition, initArrivalTime[ 0 ] = 0.

–	Otherwise, if the decoding unit is decoding unit 0, bitstream-partition-specific CPB operation is used, the decoding unit does not belong to the base bitstream partition and cbr_flag[ SchedSelIdx ] is equal to 1, the following applies:

–	initArrivalTime[ 0 ] is obtained from the bitstream partition initial arrival time SEI message applicable to TargetOp.

–	Otherwise, the following applies:

–	The variable bspSyncFlag is derived as follows:

–	If bitstream-specific CPB operation is used or the decoding unit belongs to the base bitstream partition or BspSyncFlag[ bspIdx ][ m ] is equal to 0, bspSyncFlag is set equal to 0.

–	Otherwise, bspSyncFlag is set equal to 1.

–	If cbr_flag[ SchedSelIdx ] is equal to 1, the initial arrival time for decoding unit m is equal to the final arrival time (which is derived below) of decoding unit m − 1, i.e.,

if( !subPicParamsFlag )
	initArrivalTime[ m ] = AuFinalArrivalTime[ m − 1 ]	(C-4)
else
	initArrivalTime[ m ] = DuFinalArrivalTime[ m − 1 ]

When bspSyncFlag is equal to 1, it is a requirement of bitstream conformance that initArrivalTime[ m ] is greater than or equal to the final arrival time of the decoding unit with index BspSyncDuIdx[ bspIdx ][ m ] in the bitstream partition with index BspSyncBspIdx[ bspIdx ][ m ].

–	Otherwise (cbr_flag[ SchedSelIdx ] is equal to 0), the initial arrival time for decoding unit m is derived as follows:

if( !subPicParamsFlag )
	initArrivalTime[ m ] = Max( AuFinalArrivalTime[ m − 1 ], initArrivalEarliestTime[ m ] )	(C-5)
else
	initArrivalTime[ m ] = Max( DuFinalArrivalTime[ m − 1 ], initArrivalEarliestTime[ m ], 
		refFinalArrivalTime )

where initArrivalEarliestTime[ m ] is derived as follows:

–	The variable tmpNominalRemovalTime is derived as follows:

if( !subPicParamsFlag )
	tmpNominalRemovalTime = AuNominalRemovalTime[ m ] 	(C-6)
else
	tmpNominalRemovalTime = DuNominalRemovalTime[ m ]

where AuNominalRemovalTime[ m ] and DuNominalRemovalTime[ m ] are the nominal CPB removal time of access unit m and decoding unit m, respectively, as specified in clause Error: Reference source not found.

–	If decoding unit m is not the first decoding unit of a subsequent buffering period, initArrivalEarliestTime[ m ] is derived as follows:

initArrivalEarliestTime[ m ] = tmpNominalRemovalTime − ( InitCpbRemovalDelay[ SchedSelIdx ]
		+ InitCpbRemovalDelayOffset[ SchedSelIdx ] )  90000	(C-7)

–	Otherwise (decoding unit m is the first decoding unit of a subsequent buffering period), initArrivalEarliestTime[ m ] is derived as follows:

initArrivalEarliestTime[ m ] = tmpNominalRemovalTime −
		( InitCpbRemovalDelay[ SchedSelIdx ]  90000 )	(C-8)

and where refFinalArrivalTime is the final arrival time of the decoding unit with index BspSyncDuIdx[ bspIdx ][ m ] in the bitstream partition with index BspSyncBspIdx[ bspIdx ][ m ].

The final arrival time for decoding unit m is derived as follows:

if( !subPicParamsFlag )
		AuFinalArrivalTime[ m ] = initArrivalTime[ m ] + sizeInbits[ m ]  BitRate[ SchedSelIdx ]	(C-9)
else
		DuFinalArrivalTime[ m ] = initArrivalTime[ m ] + sizeInbits[ m ]  BitRate[ SchedSelIdx ]

- where sizeInbits[ m ] is the size in bits of decoding unit m, counting the bits of the VCL NAL units and the filler data NAL units for the Type I conformance point or all bits of the Type II bitstream for the Type II conformance point, where the Type I and Type II conformance points are as shown in Figure C.1.

The values of SchedSelIdx, BitRate[ SchedSelIdx ], and CpbSize[ SchedSelIdx ] are constrained as follows:

–	If the content of the selected hrd_parameters( ) syntax structures for the access unit containing decoding unit m and the previous access unit differ, the HSS selects a value SchedSelIdx1 of SchedSelIdx from among the values of SchedSelIdx provided in the selected hrd_parameters( ) syntax structures for the access unit containing decoding unit m that results in a BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] for the access unit containing decoding unit m. The value of BitRate[ SchedSelIdx1 ] or CpbSize[ SchedSelIdx1 ] may differ from the value of BitRate[ SchedSelIdx0 ] or CpbSize[ SchedSelIdx0 ] for the value SchedSelIdx0 of SchedSelIdx that was in use for the previous access unit.

–	Otherwise, the HSS continues to operate with the previous values of SchedSelIdx, BitRate[ SchedSelIdx ] and CpbSize[ SchedSelIdx ].

When the HSS selects values of BitRate[ SchedSelIdx ] or CpbSize[ SchedSelIdx ] that differ from those of the previous access unit, the following applies:

–	The variable BitRate[ SchedSelIdx ] comes into effect at the initial CPB arrival time of the current access unit.

–	The variable CpbSize[ SchedSelIdx ] comes into effect as follows:

–	If the new value of CpbSize[ SchedSelIdx ] is greater than the old CPB size, it comes into effect at the initial CPB arrival time of the current access unit.

–	Otherwise, the new value of CpbSize[ SchedSelIdx ] comes into effect at the CPB removal time of the current access unit.

- F.7.3.2.1.3	VPS VUI syntax

| vps_vui( ){ | Descriptor |
| --- | --- |
| … |  |
| vps_vui_bsp_hrd_present_flag | u(1) |
| if( vps_vui_bsp_hrd_present_flag ) |  |
| vps_vui_bsp_hrd( ) |  |
| … |  |
| } |  |

vps_vui_bsp_hrd_present_flag equal to 0

specifies that no bitstream-partition-specific HRD parameters are present.  vps_vui_bsp_hrd_present_flag equal to 1 specifies that bitstream-specific HRD parameters are present.

| vps_vui_bsp_hrd( ){ | Descriptor |
| --- | --- |
| vps_num_bsp_hrd_parameters_minus1 | ue(v) |
| for( i = 0; i <= vps_num_bsp_hrd_parameters_minus1; i++ ) { |  |
| if( i > 0 ) |  |
| bsp_cprms_present_flag[ i ] | u(1) |
| hrd_parameters( bsp_cprms_present_flag[ i ], vps_max_sub_layers_minus1 ) |  |
| } |  |
| for( h=1; h <= vps_num_layer_sets_minus1; h++ ) { |  |
| num_bitstream_partitions[ h ] | ue(v) |
| for( i = 0; i < num_bitstream_partitions[ h ]; i++ ) { |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |
| if( layer_id_included_flag[ h ][ j ] ) |  |
| layer_in_bsp_flag[ h ][ i ][ j ] | u(1) |
| } |  |
| } |  |
| if( num_bitstream_partitions[ h ] ) { |  |
| num_bsp_sched_combinations[ h ] | ue(v) |
| for( i = 0; i < num_bsp_sched_combinations[ h ]; i++ ) { |  |
| du_based_bpb_sync_flag[ h ][ i ] | u(1) |
| for( j = 0; j < num_bitstream_partitions[ h ]; j++ ) { |  |
| bsp_comb_hrd_idx[ h ][ i ][ j ] | ue(v) |
| bsp_comb_sched_idx[ h ][ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

vps_num_bsp_hrd_parameters_minus1 plus 1 specifies the number of hrd_parameters( ) syntax structures present within the vps_vui_bsp_hrd( ) syntax structure.

bsp_cprms_present_flag[ i ] equal to 1 specifies that the HRD parameters that are common for all sub-layers are present in the i-th hrd_parameters( ) syntax structure in the vps_vui_bsp_hrd( ) syntax structure. bsp_cprms_present_flag[  i ] equal to 0 specifies that the HRD parameters that are common for all sub-layers are not present in the i-th hrd_parameters( ) syntax structure in the vps_vui_bsp_hrd( ) syntax structure and are derived to be the same as the ( i − 1 )-th hrd_parameters( ) syntax structure in the in the vps_vui_bsp_hrd( ) syntax structure. bsp_cprms_present_flag[ 0 ] is inferred to be equal to 1.

num_bitstream_partitions[ h ] specifies the number of bitstream partitions for which HRD parameters are specified for the layer set with index h.

layer_in_bsp_flag[ h ][ i ][ j ] specifies that the layer with index j is a part of bitstream partition with index i within a layer set with index h.

It is a requirement of bitstream conformance that bitstream partition with index j shall not include direct or indirect reference layers of any layers in bitstream partition i for any values of i and j in the range of 0 to num_bitstream_partitions[ h ] – 1, inclusive, such that i is less than j.

num_bsp_sched_combinations [ h ] specifies the number of combinations of delivery schedules and hrd_parameters( ) specified for bitstream partitions for layer set with index h.

du_based_bpb_sync_flag[ h ][ i ] affects the CPB operation when the i-th combination of delivery schedules and hrd_parameters( ) is used.

bsp_comb_hrd_idx[ h ][ i ][ j ] specifies the index of hrd_parameters( ) within the vps_vui_bsp_hrd( ) syntax structure used in the i-th combination.

bsp_comb_sched_idx[ h ][ i ][ j ] specifies the index of delivery schedule within the hrd_parameters( ) syntax structure with the index bsp_comb_hrd_idx[ h ][ i ][ j ] that is used in the i-th combination.

- F.14.1.3	Bitstream partition nesting SEI message syntax

| bsp_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| bsp_idx | ue(v) |
| while( !byte_aligned( ) ) |  |
| bsp_nesting_zero_bit /* equal to 0 */ | u(1) |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| } |  |

- F.14.2.3	Bitstream partition nesting SEI message semantics

The bitstream partition nesting SEI message provides a mechanism to associate SEI messages with a bitstream partition of a layer set.

When present, this SEI message shall be contained within a scalable nesting SEI message. When this SEI message is contained in a scalable nesting SEI message, it shall be the only nested SEI message. In the scalable nesting SEI message containing this SEI message bitstream_subset_flag shall be equal to 1, nesting_op_flag is equal to 1, default_op_flag shall be equal to 0 and nesting_num_ops_minus1 shall be equal to 0.

The associated bitstream partition HRD message for the bitstream partition nesting SEI message is the preceding bitstream partition HRD message, in decoding order, that is nested in a scalable nesting SEI message with the same value of nesting_op_idx[ 0 ] as that for the scalable nesting SEI message containing the bitstream partition nesting SEI message. It is a requirement of bitstream conformance that when bitstream partition nesting SEI message is present, it shall have an associated bitstream partition HRD message within the same coded video sequence.

A bitstream partition nesting SEI message contains one or more SEI messages.

bsp_idx specifies the bitstream partition index among the bitstream partitions specified in the associated bitstream partition HRD message. This SEI message applies to the bitstream partition with index bsp_idx.

- F.14.1.4	Bitstream partition initial arrival time SEI message syntax

| bsp_initial_arrival_time( payloadSize ) { | Descriptor |
| --- | --- |
| if( NalHrdBpPresentFlag ) |  |
| for( i = 0; i < SchedCombCnt; i++ ) |  |
| nal_initial_arrival_delay[ i ] | u(v) |
| else |  |
| for( i = 0; i < SchedCombCnt; i++ ) |  |
| vcl_initial_arrival_delay[ i ] | u(v) |
| } |  |

- F.14.2.4	Bitstream partition initial arrival time SEI message semantics

The bitstream partition initial arrival time SEI message specifies the initial arrival delays to be used in the bitstream-partition-specific CPB operation.

When present, this SEI message shall be contained within bitstream partition nesting SEI message that is contained in a scalable nesting SEI message. The same bitstream partition SEI message shall also contain a picture buffering SEI message.

nal_initial_arrival_delay[ i ] specifies the initial BPB arrival delays for the i-th schedule combination of the bitstream partition to which this SEI message applies, when NAL HRD parameters are in use.

vcl_initial_arrival_delay[ i ] specifies the initial BPB arrival delays for the i-th schedule combination of the bitstream partition to which this SEI message applies, when VCL HRD parameters are in use.

- F.X	Demultiplexing process for deriving a bitstream partition

Inputs to this process are a bitstream, a layer identifier list bspLayerId[ idx ] and the number of layer identifiers numBspLayerId in the layer index list bspLayerId[ idx ].

Output of this process is a bitstream partition.

Let variable minBspLayerId be the smallest value of bspLayerId[ idx ] with any value of idx in the range of 0 to numBspLayerId – 1, inclusive.

The output bitstream partition consists of selected NAL units of the input bitstream in the same order as they appear in the input bitstream. The following NAL units of the input bitstream are omitted from the output bitstream partition, while the remaining NAL units of the input bitstream are included in the output bitstream partition:

–	Omit all NAL units that have a nuh_layer_id value other than bspLayerId[ idx ] with any value of idx in the range of 0 to numBspLayerId – 1, inclusive.

–	Omit all SEI NAL units containing a scalable nesting SEI message for which no derived nestingLayerIdList[ i ] contains any layer identifier value equal to bspLayerId[ idx ] with any value of idx in the range of 0 to numBspLayerId – 1, inclusive.

–	Omit all SEI NAL units containing a scalable nesting SEI message for which a derived nestingLayerIdList[ i ] contains a layer identifier value less than minBspLayerId.