| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0172r2 |
| --- | --- |

| Title: | AHG22: Sequences with medical mixed content and medical visual content with bit depth beyond 10 bits for HEVC development |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peter Amon, Andreas Hutter Siemens Corporate Technology Imaging and Computer Vision Otto-Hahn-Ring 6 81739 Munich, Germany Uwe-Erik Martin Siemens Healthcare Imaging and Therapy Systems Hartmannstr. 16 91052 Erlangen, Germany Benno Heigl Siemens Healthcare Imaging and Therapy Systems Siemensstr. 1 91301 Forchheim, Germany | Tel: Email: | +49 89 636 54642 p.amon@siemens.com |
| Source: | Siemens |  |  |

_____________________________

# Abstract

This contribution proposes test sequences from the medical domain for the development of HEVC, especially for the specification of range extensions. The proposed set contains medical mixed content (i.e., 8-bit 2D+t RGB image data) and medical visual content with bit depth beyond 10 bits (i.e., 12-bit and 16-bit monochrome 3D volume image data). The adoption of this material as test sequences for the further development of HEVC range extensions is proposed.

# Introduction

The current test set for the development of HEVC (including the development of range extensions) includes mainly natural image data captured by a camera and screen content captured from a computer (e.g., capturing the DVI output) or rendered by a graphics card. Image content from the medical domain is not contained in the current test set. Therefore, test sequences are proposed that contain content acquired from ultrasound, computed tomography, magnetic resonance, and angiography scanners.

# Sequence description

The sequences stem from different medical imaging modalities:

- Medical mixed content, i.e., ultrasound image data and screen content elements: 8-bit RGB 2D-t image sequences

  - Sequence US_19Segments

    - Resolution: 800x600 pixels

    - Frame rate: 30 fps

    - Number of frames: 90

  - Sequence US_Doppler

    - Resolution 800x600 pixels

    - Frame rate: 12 fps

    - Number of frames: 12

  - Sequence US_FetalHeart

    - Resolution: 800x600 pixels

    - Frame rate: 40 fps

    - Number of frames: 90

- Computed tomography (CT) image data: 12-bit monochrome, continuous-tone 3D image volumes

  - Sequence CT_Cardiac

    - Resolution: 512x512 pixels

    - Frame rate: N/A

    - Number of frames: 281

    - Effective bit depth: 12

  - Sequence CT_Longrun

    - Resolution 512x512 pixels

    - Frame rate: N/A

    - Number of frames: 1463

    - Effective bit depth: 12

- Magnetic resonance (MR) image data: 14-bit monochrome, continuous-tone 3D image volumes.

  - Sequence MR_Axial1

    - Resolution: 256x256 pixels

    - Frame rate: N/A

    - Number of frames: 42

    - Effective bit depth: 11

  - Sequence MR_Axial2

    - Resolution: 512x512 pixels

    - Frame rate: N/A

    - Number of frames: 13

    - Effective bit depth: 14

  - Sequence MR_Coronal

    - Resolution: 448x512 pixels

    - Frame rate: N/A

    - Number of frames: 45

    - Effective bit depth: 11

- Angiography (AX) image data: 16-bit monochrome, continuous-tone 2D X-ray projections

  - Sequence AX_Head

    - Resolution: 1240x960 pixels

    - Frame rate: N/A

    - Number of frames: 496

    - Effective bit depth: 16

The sequences are further described in JCTVC-N0179 [1], where also sample pictures are depicted. A summary is depicted in the following table.

# Copyright information

Siemens AG remains the owner of the test sequence and any and all intellectual property rights and/or copyrights therein. The test sequence may only be used for the purposes of development, testing, promulgation, and promotion of the standards currently contemplated by the Joint Collaborative Team on Video Coding (JCT-VC). The test sequence may not be copied or distributed against payment of royalties. Siemens AG‘s liability with respect to the test sequences is excluded to the widest possible extent. This includes but is not limited to any liability for Siemens AG’s ownership of the test sequences and/or any and all intellectual property rights and/or copyrights, as well as any liability for the infringement of third party intellectual property rights and/or copyrights. Furthermore, Siemens AG makes no warranties whatsoever with regard to the test sequence’s fitness for any purpose.

# Recommendation

The presented sequences provide unique characteristics that are common for medical imaging applications. We recommend the adoption of this material as test sequences for the further development of HEVC range extensions.

# Acknowledgement

The research leading to these results has received funding from the European Union’s Seventh Framework Programme ([FP7/2007-2013]) under grant agreement n◦ 288502 (CONCERTO).

# References

- P. Amon, A. Hutter, U.-E. Martin, B. Heigl, “Selected medical imaging sequences for HEVC development”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, document JCTVC-N0179/M29810, Vienna, Austria, July 2013.

# Patent rights declaration(s)

Siemens does not have any current or pending patent rights relating to the technology described in this contribution.