| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0173r1 |
| --- | --- |

| Title: | Coding of medical mixed content and medical visual content with bit depth beyond 10 bits |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peter Amon, Andreas Hutter Siemens Corporate Technology Imaging and Computer Vision Otto-Hahn-Ring 6 81739 Munich, Germany Uwe-Erik Martin, Nikolaus Wirsz Siemens Healthcare Imaging and Therapy Systems Hartmannstr. 16 91052 Erlangen, Germany | Tel: Email: | +49 89 636 54642 p.amon@siemens.com |
| Source: | Siemens |  |  |

_____________________________

# Abstract

This contribution analyses the coding of medical mixed content and medical visual content with bit depth beyond 10 bits. Requirements are derived from use cases in medical imaging applications. The related capabilities are discussed for relevant future HEVC profiles in the context of the Range Extension work. Recommendations for the design of these profiles are made.

# Introduction

Medical imaging applications impose specific requirements on the underlying image/video compression scheme. These requirements are derived in the following, based on common medical use cases. Furthermore, the respective coding capabilities are proposed for the relevant HEVC profiles.

# Medical imaging use cases

JCTVC-M0190 [1] presents different use cases for medical imaging applications (e.g., interactive browsing in medical image content, export of medical image content from an archive). Two important use cases are detailed in the following.

## Use case “interactive browsing in medical visual content with bit depth beyond 10 bits”

A common use case in the medical domain is the interactive transmission of medical image content from a server (e.g., a PACS – Picture Archiving and Communication System) to a client. Depending of the type of the client (e.g., the respective computational resources), a client receives different representations of the archived data, e.g.:

- rich client: transmission of 12-bit/14-bit/16-bit medical (e.g., CT or MR) content, filtering (e.g., sharpening of edges) and windowing (i.e., adaptive mapping from a 12-bit/14-bit/16-bit representation to a 8-bit representation),

- thin client: transmission of 8-bit content (i.e., windowing done at the server).

For browsing through the medical images, lossy compression of the images can be accepted. However, diagnostically acceptable irreversible compression is preferred. For examining an image after browsing is finished, the image has to be transmitted with reversible (i.e., lossless) compression. The bit depth for lossless coding is 8, 12, or 16 bit, depending on the type of medical content and the type of client. In order to support interactivity, a low-delay coding mode is required.

## Use case “transmission and storage of medical mixed content”

The transmission and storage of medical mixed content (e.g., a user interface containing medical image content, rendered by a server) can be seen as an extensions of use cases 1 and 2. For several medical applications, not only the raw medical image content is transmitted and stored but artificial content is generated or added. This artificial content results, e.g., from colorized ultrasound images, screen content overlays (patient data, scale information, annotations, etc.), or volume-rendered computed tomography image data. The resulting image sequences are usually represented in 8-bit RGB format.

For this use case, the compression of 8-bit RGB mixed content (i.e., simultaneous compression of continuous-tone medical content and screen content) should be supported. Reversible (i.e., lossless) compression is required in some cases (e.g., export to a storage medium).

# Requirements for coding medical image content

Further, JCTVC-M0190 [1] derives requirements for compression of medical image content in the context of HEVC range extensions from these use cases. The following main requirements are extracted from the example use cases:

- Support for efficient reversible (i.e., lossless) and diagnostically acceptable irreversible compression (i.e., near-lossless) coding of 8-bit, 12-bit, and 16-bit monochrome, continuous-tone image content.

- Support for efficient reversible (i.e., lossless) and diagnostically acceptable irreversible compression (i.e., near-lossless) coding of 8-bit RGB image content.

# Related profiles and recommendation

JCTVC-N0178 [2] proposes HEVC profiles related to the requirements derived from medical imaging applications. The main capabilities that should be supported by the respective profiles are lossless compression and signed coding. The profiles that should support these capabilities are High 12 Intra 4:0:0, High 12 4:0:0, High 16 Intra 4:0:0, and High 16 4:0:0. (14-bit profiles are not seen as necessary.) The same capabilities should be supported by the related non-monochrome profiles in an onion-shell fashion.

Furthermore, also the definition of an 8-bit RGB profile is encouraged. Also this profile should support lossless compression.

The proposed future profiles are summarized in the following table.

# Acknowledgement

The research leading to these results has received funding from the European Union’s Seventh Framework Programme ([FP7/2007-2013]) under grant agreement n◦ 288502 (CONCERTO) and from EIT ICT Labs’ Action Line “Networking Solutions for Future Media” under activity n◦ 13090.

# References

- P. Amon, A. Hutter, U.-E. Martin, N. Wirsz, “Requirements for medical imaging applications”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, document JCTVC-M0190/M28643, Incheon, Korea, April 2013.

- P. Amon, A. Hutter, U.-E. Martin, N. Wirsz, “HEVC profiles for medical imaging applications”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, document JCTVC-N0178/M29809, Vienna, Austria, July/August 2013.

- P. Amon, A. Hutter, U.-E. Martin, B. Heigl, “Selected medical imaging sequences for HEVC development”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, document JCTVC-O0173/M30934, Geneva, Switzerland, October/November 2013.

# Patent rights declaration(s)

Siemens may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).