| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0176v3 m30941 |
| --- | --- |

| Title: | HLS: Error robust POC alignment |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Jonatan Samuelsson Ruoyang Yu Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46 761153563 rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In this contribution, a mechanism is proposed that is reported to ensure that the POC values of all pictures of each access unit are the same. The contribution targets specifically the layerwise start-up use case where IRAP pictures and non-IRAP pictures are mixed in the same access unit.

The contribution proposes the following elements:

- Remove the poc_reset_flag syntax element and replace it by a virtual POC lsb syntax element in the slice header extension of base layer IDR pictures when cross_layer_irap_aligned_flag is equal to 0. When the syntax element is present, a virtual POC value is derived for the IDR as if it was a CRA picture using the virtual POC lsb value from the slice header extension. Then every picture in the DPB is decremented by this value, including pictures belonging to higher layers. The POC value of the base layer IDR picture is set to 0.

- The POC value of an enhancement layer IDR picture is set to the POC value of the base layer picture in the same access unit when cross_layer_irap_aligned_flag is equal to 0. This is done regardless of the NAL unit type of the base layer picture.

- The contribution also proposes to add two restrictions when cross_layer_irap_aligned_flag is equal to 0. The first is to require that a base layer picture that is not a RASL, RADL, or sub-layer non-reference is present in all access units that contain an enhancement layer IDR picture. The second is that no BLA picture shall have nuh_layer_id larger than 0.

The proposal claims that the main benefit over the existing solution is increased robustness against picture losses. The proposal also claims to provide additional encoder flexibility regarding picture structures. The proposal claims that the POC values derived for the base layer now stay the same. The proposal claims to have no impact on detection on the boundary between access units.

In version 2 of the contribution, poc_lsb_idr is changed from a 4-byte to a 2-byte value, POC values of enhancement layer IDR pictures are set to the POC of base layer pictures, and the decrement value is derived based on poc_lsb_idr.

In version 3 of the contribution clarifications regarding bitstream constraints, relation with cross_layer_irap_aligned_flag, impact on AU boundary derivation were added in response to feedback given during the BoG presentation. Also, an asserted problem with the current solution was corrected and a new asserted problem was added.

# Background

A design choice to support non-aligned IRAP pictures was made at the previous JCT-VC meeting. This means that access units containing a mix of IRAP pictures and non-IRAP pictures are now supported. An IRAP picture in the base layer provides a point where it is possible to start decoding. An IRAP picture in an enhancement layer provides a point where it is possible to start decoding that layer given that the layer below is decoded.

An example is shown in Fig 1 where the first IRAP picture in the base layer is used by a decoder to start decoding the base layer. Thereafter the decoder starts decoding enhancement layers whenever there is an IRAP picture in the next enhancement layer. The gray pictures shown in Fig 1 will be decoded while the white ones will be discarded.

Fig 1 - Non-aligned IRAP picture example

Another decision made at the previous meeting was to specify that every picture in an access unit shall have identical picture order count values (PicOrderCntVal). Since IDR pictures in the base layer set PicOrderCntVal to 0, some mechanism is needed to achieve POC alignment when the base layer picture in an access unit is an IDR picture and there is a non-IRAP enhancement layer picture in that access unit.

One proposal (JCTVC-N0244) at the last meeting suggested to add a slice header extension syntax element named poc_reset_flag for the enhancement layer pictures. The decoder in that contribution performs the following steps for a current picture P:

- The POC of P is derived as usual, using the POC of a previous picture and the POC lsb as signaled in the slice header of P.

- If the picture has poc_reset_flag equal to 1, the POC of every picture in the DPB that belongs to the same layer is decremented by the derived POC value and the POC of picture P itself is set equal to 0.

The notes on JCTVC-N0244 says that “It was remarked that this has loss resilience implications (when the picture which causes the POC reset is lost), and further study of that aspect was encouraged...”

# Problem statement

One possible solution to achieve POC alignment for base layer IDR pictures is the poc_reset_flag solution which signals POC recalculation information in the enhancement layer picture(s). This seems to be broken and cause problems with lost pictures, missing temporally co-located pictures, and temporal layers.

## The poc_reset_flag=1 in the base layer problem

In the current design we have the following:

- If an enhancement layer picture is IDR, the poc_reset_flag has to be set in all other layers, including the base layer in order to achieve POC alignment and set POC value 0 for all pictures in the AU

- A version 1 decoder ignores the poc_reset_flag

It seems impossible to find slice_pic_order_cnt_lsb value for the next picture that works for both version 1 and extension decoders, see Fig 2 below:

Fig 2 - The poc_reset_flag=1 in the base layer problem

## The lost picture problem

Fig 3 - The lost picture problem

Consider Fig 3 which shows a base layer and an enhancement layer. The numbers inside the pictures are POC values. If picture X is lost, there is no way for the decoder to know how to decrement the POC values of the pictures in the DPB. The consequence is that the POC values of the pictures in the DPB are lost.

## The missing co-located picture problem

Fig 4 - The missing co-located picture problem

Fig 4 shows a picture structure where there is no picture in the enhancement layer of the access unit that contains the base layer IDR picture. Since poc_reset_flag sets the POC to 0, it can not be used for picture Y. This means that if POC recalculation data is sent in the enhancement layer, the picture structure shown in Fig 4 will be disallowed which is an unnecessary encoder constraint.

Note that in order to avoid very large access units, it may be desirable to avoid aligning IDR pictures in the base layer with enhancement layer pictures.

## The sub-layer non-reference  problem

Fig 5 - The sub-layer non-reference problem

Fig 5 shows a picture structure where the enhancement layer contains two temporal layers where the enhancement layer picture X in the access unit containing the base layer IDR picture is a sub-layer non-reference picture. Signaling POC recalculation data in the enhancement layer is fine if the decoder receives picture X. But the POC calculation would be incorrect if picture X is removed. The picture structure shown in Fig 5 will therefore be disallowed which is an unnecessary encoder constraint.

# Proposal

## Proposed POC derivation changes

We propose the following scheme for supporting POC alignment when cross_layer_irap_aligned_flag is equal to 0:

- Signal POC recalculation data in the slice header extension of the IDR picture instead of the enhancement layer picture(s).

- Set the POC values of enhancement layer IDR pictures to the POC value of the base layer picture in the same access unit.

This solves all three problems listed above:

- POC derivation works for both version 1 and extension decoders, see Fig 6.

- Lost enhancement layer pictures do not affect the POC recalculation of the pictures in DPB, therefore the correct POC values of the pictures in the DPB(s) are maintained even when enhancement layer pictures are lost.

- A missing co-located picture is not a problem since the POC recalculation is done at the base layer IDR picture.

- Removal of temporal layers in the enhancement layer does not affect the POC recalculation so these picture structures can be allowed.

Of course, the decoder is in very deep trouble if the IDR picture is lost, but this would be the case regardless of where the POC recalculation information is signalled. This proposal aims at improving the error resilience against lost enhancement layer pictures while at the same time allow for increased encoder flexibility.

Fig 6 - Two POC derivation examples

## Proposed bitstream constraints

In order to ensure that there is a base layer picture in access units that contain an enhancement layer IDR picture, we propose to add a constraint when cross_layer_irap_aligned_flag is equal to 0 that enforces a picture to be present in the base layer of those access units. The base layer picture shall not be a RASL, RADL or sub-layer non-reference picture.

We also see no use for BLA pictures in enhancement layers when cross_layer_irap_aligned_flag is equal to 0. We therefore propose a restriction saying that no BLA picture shall have nuh_layer_id larger than 0 when cross_layer_irap_aligned_flag is equal to 0. An alternative would be to allow BLA pictures in enhancement layers, but then we would propose that they also copy the POC value from the base layer picture and that there is a base layer picture in those access units.

## A note on detection of the boundary between access units

Copying the POC values from base layer pictures to enhancement layer IDR pictures may seem to possibly cause problems for detection of the boundary between access units. No such problem is however introduced since there is always a base layer picture in the access unit in which the copy was made.

## Proposed syntax changes relative to JCTVC-N1008_v3

- F.7.3.6.1	General slice segment header syntax

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( num_extra_slice_header_bits > i ) { |  |
| i++ |  |
| poc_reset_flag | u(1) |
| } |  |
| ... |  |
| slice_segment_header_extension_length | ue(v) |
| for( i = 0; i < slice_segment_header_extension_length; i++) { |  |
| if( i == 0 && nuh_layer_id == 0 && cross_layer_irap_aligned_flag == 0 && (nal_unit_type == IDR_W_RADL ¦¦ nal_unit_type == IDR_N_LP)) { |  |
| poc_lsb_idr | u(16) |
| i += 2 |  |
| } else { |  |
| slice_segment_header_extension_data_byte[ i ] | u(8) |
| } |  |
| } |  |
| } |  |
| byte_alignment( ) |  |
| } |  |

- F.7.4.2.2	NAL unit header semantics

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7 1.

When one picture picA of a layer layerA has nal_unit_type equal to TSA_N or TSA_R, each picture in the same access unit as picA in a direct or indirect reference layer of layerA shall have nal_unit_type equal to TSA_N or TSA_R.

When one picture picA of a layer layerA has nal_unit_type equal to STSA_N or STSA_R, each picture in the same access unit as picA in a direct or indirect reference layer of layerA shall have nal_unit_type equal to STSA_N or STSA_R.

When cross_layer_irap_aligned_flag is equal to 0 and there is an IDR picture with nuh_layer_id greater than 0 present in the current access unit, a base layer picture that is not a RASL, RADL, nor sub-layer non-reference picture shall be present in the current access unit.

nuh_layer_id specifies the identifier of the layer.

When nal_unit_type is equal to AUD_NUT, the value of nuh_layer_id shall be equal to the minimum of the nuh_layer_id values of all VCL NAL units in the access unit.

When nal_unit_type is equal to VPS_NUT, the value of nuh_layer_id shall be equal to 0. Decoder shall ignore NAL units with nal_unit_type equal to VPS_NUT and nuh_layer_id greater than 0.

nuh_layer id shall be equal to 0 when the current picture is a BLA picture and cross_layer_irap_aligned_flag is equal to 0.

- 7.4.7.1	General slice segment header semantics

...

slice_segment_header_extension_length specifies the length of the slice segment header extension data in bytes, not including the bits used for signalling slice_segment_header_extension_length itself. It is a requirement of bitstream conformance that slice_segment_header_extension_length shall be equal to or greater than 2 for IDR_W_RADL and IDR_N_LP NAL units when nuh_layer_id and cross_layer_irap_aligned_flag are both equal to 0. The value of slice_segment_header_extension_length shall be in the range of 0 to 256, inclusive.

poc_lsb_idr specifies a picture order count to be used in the decoding process of picture order count and compute a POC decrement value to be used for the current picture.

poc_reset_flag equal to 1 specifies that the derived picture order count for the current picture is equal to 0. poc_reset_flag equal to 0 specifies that the derived picture order count for the current picture may or may not be equal to 0. It is a requirement of bitstream conformance that when cross_layer_irap_aligned_flag is equal to 1, the value of poc_reset_flag shall be equal to 0. When not present, the value of poc_reset_flag is inferred to be equal to 0.

- 8.1	General decoding process

...

–	Variables and functions relating to picture order count are derived in clause Error: Reference source not found. This needs to be shall be invoked only for the first slice segment of a picture.

...

- F.8.3.1	Decoding process for picture order count

Output of this process is PicOrderCntVal, the picture order count of the current picture.

Picture order counts are used to identify pictures, for deriving motion parameters in merge mode and motion vector prediction, and for decoder conformance checking (see subclause C.5).

Each coded picture is associated with a picture order count variable, denoted as PicOrderCntVal.

- When the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1, or when the picture is a picture with poc_lsb_idr present in the slice header, the variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows:

- Let prevTid0Pic be the If there is a previous picture in decoding order in the bitstream that has TemporalId equal to 0 and nuh_layer_id equal to nuh_layer_id of the current picture and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture, call that picture prevTid0Pic and let prevPicOrderCnt be equal to PicOrderCntVal of prevTid0Pic.

  - The variable prevPicOrderCntLsb is set equal to prevPicOrderCnt & ( MaxPicOrderCntLsb − 1 ).

  - The variable prevPicOrderCntMsb is set equal to prevPicOrderCnt − prevPicOrderCntLsb.

- Otherwise, (there is no previous picture in decoding order in the bitstream that has TemporalId equal to 0 and nuh_layer_id equal to nuh_layer_id of the current picture and that is not a RASL picture, a RADL picture, or a sub-layer non-reference picture), prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

- Let the variable SlicePicOrderCntLsb be equal to poc_lsb_idr if present in the slice header and equal to slice_pic_order_cnt_lsb otherwise.

- The variable PicOrderCntMsb of the current picture is derived as follows:

- If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1 and poc_lsb_idr is not present in the slice header, PicOrderCntMsb is set equal to 0.

- Otherwise, PicOrderCntMsb is derived as follows:

if( (slice_pic_order_cnt_lsb SlicePicOrderCntLsb < prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − slice_pic_order_cnt_lsb SlicePicOrderCntLsb)  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(Error: Reference source not found-1)
else if( (slice_pic_order_cnt_lsb SlicePicOrderCntLsb > prevPicOrderCntLsb )  &&
		( (slice_pic_order_cnt_lsb SlicePicOrderCntLsb − prevPicOrderCntLsb ) > ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

When poc_lsb_idr is present in the slice header, the PicOrderCntVal of each picture that is in the DPB is decremented by PicOrderCntMsb + poc_lsb_idr.

PicOrderCntVal is derived as follows:

If cross_layer_irap_aligned_flag is equal to 0 and the current picture is an IDR picture with nuh_layer_id greater than 0, the PicOrderCntVal is set to the PicOrderCntVal of the picture with nuh_layer_id equal to 0 in the same access unit.

Otherwise, (cross_layer_irap_aligned_flag is equal to 1 or the picture is not an IDR picture with nuh_layer_id greater than 0), the following applies,

If the current picture is an IRAP picture with NoRaslOutputFlag equal to 1,

PicOrderCntVal = slice_pic_order_cnt_lsb

Otherwise (the current picture is not an IRAP picture with NoRaslOutputFlag equal to 1),

PicOrderCntVal = PicOrderCntMsb + slice_pic_order_cnt_lsb	(Error: Reference source not found-2)

NOTE 1 – All IDR pictures with nuh_layer_id equal to 0 will have PicOrderCntVal equal to 0 since slice_pic_order_cnt_lsb is inferred to be 0 for those pictures IDR pictures and prevPicOrderCntLsb and prevPicOrderCntMsb are both set equal to 0.

When poc_reset_flag is equal to 1, the following steps apply in the order listed:

- The PicOrderCntVal of each picture that is in the DPB and belongs to the same layer as the current picture is decremented by PicOrderCntVal.

- PicOrderCntVal is set equal to 0.

- Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).