7.3.2.2 Sequence parameter set RBSP syntax

| transform_skip_context_enabled_flag | u(1) |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| intra_screen_coding_mode_enabled_flag | u(1) |
| residual_dpcm_intra_enabled_flag | u(1) |

7.3.8.5 Coding unit syntax

| if( intra_block_copy_enabled_flag ) |  |
| --- | --- |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] && intra_scm_enable_flag ) |  |
| intra_scm_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] && !intra_scm_flag[ x0 ][ y0 ] ) { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| …… |  |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) { |  |
| mvd_coding( x0, y0, 2) |  |
| } else if( intra_scm_flag[ x0 ][ y0 ] ) { |  |
| for( cIdx=0; cIdx< ChromaArrayType==0?1:3; cIdx++ ) { |  |
| major_color_num[ x0 ][ y0 ][ cIdx ] | ae(v) |
| for( i=0; i< intra_scm_color_num[ x0 ][ y0 ][ cIdx ]; i++ ) { |  |
| major_color_value[ x0 ][ y0 ][ cIdx ][ i ] | ae(v) |
| } |  |
| } |  |
| } else { |  |
| …… |  |

7.3.8.11 Residual coding syntax

| …… |  |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && !intra_scm_flag[ x0 ][ y0 ] && ( log2TrafoSize <= Log2MaxTransformSkipSize ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| …… |  |
| if( sig_coeff_flag[ xC ][ yC ] && ( !intra_scm_flag[ xC ][ yC ] ¦¦ inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] ) && ( !sign_data_hiding_enabled_flag ¦ ¦ !signHidden ¦ ¦ ( n != firstSigScanPos ) ) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| …… |  |

7.4.3.2 Sequence parameter set RBSP semantics

sps_extension1_flag equal to 1 specifies that transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, intra_block_copy_enabled_flag, intra_scm_enabled_flag, residual_dpcm_intra_enabled_flag, residual_dpcm_inter_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, and sps_extension2_flag are present in the SPS RBSP syntax structure. sps_extension1_flag equal to 0 specifies that these syntax elements are not present.

intra_scm_enabled_flag equal to 1 specifies that intra scm may be invoked in the decoding process for intra prediction. intra_scm_enabled_flag equal to 0 specifies that intra scm is not applied. When not present, the value of intra_scm_enabled_flag is inferred to be equal to 0.

7.4.9.5 Coding unit semantics

intra_scm_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in intra scm mode. intra_scm_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in intra scm mode. When not present, the value of intra_scm_flag is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

intra_major_color_num[ x0 ][ y0 ][ cIdx ] specifies the base color number of the cIdx-th color component. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

intra_major_color_value[ x0 ][ y0 ][ cIdx ][ i ] specifies the i-th base color of the cIdx-th color component. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture.

8.4.1 General decoding process for coding units coded in intra prediction mode

Depending on the values of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for luma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SL[ xCb + i ][ yCb + j ] = 
		pcm_sample_luma[ ( nCbS * j ) + i ]  <<  ( BitDepthY − PcmBitDepthY ), with i, j = 0..nCbS − 1	(8-1)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], the variable predModeIntraScm set equal to intra_scm_flag[ xCb ][ yCb ], every element in the variable array majorColors is set to be equal to the corresponding element in the array of intra_major_color_value[ xCb ][ yCb ][ 0 ], and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0 and IntraSplitFlag is equal to 1), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xPb is set equal to xCb + ( nCbS  >>  1 ) * ( blkIdx % 2 ).

- The variable yPb is set equal to yCb + ( nCbS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ) as input.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSize − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeY[ xPb ][ yPb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], the variable predModeIntraScm set equal to intra_scm_flag[ xCb ][ yCb ], every element in the variable array majorColors is set to be equal to the corresponding element in the array of intra_major_color_value[ xCb ][ yCb ][ 0 ], and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

When ChromaArrayType is not equal to 0, the following applies.

The variable log2CbSizeC is set equal to log2CbSize − ( ChromaArrayType  = =  3 ? 0 : 1 ).

Depending on the value of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for chroma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SCb[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = 	pcm_sample_chroma[ ( nCbS / SubWidthC * j ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-2)

SCr[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = pcm_sample_chroma[ ( nCbS / SubWidthC * ( j + nCbS / SubHeightC ) ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-3)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0 or ChromaArrayType is not equal to 3, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], the variable predModeIntraScm set equal to intra_scm_flag[ xCb ][ yCb ], every element in the variable array majorColors is set to be equal to the corresponding element in the array of intra_scm_colors[ xCb ][ yCb ][ 1 ], and the variable cIdx set equal to 1 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], the variable predModeIntraScm set equal to intra_scm_flag[ xCb ][ yCb ], every element in the variable array majorColors is set to be equal to the corresponding element in the array of intra_scm_colors[ xCb ][ yCb ][ 2 ], and the variable cIdx set equal to 2 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xC ][ yC ] is equal to 0 and IntraSplitFlag is equal to 1 and ChromaArrayType is equal to 3), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xBS is set equal to xC + ( nS  >>  1 ) * ( blkIdx % 2 ).

- The variable yBS is set equal to yC + ( nS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xBS, yBS ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xBS, yBS), the variable log2TrafoSize set equal to log2CbSizeC − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], the variable predModeIntraScm set equal to intra_scm_flag[ xCb ][ yCb ], every element in the variable array majorColors is set to be equal to the corresponding element in the array of intra_scm_colors[ xCb ][ yCb ][ 1 ], and the variable cIdx set equal to 1 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xBS, yBS), the variable log2TrafoSize set equal to log2CbSizeC − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], the variable predModeIntraScm set equal to intra_scm_flag[ xCb ][ yCb ], every element in the variable array majorColors is set to be equal to the corresponding element in the array of intra_scm_colors[ xCb ][ yCb ][ 2 ], and the variable cIdx set equal to 2 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

8.4.4.1 General decoding process for intra blocks

Inputs to this process are:

–	a sample location ( xTb0, yTb0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	a variable log2TrafoSize specifying the size of the current transform block,

–	a variable trafoDepth specifying the hierarchy depth of the current block relative to the coding unit,

–	a variable predModeIntra specifying the intra prediction mode,

–	a variable predModeIntraBc specifying the intra block copying mode,

–	a variable bvIntra specifying the intra block copying vector,

–	a variable predModeIntraScm specifying the intra scm mode,

–	a variable array majorColors specifying the major colors of intra scm mode,

–	a variable cIdx specifying the colour component of the current block.

Output of this process is a modified reconstructed picture before deblocking filtering.

The luma sample location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture is derived as follows:

( xTbY, yTbY ) = ( cIdx  = =  0 ) ? ( xTb0, yTb0 ) : ( xTb0 * SubWidthC, yTb0 * SubHeightC )	(8-4)

The variable splitFlag is derived as follows:

–	If cIdx is equal to 0, splitFlag is set equal to split_transform_flag[ xTbY ][ yTbY ][ trafoDepth ].

–	Otherwise, if all of the following conditions are true, splitFlag is set equal to 1.

- cIdx is greater than 0

- split_transform_flag[ xTbY ][ yTbY ][ trafoDepth ] is equal to 1

- log2TrafoSize is greater than 2

–	Otherwise, splitFlag is set equal to 0.

Depending on the value of splitFlag, the following applies:

–	If splitFlag is equal to 1, the following ordered steps apply:

- The variables xTb1 and yTb1 are derived as follows:

  - The variable xTb1 is set equal to xTb0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

  - The variable yTb1 is set equal to yTb0 + ( 1  <<  ( log2TrafoSize − 1 ) ).

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb0, yTb0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, the intra block copying mode flag predModeIntraBc, the block copying vector bvIntra, the intra scm mode flag predModeIntraScm, the variable array majorColors, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb1, yTb0 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, the intra block copying mode flag predModeIntraBc, the block copying vector bvIntra, the intra scm mode flag predModeIntraScm, the variable array majorColors, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb0, yTb1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, the intra block copying mode flag predModeIntraBc, the block copying vector bvIntra, the intra scm mode flag predModeIntraScm, the variable array majorColors, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in this subclause is invoked with the location ( xTb1, yTb1 ), the variable log2TrafoSize set equal to log2TrafoSize − 1, the variable trafoDepth set equal to trafoDepth + 1, the intra prediction mode predModeIntra, the intra block copying mode flag predModeIntraBc, the block copying vector bvIntra, the intra scm mode flag predModeIntraScm, the variable array majorColors, and the variable cIdx as inputs, and the output is a modified reconstructed picture before deblocking filtering.

……

- Depending upon the value of predModeIntraBc and predModeIntraScm, the following applies

–	When predModeIntraBc is equal to 0 and predModeIntraScm is equal to 0, the general intra sample prediction process as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the intra prediction mode predModeIntra, the transform block size nTbS, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

–	Otherwise (predModeIntraBc is equal to 1), the intra block copying process as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the transform block size nTbS, the variable bvIntra, and the variable cIdx as inputs, and the output is an (nTbS)x(nTbS) array predSamples.

–	Otherwise (predModeIntraScm is equal to 1), every element in the (nTbS)x(nTbS) array predSamples is set to be equal to 0.

- The scaling and transformation process as specified in subclause Error: Reference source not found is invoked with the luma location ( xTbY, yTbY + yTbOffset * SubHeightC ), the variable trafoDepth, the variable cIdx, and the transform size trafoSize set equal to nTbS as inputs, and the output is an (nTbS)x(nTbS) array resSamples.

- When residualDpcm is equal to 1 and either predModeIntra is equal to 10, or predModeIntra is equal to 26, the directional residual modification process for blocks using a transform bypass as specified in subclause Error: Reference source not found is invoked with the variable mDir set equal to predModeIntra / 26, the variable nTbS, and the (nTbS)x(nTbS) array r set equal to the array resSamples as inputs, and the output is a modified (nTbS)x(nTbS) array resSamples.

When predModeIntraScm is equal to 1 and inter_rdpcm_flag[ xTb0 ][ yTb0 + yTbOffset ][ cIdx ] is equal to 1, the directional residual modification process for blocks using a transform bypass as specified in subclause Error: Reference source not found is invoked with the variable mDir set equal to inter_rdpcm_dir_flag[ xTb0 ][ yTb0 + yTbOffset ][ cIdx ], the variable nTbS, and the (nTbS)x(nTbS) array r set equal to the array resSamples as inputs, and the output is a modified (nTbS)x(nTbS) array resSamples.

When predModeIntraScm is equal to 1, the residual sample arrays values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS – 1 in the (nTbS)x(nTbS) array resSamples array are modified as follows:

r[ x ][ y ] = majorColors[ r[ x ][ y ] ].

- The picture reconstruction process prior to in-loop filtering for a colour component as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTb0, yTb0 + yTbOffset ), the variables nCurrSw and nCurrSh both set equal to nTbS, the variable cIdx, the (nTbS)x(nTbS) array predSamples, and the (nTbS)x(nTbS) array resSamples as inputs.

8.6.2 Scaling and transformation process

…

The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If cu_transquant_bypass_flag is equal to 1 or intra_scm_flag[ xTbY ][ yTbY ] is equal to 1, the (nTbS)x(nTbS) array of residual samples r is derived as follows:

–	If transform_skip_rotation_enabled_flag is equal to 1 and nTbS is equal to 4, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( TransCoeffLevel[ nTbS − x −1 ][ nTbS − y − 1 ]  <<  tsShift )	(8-5)

–	Otherwise, the (nTbS)x(nTbS) array r is set equal to the (nTbS)x(nTbS) array of transform coefficients TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

- Otherwise, the following ordered steps apply:

…

Table 9-4 Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) | cu_transquant_bypass_flag |  | 0 | 1 | 2 |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | intra_bc_flag[ ][ ] | Table 9- | 0..2 | 3..5 | 6..8 |
|  | intra_scm_flag[ ][ ] | Table 9-XX | 0..2 | 3..5 | 6..8 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 | 1..4 | 5..8 |

Table 9-XX – Values of initValue for ctxIdx of intra_scm_flag

| Initialization variable | ctxIdx of intra_scm_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 185 | 185 | 201 | 197 | 197 | 185 | 197 | 197 | 185 |

Table 9-34 Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | intra_mc_flag | FL | cMax = 1 |
|  | intra_scm_flag | FL | cMax = 1 |
|  | major_color_num | FL | cMax = 15 |
|  | major_color_value | FL | cMax = cIdx == 0 ? (1<<BitDepthY) – 1 : (1<<BitDepthC) – 1 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |

Table 9-39 Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_skip_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| pred_mode_flag | 0 | na | na | na | na | na |
| intra_scm_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| major_color_num | bypass | bypass | bypass | bypass | na | na |
| major_color_value | bypass | bypass | bypass | bypass | bypass | bypass |
| part_mode log2CbSize = = MinCbLog2SizeY | 0 | 1 | 2 | bypass | na | na |

9.3.4.2.2 Derivation process of ctxInc using left and above syntax elements

Table 9-1 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxInc |
| --- | --- | --- | --- |
| split_cu_flag[ x0 ][ y0 ] | CtDepth[ xNbL ][ yNbL ] > cqtDepth | CtDepth[ xNbA ][ yNbA ] > cqtDepth | ( condL && availableL ) + ( condA && availableA ) |
| cu_skip_flag[ x0 ][ y0 ] | cu_skip_flag[ xNbL ][ yNbL ] | cu_skip_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| intra_bc_flag[ x0 ][ y0 ] | intra_bc_flag[ xNbL ][ yNbL ] | intra_bc_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| intra_scm_flag[ x0 ][ y0 ] | intra_scm_flag[ xNbL ][ yNbL ] | intra_scm_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |