| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| transform_skip_context_enabled_flag | u(1) |
| intra_block_copy_enabled_flag | u(1) |
| intra_screen_coding_enabled_flag | u(1) |
| … |  |

7.3.2.2 Sequence parameter set RBSP syntax

7.3.8.5 Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| … |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if( !intra_bc_flag[ x0 ][ y0 ] ) { |  |
| if( screen_coding_enabled_flag) |  |
| intra_scm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_scm_flag[ x0 ][ y0 ] ) { |  |
| color_index_coding(x0, y0, nCbS, nCbS) |  |
| } else { |  |
| if( slice_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && !intra_bc_flag && !intra_scm_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| … |  |
| if( !pcm_flag[ x0 ][ y0 ] && !intra_scm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ¦ ¦ CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) |  |
| rqt_root_cbf | ae(v) |
| … |  |
| } |  |
| } |  |
| } |  |
| } |  |

Color index coding

| color_index_coding(x0, y0, CbWidth, CbHeight) { |  |
| --- | --- |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 0 )?1:3; cIdx++) { |  |
| major_color_num[ x0 ][ y0 ][ cIdx ] | ae(v) |
| for( n = 0; n < major_color_num[ cIdx ]; n++ ) { |  |
| major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| if( !major_color_pred[ cIdx ] ) |  |
| major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] | ae(v) |
| } |  |
| } |  |
| for(cIdx = 0; cIdx < ( ChromaArrayType == 3 )?1:3; n++) { |  |
| for( i = 0; i < CbHeight; i ++) { |  |
| line_mode[ x0 ][ y0 ][ cIdx ][i] | ae(v) |
| if(line_mode[ x0 ][ y0 ][ cIdx ][ i ] == HOR_MODE) { |  |
| line_index_flag[ x0 ][ y0 ][ cIdx ][ i ] | ae(v) |
| if(!line_index_flag[ x0 ][ y0 ][ cIdx ][i]) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 )?3:1; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i ][ 0 ] | ae(v) |
| } |  |
| } |  |
| } else if(line_mode[ x0 ][ y0 ][ cIdx ][i] == VER_MODE) { |  |
| for(j = 0; j < CbWidth; j++) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 )?3:1; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i ][ j ] = color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][ i-1 ][ j ] |  |
| } |  |
| } |  |
| } else { |  |
| for(j = 0; j < CbWidth; j++) { |  |
| index_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] | ae(v) |
| } |  |
| for(j = 0; j < CbWidth; j++) { |  |
| if(Indx_pred_symbol[ x0 ][ y0 ][ cIdx ][ i ][ j ] == NO_PREDICTION) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 )?3:1; cSubIdx++ ) { |  |
| color_index[ x0 ][ y0 ][ cIdx ][ I ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| for(j = 0; j < CbWidth; j++) { |  |
| for(cSubIdx = 0; cSubIdx < ( ChromaArrayType == 3 )?3:1; cSubIdx++ ) { |  |
| if(color_index[ x0 ][ y0 ][ cIdx + cSubIdx ][i][0] == major_color_num[ x0 ][ y0 ][ cIdx + cSubIdx ]) { |  |
| for(j = 0; j < CbWidth; j++) |  |
| quant_escape_value[ cIdx + cSubIdx ][ i ][ j ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.4.3.2 Sequence parameter set RBSP semantics

sps_extension1_flag equal to 1 specifies that transform_skip_rotation_enabled_flag, transform_skip_context_enabled_flag, intra_block_copy_enabled_flag, intra_screen_coding_enabled_flag, residual_dpcm_intra_enabled_flag, residual_dpcm_inter_enabled_flag, extended_precision_processing_flag, intra_smoothing_disabled_flag, and sps_extension2_flag are present in the SPS RBSP syntax structure. sps_extension1_flag equal to 0 specifies that these syntax elements are not present.

intra_screen_coding_enabled_flag equal to 1 specifies that intra screen coding may be used for intra blocks. intra_screen_coding_enabled_flag equal to 0 specifies that intra screen coding is not applied. When not present, the value of intra_screen_coding_enabled_flag is inferred to be equal to 0.

7.4.9.5 Coding unit semantics

intra_scm_flag[ x0 ][ y0 ] equal to 1 specifies that the current coding unit is coded in screen coding mode. intra_scm_flag[ x0 ][ y0 ] equal to 0 specifies that the current coding unit is not coded in screen coding mode. When intra_scm_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_num[ x0 ][ y0 ][ cIdx ] specifies the number of major colors for color component cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] equal to 1 specifies n-th major color value for color component cIdx could be predicted from the above or left CU. major_color_pred_flag[ x0 ][ y0 ][ cIdx ][ n ] equal to 0 specifies current major color couldn’t be predicted. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] specifies the prediction direction of the n-th major color for color component cIdx. major_color_pred_dir[ x0 ][ y0 ][ cIdx ][ n ] equal to 1 specifies current major color is the same as the corresponding major color in the above CU. When it equals to 0, current major color is the same as the corresponding major color in the left CU. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

major_color_value[ x0 ][ y0 ][ cIdx ][ n ] specifies the n-th major color value for color component cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

line_mode[ x0 ][ y0 ][ i ] equal to HOR_MODE specifies that the line should be decoded in horizontal line mode. line_mode[ x0 ][ y0 ][i] equal to VER_MODE specifies that the line should be decoded in vertical line mode. line_mode[ x0 ][ y0 ][ i ] equal to NOR_MODE specifies that the line should be decoded in normal line mode. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

line _index_flag[ x0 ][ y0 ][ i ] equal to 1 specifies index value can be predicted from the left-most pixel in the previous line. line_index_flag[ x0 ][ y0 ][ i ] equal to 0 specifies index value can’t be predicted from the left-most pixel in the previous line. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture

index_pred_symbol[ x0 ][ y0 ][ i ][ j ] specifies the prediction symbol of the index for the current pixel in position ( i, j ). index_pred_symbol[ x0 ][ y0 ] [ i ][ j ] equal to INDEX_PREDICTION_ABOVE specifies that index value of current pixel is the same as the above one. index_pred_symbol[ x0 ][ y0 ] [ i ][ j ] equal to 0 specifies that index value of current pixel is 0. index_pred_symbol[ x0 ][ y0 ] [ i ][ j ] equal to NO_PREDICTION specifies that index value of current pixel cannot be derived from the above or the left pixel.

quant_escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ] specifies the quantized escape pixel value for color component cIdx in at the location ( i, j ). The variable bitEscapeQuant is define as Table

- escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ] is set equal to

quant_escape_value[ x0 ][ y0 ][ cIdx ][ i ][ j ]<<(bitDepth – bitEscapeQuant)

color_index[ x0 ][ y0 ][ cIdx ][ i ][ j ] specifies the index value of the pixel at the location( i, j ) for color component cIdx.

8.4.1 General decoding process for coding units coded in intra prediction mode

Depending on the values of pcm_flag[ xCb ][ yCb ], intra_scm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for luma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SL[ xCb + i ][ yCb + j ] = 
		pcm_sample_luma[ ( nCbS * j ) + i ]  <<  ( BitDepthY − PcmBitDepthY ), with i, j = 0..nCbS − 1	(8-1)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, intra_scm_flag[ xCb ][ yCb ] is equal to 1), the decoding process is specified as follows:

For each i, j = 0..nCbS – 1

–  If color_index[0][i][j] is not equal to major_color_num[0],

SL [ xCb + i ][ yCb + j ] = major_color_value[0][ color_index[0][i][j] ]

–  Otherwise

SL [ xCb + i ][ yCb + j ] = escape_value [0][i][j]

Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, intra_scm_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xCb, yCb ), the variable log2TrafoSize set equal to log2CbSize, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, intra_scm_flag[ xCb ][ yCb ] is equal to 0 and IntraSplitFlag is equal to 1), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xPb is set equal to xCb + ( nCbS  >>  1 ) * ( blkIdx % 2 ).

- The variable yPb is set equal to yCb + ( nCbS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the intra prediction mode as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ) as input.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xPb, yPb ), the variable log2TrafoSize set equal to log2CbSize − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeY[ xPb ][ yPb ], the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

When ChromaArrayType is not equal to 0, the following applies.

The variable log2CbSizeC is set equal to log2CbSize − ( ChromaArrayType  = =  3 ? 0 : 1 ).

Depending on the value of pcm_flag[ xCb ][ yCb ] and IntraSplitFlag, the decoding process for chroma samples is specified as follows:

–	If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SCb[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = 	pcm_sample_chroma[ ( nCbS / SubWidthC * j ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-2)

SCr[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] = pcm_sample_chroma[ ( nCbS / SubWidthC * ( j + nCbS / SubHeightC ) ) + i ]  <<
				( BitDepthC − PcmBitDepthC ), with i = 0..nCbS / SubWidthC − 1, and j = 0..nS / SubHeightC − 1	(8-3)

–	Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, intra_scm_flag[ xCb ][ yCb ] is equal to 1), the decoding process is specified as follows

For each i = 0..nCbS/ SubWidthC  – 1, j = 0..nCbS / SubHeightC – 1,

–  If color_index[1][i][j] is not equal to  major_color_num[1],

SCb [ xCb + i ][ yCb + j ] = major_color_value[1][ color_index[1][i][j]

–  Otherwise

SCb [ xCb + i ][ yCb + j ] = escape_value [1][i][j]

For each i = 0..nCbS/ SubWidthC  – 1, j = 0..nCbS / SubHeightC – 1,

–  If color_index[2][i][j] is not equal to major_color_num[2],

SCr [ xCb + i ][ yCb + j ] = major_color_value[2][ color_index[2][i][j]

–  Otherwise

SCr [ xCb + i ][ yCb + j ] = escape_value [2][i][j]

Otherwise (pcm_flag[ xCb ][ yCb ] is equal to 0, intra_scm_flag[ xCb ][ yCb ] is equal to 0), if IntraSplitFlag is equal to 0 or ChromaArrayType is not equal to 3, the following ordered steps apply:

- When intra_bc_flag[ xCb ][ yCb ] is equal to 0, the derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xCb, yCb ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 1 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the variable log2TrafoSize set equal to log2CbSizeC, the variable trafoDepth set equal to 0, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 2 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

–	Otherwise (pcm_flag[ xC ][ yC ] is equal to 0 intra_scm_flag[ xCb ][ yCb ] is equal to 0 and IntraSplitFlag is equal to 1 and ChromaArrayType is equal to 3), for the variable blkIdx proceeding over the values 0..3, the following ordered steps apply:

- The variable xBS is set equal to xC + ( nS  >>  1 ) * ( blkIdx % 2 ).

- The variable yBS is set equal to yC + ( nS  >>  1 ) * ( blkIdx / 2 ).

- The derivation process for the chroma intra prediction mode as specified in Error: Reference source not found is invoked with the luma location ( xBS, yBS ) as input, and the output is the variable IntraPredModeC.

- The general decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the chroma location ( xBS, yBS), the variable log2TrafoSize set equal to log2CbSizeC − 1, the variable trafoDepth set equal to 1, the variable predModeIntra set equal to IntraPredModeC, the variable predModeIntraBc set equal to intra_bc_flag[ xCb ][ yCb ], the variable bvIntra set equal to BvIntra[ xCb ][ yCb ], and the variable cIdx set equal to 1 as inputs, and the output is a modified reconstructed picture before deblocking filtering.

Table 9-4 Association of ctxIdx and syntax elements for each initializationType in the initialization process

| Syntax structure | Syntax element | ctxTable | initType |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 1 | 2 |
| coding_unit( ) | cu_transquant_bypass_flag |  | 0 | 1 | 2 |
|  | cu_skip_flag | Error: Reference source not found |  | 0..2 | 3..5 |
|  | intra_bc_flag[ ][ ] | Table 9- | 0..2 | 3..5 | 6..8 |
|  | intra_scm_flag[ ][ ] | Table 9-XX | 0..2 | 3..5 | 6..8 |
|  | pred_mode_flag | Error: Reference source not found |  | 0 | 1 |
|  | part_mode | Error: Reference source not found | 0 | 1..4 | 5..8 |

Table 9-XX – Values of initValue for ctxIdx of intra_scm_flag

| Initialization variable | ctxIdx of intra_scm_flag |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| initValue | 185 | 185 | 201 | 197 | 197 | 185 | 197 | 197 | 185 |

Table 9-34 Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | intra_mc_flag | FL | cMax = 1 |
|  | intra_scm_flag | FL | cMax = 1 |
|  | major_color_num | FL | cMax = 15 |
|  | major_color_value | FL | cMax = cIdx == 0 ? (1<<BitDepthY) – 1 : (1<<BitDepthC) – 1 |
|  | major_color_pred_flag | FL | cMax = 1 |
|  | major_color_pred_dir | FL | cMax = 1 |
|  | line_mode | FL | cMax = 2 |
|  | line_index_flag | FL | cMax = 1 |
|  | index_pred_symbol | FL | cMax = 2 |
|  | quant_escape_value | FL | cMax = cIdx == 0 ? (1<<BitDepthY) – 1 : (1<<BitDepthC) – 1 |
|  | color_index | FL | cMax = 16 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 31 |

Table 9-39 Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cu_skip_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| zpred_mode_flag | 0 | na | na | na | na | na |
| intra_scm_flag | 0,1,2 (subclause Error: Reference source not found) | na | na | na | na | na |
| major_color_num | bypass | bypass | bypass | bypass | na | na |
| major_color_value | bypass | bypass | bypass | bypass | bypass | bypass |
| major_color_pred_flag | 0,1 (subclause Error: Reference source not found) | na | na | na | na | na |
| major_color_pred_dir | 0 | na | na | na | na | na |
| line_mode | 0,1 | 0,1,2 | na | na | na | na |
| line_index_flag | 0 | na | na | na | na | na |
| index_pred_symbol | 0,1 | 0,1,2 | na | na | na | na |
| quant_escape_value | bypass | bypass | bypass | bypass | bypass | bypass |
| part_mode log2CbSize = = MinCbLog2SizeY | 0 | 1 | 2 | bypass | na | na |

9.3.4.2.2 Derivation process of ctxInc using left and above syntax elements

Table 9-1 – Specification of ctxInc using left and above syntax elements

| Syntax element | condL | condA | ctxInc |
| --- | --- | --- | --- |
| split_cu_flag[ x0 ][ y0 ] | CtDepth[ xNbL ][ yNbL ] > cqtDepth | CtDepth[ xNbA ][ yNbA ] > cqtDepth | ( condL && availableL ) + ( condA && availableA ) |
| cu_skip_flag[ x0 ][ y0 ] | cu_skip_flag[ xNbL ][ yNbL ] | cu_skip_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| intra_bc_flag[ x0 ][ y0 ] | intra_bc_flag[ xNbL ][ yNbL ] | intra_bc_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| intra_scm_flag[ x0 ][ y0 ] | intra_scm_flag[ xNbL ][ yNbL ] | intra_scm_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |