| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0183_r1 |
| --- | --- |

| Title: | On Intra BC mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Gary. J. Sullivan | Tel: Email: | [phone contact number] [email address(es)] |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution proposes following modifications for Intra BC mode. 1) to use DST for luma 4x4 blocks of Intra BC CU; 2) to use inter-like deblocking for Intra BC CU; 3) constraints of block vectors for Intra BC CU; 4) modifications on block vector coding; 5) constraints when using Intra BC when constraint intra is enabled.

# Introduction

Intra BC mode was adopted at Vienna meeting, and this contribution proposes following modifications for Intra BC mode. 1) use DST for luma 4x4 blocks of Intra BC CU; 2) use inter-like deblocking for Intra BC CU; 3) constraints of block vectors for Intra BC CU; 4) modifications on block vector coding; 5) constraints when using Intra BC with constraint intra enabled.

# Problems and Solutions

At inverse transform stage, Intra BC is not treated as intra coded CU, and DCT is used for luma 4x4 blocks. But at the deblocking stage, Intra BC is treated as intra coded CU. Thus this document proposes to apply either 2.1 or 2.2 for Intra BC.

## Use DST for luma 4x4 blocks of Intra BC CU

## Use inter-like deblocking for Intra BC CU

## Constraint of block vectors for Intra BC CU

The current constraint (The value of BvIntra[ x0 ][ y0 ] shall be in the range of 0 to 128, inclusive.) is not correct. In the following case, BvIntra[ x0 ][ y0 ][ 0 ] is positive but BvIntra[ x0 ][ y0 ][ 1 ] is negative. And there are no other constraints on the range of block vectors.

This document proposes to remove the above constraint and add the following constraints in the text to help the decoder design.

- The pixels not in the current CTU or the left CTU should not be used for Intra BC prediction;

- The pixels in a different tile or slice should not be used for Intra BC prediction;

- The pixels not reconstructed yet should be used for Intra BC prediction;

The text modifications are provided in Section 4.3.

## BC vectors coding

If we divide the whole region into 9 parts according to the position of the current CU, the available BC vectors region are the top, left, top left, top right, and bottom left part.

When the vertical block vector is greater than the –CUSize, only the left part and the bottom left part can be used for prediction. Thus, this document proposes to encode horizontal block vector in the following way.

If vertical block vector is greater than the –CUSize, encode horizontal block vector + CUSize;

Otherwise, encode horizontal block vector directly.

If introducing CUSize in block vector coding is not welcome, this document also proposes to use a fix value 8.

There is no change on the syntax table and only the semantics part is modified. The experimental results are provided in Section 3.4 and the text modification is provided in Section 4.4.

## Constraints when using Intra BC mode with constraint intra enabled

When constraint intra prediction is used, reconstructed pixels in inter coded CUs cannot be used for intra prediction. Intra BC should also obey this rule, but there is no such constraint in the spec. This document proposes two ways to solve this problem. The first method is to add bitstream conformance that when constraint intra is used, reconstructed pixels in inter coded CUs cannot be used for Intra BC prediction.

The second method is to just disable Intra BC in non-Intra slices when constraint intra is enabled.

# Experimental results

The RCE common test condition specified in JCTVC-N1123 [1] is followed.

## Use DST for luma 4x4 blocks of Intra BC CU

The detailed experimental results can be found in JCTVC-O0183 - RCE IntraBCDST.xlsm. The summarized results can be found in Table 1.

Table 1 Use DST for luma 4x4 blocks of Intra BC CU

An independent implementation of the proposed technique (using DST for luma 4x4 block of intra BC CU) is also performed in JCTVC-O0053 [2]. Identical results have been obtained in JCTVC-O0053.

## Use inter-like deblocking for Intra BC CU

The detailed experimental results can be found in JCTVC-O0183 - RCE IntraBCDBL.xlsm. The summarized results can be found in Table 2.

Table 2 Use inter-like deblocking for Intra BC CU

## Constraint of block vectors for Intra BC CU

Not provided. This modification does not have any impact on the coding efficiency.

## BC vectors coding

When using CU size adaptive offset, the detailed experimental results can be found in JCTVC-O0183 - RCE BCBVW.xlsm. The summarized results can be found in Table 3.

Table 3 CU size adaptive offset BV coding

When using fixed offset of 8, the detailed experimental results can be found in JCTVC-O0183 - RCE BCBV8.xlsm. The summarized results can be found in Table 4.

Table 4 Fixed offset of 8 BV coding

For lossless coding, when using CU size adaptive offset, the detailed experimental results can be found in JCTVC-O0183 - RCE lossless BCBVW.xlsm. The summarized results can be found in Table 5.

Table 5 CU size adaptive offset BV coding (lossless coding)

For lossless coding, when using fixed offset of 8, the detailed experimental results can be found in JCTVC-O0183 - RCE lossless BCBV8.xlsm. The summarized results can be found in Table 6.

Table 6 Fixed offset of 8 BV coding (lossless coding)

## Constraints when using Intra BC mode with constraint intra enabled

Not provided. This modification does not have any impact on the coding efficiency under CTC or RCE test conditions.

# Text Modifications

The text modifications are based on JCTVC-N1005_v3.doc.

## Use DST for luma 4x4 blocks of Intra BC CU

No modifications to the current text. DST is used for luma 4x4 blocks of Intra BC CU as specified in 8.6.4.1.

## Use inter-like deblocking for Intra BC CU

8.7.2.4

    - If the sample p0 or q0 is in the luma coding block of a coding unit coded with intra prediction mode but not Intra BC prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

## Constraint of block vectors for Intra BC CU

7.4.9.9 Motion vector difference semantics

The variable BvIntra[ x0 ][ y0 ][ compIdx ] specifies the vector used for the intra block copying prediction mode.  The value of BvIntra[ x0 ][ y0 ] shall be in the range of 0 to 128, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 1), MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 2), BvIntra[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

It is a requirement of bitstream conformance that the following conditions apply:

- The value of BvIntra[ x0 ][ y0 ][ 0 ] shall be greater than or equal to – (x0 % CtbSizeY + CtbSizeY).

- The value of BvIntra[ x0 ][ y0 ][ 1 ] shall be greater than or equal to – (y0 % CtbSizeY).

- When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring luma location ( x0 + BvIntra[ x0 ][ y0 ][ 0 ], y0 + BvIntra[ x0 ][ y0 ][ 1 ]) as inputs, the output shall be equal to TRUE.

- When the derivation process for z-scan order block availability as specified in subclause Error: Reference source not found is invoked with ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring luma location ( x0 + BvIntra[ x0 ][ y0 ][ 0 ] + (1 << log2CbSize – 1), y0 + BvIntra[ x0 ][ y0 ][ 1 ] + (1 << log2CbSize) – 1) as inputs, the output shall be equal to TRUE.

- One or two of the following two conditions shall be satisfied

    - BvIntra[ x0 ][ y0 ][ 0 ] + (1 << log2CbSize) <= 0

    - BvIntra[ x0 ][ y0 ][ 1 ] + (1 << log2CbSize) <= 0

If Intra BC prediction is performed at TU level (the current spec specifis TU based intra BC prediction but the current software implements CU based intra BC prediction), the last condition should be replaced by

8.4.4.2.7 Specification of intra block copyping prediction mode

Inputs to this process are:

–	a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

–	a variable nTbS specifying the transform block size,

–	a variable bvIntra specifying the block copying vector,

–	a variable cIdx specifying the colour component of the current block.

Output of this process is the predicted samples predSamples[ x ][ y ], with x, y = 0..nTbS − 1.

The variable bv representing the block vector for prediction in full-sample units is derived as follows:

bv[ 0 ] = bvIntra[ 0 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubWidthC ) − 1 )	(8-1)

bv[ 1 ] = bvIntra[ 1 ]  >>  ( ( ( cIdx  = =  0 ) ? 1 : SubHeightC ) − 1 )	(8-2)

It is a requirement of bitstream conformance that one or two of the following two conditions shall be satisfied

    - bv[ 0 ] + nTbS <= 0

    - bv[ 1 ] + nTbS <= 0

## BC vectors coding

7.4.9.9 Motion vector difference semantics

The variable BvIntra[ x0 ][ y0 ][ compIdx ] specifies the vector used for the intra block copying prediction mode.  The value of BvIntra[ x0 ][ y0 ] shall be in the range of 0 to 128, inclusive. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered prediction block relative to the top-left luma sample of the picture. The horizontal block vector component is assigned compIdx = 0 and the vertical block vector component is assigned compIdx = 1.

- If refList is equal to 0, MvdL0[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 1), MvdL1[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

- Otherwise (refList is equal to 2), BvIntra[ x0 ][ y0 ][ compIdx ] is set equal to lMvd[ compIdx ] for compIdx = 0..1.

BvIntra[ x0 ][ y0 ][ 1 ] is set equal to lMvd[ 1 ], and

If BvIntra[ x0 ][ y0 ][ 1 ] is greater than –8, BvIntra[ x0 ][ y0 ][ 0 ] is set equal to lMvd[ 0 ] – 8;

Otherwise, BvIntra[ x0 ][ y0 ][ 0 ] is set equal to lMvd[ 0 ].

8 is replaced by (1 << log2CbSize) for CU size adaptive block vector coding.

The offset can also be explicitly signalled at SPS, PPS, or slice header level.

## Constraints when using Intra BC mode with constraint intra enabled

Method 1: Add a bitstream conformance requirement.

7.4.9.9 Motion vector difference semantics

……

It is a requirement of bitstream conformance that when constrained_intra_pred_flag is equal to 1, any pixel p[x][y], with x=x0+ BvIntra[ x0 ][ y0 ][ 0 ].. x0+ BvIntra[ x0 ][ y0 ][ 0 ]+ (1 << log2CbSize) – 1, y=y0+ BvIntra[ x0 ][ y0 ][ 1 ].. y0+ BvIntra[ x0 ][ y0 ][ 1 ]+ (1 << log2CbSize) – 1 shall be in a coding unit coded with intra prediction mode.

Method 2: disable Intra BC mode in non-Intra slice if constraint intra is enabled.

7.3.8.5 Coding unit syntax

| if( intra_block_copy_enabled_flag && (!constrained_intra_pred_flag ¦¦ slice_type == I) ) |  |
| --- | --- |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |

# References

- JCTVC-N1123, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques,” Vienna, Austria, 25 Jul. – 2 Aug. 2013.

- JCTVC-O0053, “RExt: On transform selection for Intra-BlockCopy blocks,” Geneva, CH, 23 Oct. – 1 Nov. 2013.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).