| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0184 |
| --- | --- |

| Title: | Comparison of Compression Performance of HEVC Range Extensions Draft 4 with AVC High 4:4:4 Predictive profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Gary. J. Sullivan | Tel: Email: | [phone contact number] [email address(es)] |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution is a study of the relative objective (i.e. PSNR-based) compression performance of HEVC RExt Profile and AVC High 4:4:4 Predictive Profile. It builds upon the prior work reported in JCTVC-G399 JCTVC-H0360, JCTVC-I0409, JCTVC-J0236, JCTVC-K0279, JCTVC-L0322, JCTVC-M0329, and updating the results by using the latest available reference software (JM-18.5 and HM-12.0+RExt-4.1) and the RExt test sequences.

# Performance Comparison

We use HM-12.0+RExt-4.1and JM-18.5 in the tests. The JM software was configured with the same coding structure and referencing structure as HM-12.0+RExt-4.1 using the “HM-like” configurations. High 4:4:4 Predictive profile is used in JM. Both the RExt CTC [8] and RCE [9] test conditions (e.g. internal bit depth, intra period, QP setting, etc.) are followed in the comparison.

# Experimental result

Table 1 JM VS. HM-12.0+RExt-4.1 CTC

Table 2 JM VS. HM-12.0+RExt-4.1 RCE

It seems that there are some strange BD-Rate numbers in Table 2. It is caused by the PSNR of JM is relatively smaller than that of HM, especially for the screen content. Thus, the valid range to calculate BD-Rate is small or even empty. And there are some strange RD curve when encoding with JM. For example, the RD curve of Y component of sc_ppt_doc_xls_1920x1080_20_8bit_200_rgb under LB-HT coding structure is shown below. The PSNR when encoding with QP 17 is lower than that encoded with QP 22.

To provide more meaningful results, we adjust the QP range values sequence by sequence to enlarge the PSNR overlap between JM and HM and to avoid the case shown in the above figure. The experimental results with JM QP adjustment (the detailed QP used in the comparison can be found in the attached spreadsheet) are provided in Table 3.

Table 3 JM with modified QP VS. HM-12.0+RExt-4.1 RCE

# References

- JCTVC-G399, “Comparison of Compression Performance of HEVC Working Draft 4 with AVC High Profile,” Geneva, CH, Nov. 2011.

- JCTVC-H0360, “Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile,” San José, CA, USA, Feb. 2012.

- JCTVC-I0409, “Comparison of Compression Performance of HEVC Draft 6 with AVC High Profile,” Geneva, CH, 27 April – 7 May 2012.

- JCTVC-J0236, “Comparison of Compression Performance of HEVC Draft 7 with AVC High Profile,” Stockholm, SE, 11–20 July 2012.

- JCTVC-K0279, “Comparison of Compression Performance of HEVC Draft 8 with AVC High Profile and Performance of HM8.0 with Different Delay Characteristics,” Shanghai, CN, 10–19 Oct. 2012.

- JCTVC-L0322, “Comparison of Compression Performance of HEVC Draft 9 with AVC High Profile and Performance of HM9.0 with Temporal Scalability Characteristics,” Geneva, CH, 14–23 Jan. 2013.

- JCTVC-N1005, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 4,” Vienna, Austria, 25 Jul. – 2 Aug. 2013.

- JCTVC-N1006, “Common test conditions and software reference configurations for HEVC range extensions,” Vienna, Austria, 25 Jul. – 2 Aug. 2013.

- JCTVC-N1123, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques,” Vienna, Austria, 25 Jul. – 2 Aug. 2013.