| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0186 |
| --- | --- |

| Title: | On residual rotation for Inter and Intra BC modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiulian Peng Bin Li Jizheng Xu | Tel: Email: | [phone contact number] [email address(es)] |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document reports results of disabling residual rotation in inter CUs and Intra BC CUs and proposes to remove the residual rotation process for non-Intra CUs.

# Introduction

Residual rotation was adopted at Vienna meeting for 4x4 TU with transform bypass. Residual rotation aim to place the potential large residue at the top left of the TU, which may be helpful to the entropy coding. For intra predicted blocks with transform bypass, the residue at bottom right of the block may be relatively larger, because of the distance between the reconstructed pixels and predicted pixels. Thus residual rotation may be helpful to improve the coding efficiency. But for Inter CUs, the residue at bottom right will no longer be relatively larger than the residue at top left. Thus, this document proposes to remove the residual rotation for non-Intra predicted blocks to reduce the complexity.

# Experimental results

The RCE common test condition specified in JCTVC-N1123 [1] is followed. Both experimental results on lossy coding and lossless coding are provided.

## Disable rotation for Inter CUs

Table 1 Disable rotation for Inter CUs (lossy coding)

Table 2 Disable rotation for Inter CUs (lossless coding)

## Disable rotation for Inter CUs and Intra BC CUs

Table 3 Disable rotation for Inter CUs and Intra BC CUs (lossy coding)

Table 4 Disable rotation for Inter CUs and Intra BC CUs (lossless coding)

# Text Modifications

The text modifications are based on JCTVC-N1005_v3.doc.

## Disable rotation for Inter CU

8.6.2 Scaling and transformation process

–	If transform_skip_rotation_enabled_flag is equal to 1 and nTbS is equal to 4 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( TransCoeffLevel[ nTbS − x −1 ][ nTbS − y − 1 ]  <<  tsShift )	(8-1)

–	Otherwise, the (nTbS)x(nTbS) array r is set equal to the (nTbS)x(nTbS) array of transform coefficients TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the quantization parameter qP as inputs, and the output is an (nTbS)x(nTbS) array of scaled transform coefficients d.

- The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

–	The variable tsShift is derived as follows:

tsShift = extended_precision_processing_flag ? Max( 7, bdShift ) : 7	(8-2)

–	If transform_skip_rotation_enabled_flag is equal to 1 and nTbS is equal to 4,

r[ x ][ y ] = ( d[ nTbS − x −1 ][ nTbS − y − 1 ]  <<  tsShift )	(8-3)

–	Otherwise,

r[ x ][ y ] = ( d[ x ][ y ]  <<  tsShift )	(8-4)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

- The residual sample values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are modified as follows:

r[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-5)

## Disable rotation for Inter CUs and Intra BC CUs

–	If transform_skip_rotation_enabled_flag is equal to 1 and nTbS is equal to 4 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA and intra_bc_flag[ xTbY ][ yTbY ] is equal to 0, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

r[ x ][ y ] = ( TransCoeffLevel[ nTbS − x −1 ][ nTbS − y − 1 ]  <<  tsShift )	(8-6)

–	Otherwise, the (nTbS)x(nTbS) array r is set equal to the (nTbS)x(nTbS) array of transform coefficients TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

- Otherwise, the following ordered steps apply:

- The scaling process for transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the quantization parameter qP as inputs, and the output is an (nTbS)x(nTbS) array of scaled transform coefficients d.

- The (nTbS)x(nTbS) array of residual samples r is derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1, the residual sample array values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are derived as follows:

–	The variable tsShift is derived as follows:

tsShift = extended_precision_processing_flag ? Max( 7, bdShift ) : 7	(8-7)

–	If transform_skip_rotation_enabled_flag is equal to 1 and nTbS is equal to 4 and CuPredMode[ xTbY ][ yTbY ] is equal to MODE_INTRA and intra_bc_flag[ xTbY ][ yTbY ] is equal to 0,

r[ x ][ y ] = ( d[ nTbS − x −1 ][ nTbS − y − 1 ]  <<  tsShift )	(8-8)

–	Otherwise,

r[ x ][ y ] = ( d[ x ][ y ]  <<  tsShift )	(8-9)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0), the transformation process for scaled transform coefficients as specified in subclause Error: Reference source not found is invoked with the transform block location ( xTbY, yTbY ), the size of the transform block nTbS, the colour component variable cIdx, and the (nTbS)x(nTbS) array of scaled transform coefficients d as inputs, and the output is an (nTbS)x(nTbS) array of residual samples r.

- The residual sample values r[ x ][ y ] with x = 0..nTbS − 1, y = 0..nTbS − 1 are modified as follows:

r[ x ][ y ] = ( r[ x ][ y ] + ( 1  <<  ( bdShift − 1 ) ) )  >>  bdShift	(8-10)

# References

- JCTVC-N1123, “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques,” Vienna, Austria, 25 Jul. – 2 Aug. 2013.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).