| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0188 |
| --- | --- |

| Joint Collaborative Team on 3D Video Coding Extensions of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 6th Meeting: Geneva, CH, 25 Oct. – 1 Nov. 2013 | Document: JCT3V-F0070 |
| --- | --- |

| Title: | AHG20: Ultra Low Delay for SHVC, MV-HEVC and 3D-HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC and JCT-3V |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Karsten Sühring Yago Sanchez Thomas Schierl | Email: | robert.skupin@hhi.fraunhofer.de karsten.suehring@hhi.fraunhofer.de yago.sanchez@hhi.fraunhofer.de thomas.schierl@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The current bitstream order in SHVC, MV-HEVC and 3D-HEVC prohibits ultra-low delay applications. This contribution proposes to enable ultra-low delay operation with a corresponding bitstream order and decoding picture buffer operation and discusses implications of the CPB layout on legacy devices.

# Introduction

The HEVC base specification contains the necessary tools to allow decoding of bitstreams with ultra-low delay, i.e. through CPB operation on DU level and CPB timing information with DU granularity as opposed to CPB operation on AU level as in H.264/AVC. Thus, a device can operate on sub-portions of pictures in order to reduce occurring processing delays.

The following provides a short overview on two use cases for ultra-low delay applications with different HEVC extensions:

- 3D Cloud Gaming: 3D video game mechanics and graphics are computed on a powerful cloud server that is connected to the remote end user equipment by means of the public internet. A small end-to-end delay in the scenario, e.g. time between a user keystroke to display of character movement, is vital for a favourable user experience. The necessary encoding of game graphics on the cloud server and corresponding decoding on the end user equipment significantly contributes to said end-to-end delay.

- Remote Surgery: A well-known cardiologist is requested to support heart surgery procedure through means of remotely controlled surgical instruments and live video feedback. The precision of instrument control and thus the well-being of the patient highly depend on the end-to-end delay between the surgeons input and the corresponding visual feedback. . The necessary encoding of surgery procedures on the surgery site and the decoding on the surgeons site significantly contributes to said end-to-end delay. During the procedure, a low quality representation of the multiple hours of video material is streamed to lecture rooms and archived for legal protection.

Such ultra-low delay operations with extensions of HEVC require a flexible bitstream order that does not prohibit layer-wise interleaving of DUs within an AU is necessary as proposed in JCTVC-M0204 [1] or JCTVC-N0290 [2]. In order to enable an interleaved bitstream order and inter-layer prediction in ultra-low delay mode, the technical proposal is given in section 2.

At the last meeting, the following was noted:

“A decoder that is not designed to be able to decode part of picture and then decode part of some other layer, then come back and decode more of the base layer would need to have buffering to re-order the DUs into whole-picture order.

The concept was generally agreed in spirit, but some participants were unsure that all issues had been fully thought through. We are inclined to adopt, but feel that some further study is needed to confirm that this does not introduce problems. Further study to confirm, with intent to adopt if not found problematic.”

In order to address these comments, implications on the construction of the reference picture lists for inter-layer references were studied and the proposal was accordingly extended. Furthermore, a discussion of legacy device impact is given in section 3.

A revision of this contribution adds a slide deck.

# Proposal

This document proposes to add a flag to the VPS extension syntax to indicate the interleaving of DUs across layers within an AU. When the du_interleaving_enabled_flag is enabled, DUs with a nuh_layer_id nuhLayerIdA can follow DUs with a nuh_layer_id nuhLayerIdB in an AU in bitstream order with nuhLayerIdB > nuhLayerIdA.

| vps_extension( ) { | Descriptor |
| --- | --- |
| … |  |
| du_interleaving_enabled_flag | u(1) |
| ... |  |

du_interleaving_enabled_flag equal to 1 specifies that all VCL NAL units of decoding unit m with nuh_layer_id equal to nuhLayerIdA shall precede in decoding order all VCL NAL units of decoding unit n with nuh_layer_id greater than nuhLayerIdA in the same access unit for which the decoding process depends on syntax elements or decoded sample values that belong to the decoding unit m. du_interleaving_enabled_flag equal to 0 specifies that all VCL NAL units of a coded picture with nuh_layer_id equal to nuhLayerIdA shall precede in decoding order the VCL NAL units of all coded pictures with nuh_layer_id greater than nuhLayerIdA in the same access unit.

## Bitstream order

Section F.7.4.2.4 of JCTVC-E1007_v6 as common to SHVC, 3D-HEVC and MV-HEVC adds the following constraint regarding the bitstream order of coded pictures.

A coded picture with nuh_layer_id equal to nuhLayerIdA shall precede, in decoding order, all coded pictures with nuh_layer_id greater than nuhLayerIdA in the same access unit.

Section 7.4.2.4.5 Order of VCL NAL units and association to coded pictures of the HEVC base specification applies in the current extension drafts and this document proposes the following changes highlighted in yellow.

Order of VCL NAL units and association to coded pictures

This subclause specifies the order of VCL NAL units and association to coded pictures.

Each VCL NAL unit is part of a coded picture.

The order of the VCL NAL units within a coded picture is constrained as follows:

–	The first VCL NAL unit of the coded picture shall have first_slice_segment_in_pic_flag equal to 1.

–	Let sliceSegAddrA and sliceSegAddrB be the slice_segment_address values of any two coded slice segment NAL units A and B within the same coded picture. When either of the following conditions is true, coded slice segment NAL unit A shall precede the coded slice segment NAL unit B:

–	TileId[ CtbAddrRsToTs[ sliceSegAddrA ] ] is less than TileId[ CtbAddrRsToTs[ sliceSegAddrB ] ].

–	TileId[ CtbAddrRsToTs[ sliceSegAddrA ] ] is equal to TileId[ CtbAddrRsToTs[ sliceSegAddrB ] ] and CtbAddrRsToTs[ sliceSegAddrA ] is less than CtbAddrRsToTs[ sliceSegAddrB ].

If an access unit consists of more than one coded picture the order of the associated VCL NAL units is constrained as follows:

– 	If du_interleaving_enabled_flag is equal to 0, all VCL NAL units of a A coded picture with nuh_layer_id equal to nuhLayerIdA shall precede in decoding order the VCL NAL units of all coded pictures with nuh_layer_id greater than nuhLayerIdA in the same access unit.

–	Otherwise (du_interleaving_enabled_flag is equal to 1), all VCL NAL units of decoding unit m with nuh_layer_id equal to nuhLayerIdA shall precede in decoding order all VCL NAL units of decoding unit n with nuh_layer_id greater than nuhLayerIdA in the same access unit for which the decoding process depends on syntax elements or decoded sample values that belong to the decoding unit m.

## DPB Operation

The following change enables inter-layer prediction from DUs with nuh_layer_id equal to nuhLayerIdA before all DUs with nuh_layer_id equal to nuhLayerIdA are decoded. The intention here is to ensure the presence of “partially” decoded pictures in the DPB for F.8.1.2/H.8.1.2 (Decoding process for inter-layer reference picture set) via an “update” of the respective picture buffer storage after finishing decoding of each DU.

- F.13.3.4	Current decoded picture marking and storage

The process specified in this subclause happens instantaneously at the CPB removal time of anythe last decoding unit m of picture n, CpbRemovalTime[ n ].

- If du_interleaving_enabled_flag is equal to 1 then the following applies.

  - If decoding unit m is the first decoding unit of the current decoded picture, the current decoded picture is stored in the DPB in an empty picture storage buffer picStorBufA, the DPB fullness is incremented by one, and the current picture is marked as "used for short-term reference".

  - Otherwise, the current decoded picture is stored in the DPB in the in picture buffer storage picStorBufA.

- Otherwise (du_interleaving_enabled_flag is equal to 0) and decoding unit m is the last decoding unit of picture n, Tthe current decoded picture is stored in the DPB in an empty picture storage buffer, the DPB fullness is incremented by one, and the current picture is marked as "used for short-term reference".

# Discussion

The impact of interleaved DUs across layers within an AU for the processing delay is illustrated in the following. In Error: Reference source not found the arrival of DUs (1) of an AU n with non-interleaved bitstream order is shown. The numbers in the box refers to the value of nuh_layer_id (or layer) of the NAL units of the DU. As shown, first all DUs with nuh_layer_id equal to 0 arrive before DUs with nuh_layer_id equal to 0 arrive and so forth.

Figure 1: Bitstream order of DUs of with three layers over time (non-interleaved DUs).

In Error: Reference source not found the arrival of DUs (1) of an AU n is shown for an interleaved bitstream according to the proposed interleaved bitstream order. As shown, DUs of different layers can be mixed within the AU.

Figure 2: Bitstream order of DUs of three layers over time (interleaved DUs).

A CPB removal time is associated with each DU. The CPB removal time cannot be lower than the final arrival time of a DU, exemplarily shown as (2) for the first DU. The final arrival time of the first DU of the second layer, which is labelled with (3), can be lowered by using an interleaved bitstream order as shown in Error: Reference source not found compared to the bitstream order shown in Error: Reference source not found, which allows an earlier decoding begin of DUs with higher values of nuh_layer_id.

Adoption of the proposal would imply that the decoding unit information SEI message may carry CPB timings for the case that DUs of an AU are interleaved across layers. A device may base its decision on sub-picture level CPB operation on the du_interleaving_enabled_flag and always fall back to AU based operation when it is incapable or reluctant of interleaved DU decoding.

In the latter case, reconstructing a non-interleaved bitstream order of DUs is trivial using the value of nuh_layer_id and the order of NAL units in the interleaved bitstream as ordering constraints. The fall back solution for legacy devices depends in part of the CPB approach of which two alternatives are described in the following.

### Single CPB solution

When using a single CPB for the DUs of an AU, the memory fragmentation can be avoided by constraining the signalled timing in the following way: for any DU n that precedes the DU m in bitstream order, the CPB removal time of DU n shall be lower than or equal to the CPB removal time of DU m. When arriving packets are stored at consecutive memory addresses in the CPB (as typical in a ring buffer), this constraint avoids fragmentation of free memory in the CPB.

On an AU scale, data is removed in the same order as it is received. The same holds true for DU based sub-picture CPB operation when the above constraint is obeyed. Keeping track of the start and end addresses of used memory blocks instead of a list of used and free memory blocks ensures that newly arriving DUs are not fragmented. A minor burden for the fall back AU based CPB operation are the necessary non-continuous read operations to recombine an AU with non-interleaved DUs.

### Multi CPB solution

Alternatively multiple CPBs can be used for the DUs of each layer, i.e. depending on the value of nuh_layer_id, a NAL unit is written to the respective CPB, as illustrated in Figure 3.

Figure 3: Bitstream order, timing and memory usage with multiple CPBs.

The left side of Figure 3 shows the assignment of DUs to different CPBs. For each layer (number), a separate CPB is operated and DUs are stored at consecutive memory locations within each CPB. Exemplarily the arrival timing of an interleaved bitstream is shown. The right side of Figure 3 shows the memory status after arrival of the AU. DUs that belong to the same layer are stored in consecutive memory locations. The same holds true for non-interleaved bitstreams. A multi-layer decoder benefits from such a memory layout as the DUs of a layer are accessible at consecutive memory addresses. DUs arrive in decoding order for each layer and used memory block cover continuous memory areas in each CPB. A multi CPB solution also reduces efforts in layer-wise transport, as individual stream (TS, RTP) do not have to be remultiplexed.

The recombining of DUs for fall back AU based CPB operation follows the same constraints as the single CPB solution, i.e. using the value of nuh_layer_id (or CPB identifier) and the order of NAL units as an ordering constraint within the AU. However, AU based CPB operation does not require non-continuous memory access as in the single CPB solution and thus is not associated with any extra cost.

# References

- K. Suehring, R. Skupin, Y. Sanchez, T. Schierl, ”Ultra-low delay multi-layer decoding by interleaving of decoding units in SHVC and 3D/MV-HEVC”, JCTVC-M0204, Incheon, KR, April 2013.

- R. Skupin, K. Suehring, Y. Sanchez, T. Schierl, ”Ultra-low delay with SHVC, MV-HEVC and 3D-HEVC”, JCTVC-N0290, Vienna, AT, July 2013.

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e.V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).