| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0190 m30960 |
| --- | --- |

| Title: | AHG15: AVC and HEVC encapsulation for hybrid codec scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Jack Enhorn Rickard Sjöberg Ericsson Research Färögatan 6 16480 Stockholm | Tel: Email: | +46761263591 jonatan.samuelsson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes syntax for enhancement layer HEVC NAL units to support hybrid codec scalability where the base layer is AVC. More specifically it is proposed that an additional prefix byte is added to enhancement layer HEVC NAL units when the stream conforms to the hybrid codec scalability profile. The prefix byte is selected so that it corresponds to the reserved nal_unit_type 22 in the AVC specification which is proposed to be used for indicating the HEVC NAL units for an AVC decoder. Using a reserved nal_unit_type will make legacy AVC decoders discard all HEVC NAL units.

The prefix byte also happen to correspond to the reserved nal_unit_type 11 in the HEVC specification, which is proposed to act as an identifier to a scalable HEVC decoder that this NAL unit contains HEVC enhancement layer data to an AVC base layer.

It is asserted that the proposed scheme enables creation of a single NAL unit stream containing both AVC NAL units and HEVC NAL units without any NAL unit codec identification problems.

# Problem Statement

The scalable extension of HEVC with an AVC base layer is an important feature and has been highlighted by MPEG requirements as something that should be supported [1]. JCTVC-N0050 [2] provides a suggested specification text for this "hybrid mode" in which the base layer AVC NAL units are encapsulated in HEVC NAL units with an additional HEVC NAL unit header. A disadvantage with that solution is that a legacy AVC decoder cannot decode the stream "as is" but is required to unwrap the AVC NAL units first. This contribution proposes a different scheme for the encapsulation; base layer AVC NAL units are kept unmodified in the joint NAL unit stream and enhancement layer HEVC NAL units are required to have an additional prefix byte in order for legacy AVC decoders to identify them as NAL units they cannot decode (and should discard).

It is proposed that the additional prefix byte shall be present in all HEVC NAL units if and only if the profile is a "hybrid AVC-HEVC" profile (alternatively if and only if avc_base_layer_flag is equal to1), and that only one single value for the prefix byte shall be allowed (0x16).

The value of the prefix byte is selected so that it corresponds to a nal_unit_type that is "reserved for future use by ITU-T | ISO/IEC" in the AVC specification. Also in the HEVC specification the value of the prefix byte corresponds to a nal_unit_type that is "reserved for future use by ITU-T | ISO/IEC". It is proposed that that nal_unit_type value in HEVC is defined as forbidden in order to make it easy to determine directly at NAL unit header level if the stream is a "hybrid AVC-HEVC" stream or a normal HEVC/SHVC stream.

Finally it is suggested to add a variable, HevcPrefixBytePresent, which is set by external means, to indicate that the decoder shall parse the prefix byte. In practice HevcPrefixBytePresent must be set to one when decoding a "hybrid AVC-HEVC" stream and set to zero when decoding other streams.

The proposed change does not have any effect on HEVC version 1 bitstreams or normal (non-hybrid) scalable bitstreams as is highlighted in Figure 1.

Figure 1. Illustration of the effect of the proposed prefix byte.

# Software

The proposed solution has been implemented in SHM 3.0.1 and a patch is attached to this contribution. The modified SHM encoder takes as additional input the AVC base layer bitstream and creates a joint bitstream containing both AVC NAL units and HEVC enhancement layer NAL units with the additional prefix byte. The modified SHM decoder takes as additional argument -hevc-prefix-byte. When hevc-prefix-byte is set to 1 the decoder will parse the additional prefix byte in the HEVC NAL units and ignore all NAL units that do not start with the prefix byte (i.e. the AVC NAL units).

As an additional feature the decoder is modified such that when hevc-prefix-byte is not set to 1 and the prefix byte is detected, a warning message is printed and the value of hevc-prefix-byte is overridden. This feature makes it possible to decode hybrid AVC-HEVC streams even when hevc-prefix-byte is not set to 1 by external means.

It is verified that the AVC reference decoder (JM) is capable of decoding the AVC base layer when the joint bitstream is given as input, without any modifications at all.

# Proposed HEVC specification text

Syntax:

| nal_unit( NumBytesInNalUnit ) { | Descriptor |
| --- | --- |
| if( HevcPrefixBytePresent = = 0 ¦¦ next_bits( 8 ) = = 0x16) { |  |
| if( next_bits( 8 ) = = 0x16 ) { |  |
| hevc_prefix_byte | u(8) |
| RbspStartOffset = 3 |  |
| } else |  |
| RbspStartOffset = 2 |  |
| nal_unit_header( ) |  |
| NumBytesInRbsp = 0 |  |
| for( i = 2RbspStartOffset; i < NumBytesInNalUnit; i++ ) |  |
| if( i + 2 < NumBytesInNalUnit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRbsp++ ] | b(8) |
| rbsp_byte[ NumBytesInRbsp++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRbsp++ ] | b(8) |
| } else |  |
| Rec. ITU-T H.264 ¦ ISO/IEC 14496-10 nal_unit( NumBytesInNalUnit ) |  |
| } |  |

Semantics:

hevc_prefix_byte indicates that the current NAL unit is an HEVC NAL unit in a bitstream containing a mixture of AVC NAL units and HEVC NAL units. If profile_idc in the VPS that is active for the CVS is equal to HYBRID_CODEC, hevc_prefix_byte shall be present for each HEVC NAL unit in the CVS and the value of hevc_prefix_byte shall be equal to 0x16. Otherwise (profile_idc in the VPS that is active for the CVS is not equal to HYBRID_CODEC), hevc_prefix_byte shall not be present for any NAL unit in the CVS.

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| … | … | … | … |
| 10 12 14 | RSV_VCL_N10 RSV_VCL_N12 RSV_VCL_N14 | Reserved non-IRAP sub-layer non-reference VCL NAL unit types | VCL |
| 11 | FORBIDDEN11 | Forbidden | N/A |
| 11 13 15 | RSV_VCL_R11 RSV_VCL_R13 RSV_VCL_R15 | Reserved non-IRAP sub-layer reference VCL NAL unit types | VCL |
| … | … | … | … |

It is a requirement of bitstream conformance that the bitstream shall not contain any NAL unit with nal_unit_type equal to FORBIDDEN11.

NOTE – When HevcPrefixBytePresent is equal to 0 and a NAL unit with nal_unit_type equal to FORBIDDEN11 is detected in a bitstream then the bitstream is a non-conforming bitstream for that mode of operation. However, when this situation occurs, decoders are encouraged to request or emulate that HevcPrefixBytePresent is set to 1 by external means for such a bitstream as it may make the bitstream conforming.

Decoding process:

The variable HevcPrefixBytePresent, is specified as follows:

- If some external means, not specified in this Specification, is available to set HevcPrefixBytePresent, HevcPrefixBytePresent is set by the external means.

- Otherwise, HevcPrefixBytePresent is set equal to 0.

NOTE – When avc_base_layer_flag is equal to 1 in the VPS that is active for the CVS that is to be decoded, then HevcPrefixBytePresent should be set to 1 by external means.

# Proposed AVC specification text (included here for information)

The following changes are proposed for the H.264 specification in a VCEG document and to the 14496-10 specification in an MPEG document.

- Table 7-1 – NAL unit type codes, syntax element categories, and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | C | Annex Error: Reference source not found NAL unit type class | Annex Error: Reference source not found and Annex Error: Reference source not found NAL unit type class | Annex Error: Reference source not found NAL unit type class |
| --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … |
| 16..18 | Reserved |  | non-VCL | non-VCL | non-VCL |
| 19 | Coded slice of an auxiliary coded picture without partitioning slice_layer_without_partitioning_rbsp( ) | 2, 3, 4 | non-VCL | non-VCL | non-VCL |
| 20 | Coded slice extension slice_layer_extension_rbsp( ) | 2, 3, 4 | non-VCL | VCL | VCL |
| 21 | Coded slice extension for depth view components /*specified in Annex Error: Reference source not found */ slice_layer_extension_rbsp( ) /* specified in Annex Error: Reference source not found */ | 2, 3, 4 | non-VCL | non-VCL | VCL |
| 22 | HEVC NAL unit /*specified in ITU-T H.265 ¦ ISO/IEC 23008-2 */ |  | non-VCL | non-VCL | non-VCL |
| 22..23 | Reserved |  | non-VCL | non-VCL | VCL |
| 24..31 | Unspecified |  | non-VCL | non-VCL | non-VCL |

When NAL units with nal_unit_type equal to 22 are present in a coded video sequence, decoders shall either perform the (optional) decoding process specified for these NAL units in ITU-T H.265 | ISO/IEC 23008-2 or shall ignore (remove from the bitstream and discard) the contents of these NAL units.

# References

[1] 	N12956 “Requirements of the scalable enhancement of HEVC” Ajay Luthra, Jens-Rainer Ohm, Jörn Ostermann, July 2012

[2]	JCTVC-N0050 "Specification text to support AVC base layer in HEVC layered extensions" 
J. Boyce, K. Kawamura, July 2013

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).