# 2 Normative references

…

## 2.4 Additional references

–	Recommendation ITU-T T.35 (in force), Procedure for the allocation of ITU-T defined codes for non-standard facilities.

–	ISO/IEC 11578: in force, Information technology — Open Systems Interconnection — Remote Procedure Call (RPC).

–	ISO 11664-1: in force, Colorimetry  — Part 1: CIE standard colorimetric observers.

–	ISO 12232: in force, Photography – Digital still cameras – Determination of exposure index, ISO speed ratings, standard output sensitivity, and recommended exposure index.

–	IETF RFC 1321 (in force), The MD5 Message-Digest Algorithm.

–	Recommendation ITU-T H.264 | ISO/IEC 14496-10: in force, Advanced video coding for generic audiovisual services.

#### 7.3.1.1 General NAL unit syntax

| nal_unit( NumBytesInNalUnit ) { | Descriptor |
| --- | --- |
| if( HevcPrefixBytePresent = = 0 ¦¦ next_bits( 8 ) = = 0x16) { |  |
| if( next_bits( 8 ) = = 0x16 ) { |  |
| hevc_prefix_byte | u(8) |
| RbspStartOffset = 3 |  |
| } else |  |
| RbspStartOffset = 2 |  |
| nal_unit_header( ) |  |
| NumBytesInRbsp = 0 |  |
| for( i = 2RbspStartOffset; i < NumBytesInNalUnit; i++ ) |  |
| if( i + 2 < NumBytesInNalUnit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRbsp++ ] | b(8) |
| rbsp_byte[ NumBytesInRbsp++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRbsp++ ] | b(8) |
| } else |  |
| Rec. ITU-T H.264 ¦ ISO/IEC 14496-10 nal_unit( NumBytesInNalUnit ) |  |
| } |  |

#### 7.4.2.1 General NAL unit semantics

- hevc_prefix_byte indicates that the current NAL unit is an HEVC NAL unit in a bitstream containing a mixture of AVC NAL units and HEVC NAL units. If avc_base_layer_flag is equal 0 in the VPS that is active for the CVS, hevc_prefix_byte shall not be present for any NAL unit in the CVS. When hevc_prefix_byte is not present, it is inferred to be equal to 0.

…

NOTE – The value 11 is reserved to prevent emulation of the syntax element hevc_prefix_byte and may be used to detect that avc_base_layer_flag is equal to 1 in the VPS that is active or will be actived for the current CVS.

nuh_layer_id shall be equal to 0. Other values of nuh_layer_id may be specified in the future by ITU-T | ISO/IEC. For purposes other than determining the amount of data in the decoding units of the bitstream (as specified in Annex Error: Reference source not found), decoders shall ignore (i.e., remove from the bitstream and discard) all NAL units with values of nuh_layer_id not equal to 0. When nuh_layer_id is not present, it is inferred to be equal to 0.

- NOTE 1 – It is anticipated that in future scalable or 3D video coding extensions of this specification, this syntax element will be used to identify additional layers that may be present in the CVS, wherein a layer may be, e.g., a spatial scalable layer, a quality scalable layer, a texture view or a depth view.

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit. The value of nuh_temporal_id_plus1 shall not be equal to 0. When nuh_temporal_id_plus1 is not present, it is inferred to be equal to 1.

## 8.1 General decoding process

…

The variable HevcPrefixBytePresent, is specified as follows:

- If some external means, not specified in this Specification, is available to set HevcPrefixBytePresent, HevcPrefixBytePresent is set by the external means.

- Otherwise, HevcPrefixBytePresent is set equal to 0.

…

For each NAL unit the decoding process operates as follows for the current picture CurrPic:

The variable AvcNalUnit, is specified as follows:

- If some external means, not specified in this Specification, is available to set AvcNalUnit, AvcNalUnit is set by the external means.

- Otherwise, if avc_base_layer_flag is equal to 1 and hevc_prefix_byte is equal to 0, AvcNalUnit is set equal to 1.

- Otherwise, AvcNalUnit is set equal to 0.

If AvcNalUnit is equal to 1:

- The decoding process of Rec. ITU-T H.264 | ISO/IEC 14496-10 is invoked with the NAL unit as input for Rec. ITU-T H.264 | ISO/IEC 14496-10 nal_unit( ) for current picture CurrPic.

- PicOrderCntVal is set equal to Rec. ITU-T H.264 | ISO/IEC 14496-10 PicOrderCnt( CurrPic ).

- PicOutputFlag is set equal to Rec. ITU-T H.264 | ISO/IEC 14496-10 output_flag.

- After all slices of CurrPic have been decoded, an additional picture marking used in this Specification is applied to the picture:

–	If Rec. ITU-T H.264 | ISO/IEC 14496-10 discardable_flag is equal to 1, the decoded picture is marked as "unused for reference".

–	Otherwise, the decoded picture is marked as "used for short-term reference".

NOTE – The additional marking used in this specification does not affect the picture marking applied to the picture in Rec. ITU-T H.264 | ISO/IEC 14496-10.

Otherwise, (AvcNalUnit is equal to 0):

- The decoding of NAL units is specified in subclause 8.2.

- The processes in subclause 8.3 specify the following decoding processes using syntax elements in the slice segment layer and above:

–	Variables and functions relating to picture order count are derived in subclause Error: Reference source not found. This needs to be invoked only for the first slice segment of a picture.

–	The decoding process for RPS in subclause 8.3.2 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first slice segment of a picture.

–	When the current picture is a BLA picture or is a CRA picture with NoRaslOutputFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 8.3.3 is invoked, which needs to be invoked only for the first slice segment of a picture.

–	PicOutputFlag is set as follows:

–	If the current picture is a RASL picture and NoRaslOutputFlag of the associated IRAP picture is equal to 1, PicOutputFlag is set equal to 0.

–	Otherwise, PicOutputFlag is set equal to pic_output_flag.

–	At the beginning of the decoding process for each P or B slice, the decoding process for reference picture lists construction specified in subclause 8.3.4 is invoked for derivation of reference picture list 0 (RefPicList0) and, when decoding a B slice, reference picture list 1 (RefPicList1).

- The processes in subclauses 8.4, 8.5, 8.6, and 8.7 specify decoding processes using syntax elements in all syntax structure layers. It is a requirement of bitstream conformance that the coded slices of the picture shall contain slice segment data for every coding tree unit of the picture, such that the division of the picture into slices, the division of the slices into slice segments, and the division of the slice segments into coding tree units each form a partitioning of the picture.

- After all slices of the current picture have been decoded, the decoded picture is marked as "used for short-term reference".

F.7.3.2.1 Video parameter set RBS

| vps_extension( ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| vps_extension_byte_alignment_reserved_one_bit | u(1) |
| avc_base_layer_flag | u(1) |
| if (avc_base_layer_flag) |  |
| avc_base_profile_level_idc | u(24) |
| splitting_flag | u(1) |
| … |  |

avc_base_profile_level_idc indicates the conformance point of the Rec. ITU-T H.264 | ISO/IEC 14496-10 base layer.  avc_base_profile_level_idc[ i ] is the exact copy of the three bytes Rec. ITU-T H.264 | ISO/IEC 14496-10 comprised of profile_idc, constraint_set0_flag, constraint_set1_flag, constraint_set2_flag, constraint_set3_flag, constraint_set4_flag, constraint_set5_flag, reserved_zero_2bits and level_idc, as if these syntax elements were used to specify the profile and level conformance of the representation of the Rec. ITU-T H.264 | ISO/IEC 14496-10 base layer.

H.11 Profiles, tiers, and levels

- H.11.1.2 Scalable Main profile

Bitstreams conforming to the scalable main profile shall obey the following constraints:

–	The picture resampling process of picture sample values as specified in subclause Error: Reference source not found shall not be invoked more than once for decoding of each particular picture and the resampling process of picture motion field as specified in subclause Error: Reference source not found shall not be invoked more than once for decoding of each particular picture. When both picture sample values and picture motion field resampling processes are invoked for decoding of a particular picture, they shall be applied to the same reference layer picture.

–	When DependencyId[ i ] is equal to 1, ScalabilityId[ LayerIdxInVps [ i ]][ smIdx ] shall be equal to 0 for any smIdx value from 0 to 15, inclusive, that is not equal to 2, for any coded picture with nuh_layer_id equal to i.

- VPSs shall have avc_base_layer_flag equal to 0.

When avc_base_layer_flag equal to 1, it is a requirement of bitstream conformance that MotionPredEnabledFlag[ iNuhLId ][ 0 ] shall be equal to 0 for iNuhLId equal to any value of nuh_layer_id present in the bitstream.

- H.11.1.3 Hybrid Scalable Main profile

Bitstreams conforming to the hybrid scalable main profile shall obey the following constraints:

- The picture resampling process as specified in subclause Error: Reference source not found shall not be invoked more than once for decoding of each particular picture.

- VPSs shall have avc_base_layer_flag equal to 1.

- It is a requirement of bitstream conformance that MotionPredEnabledFlag[ iNuhLId ][ 0 ] shall be equal to 0 for iNuhLId equal to any value of nuh_layer_id present in the bitstream.

- The Rec. ITU-T H.264 | ISO/IEC 14496-10 base layer bitstream shall obey all constraints specified in Rec. ITU-T H.264 | ISO/IEC 14496-10 A.2.4.1 for the High profile.