| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Geneva, CH, 23 Oct. – 1 Nov. 2013 | Document: JCTVC-O0192 |
| --- | --- |

| Title: | AHG5/AHG6/AHG9: Text changes for HEVC Range Extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | PoLin Lai, Shan Liu, and Shawmin Lei MediaTek USA 2860 Junction Ave San Jose, CA 95134, USA | Tel: Email: | polin.lai@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document presents text changes for HEVC Range Extensions in the following three aspects: 1. Inter-RDPCM fix to reflect its behavior under both lossy and lossless conditions. 2. Extending inter-RDPCM to IntraBC blocks, and 3. Simplifying CU-level transquant bypass signalling with an additional PPS flag.

# Introduction

In the 14th JCTVC meeting, several coding tools were adopted together, including Inter-RDPCM [1], residue rotation [2], and Intra block copy (IntraBC) [3]. With the concurrent adoption, it is noticed that there exist discrepancy between the Range Extension text specification [4] and software. This document provides text changes with experimental data to improve the specification of Range Extension, taking into account the newly adopted tools from the 14th JCTVC meeting.

# Proposed text changes

## Inter-RDPCM text fixes and suggestions

Inter-RDPCM from JCTVC-N0074 [1] was adopted in the 14th JCTVC meeting, which extend RDPCM from Intra CU to Inter CU with flags signalling the usage and directionality of Inter-RDPCM. Both lossy and lossless Inter-RDPCM methods were adopted, which were integrated in the reference software HM-12.0+RExt-4.1. However the text in the specification of Range Extension [4] only reflects the usage of Inter-RDPCM lossy. Below are the proposed fixes to allow Inter-RDPCM lossless, reflecting the agreed Inter-RDPCM methods in the 14th JCTVC meeting.

- 7.3.8.11	Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize <= Log2MaxTransformSkipSize ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER && residual_dpcm_inter_enabled_flag ) ) && ( transform_skip_flag[ x0 ][ y0 ][ cIdx ] ¦ ¦ cu_transquant_bypass_flag ) ) { |  |
| inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| inter_rpdcm_dir_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |
| … |  |
| } |  |

8.5.4.2	Decoding process for luma residual blocks

…

…

–	Otherwise (split_transform_flag[ xCb + xB0 ][ yCb + yB0 ][ trafoDepth ] is equal to 0), the following ordered steps apply:

- …

- …

- When residual_dpcm_inter_enabled_flag is equal to 1 and transform_skip_flag[ xCb + xB0 ][ yCb + yB0 ][ 0 ] inter_rdpcm_flag[ xCb + xB0 ][ yCb + yB0 ][ 0 ] is equal to 1, the directional residual modification process for blocks using a transform bypass as specified in subclause Error: Reference source not found is invoked with the variable mDir set equal to inter_rdpcm_dir_flag[ xCb + xB0 ][ yCb + yB0 ][ 0 ], the variable nTbS, and the (nTbS)x(nTbS) array r set equal to the array transformBlock as inputs, and the output is a modified (nTbS)x(nTbS) array transformBlock.

- …

8.5.4.3	Decoding process for chroma residual blocks

…

…

–	Otherwise (splitChromaFlag is equal to 0), for the variable blkIdx proceeding over the values 0..( ChromaArrayType  = =  2 ? 1 : 0 ), the following ordered steps apply:

- The variable nTbS is set equal to ( 1  <<  log2TrafoSize ) / SubWidthC.

- The variable yBN is set equal to yB0 + blkIdx * nTbS * SubHeightC.

- ...

- When residual_dpcm_inter_enabled_flag is equal to 1 and transform_skip_flag[ xCb + xBN ][ yCb + yBN ][ cIdx ] inter_rdpcm_flag[ xCb + xBN ][ yCb + yBN ][ cIdx ] is equal to 1, the directional residual modification process for blocks using a transform bypass as specified in subclause Error: Reference source not found is invoked with the variable mDir set equal to inter_rdpcm_dir_flag[ xCb + xBN ][ yCb + yBN ][ cIdx ], the variable nTbS, and the (nTbS)x(nTbS) array r set equal to the array transformBlock as inputs, and the output is a modified (nTbS)x(nTbS) array transformBlock.

- …

## Extending Inter-RDPCM to IntraBC blocks

Originally, the adopted Inter-RDPCM from JCTVC-N0074 [1] was applied only to Inter blocks. Meanwhile, the adopted IntraBC [3] follows Inter residue coding while IntraBC blocks are regarded as Intra blocks. Following these notions, in the Range Extension text specification [4], only Inter blocks utilize Inter-RDPCM. However in the reference software HM-12.0+RExt-4.1, contradict to the adoptions and specification, Inter-RDPCM is also applied to IntraBC blocks. In fact, the effect of Inter-RDPCM under IntraBC has not been studied in previous meetings.

To resolve the discrepancy between spec and software, one way is to align the software to the text specification, meaning disallowing Inter-RDPCM for IntraBC blocks. Another way is to align the text specification to the software, extending Inter-RDPCM to IntraBC blocks. The latter approach requires justification of Inter-RDPCM for IntraBC blocks.

Table 1 below provide 1-sec results of disallowing Inter-RDPCM for IntraBC blocks in the software, using the test conditions specified in JCTVC-N1023 [5].

Table 1. Results of disallowing Inter-RDPCM for IntraBC blocks in HM-12.0+RExt-4.1

It can be seen that Inter-RDPCM for IntraBC blocks provides noticeable gains for screen content. Thus, this document proposes to extend Inter-RDPCM to IntraBC blocks in the Range Extension specification. Below are the proposed changes. Green text corresponds to those already proposed in Section 2.1.

- 7.3.8.11	Residual coding syntax

| residual_coding( x0, y0, log2TrafoSize, cIdx ) { | Descriptor |
| --- | --- |
| if( transform_skip_enabled_flag && !cu_transquant_bypass_flag && ( log2TrafoSize <= Log2MaxTransformSkipSize ) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( ( ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER && residual_dpcm_inter_enabled_flag ) ¦¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] && residual_dpcm_intra_enabled_flag ) ) && ( transform_skip_flag[ x0 ][ y0 ][ cIdx ] ¦¦ cu_transquant_bypass_flag ) ) { |  |
| inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| if( inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| inter_rpdcm_dir_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| } |  |
| last_sig_coeff_x_prefix | ae(v) |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| lastScanPos = 16 |  |
| lastSubBlock = ( 1 << ( log2TrafoSize − 2 ) ) * ( 1 << ( log2TrafoSize − 2 ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = 16 |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ lastSubBlock ][ 0 ] |  |
| yS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ lastSubBlock ][ 1 ] |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| xS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ i ][ 0 ] |  |
| yS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| for( n = ( i = = lastSubBlock ) ? lastScanPos − 1 : 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| } |  |
| firstSigScanPos = 16 |  |
| lastSigScanPos = −1 |  |
| numGreater1Flag = 0 |  |
| lastGreater1ScanPos = −1 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| if( numGreater1Flag < 8 ) { |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| numGreater1Flag++ |  |
| if( coeff_abs_level_greater1_flag[ n ] && lastGreater1ScanPos = = −1 ) |  |
| lastGreater1ScanPos = n |  |
| } |  |
| if( lastSigScanPos = = −1 ) |  |
| lastSigScanPos = n |  |
| firstSigScanPos = n |  |
| } |  |
| } |  |
| if( cu_transquant_bypass_flag ¦ ¦ ¦ ¦ ( CuPredMode[ xC ][ yC ] = = MODE_INTRA && residual_dpcm_intra_enabled_flag && transform_skip_flag[ x0 ][ y0 ][ cIdx ] && ( predModeIntra = = 10 ¦ ¦ predModeIntra = = 26 ) ) ¦ ¦ ( CuPredMode[ xC ][ yC ] = = MODE_INTER && residual_dpcm_inter_enabled_flag && inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ] ) ) ¦ ¦ ( CuPredMode[ xC ][ yC ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] && inter_rdpcm_flag[ x0 ][ y0 ][ cIdx ]) ) |  |
| signHidden = 0 |  |
| else |  |
| signHidden = ( lastSigScanPos − firstSigScanPos > 3 ) |  |
| if( lastGreater1ScanPos != −1 ) |  |
| coeff_abs_level_greater2_flag[ lastGreater1ScanPos ] | ae(v) |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] && ( !sign_data_hiding_enabled_flag ¦ ¦ !signHidden ¦ ¦ ( n != firstSigScanPos ) ) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| numSigCoeff = 0 |  |
| sumAbsLevel = 0 |  |
| for( n = 15; n >= 0; n− − ) { |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ n ][ 1 ] |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| baseLevel = 1 + coeff_abs_level_greater1_flag[ n ] + coeff_abs_level_greater2_flag[ n ] |  |
| if( baseLevel = = ( ( numSigCoeff < 8 ) ? ( (n = = lastGreater1ScanPos) ? 3 : 2 ) : 1 ) ) |  |
| coeff_abs_level_remaining[ n ] | ae(v) |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( coeff_abs_level_remaining[ n ] + baseLevel ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( sign_data_hiding_enabled_flag && signHidden ) { |  |
| sumAbsLevel += ( coeff_abs_level_remaining[ n ] + baseLevel ) |  |
| if( ( n = = firstSigScanPos ) && ( ( sumAbsLevel % 2 ) = = 1 ) ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = −TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] |  |
| } |  |
| numSigCoeff++ |  |
| } |  |
| } |  |
| } |  |
| } |  |

8.5.4.2	Decoding process for luma residual blocks

…

…

–	Otherwise (split_transform_flag[ xCb + xB0 ][ yCb + yB0 ][ trafoDepth ] is equal to 0), the following ordered steps apply:

- …

- …

- When residual_dpcm_inter_enabled_flag or residual_dpcm_intra_enabled_flag is equal to 1 and transform_skip_flag[ xCb + xB0 ][ yCb + yB0 ][ 0 ] inter_rdpcm_flag[ xCb + xB0 ][ yCb + yB0 ][ 0 ] is equal to 1, the directional residual modification process for blocks using a transform bypass as specified in subclause Error: Reference source not found is invoked with the variable mDir set equal to inter_rdpcm_dir_flag[ xCb + xB0 ][ yCb + yB0 ][ 0 ], the variable nTbS, and the (nTbS)x(nTbS) array r set equal to the array transformBlock as inputs, and the output is a modified (nTbS)x(nTbS) array transformBlock.

- …

8.5.4.3	Decoding process for chroma residual blocks

…

…

–	Otherwise (splitChromaFlag is equal to 0), for the variable blkIdx proceeding over the values 0..( ChromaArrayType  = =  2 ? 1 : 0 ), the following ordered steps apply:

- …

- …

- …

- When residual_dpcm_inter_enabled_flag or residual_dpcm_intra_enabled_flag is equal to 1 and transform_skip_flag[ xCb + xBN ][ yCb + yBN ][ cIdx ] inter_rdpcm_flag[ xCb + xBN ][ yCb + yBN ][ cIdx ] is equal to 1, the directional residual modification process for blocks using a transform bypass as specified in subclause Error: Reference source not found is invoked with the variable mDir set equal to inter_rdpcm_dir_flag[ xCb + xBN ][ yCb + yBN ][ cIdx ], the variable nTbS, and the (nTbS)x(nTbS) array r set equal to the array transformBlock as inputs, and the output is a modified (nTbS)x(nTbS) array transformBlock.

- …

## Simplifying CU-level transquant bypass signalling with an PPS flag

Currently for lossless coding conditions, an encoding parameter from the configuration file is used which forces every CU to be coded using transquant bypass mode. At decoder, for each CU, a flag cu_transquant_bypass_flag needs to be parsed, even though it’s always true for the coding conditions that force every CU to be coded using transquant bypass mode.

For such lossless coding conditions, the following changes are proposed such that the CU-level parsing of cu_transquant_bypass_flag is avoided, with the addition of a PPS flag indicating that every CU is coded using transquant bypass.

#### 7.3.2.3	Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(1) |
| transquant_bypass_enabled_flag | u(1) |
| if(transquant_bypass_enabled_flag) |  |
| force_cu_transquant_bypass_flag | u(1) |
| … |  |
| } |  |

transquant_bypass_enabled_flag equal to 1 specifies that force_cu_transquant_bypass_flag cu_transquant_bypass_flag is present. transquant_bypass_enabled_flag equal to 0 specifies that force_cu_transquant_bypass_flag cu_transquant_bypass_flag is not present.

force_cu_transquant_bypass_flag equal to 1 specifies that cu_transquant_bypass_flag is not present, and cu_transquant_bypass_flag is inferred to be equal to 1. force_cu_transquant_bypass_flag equal to 0 specifies that cu_transquant_bypass_flag is present . If force_cu_transquant_bypass_flag is not present, it is inferred to be equal to 0.

#### 7.3.8.5	Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag && !force_cu_transquant_bypass_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| … |  |
| } |  |

cu_transquant_bypass_flag equal to 1 specifies that the scaling and transform process as specified in subclause Error: Reference source not found and the in-loop filter process as specified in subclause Error: Reference source not found are bypassed. When cu_transquant_bypass_flag is not present, if force_cu_transquant_bypass is equal to 1, it is inferred to be equal to 1, otherwise, it is inferred to be equal to 0.

Table 2 below provide 1-second results of the proposed method to symplify CU-level transquant bypass signalling, using the test conditions specified in JCTVC-N1023 [5]. Since in the anchor, the per-CU flag cu_transquant_bypass_flag is always 1 and coded with CABAC, and lossless coding rate are relatively high, the bitrate reduction are negligible.

# References

[1] “RCE2: Experimental results for Test C.1,” JCTVC-N0074, Jul. 2013

[2] “RCE2 Test B.1: Residue rotation and significance map context,” JCTVC-N0044, Jul. 2013

[3] “Non-RCE3: Intra Motion Compensation with 2-D MVs,”JCTVC-N0256, Jul. 2013

[4] “Range Extensions Draft 4,” JCTVC-N1005, Jul. 2013

[5] “HEVC Range Extensions Core Experiment 3 (RCE3): Intra Prediction techniques”, JCTVC-N1023, Jul. 2013

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).